MANUFACTURING PERFORMANCE EDITION

Tutorial Exercise

Workbook and Guide

2005 Edition

Lanner trademarks represent valuable assets for our organization, and we protect them against unauthorized use according to international intellectual property laws.

The following are Lanner Group’s trademarks, and are not to be used without the express permission of our organization:  Lanner Group, the Lanner logo, WITNESS.

This list is subject to change without notice and is not a complete list of Lanner trademarks or registered trademarks for the United Kingdom, United States and/or other countries.

Other trademarks or service marks mentioned in this document are trademarked or service marked by their respective owners

The information in this document is believed to be accurate.  However, no responsibility for its use is assumed by Lanner Group Ltd. or Lanner Group, Inc. and it must not be construed as a commitment by Lanner Group.

Copyright © 2005 Lanner Group Ltd.

All rights reserved.  No part of this publication may be reproduced or transmitted in any form or by any means, including photocopying and recording, without written permission of Lanner Group Ltd. or Lanner Group Inc., application for which should be addressed to the publisher.  Such written permission must also be obtained before any part of this publication is stored in a retrieval system of any nature.

Printed in the U.S.			       REF 2005/1/WKBM

Contents				   Page

Introduction	5

Tracks and Vehicles Modeling	7

Model Build	7

Introduction	7

Stage 1	9

Stage 2	14

Further Tracks and Vehicles Model Examples	24

WAREHOUSE.MOD	25

SHUTTLE.MOD	28

Power and Free Modeling	30

Model Build	30

Introduction	30

Stage 1	31

Stage 2	38

Further Power and Free Model Examples	42

Standard Module ADVANCED Tab	42

DROPLINE.MOD	44

Multi-Cycle Machine & Labor Modeling	46

Model Build	46

Introduction	46

Stage 1	47

Stage 2	53

Stage 3	59

Further Multi-Cycle Machine & Labor Examples	63

MULTICYCLE.MOD	64

MAINTENANCE.MOD	67

Contents				   Page

Supply Chain Modeling	69

Model Build	69

Introduction	69

Stage 1	70

Stage 2	75

Stage 3	84

Shift & Arrival Profile Modeling	89

Model Build	89

Introduction	89

Stage 1	90

Stage 2	95

Stage 3	101

Further Shifts Example	109

SHIFT.MOD	110

Tanks and Fluids Modeling	113

Model Build	113

Introduction	113

Stage 1	114

Stage 2	118

Further Continuous Examples	122

CHEMIST.MOD	122

RECIPE.MOD	125

# Introduction

Welcome to this WITNESS Tutorial Booklet. The aims of the booklet are to provide a good introduction to specific modeling using WITNESS simulation software from the Lanner Group.

The first modeling area is the use of tracks and vehicles elements in WITNESS. These are powerful elements for representing AGV’s, Warehouse and Racking areas, Forklift trucks, Shuttle cars and more. Further models supplied with WITNESS are discussed including WAREHOUSE.MOD and SHUTTLE.MOD

The second modeling area is the use of power and free conveyor elements in WITNESS. This specific type of transport is found in many factories from automotive manufacture to electronics. Further modules and models supplied with WITNESS are discussed including the use of a module from the advanced manufacturing designer elements tab and the model DROPLINE.MOD.

The third modeling area is the use of multi-cycle machines and labor in WITNESS. Multi-cycle machines are very powerful as they allow staged manufacture with different labor rules, inputs and outputs at each stage. Many different features of labor allocation in WITNESS are explored including priorities of work and pre-emption from other tasks. Further models supplied with WITNESS are discussed including MULTICYCLE.MOD and MAINTENANCE.MOD.

The fourth modeling area is the use of WITNESS for supply chain models. In this both the flows of goods and orders are modeled. The model uses the powerful MATCH rule and IF constructs and makes use of different inbuilt system attributes and variables. This model also includes the use of the picture gallery for using different icons in WITNESS and the definition and use of timeseries element to provide a dynamic view of time based reports such as stock levels in this case.

The fifth modeling area is the use of shifts and arrival profiles in WITNESS models. It includes the use of a shift pattern composed of sub shift elements in order to illustrate the complex patterns which can be built up hierarchically.  A 24 hour pattern of half hourly arrival rates is used for the arrival of components into the model and the creation and   use of a histogram and a repeat period timeseries is also included.

The sixth modeling area is the use WITNESS continuous elements including tanks, fluids, pipes and processors. The model built determines the size that a tank needs to be for an oil based process. Further models are discussed including CHEMIST.MOD and RECIPE.MOD.

# Tracks and Vehicles Modeling

## Model Build

### Introduction

- This is a description of how to build a simple WITNESS tracks and vehicles model. Much larger and more detailed models can be developed in WITNESS. The model is built incrementally. In this way you can ensure that each stage of the model is correct before you go on to the next stage, and it is possible to see clearly the effects of any change that you make.

- If you don’t want to build the model, but would like to see the model, open the model TRACKS.MOD in the demo\tutorial\tracks subdirectory of your WITNESS installation.

Model summary

- In the model two types of pallet are transported from one buffer to two different buffers using an AGV system. In the model the AGV’s are vehicles and the routes they take are represented by tracks. The AGV’s always pick up at the loading point and travel around the circuit back to the loading point to wait for work. WITNESS offers many other more complex ways to control vehicles (CALL systems, work search methods, etc) – this is the simplest method in this example.

### Stage 1

- Open the model TRACKSTART.MOD, which is located in the demo\tutorial\tracks subdirectory of your WITNESS installation directory.

- This designer elements model allows you to add WITNESS elements to your model quickly and easily. The first stage of the model you are about to build contains two parts called PalletA and PalletB and a buffer called QAGV to hold pallets waiting for the AGV system.

- Click on the part icon in the designer elements window, it will become selected (you will know when it is selected as the cursor changes to a cross and an outline rectangle is shown). Position the cursor in the layout window and click with the left-hand mouse button. A part (called Part001) appears selected in the simulation window. Hold down the left mouse button whilst the cursor is inside the net and you can drag the new element around the screen to position it. Click outside the net to remove it. The part is now part of the model you are building.

- Add another part in the same way and a buffer by clicking on the buffer icon in the designer elements window. Once it is selected move the cursor into the layout window and click again to place the counter which represents the buffer. Drag the buffer to the position that you want.

- The three elements for this first stage are now all added to the model and the next step is to add the detail needed to run the model.

- Detailing an element

- Elements can be detailed in several ways:

- The easiest way to access and change the details of an element is to double-click on the element displayed on the screen (that is, the machine icons or the buffer numerical display).

- Other options include:

- Clicking once on the element’s icon, then clicking on the detail button on the element toolbar

- Using the right mouse button on the element in the element selector.

- Part detail information

- Double click on Part001 to get the detail dialog for this element.

- Change the name of the part to PalletA by simply typing over the existing name

- In the arrivals section select Active from the pull down list

- Enter 60 in the Inter Arrival time field.

- Click on the OK button to accept these changes.

Repeat the above for Part002 – changing the name to PalletB and setting an interarrival time of 45.

Change the style of PalletB by right clicking on the icon for the part and selecting the display option using the left hand mouse. This will bring up the display bar for the part as follows.

Selecting the first button (the draw) will bring up the display style dialog. Change the fixed color to magenta (purple) by clicking on the color and choosing magenta from the palette displayed.

Click OK to select the color and then Update to update the part style.  Finally click on the tick on the display bar to close this.

- Buffer detail information

- Double click on counter representing the buffer enter:

- Name: QAGV

- Click on the OK button on the detail dialog to accept the change.

- The display in the layout window should now look like this:

- The next step in this model is to define logic rules linking these elements together. Rules may be input either by the toolbars and the mouse, or via the detail dialog for elements.

- Part Output rule information

- Select the part PalletA by clicking on its name or icon. Now click on the visual output rules button on the element toolbar.

- The default rule should be push – leave this in this case but note that others are selectable from the pull down combo box.

- Click on the buffer counter display and then click on the OK button to accept the rule.

- Repeat this rule method for PalletB

Running the model

- Set the end time for the run in the text box next to the stop run at button (alarm clock symbol) on the run toolbar by clicking in the box and then typing in 100. Now click on the run button, (single arrow pointing right), to start the model running. The model has been set to run in timescale mode mainly for the track displays later in the model – if you want the model to run faster then please de-select timescaling from the run menu.

Anticipated results at this stage

- With interarrival times of 60 and 45 and the first arrivals at time 0.0 it is apparent that after 100 minutes you would expect 5 parts to be in the buffer waiting for the AGV system.

- Observe this and also have a look at the buffer explode report. Use the right hand mouse button to click on the buffer count of 5 and then select Explode from the menu with the left hand mouse button.

- This will show the details of the parts in the buffer making it easy to validate that there are 2 PalletA and 3 PalletB parts in the buffer.

- This corresponds to the model in Tracks Model STAGE1.MOD.

### Stage 2

- Now we will create the AGV layout.

- Select the Transport tab in the designer elements window and then select and place a track element on the screen.

- Place a further 4 track elements on the screen in the same way.

- Your display will look something like this:

- We now need to arrange the tracks into the required layout.

- Click again on each track in turn and then use the Ctrl key on the keyboard in conjunction with the mouse left hand button to drag the nodes of the track to the correct places. The mouse is positioned correctly to move a node when the 4 way arrow display changes to a 2 way arrow display. Selecting and dragging the directional arrows in the center of each section of track in this way (with the Ctrl key) creates additional sections and allows the creation of corners.

- Lay out your tracks roughly as shown – with the co-ordinate displays turned on (Window/Co-ordinates menu item) note that the dimensions are shown for accurate depiction. Note also that the display grid / display / snap to grid are all options (the grid is turned off in this model example) – please see on-line help for full details.

Select each track in turn by double clicking on the display and change the names of the tracks to T1, T2, T3, T4 and T5.

From the basic tab in the designer elements palette place two more buffers on the display where shown below and name these to QPalletA for the upper buffer and QPalletB for the lower buffer.

Now we need to set the lengths of the tracks. The default for each in this model is that there are 7 AGV positions on each track. We will change this number for tracks T3, T4 and T5. The length is held as a display item in WITNESS and hence we need to use the display bar.

Select T3 and using the right hand mouse menu select display. Change the first pull down list to Draw and then select length from the second pull down list. Then press the draw button.

A dialog will appear as follows:

Change the length to 8 and click OK.

Do the same for tracks T4 and T5 changing the lengths to 2 and 17 respectively.

What does length mean?

In a WITNESS model the length is simply the number of vehicles that can queue on the track.

Now we will define the AGV’s

Select the Transport tab in the designer elements window and select the vehicles element.

Place this element on the layout window (by clicking twice) and then detail the vehicle as follows by double clicking on the display.

- Name: AGV

- Start Delay:   0.5

- Speed Unloaded:  1.0

- Speed Loaded:   1.0

- Stop Delay:   0.5

The speeds are the number of track positions that the vehicle can travel per time unit. The delays are extra time units applied when a vehicle starts or stops.

Track and Vehicle Routing

Now we need to add routing to the model.

First we must introduce the AGV’s to the track system.

Select the vehicle AGV by clicking on its name or icon. Now click on the visual output rules button on the element toolbar

Click on track T5 and accept the rule PUSH T5 by clicking OK.

This will introduce the AGV to track 5 at the start of the simulation.

Now we set the track rules:

- Click on T5 and use the visual output button to create a rule PUSH T1

- Click on T4 and use the visual output button to create a rule PUSH T5

- Click on T3 and use the visual output button to create a rule PUSH T5

- Click on T2 and use the visual output button to create a rule PUSH T4

The final track rule is more complicated. In order to enter complex logic we enter the rule using the tracks detail dialog.

Double click on track T1 and choose the Output to rule button on the General tab. Enter the rule:

If Type=PalletA

Push to T2

ELSE

Push to T3

Endif

and press OK –  WITNESS will check your rule for validity – therefore if you make a mistake in the syntax and make an invalid rule you will be prompted to change it.

WITNESS makes checks like this all the time while you are entering rules, changes, data – it helps in many places to ensure your models are valid.

If you press the rule button again after entering data you will notice that the layout is altered automatically by the editor to show items more clearly – e.g. tab settings, colors, etc – some of these options can be changed using the editor menu option in the dialog.

The dialog display will look by default as the following:

On exiting the editor note that an indication of the rule chosen is shown on the track dialog (the If next to the button indicating an If rule).

Now all that remains is to detail the loading and unloading of the vehicles.

All loading will take place at the head of track T5. Therefore track T5 is where we detail the loading information.

- Double click on T5 to get the detail dialog for this element.

- Select the Loading tab and tick the enable loading button.

- Choose Always from the transfer mode pull down list.

- Select the Input Loading Rule button and type PULL QAGV in the editor.

- Click OK to accept the rule.

- Enter Time to Load as 9

- Click OK to accept the changes

Unloading will take place at the head of tracks T2 and T3.

To set the unloading rule for T2:

- Double click on T2 to get the detail dialog for this element.

- Select the Unloading tab and tick the enable unloading button.

- Choose Always from the transfer mode pull down list.

- Select the Output Unloading Rule button and type PUSH QPalletA in the editor.

- Click OK to accept the rule.

- Enter Time to Unload as 9

- Click OK to accept the changes

Repeat the process for track T3 to unload to QPalletB

The model is now ready to run with the full logic in for the AGV track.

If you run the model you will see the AGV moving around the system. (Remember to turn timescaling back on if you turned it off earlier in order for the speed to be slow enough to see in detail what is going on).

In order to easily see whether the AGV is loaded we will now change the icons used to display the AGV when it is loaded and unloaded.

- Double click on T5 to get the detail dialog for this element.

- Select the Loading tab and press the actions on loading button.

- Type the following into the actions box:

IF TYPE = PalletA

VEHICLE (T5,1) AT 0:ICON = 26

ELSE

VEHICLE (T5,1) AT 0:ICON = 30

ENDIF

This will change the display of the vehicle when loaded to different icon sets.

ICON SETS

In WITNESS it is possible to define icon sets to display elements on path displays (such as used for the tracks in this example).

If a vehicle style dialog is set to allow 4 orientation options then wherever that vehicle is displayed one of four icons will be chosen for the display of that vehicle. The style dialog shows the selected starting icon.

If the starting icon chosen is icon 6 and the number of orientation options is 4 then either 6,7,8, or 9 will be chosen as the icon to display the vehicle. Which one is chosen depends on the orientation of the display item. The 360 degrees is split into the number of segments chosen with the first segment centred on vertically up.

For this example:

- For path segments heading between 315 and 45 degrees (clockwise from vertically up) icon 6 will be displayed

- For path segments heading between 45 and 135 degrees (clockwise from vertically up) icon 7 will be displayed

- For path segments heading between 135 and 225 degrees (clockwise from vertically up) icon 8 will be displayed

- For path segments heading between 225 and 315 degrees (clockwise from vertically up) icon 9 will be displayed

Now when the model is run the loaded vehicles will be displayed with an icon that shows the part which it is carrying.

Now we need to reset the icon display when a vehicle unloads:

- Double click on T2 to get the detail dialog for this element.

- Select the Unloading tab and press the actions on Unloading button.

- Type the following into the actions box:

VEHICLE (T2,1) AT 0:ICON = 22

And OK all changes

- Double click on T3 to get the detail dialog for this element.

- Select the Unloading tab and press the actions on Unloading button.

- Type the following into the actions box:

VEHICLE (T3,1) AT 0:ICON = 22

And OK all changes

The model is now completed and corresponds to TRACKS.MOD in the tutorial directory.

Running the model

Run the model to time 1000. To speed up the model run either remove time scaling (using the Run/Timescaling menu option) or batch using the batch button on the run toolbar (this runs the model with no display).

You will notice that the queue in QAGV builds up. The AGV network cannot transfer the pallets quickly enough to keep up with the demand.

As an experiment we might decide to see whether the network could manage if there were 2 AGV’s.

To make this change simply double click on the vehicle name or icon. Enter 2 in the Quantity field and then click OK to accept the change.

Now run the model again to time 1000. Again the performance of the system is not quite good enough to cope with the demand.

If you watch the model run then there is another idea that might occur to you – and this highlights the importance of watching a model carefully – you may notice an aspect of control that can be altered. In this case you may notice that only one AGV occupies each track at a time. Now this may be a realistic consideration – however let us assume not in this case. Double click on track T5 and change the capacity to 2. This will enable AGV’s on this track at once.

Run the model again and observe the change.

This increases the capability of the AGV system as returning AGV’s no longer are delayed from entering T5 until after the other has loaded.

Now the system manages to keep up with the demand (just!!).

Note that if you batch the model to time 46000 or so the output buffer for Pallet type B (QPalletB) fills to its capacity of 1000. This then block the whole network as the AGV cannot unload into the buffer.

Try altering other parameters and noting the effects:

For example:

- Vehicle speeds

- Load/Unload times

- Reduced buffer capacity for QAGV (if too small pallets will be rejected from entering the model – this is reported in the part statistics report. Obviously the throughput will decrease too.

- AGV capacity – although a more complex routing or loading rule is needed if this is to be accurate as there are two types of pallet. However note the default behavior which will mix up the types of pallet in each output queue.

## Further Tracks and Vehicles Model Examples

Tracks and vehicles can be used effectively in many different model types.

They can be used for:

- AGV Systems

- Forklift Trucks

- Cranes and Gantries

- Shuttle Mechanisms

- And many more types of storage and retrieval systems.

As with all WITNESS modeling however the advice from Lanner is to model the detail of a system that is necessary to achieve your modeling objectives. Therefore think carefully before modeling in detail a storage system if this is not the primary interest of a model.

Level of Modeling

In many cases it is possible to achieve good model results by simplification. For example a distribution of timings into and out of a storage area may suffice if a model is looking mainly at a production capability of a line.

Another example may be a warehouse being modeled as a single activity in a supply chain model with a defined capability to store or retrieve a maximum number of goods each day.

Of course if the model is designed primarily to look at how many AGV’s are needed in a system or to determine selection rules then it is indeed necessary to model the facility in detail.

There are a number of the standard WITNESS demonstration models installed in the DEMO directory under your WITNESS installation that include tracks and vehicles in use.

These include WAREHOUSE.MOD and SHUTTLE.MOD

### WAREHOUSE.MOD

This model is in the standard demo directory which is situated under the directory where you installed WITNESS. It is a simple warehousing model designed primarily to show the use of tracks and vehicles in WITNESS.

A flow of goods from a factory production unit in picked up by an AGV system and deposited in a high bay pick and drop area. From here the goods are transported by a crane to a storage position.

Pick orders are generated by a dummy generator which samples randomly for the position in the high bay storage from where parts should be taken. The extracted parts are delivered to a dedicated output pick and drop station where the AGV system collects and takes them to a sortation lane system.

At the sortation lanes a shuttle car deposits the goods in a random lane.

When a lane fills the goods are output to a conveyor leading to dispatch and leave the model.

There are three different tracks and vehicles systems in the model:

- The first is the main AGV transfer system. Here AGV’s circulate and transfer both parts P1 and P2 from the conveyors to the high bay storage area and also from the high bay area to the sortation lanes.

The rules for loading are on the following tracks :

  - AGVSystem.T1

  - AGVSystem.T5

  - AGVSystem.T6

  - AGVSystem.T9

These are the tracks which lead up to the conveyors where parts queue after being produced in the factory. If you look at the loading tab on the detail dialog of any of these tracks you can see that the loading rule is an IF rule. The condition is a function (Canload1() or Canload2()). This function simply checks for whether there is enough free space at the destination High Bay lane – if not then the part is not loaded. Parts are dropped at the HighBay.StoreIn buffer for the correct lane. The unload rule for each of the AGVSystem.TS3 tracks is again IF which simply checks whether the correct lane has been reached.

Again at each of the TS3 tracks an empty AGV will check for loading from a HighBay.StoreOut buffer – see the loading tab on the TS3 detail.

- The second is the Highbay area crane system. There are six cranes and six tracks (T11 to T16) each with 14 track positions.

Each crane is called using the AGV call and VSEARCH method (see overleaf). Cranes are called in the actions when parts are placed in the StoreIn buffers and also when pick list parts are placed in the HighBay.Pick.PickList buffer. In each of these places a CALL action is used to place a task in the task list for a crane and then a VSEARCH action is used to check for a current idle vehicle.

- The third is the shuttle mechanism which takes parts from the infeed conveyor in the sortation area (SORTATION.C3) and places them in a lane. Again each crane is called using the AGV call and VSEARCH method. Look for these in the Actions on Front of the conveyor SORTATION.C3.

The VSEARCH method

If you have used a CALL action to request a vehicle to transport parts from one track to another, the VSEARCH action tells WITNESS which tracks to search for an idle vehicle to satisfy the request.

The list of tracks specified in the VSEARCH action is called the call’s domain.

If a vehicle cannot satisfy the request immediately, the CALL and domain are stored in the vehicle’s demand list. As soon as an idle vehicle comes on to (or becomes idle on) one of the tracks specified in the domain, WITNESS sends it to satisfy the request.

### SHUTTLE.MOD

This model is in the standard demo directory which is situated under the directory where you installed WITNESS.

In this model parts are produced on four assembly machines from components held in the stores. The parts are then taken at random but in equal proportions to one of five paint booths.

The transfer of parts is via a shuttle AGV using WITNESS tracks and vehicles elements. Vehicles are called by the assembly machines when a transfer is necessary and the VSEARCH method is used to look for vehicles that can do the work.

In the model colored icons are used to distinguish finished parts from the unpainted ones. Finally the parts are baked, tested and dispatched using a conveyor system.

A graph shows production per eight hour shift. Note that in this example shift elements are not required to control working periods because all facilities in the model work the same hours. Therefore to add shifts would be an unnecessary complication that would take time and may adversely affect run speed with the extra shift events.

The key which is displayed is created by the modeler and not one of the standard WITNESS keys, although standard status colors are used.

A process representation of the model can be seen on the Process View window.  This has been generated using the View/Process View command.

# Power and Free Modeling

## Model Build

### Introduction

- This is a description of how to build a simple WITNESS model incorporating the four specialist power and free elements (networks, stations, sections and carriers). Much larger and more detailed models can be developed in WITNESS. The model is built incrementally. In this way you can ensure that each stage of the model is correct before you go on to the next stage, and it is possible to see clearly the effects of any change that you make.

- If you don’t want to build the model, but would like to see the model, open the model POWERFREE.MOD in the demo\tutorial\power subdirectory of your WITNESS installation.

Model summary

- In the model a door is to be transported from one point to another using a power and free conveyor system. The model will contain:

- A door part

- A Network element

- A Carriers element

- 2 power and free sections

- A loading station

- An unloading station

- The simple model layout will look as follows:

### Stage 1

- Power and Free networks transport parts from one point to another point using carriers. The network is composed of sections and parts are loaded and unloaded from sections using stations (load and unload). There are two other types of stations – one to perform tasks on a section (an actions station) and one where carriers can wait and not block a section (a parking station).

- Open the model STARTUP.MOD, which is located in the demo\tutorial subdirectory of your WITNESS installation directory.

- This designer elements model allows you to add WITNESS elements to your model quickly and easily. The first stage of the model you are about to build contains one part called Door, and specialized power and free elements.

- Click on the part icon in the designer elements window, it will become selected (you will know when it is selected as the cursor changes to a cross and an outline rectangle is shown). Position the cursor in the layout window and click with the left-hand mouse button. A part (called Part001) appears selected in the simulation window. Hold down the left mouse button whilst the cursor is inside the net and you can drag the new element around the screen to position it. Click outside the net to remove it. The part is now part of the model you are building.

- Change tabs in the designer elements window to Transport and add the following in the same way:

- 1 PFNetwork element

- 2 PFStation elements

- 1 PFCarrier element

- 1 PFSection element

- The next step is to add the detail needed to run the model.

- Detailing an element

- Elements can be detailed in several ways:

- The easiest way to access and change the details of an element is to double-click on the element displayed on the screen (that is, the machine icons or the buffer numerical display).

- Other options include:

- Clicking once on the element’s icon, then clicking on the detail button on the element toolbar

- Using the right mouse button on the element in the element selector.

- Part detail information

- Double click on Part001 to get the detail dialog for this element.

- Change the name of the part to Door by simply typing over the existing name

- Click on the OK button to accept this change.

- Network detail information

- Double click on Network001 to get the detail dialog for this element

- Change the name of the network to MainNetwork by simply typing over the existing name

- Click on the OK button to accept this change.

- Carrier detail information

- Double click on Carrier001 to get the detail dialog for this element

- Change the name of the carrier to Carrier by simply typing over the existing name

- Select the down arrow by the network selector and double click on MainNetwork in the Select Element box dialog that appears. This sets the parent network for the carriers (although carriers can travel from one network to another).

- Click on the OK button to accept this change.

- Loading Station detail information

- Double click on Station001 to get the detail dialog for this element

- Change the name of the station to Loader by simply typing over the existing name.

- Select the down arrow by the network selector and double click on MainNetwork in the Select Element box dialog that appears. This sets the parent network for the station.

- Choose Loading from the station type pull down list.

- Select the Loading tab and enter 3 in the cycle time field and select Free from the pull down method control. Free means that the part is loaded while the carrier waits at the station – powered means loading whilst the carrier moves along the next section.

- Click on the OK button to accept this change.

- Unloading Station detail information

- Double click on Station002 to get the detail dialog for this element

- Change the name of the station to Unloader by simply typing over the existing name

- Select the down arrow by the network selector and double click on MainNetwork in the Select Element box dialog that appears. This sets the parent network for the station.

- Choose Unloading from the station type pull down list

- Select the Unloading tab and enter 3 in the cycle time field and select Free from the pull down method control.

- Click on the OK button to accept this change.

- Section detail information

- Double click on Section001 to get the detail dialog for this element

- Change the name of the section to S1 by simply typing over the existing name.

- Select the down arrow by the network selector and double click on MainNetwork in the Select Element box dialog that appears. This sets the parent network for the station.

- Set the Section Length to 14

- Set the Drive speed to 1

- Set the Dog Spacing to 1

- Click on the OK button to accept this change.

- You will have noted many other fields in these elements offering additional options. The carrier speed in this model is set against the sections as this will be a section powered network (set in the network dialog). Hence the carrier speed field is left unset in the carrier detail.When all these elements are added, and the above changes made, your display will look something like this:

- The next step is to add the logic for the routing.

- Loader Input rule information

- Select the station Loader by clicking on its icon. Now click on the visual input rules button on the element toolbar.

- The default rule should be pull – leave this in this case but note that others are selectable from the pull down combo box.

- Click on the Door display and then the world button and you should see the following:

- Click on the OK button to accept the rule.

- Now click on the Visual output rules button.

- The default rule should be push – leave this in this case but note that others are selectable from the pull down combo box.

- Click on S1 and then click on the OK button to accept the rule.

- Carrier Output rule information

- Select the carrier Carrier by clicking on its icon.

- Now click on the Visual output rules button.

- Click on Loader and then click on the OK button to accept the rule.

- Section Output rule information

- Select the section S1 by clicking on its icon.

- Now click on the Visual output rules button.

- Click on Unloader and then click on the OK button to accept the rule.

- Unloader Output rule information

- Select the station Unloader by clicking on its icon.

- Now click on the Visual output rules button.

- Click on the SHIP button and then click on the OK button to accept the rule.

- (in this last case note that the rule that is being set is the output of the part and not the carrier).

Running the model

- Run the model using the commands on the Run toolbar.

- A single carrier should flow through the system and end up blocked, but unloaded, in the Unloader station. The carrier will turn magenta.

- This corresponds to the model in STAGE1.MOD.

### Stage 2

- Now we will add more carriers and a return loop for the carriers to return to the loading station.

- To add more carriers simply double click on Carrier to select its detail dialog and enter 5 in the quantity field.

- Then add another section from the transport tab in the designer elements window. Place this below the first section on the display and change its name to S2 and allocate it to the network MainNetwork as before.

- Set the Section Length to 30

- Set the Drive speed to 1

- Set the Dog Spacing to 1

- We will now edit the path displays for the sections to better represent the actual layout of the network.

- Click again on each section in turn and then use the Ctrl key on the keyboard in conjunction with the mouse left hand button to drag the nodes of the section to the correct places. The mouse is positioned correctly to move a node when the 4 way arrow display changes to a 2 way arrow display. Selecting and dragging the directional arrows in the center of each segment of section in this way (with the Ctrl key) creates additional segments and allows the creation of corners.

- Lay out your sections as shown overleaf – with the co-ordinate displays turned on (Window/Co-ordinates menu item) note that the dimensions are shown for accurate depiction. Note also that the display grid / display / snap to grid are all options (the grid is turned off in this model example) – please see on-line help for full details.

- Section S1 should be one straight segment length 14 units, Section S2 is three segments with total length 30.

Complete the routing of the network to incorporate S2 as follows:

- Section Output rule information

- Select the section S2 by clicking on its icon.

- Now click on the Visual output rules button.

- Click on Loader and then click on the OK button to accept the rule.

- Unloader Output rule information

- Select the station Unloader by clicking on its icon.

- Now click on the Visual output rules button.

- Before setting a rule select connect from the operation type pull down list. We are setting the rule for the carrier this time and not the part output rule which is already set.

- Click on S2 and then click on the OK button to accept the rule.

Amending the carrier display options

There are many different options for the display of the carriers, and the parts they are carrying, in the model.

Right click on Carrier and select display from the right mouse button menu (with the left mouse button).

Select Draw from the first pull down list and Style from the second pull down list. Then press the draw button. This should bring up the following dialog:

You will see many options on the pull down list.

There are many options for you to explore – however here pick the simple part option. This will continue to display the part icon when the part is loaded but will display an index display of the carrier when unloaded. Accept the change using the draw button and then the tick on the display bar.

- This corresponds to the final model – POWERFREE.MOD.

Running the model

- Run the model using the commands on the Run toolbar.

- Now all the carriers pick up the parts and unload them at the unloading point and then return for more. Batch the model for a long time. You can batch thousands of minutes in seconds.

- Looking at the statistics for the doors after this run you will see that the time in the system is approaching 20 or may even be 20 for each of the doors. This is as expected (with no delays) as each door takes 6 minutes loading and unloading in total and 14 minutes for travel (as the dogs are synchronized).

The reason why the statistic may not be exactly 20 is that  it includes doors still in the system.

Note that here there is always a part available to transport which is a little unrealistic. You may care to make the arrivals active and place a buffer (B1) in front of the load station and change the input rule for the loader to Pull B1. Then you may see a change in the carrier statistics.

Note also that stations and sections can break down affecting the smooth operation of the system.

## Further Power and Free Model Examples

### Standard Module ADVANCED Tab

There is a standard module in the Advanced tab of the designer elements that creates a whole power and free network automatically defining 10 elements inside a module.

This illustrates the power of modules in WITNESS and we encourage you to develop your own for pieces of models that you develop time and time again.

- To try this module open the model STARTUP.MOD. This is located in the demo\tutorial subdirectory of your WITNESS installation directory.

Define a part from the basic tab in the usual way.

Then select the advanced tab in the designer elements window and select and place a PFModule Element in the layout window.

- Select the carrier Part001 by clicking on its icon.

- Now click on the Visual output rules button.

- Select YES to continue through the warning on active arrivals.

- Click on InputQ and then click on the OK button to accept the rule.

The model may immediately be run as it includes most default rules and data. It will simply prompt for the interarrival time of parts – enter a number such as 12. The network includes:

- An input buffer

- A loading station

- Three sections

- A basic actions stations element (quantity 5)

- An unloading station

- An output buffer

- A carrier element (quantity 20)

- A network element

View the statistics and explore the model logic. You will notice similarities to the example in model build described here. Note the routing of the parts through the basic actions stations (the output rule of these sections uses the internal variable N to know which of the 5 stations the rule is referring to).

This model does not ship parts – they end up in the OutputQ – you will see the counter steadily increase. These parts are then awaiting further model sections. It is a good rule with modules to develop easy ways to connect into and out of them in this way.

Modules can also be developed to contain their own custom reports and timeseries, histograms, pie charts etc to provide further information on the performance of the modular component.

This shows the model created in seconds using the advance module designer elements. You can add other modules you find useful here or in your own designer libraries.

### DROPLINE.MOD

There is another power and free demonstration model in the standard WITNESS demonstration models installed in the DEMO directory under your WITNESS installation.

This is called DROPLINE.MOD

This model is a simple representation of part of a car plant circulation system. Cars of three types are loaded onto a Power and Free conveyor system at a loading station and are transported to an unload station. Carriers (or slings) are then returned to the load station via a return loop.

Breakdown patterns are applied to the loading station to illustrate the queuing mechanism for the carriers and the user is encouraged to experiment with different arrival profiles and different numbers of carriers in the system.

Various reports are available on selected windows – A histogram of the time in the system for the car bodies and a timeseries of the number of carriers on the first section. These supplement the standard reports.

# Multi-Cycle Machine & Labor Modeling

## Model Build

### Introduction

- This is a description of how to build a simple WITNESS model incorporating a multi-cycle machine and a labor resource. Much larger and more detailed models can be developed in WITNESS. The model is built incrementally. In this way you can ensure that each stage of the model is correct before you go on to the next stage, and it is possible to see clearly the effects of any change that you make.

- If you don’t want to build the model, but would like to see the model, open the model MULTIMACHINE.MOD in the demo\tutorial\multi subdirectory of your WITNESS installation.

Model summary

- In the model a valve is to be produced on a milling machine. The milling machine is manned by an operator who only needs to be present to initialize the machine and to remove and clean the part – i.e. the operator does not need to be present during the main machining cycle.

- As an added complication this model also looks at the various ways in WITNESS to handle other tasks that the operator also has to perform that are outside the scope of this model.

- The simple model layout will look as follows:

### Stage 1

- Open the model STARTUP.MOD, which is located in the demo\tutorial subdirectory of your WITNESS installation directory.

- This designer elements model allows you to add WITNESS elements to your model quickly and easily. The first stage of the model you are about to build contains one part called VALVE and a machine called MILLING to machine the VALVEs.

- Click on the part icon in the designer elements window, it will become selected (you will know when it is selected as the cursor changes to a cross and an outline rectangle is shown). Position the cursor in the layout window and click with the left-hand mouse button. A part (called Part001) appears selected in the simulation window. Hold down the left mouse button whilst the cursor is inside the net and you can drag the new element around the screen to position it. Click outside the net to remove it. The part is now part of the model you are building.

- Add a machine in the same way. The next step is to add the detail needed to run the model.

- Detailing an element

- Elements can be detailed in several ways:

- The easiest way to access and change the details of an element is to double-click on the element displayed on the screen (that is, the machine icons or the buffer numerical display).

- Other options include:

- Clicking once on the element’s icon, then clicking on the detail button on the element toolbar

- Using the right mouse button on the element in the element selector.

- Part detail information

- Double click on Part001 to get the detail dialog for this element.

- Change the name of the part to Valve by simply typing over the existing name

- Click on the OK button to accept this change.

- Machine detail information

- Double click on the machine and enter:

- Name: Milling

- From the Type pull down list select Multiple Cycle

- Position the cursor in the Cycle Name field and type

- Initialize

- Now add a second cycle to the machine by clicking on the Add/Remove button, typing a description Machining in the cycle name field and clicking the Add button.

- Now add a third cycle to the machine by clicking on the Add/Remove button, typing a description Clean and Remove in the cycle name field and clicking the Add button.

- Notice the selection mechanism for each cycle through the pull down selector and the summary display obtained using the summarize button.

- Click on the OK button on the detail dialog to accept the changes.

- We will now define the rules to govern the flow of valves into and out of the machine. Remember rules may be input either by the toolbars and the mouse, or via the detail dialog for elements. We will use the toolbar method.

- Machine Input rule information

- Select the machine Milling by clicking on its icon. Now click on the visual input rules button on the element toolbar.

- You will be warned that this will define an input rule for the first cycle of this machine – click the Continue button.

- The default rule should be pull – leave this in this case but note that others are selectable from the pull down combo box.

- Click on the valve display and then the world button and you should see the following:

- Click on the OK button to accept the rule.

- Now click on the Visual output rules button.

- You will be warned that this will define an Output rule for the final cycle of this machine – click the Continue button.

- The default rule should be push – leave this in this case but note that others are selectable from the pull down combo box.

- Click on the Ship button in the toolbar and then click on the OK button to accept the rule.

- It is possible to define input and output rules for intermediate cycles in a multi-cycle machine too to ask for or output extra parts – this should be done via the detail dialog if required.

Running the model

- Initially run the model in step mode using the step button on the run toolbar.

- WITNESS has been designed with an in-built safety mechanism to prevent you from running a model if vital information is missing. You have not entered any timings for this machine, so as soon as you start to step the model, it prompts you with the message:

- In this case we do not want to accept the default time of 1.0 that is suggested – we wish to enter a distributional timing using the assistant tool bar. Make it available by view/ toolbars menu and clicking on assistant (if not already on).

- Click on the distributions button of the assistant toolbar.

- Choose the UNIFORM distribution (scroll down if necessary) and click on the prompt button.

- Enter the limits as 2.5 and 4.5 and leave the prn numbers blank. PRN numbers are pseudo random number streams – leaving them blank means that WITNESS will handle this for you – automatically assigning streams to create the randomness in the sample.

- Click OK to accept the information entered.

After entering the first timing the interact box will open (as you are in step mode). Note the information that it tells you as you step on from event to event by clicking in it or the model display window.

- Step on (just continue clicking in the model window or interact box) and enter two more uniform distributions in the same way when prompted: Machining Uniform(9,13) and Clean and Repair Uniform(2,4).

- The TIME UPDATE interval reported in the interact box can be amended as wished in the Model/Options menu dialog.

- After you have stepped through a few cycles of the machine stop the model using the stop button on the run toolbar.

Anticipated results at this stage

- With a total average cycle time of 17.5 you would expect 5 or 6 parts to be processed in 100 minutes. Run the model to 100 minutes and look at the statistics report for the valve element (right hand mouse menu – statistics). To set the end time for the run to 100 type 100 in the text box next to the stop run at button (alarm clock symbol) on the run toolbar. Now as long as the alarm clock button is toggled in the model will stop at time 100 when running or batching.

- Of course there are distributional timings in the model and therefore each cycle theoretically could be anywhere between 13.5 and 21.5 minutes although these extremes are unlikely.

- As you run the model for longer and longer the average time in model statistic for valves will approach the mean of 17.5

- This corresponds to the model in STAGE1.MOD.

### Stage 2

- Now we will add an operator and also a string variable to show us on the screen the cycle that the multi cycle machine is currently on.

From the designer elements window select a labor element from the basic tab and place this element on the layout window (by clicking twice).  Then detail the labor element as follows by double clicking on the display.

- Name: Operator

- Click on the OK button to accept this change.

- Your display will look something like this:

- Now we will add the rule for the first and third cycles of the Milling machine to use labor to cycle:

- Click on the Milling machine to select it.

- Click on the visual labor rule button  in the element toolbar and then click on the Operator labor element.

- This assigns the labor element to the Initialize cycle

- Select the Clean and Remove cycle from the pull down cycle list.

- This will prompt you to save the first labor rule – choose Yes

- Now that the third cycle is selected click again on the Operator labor element.

- Click on SAVE and then CLOSE in the rule bar to accept this rule.

- Now we will define the variable to display the current cycle of the machine.

- Select the Variables tab in the designer element window.

- Select VString and place the element in the modeling window.

- Double click on VString001 and rename to CurrentCycle (no spaces).

- Click OK to accept the change.

To use this to detail the current cycle we will set the value of the variable at the start of each cycle of the milling machine.

Double click on the Milling machine and click on the actions on input button for the initialize cycle.

- Enter the following text: CurrentCycle=”Initialize”

- Click OK

- Select the second cycle from the pull down list

- Select the Actions on Start tab

- Enter the following text: CurrentCycle=”Machining”

- Click OK

- Select the third cycle from the pull down list

- Select the Actions on Start tab

- Enter the following text: CurrentCycle=”Clean and Remove”

- Click OK

- Click  OK on the main dialog to accept the changes.

In order to see the all the text entered above we need to change the default display for the variable.

Right click on the Current Cycle Variable and select display from the right hand mouse menu using the left hand mouse button.

Choose Variable Value(s) from the second pull down list:

Now select the draw button to the right of that pull down list. This will display the value display dialog:

It is the width field that needs to be changed – alter the value in this field (either type or use the buttons) to a value of 16 which will cope with all the different cycles. Click on update and then on the tick in the display bar to accept the change.

Running the model

- Step the model again and note the use of the operator and the updating of the variable. The visual checking of logic in this way is very important and is a key benefit of visual simulation with WITNESS.

- This corresponds to the model in STAGE2.MOD.

- Running the model for 1000 minutes shows that the operator is busy less than 40% of the time.

- However this assumes that the operator is always immediately available when required. If this is true, i.e. if the operator can just fill in with other activities for the other 60% of the time whilst always keeping an eye out for the machine then this is a valid result. In many situations this will not be true.

- The factors that need to be considered are those from the real life situation:

- What else will the operator be doing

- How will the operator know that the machine needs attention

- Will the operator finish any other task first

- How long is the delay before the machining task can start

- When modeling these factors when determined can be modeled in different ways :

- if the delays are small and the response fairly immediate then the extra timings can usually be built into the cycle times and result in accurate modeling.

- For large delays the other activities should usually be modeled although if there are several different activities it is not always necessary to model each – a single machine with a distributional cycle time that represents the range of tasks that the operator will perform may suffice (note this will not work if a lot of long jobs are followed by a lot of small jobs – as the distribution will sample each time randomly).

- If movement times/locations are important then a path network can be defined and movement times incorporated into the model. (see MAINTENANCE.MOD in the demo directory for a good example of this).

We shall explore method ii) here in stage 3 below with a single dummy activity defined to represent other work that needs to be performed.

### Stage 3

Define another part and machine from the basic designer tab

Call the part OtherWork and the machine OtherActivity

- Click on OtherWork

- Now click on the Visual output rules button.

- You will be warned that this will create active arrivals for this part – choose Yes to continue.

- Click on the OtherActivity machine

- Click on OK to accept the rule.

Double click on OtherWork and enter:

- Interarrival time : Uniform(5,15)

- Click OK to accept change

Double click on OtherActivity and enter:

- Cycle time : Uniform(1,5)

- Input Rule : Buffer(1000)

- This will hold waiting work from the active arrivals

- Output Rule: Push to Served

- Click OK to accept change

- Click on the visual labor rule button  in the element toolbar and then click on the Operator labor element.

- This assigns the labor element to the OtherActivity cycle

- Click on SAVE and then CLOSE in the rule bar to accept this rule.

We have set the OtherActivity to use the operator for an average of 3 minutes every 10 minutes.

Run the model to see the effect (make sure the walk button is turned on to see the logic clearly).

- This corresponds to the model in MULTIMACHINE.MOD.

At different times it can be seen that each of the two machines are waiting for labor. This is indicated by the blue color of the machine status icon and in the machine statistics chart report.

Selecting Reports/Charts

To select the reports for all the machines in a model the easiest way is to expand the tree selector Types section. Then use the right hand mouse menu when clicking on Machines to select statistics. Charts are available from the results tables via the buttons to the right of the dialog.

With the above results there are considerable delays for the milling process. However it is important to be careful with the model logic.

WITNESS in this case is taking all work in the order it arrives. What will happen if Milling is always given priority?

Double click on Milling and change the priority to 1.

Now running the model gives a much smaller delay for the milling activity as can be seen from the following results chart.

WITNESS allows labor pre-emption too.

Double Click on the Milling machine.

Click on the Labor Rule for the initialize cycle:

Now tick the box for labor pre-emption on the left and enter a level of 1, an allowance of 0 and a penalty of 0.

Click OK to accept the changes and then make the same changes for cycle 3 – Clean and Remove.

Now when you run or step the model the Milling machine will pull the labor away from the other activities in the middle of work. However this essentially is the same as saying that the labor is always available and hence if this is the case the other activities need not have been modeled.

Try different values in the allowance and penalty fields for the labor pre-emption and observe the results.

## Further Multi-Cycle Machine & Labor Examples

There are a number of the standard WITNESS demonstration models installed in the DEMO and EXAMPLES directory under your WITNESS installation that include multi-cycle machines and labor elements.

These include MULTICYCLE.MOD in the Examples directory

and

MAINTENANCE.MOD in the Demo directory

### MULTICYCLE.MOD

A, B and C are passive parts, pulled into the model when required by machine Machine1. Machine1 is a multi-cycle machine that has 7 cycles:

1.	Load A

2.	Load B

3.	Assemble B to A

4.	Reposition A

5.	Load C

6.	Assemble C to A

7.	Clean Assembly

Machine1 also has a setup called clean up machine that it must execute before it can cycle. It lasts for 5.0 time units and occurs every 1 operation (that is, once every sequence of 7 cycles on this machine).

When the machine has completed the setup, it commences the series of cycles.

Cycle 1: Load A

A single A part is pulled into the machine via the cycle’s input rule:

PULL from A out of WORLD

Machine1 cycles for 2.0 time units then the cycle finishes. The part is not output from the machine, so the finish quantity at the end of this cycle is 1 and the output quantity is 0.

Cycle 2: Load B

A single B part is pulled into the machine via the cycle’s input rule:

PULL from B out of WORLD

Machine1 cycles for 2.0 time units then the cycle finishes. The part is not output from the machine, so the finish quantity at the end of this cycle is 2 (that is, 1 x part A and 1 x part B).

Cycle 3: Assemble B to A

During this cycle part B is assembled to part A, an operation that takes 5.0 time units. No parts are pulled into the machine as the necessary parts are already there. Again, no parts are output but the finish quantity changes to 1 again as parts A and B have been combined.

Cycle 4: Reposition A

During this cycle part A is repositioned within the machine, an operation that takes 4.0 time units. No parts are pulled into the machine as the necessary parts are already there. Again, no parts are output and the finish quantity remains 1.

Cycle 5: Load C

A single C part is pulled into the machine via the cycle’s input rule:

PULL from C out of WORLD

Machine1 cycles for 2.0 time units then the cycle finishes. The part is not output from the machine, so the finish quantity at the end of this cycle is 2 (that is, 1 x part A and 1 x part C).

Cycle 6: Assemble C to A

During this cycle part C is assembled to part A, an operation that takes 4.0 time units. No parts are pulled into the machine as the necessary parts are already there. Again, no parts are output but the finish quantity changes to 1 again as parts A and C have been combined.

Cycle 7: Clean Assembly

During this cycle part A is cleaned, an operation that takes 5.0m time units. All the cycles have been completed, so the machine’s output rule can expel the part:

PUSH to SHIP

Model Display

Each part’s Actions On Create sets the icon for that part.

A machine element key is displayed to the right of the main display.

A part key has been created by displaying the part names and icons.

Machine1 also displays the number of the current cycle. This is achieved by using a text and an expression display item for the machine. The text item displays the text:

Running the model

You can see parts (red A parts, green B parts or yellow C parts) arriving at the machine. Text below the machine keeps track of the current cycle number. The machine icon changes color according to the machine’s state. After 7 cycles, the icon is colored cyan, showing that the machine is setting up. Overall, the machine spends approximately 17% of its time setting up and approximately 83% processing.

### MAINTENANCE.MOD

This is an example model to show resource rules using paths for a typical manufacturing maintenance team.

The model contains several breakdowns where the resource rule for helping a stopped element to resume is set as 'Man Using Path'.

When a stoppage occurs the 'Man' will search for the shortest route using the path network defined. (Note that this is usually several 'Path' elements linked together - Each path element is defined with start and end nodes).

Overall model options for paths in this model include bi-directional paths and resources walk to and from their free queuing positions. Entities in the model transfer between activities using standard WITNESS queuing conveyors (path displays which in this example are shown as grey lines outlined in blue) Paths for the movement of the men who are helping elements to resume are shown in thin blue lines.

The model has been slowed down using the time interval option (from the model/options menu - set to 0.001). This is in order to observe the movement of resources to the activities on reasonably fast activities. For slow computers (or to observe the model running faster) reset this option to either 0.01 or even 0.1.

Running the Model

The first model stoppage occurs at roughly time 70:

1.	For the first stoppage a man travels from the free queue to the activity arriving at roughly time 72.

2.	At time 117 a second activity stops and another man leaves the free queue to attend.

3.	At time 125 a third activity stops and the third and last man leaves to attend.

4.	At time 143 another stoppage occurs and there is no-one to attend

5.	At time 147 the first stoppage ends and the man immediately leaves to attend to the other stoppage.

6.	At time 167 another stoppage occurs and there is no-one to attend

7.	This stoppage is not the first attended by the first man becoming free – as it is not the highest priority

Further control options are many - for example:

1.	Different free queue positions can the tried to better position the response.

2.	Different free queues and resources can try zoning for effectiveness.

3.	Different pre-emption options can be tried (with different priorities, allowances and penalties) to target the more problematical stoppages).

4.	Different resources and free queues can be coupled with complex selection (IF) rules for the stoppages where the position of the resources can be taken into account in the selection

# Supply Chain Modeling

## Model Build

### Introduction

- This is a description of how to build a simple WITNESS supply chain model incorporating a manufacturer, a warehouse and a retailer. Much larger and more detailed models can be developed in WITNESS. The model is built incrementally. In this way you can ensure that each stage of the model is correct before you go on to the next stage, and it is possible to see clearly the effects of any change that you make.

- If you don’t want to build the model, but would like to see the model, open the model SUPPLYCHAIN.MOD in the demo\tutorial\supply subdirectory of your WITNESS installation.

Model summary

- In the model the flow of goods and orders are both represented by parts. Customer demand is modeled taking goods from the retailer. When the stock level at the retailer falls below a set amount an order is generated and placed on the warehouse. The order is picked and delivered. The warehouse is fed with the same level of goods as the customer demand from the manufacturer.

- The simple model layout will look as follows:

### Stage 1

- Open the model STARTSUPPLY.MOD, which is located in the demo\tutorial\supply subdirectory of your WITNESS installation directory.

- This designer elements model allows you to add WITNESS elements to your model quickly and easily.

- This startup model differs a little from the standard WITNESS startup model in the following ways:

- The simulation clock has been altered to work in days as units – suitable for a supply chain model. To see how this is setup, please look at the model/clock menu option. You will notice the difference in the Time window display.

- Three icons to represent the factory, warehouse and retail outlet have been added in the picture gallery for use in the model build.

- Machine and Buffer displays have been redesigned with counters – again more suitable for supply chain modeling and labor queues for machines have been removed as labor elements are not to be used in this example.

- The grid display and co-ordinates are turned off although snap to grid is still active.

To start the model the first stage is to define two parts – one to represent goods and one to represent orders.

- Click on the part icon in the designer elements window, it will become selected (you will know when it is selected as the cursor changes to a cross and an outline rectangle is shown). Position the cursor in the layout window and click with the left-hand mouse button. A part (called Part001) appears selected in the simulation window. Hold down the left mouse button whilst the cursor is inside the net and you can drag the new element around the screen to position it. Click outside the net to remove it. The part is now part of the model you are building.

- Add another part in the same way.

- Then add a machine element and a buffer element.

- The next step is to add the detail needed to run the model.

- Detailing an element

- Elements can be detailed in several ways:

- The easiest way to access and change the details of an element is to double-click on the element displayed on the screen (that is, the machine icons or the buffer numerical display).

- Other options include:

- Clicking once on the element’s icon, then clicking on the detail button on the element toolbar

- Using the right mouse button on the element in the element selector.

- Part detail information

- Double click on Part001 to get the detail dialog for this element.

- Change the name of the part to Goods by simply typing over the existing name

- Click on the OK button to accept this change.

- Double click on Part002 to get the detail dialog for this element.

- Change the name of the part to Orders by simply typing over the existing name

- Click on the OK button to accept this change.

We will also change the display of the order part. Right click on the icon for the Orders part. This should bring up the display bar with Update Style as the default selections (as shown below):

Selecting the Draw button (the first from the left) the style change dialog will be opened. Double click on the icon in the selector shown and the picture gallery will open. Click on icon number 92 and then click OK. Then click Update and the tick on the display bar. Your icon display on the layout window should now be updated as shown overleaf.

Machine detail information

- Double click on Machine001 and enter:

- Name: Manufacturer

- From the Type pull down list select Batch

- Position the cursor in the Cycle Time field and type 1

- Set the Batch Min and the Batch Max to 8

- Click on the OK button on the detail dialog to accept the changes.

- Adjust the display slightly so that the new name does not overlap the status icon.

- Now change the icon for the machine – right mouse click on the machine icon and select display using the left hand mouse button. You should have update and Main icon as the default selection – if so click on the draw button and then double click on the icon. From the picture gallery select icon number 32 followed by OK, Update and the tick button on the display bar.

- Use the Shift or Ctrl keys and the left mouse button to drag the corners of the icon to make it bigger.

- Buffer detail information

- Double click on Buffers001 and enter:

- Name: Warehouse

- Click on the OK button on the detail dialog to accept the changes.

We will now define the rules to govern the production of goods by the manufacturer. Remember rules may be input either by the toolbars and the mouse, or via the detail dialog for elements. We will use the toolbar method.

- Machine Input rule information

- Select the machine Manufacturer by clicking on its icon. Now click on the visual input rules button on the element toolbar.

- The default rule should be pull – leave this in this case but note that others are selectable from the pull down combo box.

- Click on the Goods display and then the world button and you should see the following:

- Click on the OK button to accept the rule.

- Now click on the Visual output rules button.

- The default rule should be push – leave this in this case but note that others are selectable from the pull down combo box.

- Click on Warehouse and then click on the OK button to accept the rule.

Running the model

- Initially run the model in step mode using the step button on the run toolbar.

- This will open up the interact box to give you information on each step that takes place. As you step you should see 8 goods parts being placed in the Warehouse buffer each 1 time unit. (remember 1 time unit here represents one day).

- This corresponds to the model in STAGE1.MOD.

## Stage 2

- Now we will add the other elements necessary for this model.

- Firstly in the warehouse we will add the following:

- A buffer to hold orders needing to be picked

- A machine to model the picking operation

- A buffer to hold the orders waiting for delivery

- Secondly we will add a delivery machine to model the transport of goods from the warehouse to the retailer.

- At the retailer we will add the following:

- A buffer to hold the goods ready for sale

- A machine to model the customer demand

- A machine to generate orders when necessary

- Finally we will add two variables to act as counters of goods needed and goods ordered.

From the designer elements window define the other warehouse elements of two buffers and a machine.

Change the icon of the warehouse to icon number 33 and adjust its size using the left mouse button and the Ctrl or Shift keys. Name the elements as follows:

The two buffers should be called OrdersToPick and GoodsforDelivery and the machine should be called Picking.

- With a little arrangement your display should now look something like this:

From the designer elements window now define the Delivery machine.

Change the name of the machine to Delivery.

Delete the colored icon of this machine using the Delete Graphics option on the right hand mouse menu when selecting the colored icon.

Next define the retail elements of two machines and a buffer.

The two machines should be called OrderGenerator and CustomerDemand and the buffer should be called Retailer.

Change the icon of Customer Demand to icon number 34 and adjust its size using the left mouse button and the Ctrl or Shift keys and delete the colored icon of Order Generator using the Delete Graphics option on the right hand mouse menu when selecting the colored icon.

- With a little arrangement your display should now look something like this:

- Select the Variables tab in the Designer elements window.

- Define two integer variables (VInteger).

- Change the names of these variables to NumberNeeded and NumberonOrder.

- We have now completed adding all the elements that we need. However now we add the logic to make the model work.

- Detailing the Retail Elements

- The first step is to load the retailer with goods to sell at the start of the model.

- Double click on Goods and make the arrivals active. Fill in the form as below with maximum arrivals of 50, first arrival at 0.0 and lot size of 50. Click on the To… button and enter the rule Push to Retailer.

- Click on the OK button to accept this rule and on OK again to accept all the changes.

- Double click on CustomerDemand and make the following changes:

- Select Batch from the type pull down list

- Set Batch Min to 1

- Set Batch Max to 8

- Set Cycle Time to 1

- Click on the From button and type Pull from Retailer

- Click on the To button and type Push to Ship

- Click on the OK button to accept these changes.

This equates to a customer demand of up to 8 goods per day (a full 8 if available).

- Double click on Retailer and make the following changes:

- Click on the Actions on Input button and type

IF TIME > 0

NumberonOrder = NumberonOrder - 1

ENDIF

- Click on the OK button to accept these changes.

- Click on the Actions on Output button and type

IF NPARTS (Retailer) + NumberonOrder < 50

NumberNeeded = 50

ENDIF

- Click on the OK button to accept these changes.

The actions on output set here check the levels of goods at the retailer (taking into account those on order already) and record that a number of goods are needed if necessary. In this example 50 is the re-order level and 50 are ordered at a time. Setting the counter does not create the order but we will see later that the counter will cause the goods to be ordered by the machine OrderGenerator.

- Double click on OrderGenerator and make the following changes:

- Select Batch from the type pull down list

- Set Batch Min to NumberNeeded

- Set Cycle Time to 1

- Click on the From button and enter the following rule:

IF NumberNeeded > 0

PULL from Orders out of WORLD

ELSE

Wait

ENDIF

- Click on the OK button to accept these changes.

- Click on the Actions on Start button and enter the following actions:

NumberonOrder = NumberonOrder + NumberNeeded

NumberNeeded = 0

- Click on the OK button to accept these changes.

- Click on the To button and enter the following rule:

Push to OrderstoPick

- Click on the OK button to accept this rule

- Click on the OK button to accept all the changes on the dialog.

In this element we have made use of some very powerful features of WITNESS.

- The first is the use of an IF rule – these offer powerful ways to express logic – here orders will only be created when needed.

- The second is the use of a variable in a field. WITNESS allows variables or even complex expressions in many fields – making complex models much easier to set up.

- Detailing the Delivery and Warehousing Elements

- Select the machine Delivery by clicking on its icon. Now click on the visual input rules button on the element toolbar.

- The default rule should be pull – leave this in this case but note that others are selectable from the pull down combo box.

- Click on GoodsforDelivery and click OK button to accept the rule.

- Now click on the Visual output rules button.

- The default rule should be push – leave this in this case but note that others are selectable from the pull down combo box.

- Click on Retailer and then click on the OK button to accept the rule.

- Double click on Delivery and make the following changes:

- Select Batch from the type pull down list

- Set Batch Min to 5

- Set Batch Max to 30

- Set the Cycle Time to 3

The detail dialog should look as follows:

With these settings for Delivery the delivery of goods from the warehouse takes 3 days and a minimum of 5 and a maximum of 30 goods can be delivered at once.

The final logic to add is that for the Picking in the warehouse.

- Double click on Picking and make the following changes:

- Select Batch from the type pull down list

- Set Batch Min to 1

- Set Batch Max to 100

- Set the Cycle Time to 1

For the input we will use a very powerful WITNESS rule – the MATCH rule – to match up goods to orders. This rule can be used to match up on attributes or indeed any condition but here we will simply use it to match up any goods to any order.

- Click on the From button and enter:

MATCH/ANY

OrderstoPick #(1)Warehouse #(1)

- Click on the OK button to accept these changes.

- Click on the To button and enter:

IF TYPE = Goods

PUSH to GoodsforDelivery

ELSE

PUSH to SHIP

ENDIF

- Click on the OK button to accept these changes.

- Click on the OK button to accept all the changes on the dialog.

Note the use of the WITNESS variable TYPE to identify the parts. Goods here are output to PickedOrders but orders have finished their use at this point and so are shipped.

Now the model is ready to run!!!

- This corresponds to the model in STAGE2.MOD.

Running the model

- Run the model. Use the Walk button to make sure that you see the movement of goods and orders between the queues (should be on by default). The slider next to the walk button changes walk speed. To stop a very slow walk press the escape button.

- Follow the logic carefully – these models are a little more complex than many other types of model.

### Stage 3

- For a final stage of this example let us add a timeseries looking at stock levels and arrange a number of different windows to view the model.

- First rearrange the layout window display as follows:

- Then drag up the base of the layout window to above the Goods and Orders, etc and open a second window.

- Arrange this window below the first and adjust the view with the scroll bars and zoom buttons to show the line of 4 counters and parts.

- Make the window2 active and then select the Window/Control menu option and change the title of the window to Key and Counters.

- The display should now look something like this:

- Now reposition the time window alongside the key window. Open up window 3 and for now leave this over the other windows.  Change the title of window 3 to Stock Levels. Scroll down this window until the whole window is blank.

- In the designer elements window select the reports tab and click on the timeseries to select it. (Remember it is selected when you can see a box around the designer element).

- Place the timeseries on the left hand side of the new stock levels window by clicking in the window once to place and again to position it.

- Now right click on the Timerseries001 graph display and select Display from the right mouse menu. Then click the draw button. Change the following parameters:

- Height of Display: 8

- Number of Observations: 40

- Minimum Display Value: 0

- Maximum Display Value: 40

- Then click on Update to accept the changes

- Now double click on Timeseries001 to bring up its detail dialog.

- Enter the following data:

- Name: StockLevels

- Plot1: NPARTS(Retailer)

- Plot2: NPARTS(Warehouse)

- Recording Interval: 1

- Click on the OK button to accept these changes.

Turn off the designer elements window and reposition and resize the Stock Levels window underneath the other windows as shown below (Delete the word timeseries001 – right mouse menu delete graphics – if you need the space). Remember to leave space for the time axis display at the bottom of the window:

This completes the model build for SUPPLYCHAIN.MOD

Running the model

Run the model and observe the predicted saw-tooth patterns of the timeseries. Experiment with different timings in the model and observe the effects.

In this model if you set your re-order levels too low then customers have no goods to buy sometimes and there is no compensation in manufacture. Hence Warehouse stock levels rise and rise in this scenario. Try setting the reorder level to 10 (this is in the Actions on output of the Retailer Buffer – simply set NumberNeeded=10)

The green level of goods in the warehouse is rising

A valid question here is why is the red line, stock at the retailer, not showing zero sometimes? This is due to the sequence of events in the model. Most of the events are taking place at the same time – therefore the order is crucial in determining the result recorded. The reading is taking place just after delivery and as the stockout does not last over a day it is not recorded.

To see the difference, try setting the update interval of the timeseries to 0.999999 – essentially the same for the first 1000 or more days – now the zero stock positions are recorded.

In models of supply chains like this which are essentially timeslice simulations the sequence of events is important. Of course you have full choice – some events can be more accurately modeled – e.g. customer demand could occur at exact times within the day – as could re-ordering. It is important to find out how the real system works before modeling it.

# Shift & Arrival Profile Modeling

## Model Build

### Introduction

- This is a description of how to build a simple WITNESS model incorporating a shift pattern and arrival profile. Much larger and more detailed models can be developed in WITNESS. The model is built incrementally. In this way you can ensure that each stage of the model is correct before you go on to the next stage, and it is possible to see clearly the effects of any change that you make.

- If you don’t want to build the model, but would like to see the model, open the model SHIFTS.MOD in the demo\tutorial\shifts subdirectory of your WITNESS installation.

Model summary

- In the model a component is to be produced on a lathe machine and then hand finished. The components arrive for the lathe operation at a rate of 6 each half hour between 08.00 and 17.30 each day. Each of the 6 however can arrive at any time within the half hour period – i.e. the arrival rate is not constant.

- The lathes operate on a more complex shift structure. From Monday to Friday a full day shift is worked, with breaks during the day, and at the weekend only the mornings are worked.

- The lathes break down fairly frequently (on average each 2 hours that they are busy) and take between 30 and 60 minutes to repair.

- There are four lathes and six hand finishing benches.

- In the model the arrivals of components will be modeled using an arrival profile, the lathes and hand finishing operation as machines, the lathe shift patterns using shift elements. A repeat period timeseries will be used to view the queue size that builds up before the lather operation and a histogram used to show the time for production including delays.

- The completed model will look like this:

### Stage 1

- Open the model STARTSHIFT.MOD, which is located in the demo\tutorial\shifts subdirectory of your WITNESS installation directory.

- This designer elements model allows you to add WITNESS elements to your model quickly and easily. The first stage of the model you are about to build contains one part called COMPONENT, a buffer called QLATHE and a machine called LATHE.

- Click on the part icon in the designer elements window, it will become selected (you will know when it is selected as the cursor changes to a cross and an outline rectangle is shown). Position the cursor in the layout window and click with the left-hand mouse button. A part (called Part001) appears selected in the simulation window. Hold down the left mouse button whilst the cursor is inside the net and you can drag the new element around the screen to position it. Click outside the net to remove it. The part is now part of the model you are building.

- Add a buffer and a machine in the same way.

- The next step is to add the detail needed to run the model.

- Detailing an element

- Elements can be detailed in several ways:

- The easiest way to access and change the details of an element is to double-click on the element displayed on the screen (that is, the machine icons or the buffer numerical display).

- Other options include:

- Clicking once on the element’s icon, then clicking on the detail button on the element toolbar

- Using the right mouse button on the element in the element selector.

- Part detail information including Arrival Profile

- Double click on Part001 to get the detail dialog for this element.

- Change the name of the part to Component by simply typing over the existing name

- Select Active with Profile from the Arrivals pull down list

- Select the Arrival Profile tab on the dialog

- We will use a 24 hour cycle (the default). Type 0 into the first box next to 0:00 (i.e. the quantity field)

- Use the down arrow to move to the next box and type 0 again.

- In the same way add 0 to all the boxes – except for those from 8:00 to 17:00 – against all these boxes type 6

- Click on the OK button to accept this change.

This equates to an arrival pattern of 6 arrivals every half hour between 8:00 and 17:30. This leads to a total of 114 arrivals per day. The arrival profile will randomize the arrivals within each half hour period.

An alternative here for regular arrivals would be to select active in the general part detail tab, a first arrival at time 15, an inter-arrival time of 30 and a lot size of 6. This combined with a shift pattern to only allow arrivals between the start and end times of the pattern required.

However we will use the arrival pattern here as being simpler and more flexible. Note the ability to import complex patterns from files and the ability to define a multiplier to scale up and down the arrivals whilst maintaining the pattern.

Buffer detail information

- Double click on the buffer and enter:

- Name: QLathe

- Click on the OK button on the detail dialog to accept the changes.

- Machine detail information

- Double click on the machine and enter:

- Name: Lathe

- Quantity: 4

- Cycle Time: Uniform(2,18)

- Click on the OK button on the detail dialog to accept the changes.

The time for the lathe operation has been set to be any time between 2 and 18 minutes with equal likelihood.

The display will look like this:

Rearrange the display (by dragging the elements) to look more like this:

You will find that as the display is locked the individual lathe display items will move together when dragged):

We will now define the rules to govern the flow of components through the QLathe and Lathe elements. Remember rules may be input either by the toolbars and the mouse, or via the detail dialog for elements. We will use the toolbar method.

- Part Output rule information

- Select the part Component by clicking on its icon.

- Now click on the Visual output rules button.

- The default rule should be push – leave this in this case but note that others are selectable from the pull down combo box.

- Click on QLathe and then click on the OK button to accept the rule.

- Machine Input rule information

- Select the machine Lathe by clicking on its icon. Now click on the visual input rules button on the element toolbar.

- The default rule should be pull – leave this in this case but note that others are selectable from the pull down combo box.

- Click on QLathe and then on the OK button to accept the rule.

- Now click on the Visual output rules button.

- Click on the Ship button in the toolbar and then click on the OK button to accept the rule.

Running the model

- Run the model at different speeds using the run toolbar controls and adjusting the walk rate and turning walk on and off using the run toolbar button and slide control.

- Note the changing color of the LATHE status icons – yellow is idle (ready to work), green is working. Later we will see white – the color for off shift.

- Given the demands in the model so far the lathes easily cope with the arrival rate of components at this stage.

- This corresponds to the model in STAGE1.MOD.

### Stage 2

- Now we will add another buffer and machine for the hand finishing operation and will add a shift pattern and breakdown rate for the lathe machines.

From the designer elements window select a new buffer element from the basic tab and place this element on the layout window (by clicking twice).  Then detail the buffer element as follows by double clicking on the display.

- Name: QFinish

- Click on the OK button to accept this change.

From the designer elements window select a new machine element from the basic tab and place this element on the layout window (by clicking twice).  Then detail the machine element as follows by double clicking on the display.

- Name: HandFinish

- Quantity: 6

- CycleTime: UNIFORM(6,23)

- Click on the OK button to accept this change and rearrange the display as for the lathe.

- Now change the icon for the machine – right mouse click on the main machine icon and select display using the left hand mouse button. You should have update and Main icon as the default selection – if so click on the draw button and then double click on the icon. From the picture gallery select icon number 15 followed by OK, Update and the tick button on the display bar.

- Your display will look something like this:

- Now we will add the additional flow rules for the model:

- Click on the Lathe machine to select it.

- Click on the visual output rules button.

- Delete SHIP in the rule

- Then click on the QFinish display.

- Click on OK to accept the rule

- Select the machine HandFinish machine by clicking on its icon.

- Now click on the visual input rules button.

- The default rule should be pull - Click on QFinish and then on the OK button to accept the rule.

- Now click on the Visual output rules button.

- Click on the Ship button in the toolbar and then click on the OK button to accept the rule.

Next we will add a breakdown pattern for the lathe machines:

- Double click on the Lathe machine and select the Breakdowns tab.

- Click on the add/remove button and use the add button to add the default breakdown number 1. Click OK to accept the new breakdown.

- Change the breakdown mode to busy time.

- Locate the cursor in the time between failures field of the breakdown interval pane.

- Type in NEGEXP(120)

- Click on the repair time field.

- Type in UNIFORM(30,60)

- Click OK to accept this detail and close the dialog.

This has added a breakdown interval on busy time with a negative exponential distribution with a mean of 120 minutes and a repair time of a uniform distribution – minimum 30 minutes, maximum 60 minutes.

Next we will add a shift pattern.

From the designer elements window select the Shifts Tab. This contains a number of different shift patterns to choose from. These are just examples – you can of course add your own.

Select Weekly_7_Days_1 and place this element on the layout window (by clicking twice).

This element is a module that contains a shift and two sub-shifts.

It is worthwhile exploring the structure of this shift pattern:

The structure of the module can be seen in the screenshot below. The detail dialog in the screenshot is for the main shift element in the module. It can be seen to be composed of seven sub shifts representing a seven day week pattern of five full day shifts followed by two morning shifts. These sub shifts are the other elements in the module.

- The detail of the day shift shows a complicated pattern of work and rest representing a single day with a morning, lunch and afternoon break structure. This is defined within four work periods.

To apply the shifts to the machine elements:

- Double click on Lathe to select its detail dialog

- Select the Shift tab and click on enable shifts

- Select the down arrow by the shift control and then double click on the Week_7Days_1 shift element:

- Click OK to accept this detail and close the dialog.

Repeat the same shift assignment for the HandFinish machine

The model display should now look like this:

The display for the shift shows you whether the shift is on or off, the current period number and an icon and name.

Running the model

- Run or Step the model again and note the flow of parts through the system. When the shifts are not in operation the machines will show a white status icon. The visual checking of logic in this way is very important and is a key benefit of visual simulation with WITNESS.

- This corresponds to the model in STAGE2.MOD.

### Stage 3

In order to see selected results from the model on the screen we will now define a Timeseries element and a Histogram element.

- In the designer elements window select the reports tab and click on the timeseries to select it. (Remember it is selected when you can see a box around the designer element).

- Place the timeseries on the right hand side of the model window by clicking in the window once to place and again to position it.

- Define a histogram in the same way (below the timeseries)

- Now right click on the Timerseries001 graph display and select Display from the right mouse menu. Then click the draw button. Change the following parameters:

- Number of Observations: 21

- Maximum Display Value: 200

- Time Axis Plot Options: Expression

- Expression: TIME/60

- Total Characters (each axis): 3

- Then click on Update to accept the changes as shown overleaf

Now we will detail the timeseries

Double click on the timeseries and enter:

- Name : NumberinQLathe

- Recording Interval: 480

- Plot 1: NPARTS(QLathe)

- And tick the repeat option.

- Click on the OK button on the detail dialog to accept the changes.

This has defined a timeseries that will plot the number of parts in QLathe every 8 hours over a 168 hour (1 week). This will be a repeat period timeseries which will show 3 plots – the maximum, the average and the minimum reading for each plot point.

Now we will set up the histogram. Right click on the Histogram001 graph display and select Display from the right mouse menu. Then click the draw button. Change the following parameters:

- Maximum Display Value: 48

- Number of Cells: 16

- Total Characters Cell Axis: 2

- Then click on Update to accept the changes

Now we will detail the histogram

Double click on the histogram and enter:

- Name : ProductionTimes

- Click on the OK button on the detail dialog to accept the changes.

This has defined a histogram in which to record the time that each part takes to be manufactured in the model.

In a WITNESS model each part has a system attribute automatically which records the time that it entered the model. To record the statistic that we want is a simple matter:

Double click on the machine HandFinish

Click on the button Actions on Output

Enter the text:

Record (Time-TimeEnteredModel())/60 in ProductionTimes

This calculates the difference in time now (Time) with the time when the part entered (TimeEnteredModel()) and divides by 60 to record this in hours.

To finish the model close the clock window and then select the Time window from the Windows menu and position approximately where the clock window was. The model SUPPLYCHAIN.MOD is now completed and should look like this:

Running the Model

Try running the model for a while and see the results build up (they build up more quickly with the walk mode turned off). By batching (the double forward arrow) you can fast forward the model very quickly. Try batching for 20 weeks. To do this enter 20*7*24*60 in the time box next to the alarm clock – then make sure the alarm clock button is depressed and when it is press the batch button. The time window will remain open to keep you apprised of progress.

From the timeseries it can be seen that the queue for the lathe build up as expected at the weekend and then is steadily reduced again during the week. The simulation helps to quantify how large the queue gets.

Similarly the histogram shows that a lot of parts take over 12 hours to produce and just a few take over 36 hours. Is this acceptable? – You now have the information on which to decide.

The lathes are well utilized as can be seen from the standard chart of machine status:

For an experiment try improving the repair time for the lathes to UNIFORM(10,20) – do this on the breakdown tab of the lathe detail. Perhaps this is a possible change with dedicated repair teams.

Run the model again and see that this improvement would mean that no components now take over 27 hours to product and the lathes have much spare capacity. (To see the figures behind the histogram right click on the histogram display and choose the statistics option).

Similarly explore all the model statistics. To see them all use the right hand mouse menu on the tree Simulation section. Select expand all and then after selecting all by ticking the simulation box choose the right hand mouse menu again and choose statistics.

All the reports screens can now be stepped through using the << and >> buttons.

Examples:

## Further Shifts Example

The model SHIFT.MOD in the Examples directory – located underneath the WITNESS installation directory contains another example of the use of shifts.

### SHIFT.MOD

In this model parts arrive from another area of the factory during the night as well as during the day. In this part of the factory, the machine only works during the day and the buffer stores parts arriving from operations working different shifts.

2 separate shift patterns have been set up: one for the machine and one for the parts.

Shift Pattern for the Machine

The 4 identical days can be modeled using the same shift pattern (MonThurs), but Friday is different so it needs a different shift pattern (Friday).

On Mondays, Tuesdays, Wednesdays and Thursdays, there are 2 periods in the shift:

1:		4 hours of work time + 1 hour of rest time + + 0 overtime

2:		4 hours of work time + 15 hours of rest time + 0 overtime

On Fridays, there are also 2 periods in the shift:

1:		4 hours of work time + 1 hour of rest time + + 0 overtime

2:		4 hours of work time + 17 hours of rest time + 0 overtime

Saturdays and Sundays are accounted for in the Week shift.

Although two shift patterns are used to model the machine, only one shift can be applied to the machine. Therefore, another shift (called Week) was created. Week contains:

4 x MonThurs shifts

1 x Friday shift

1 x extra period of rest time of 2880 minutes (this models the weekend).

It is the Week shift that is applied to the machine on the Shift page of the machine’s detail dialog.

Shift pattern for parts arriving in the buffer

Each day, there are 4 periods in the shift:

1:		4 hours of work time + 1 hour of rest time + 0 overtime

2:		4 hours of work time + 3 hours of rest time + 0 overtime

3:		4 hours of work time + 1 hour of rest time + 0 overtime

4:		4 hours of work time + 3 hours of rest time + 0 overtime

At the beginning of the simulation, 8 hours are worked without a break (this is specified in the shift’s initial offset period). This gives the buffer a chance to catch up with the machine’s need for parts.

The DayNight shift that incorporates this pattern is applied to the parts on the part detail dialog’s General page.

For more details on this model please see the WITNESS help text.

# Tanks and Fluids Modeling

## Model Build

### Introduction

- This is a description of how to build a simple WITNESS model incorporating WITNESS continuous modeling elements including tanks, fluids, pipes and processors. Much larger and more detailed models can be developed in WITNESS. The model is built incrementally. In this way you can ensure that each stage of the model is correct before you go on to the next stage, and it is possible to see clearly the effects of any change that you make.

- If you don’t want to build the model, but would like to see the model, open the model TANKSIZING.MOD in the demo\tutorial\continuous subdirectory of your WITNESS installation.

Model summary

- Oil flows at a constant rate into a holding tank. From here a pipeline feeds to a processing operation where the processor fills, processes and then empties.

- The pipeline is quite unreliable with frequent breakdowns affecting flow and the processor too breaks down and needs repair. The processor also needs cleaning between each individual process.

- The purpose of the model is to determine the size of tank that is needed in this process.

### Stage 1

- Open the model TankStart.MOD, which is located in the demo\tutorial \continuous subdirectory of your WITNESS installation directory.

- This designer elements model allows you to add WITNESS elements to your model quickly and easily. The first stage of the model you are about to build will contain one fluid, one tank, one pipe and one processor.

- Continuous elements are not present in the normal WITNESS startup model as there is a version of WITNESS that does not contain them. This tutorial should only be attempted by those customers with the appropriate version of WITNESS. To add this tab to the normal startup model it is simply a case of adding another tab in the designer elements window (right mouse menu in that window), and then again from the right mouse menu on the new tab selecting load designer set and selecting continuous.des from the witness/demo directory.

- However the continuous elements are present in the startup model here.

- Click on Fluid on the continuous tab in the designer elements window, it will become selected (you will know when it is selected as the cursor changes to a cross and an outline rectangle is shown). Position the cursor in the layout window and click with the left-hand mouse button. A fluid (called Fluid001) appears selected in the simulation window. Hold down the left mouse button whilst the cursor is inside the net and you can drag the new element around the screen to position it. Click outside the net to remove it. The fluid is now part of the model you are building.

- Add a tank, a pipe and a processor in the same way.

- The next step is to add the detail needed to run the model.

- Detailing an element

- Elements can be detailed in several ways:

- The easiest way to access and change the details of an element is to double-click on the element displayed on the screen.

- Other options include:

- Clicking once on the element’s icon, then clicking on the detail button on the element toolbar

- Using the right mouse button on the element in the element selector.

- Fluid detail information

- Double click on Fluid001 to get the detail dialog for this element.

- Change the name of the fluid to Oil by simply typing over the existing name

- Click on the OK button to accept this change.

Tanks detail information

- Double click on Tank001 and enter:

- Name: TankA

- Note that the capacity of the tank is 100 units which will suit us here.

- Click on the OK button on the detail dialog to accept the changes.

- Similarly change the name of the pipe to P1 and the processor to ProcessorA.

Now change the pipe display to be more interesting (just for illustration purposes).

Right click on the actual pipe display (not the name) and select display.

Now click on the draw button

Select draw and then draw a new display for the pipe by clicking with the left mouse button in a sequence of places on the display. End by clicking the end of the final segment with the right mouse button. If you make a mistake simply repeat the drawing. Try to connect the tank and the processor. Then click OK and the tick button on the display bar.

Your display should look something like:

We will now define the rules to govern the flow of oil through the system.

- Double click on Oil

- Tick active arrivals and enter an arrival rate of 0.7

- Click on the To… button and enter the following rule:

FLOW to TankA RATE (0.7)

- Click OK twice to accept the changes.

You may run the model to check that oil is indeed flowing into the tank. Use the video style buttons on the run toolbar.

- Double click on TankA

- Click on the Empty To… button and enter the rule

FLOW to P1 rate (2.0)

- Click OK twice to accept the changes.

- Double click on P1

- Enter 2.0 in the flow rate box (this is an additional control to constrain the rate of flow).

- Click on the Empty To… button and enter the rule

FLOW to ProcessorA rate (2.0)

- Click OK twice to accept the changes.

- Double click on ProcessorA

- Enter 10.0 in the process time box

- Click on the Empty To… button and enter the rule

FLOW to SHIP rate (5.0)

- Click OK twice to accept the changes.

- This corresponds to the model in STAGE1.MOD.

Running the model & Anticipated Results

If you run the model you will see a rhythmic rising and falling of the levels in the tank and the processor.

Running the model to time 1000 it can be seen from the Oil statistics (right mouse menu/statistics on oil name) that 700 units of oil have arrived and 500 have been shipped through the system.

### Stage 2

- In stage 2 we will complete the model with the following:

- A breakdown rate for the pipeline

- A breakdown rate for the processor

- A cleaning cycle for the processor

- The creation of a timeseries to monitor tank contents

- Double click on P1 and select the breakdowns tab

- Enable them by ticking in the box and then fill in a breakdown interval of 100 and a repair time of 40 (leave the mode as available time).

- Click OK to accept the changes.

- Double click on ProcessorA and select the breakdowns tab

- Enable them by ticking in the box and then select ProcessTime from the Breakdown Mode pull down list.

- Enter a distribution UNIFORM(100,200) in the breakdown interval

- Enter a distribution UNIFORM (20,40) in the repair time

- Click on the Cleaning tab

- Enable them by ticking in the box and leave the mode of Empty in the pull down box (but do note other methods exist)

- Enter 10 in the Cleaning time field

- Click OK to accept the changes.

In the designer elements window select the Reports tab and define a timeseries by clicking on it and placing in the layout window to the right of the processor.

- Double click on Timeseries001

- Enter Name: SizeTracker

- Enter Recording Interval: 1

- Enter Plot 1 expression as: VCONTS(TankA)

- Click OK to accept the changes.

Now right click on the SizeTracker graph display and select Display from the right mouse menu. Then click the draw button. Change the following parameter:

- Maximum Display Value: 100

- Then click on Update to accept the changes

- Now select draw from the first selection box on the display bar and select Max/Min in the second.

- Then press the draw button.

- Tick Max on the first row as shown in the picture and then press the Draw button. Place the max display above the timeseries in the layout window.

- This completes the model building.

Running the model

- Now when the model is run the breakdowns and cleaning cycles interrupt the smooth and regular rise and fall of the levels and there is more variation in the levels.

- As the model runs observe the different colors of the outlines of the pipe and the processor. These all indicate different states. The following tables indicate the states seen in this model.

- For the pipe:

| - State | - Color |
| --- | --- |
| - Flowing Normally | - Green |
| - Broken Down | - Red |

- For the processor:

| - State | - Color |
| --- | --- |
| - Cleaning | - Cyan (light blue) |
| - Broken Down | - Red |
| - Filling / Flowing | - Yellow |
| - Processing | - Green |
| - Emptying / Flowing | - Dark Green |

- It is important to check the display of a model in order to check the logic that has been entered. Once this has been done the model can be run more quickly by fast forwarding the model using the  key.

- By running the model for a very long time it can be seen that a tank size of 93 units capacity is needed for this process to run smoothly.

- However of course before such a tank is built or brought into operation the model may be used to quantify other options. For example: what if the pipe is made more reliable?

- Try this by increasing the pipe breakdown interval to 200. You will see that this indeed does decrease the size of tank needed.

## Further Continuous Examples

There are a number of the standard WITNESS demonstration models installed in the DEMO and EXAMPLE directories under your WITNESS installation that include WITNESS continuous elements in use.

These include CHEMIST.MOD and RECIPE..MOD

### CHEMIST.MOD

This model represents a chemical processing plant.

Raw materials flow into the RawMaterial tanks. When the tanks are full the input stops and it starts again when they reach a certain level whilst emptying. This flow is controlled by a variable Flow that is set on warning levels for the Raw material tanks.(see the warning level actions on one of these tanks detail dialogs on the warning levels tab).

60 orders arrive during every shift. Every order requires a different mixture of five disposal substances. The compound is defined by a UNIFORM distribution. You can see this process in the Actions on Create for the part Order.

The order is then assigned to the reactor with the lowest number of orders waiting. This is effected by the Logistic machine using a MOST output rule.

The flows from the raw material tanks to the mix / blend / react processors to the filling tanks are controlled and effected by a multi- cycle machine called Control. This has four cycles with different timings, logic and labor requirements. The movement from the raw material buffers is not done with a flow rule but with a MOVE action. This is a useful option for continuous models. It requires extra checking logic for volumes/capacities but will generally result in faster executing models than pure continuous flows.

Examples of the MOVE action can be seen in this model in the Control machine dialog – either on Actions on start of cycle2 (filling) or Actions on finish of cycle 3 (Cycle).

In this way, when it is required, the raw material is taken from the raw material buffers and carried to the reactor. This will be done by an available worker from worker group 1. Then one of these workers fills the reactor with the required mixture. After an additional time the fluid is transfered to a tank (T1) for the filling operation and the reactor gets cleaned for the next order.

The filling operation is controlled by the machine Filling. This uses the WITNESS modeling ability to fill parts with fluid. Look at the fluid rules tab for the machine Filling. This controls the flow from the tanks T1 into a part called Drum.

Note too on the Drum detail dialog on the attributes tab that the part is tagged as being able to contain fluids.

### RECIPE.MOD

This is a simple model, illustrating the use of the Recipe rule.

The RECIPE rule allows you to input a specified volume of fluid from a number of pipes, tanks and processors, or to output a specified volume of fluid to a number of pipes, tanks and processors. You can also use the RECIPE rule for machines that fill or empty parts with fluid.

A MIXER processor element is producing five products from different raw materials in differing quantities and storing them in different tanks.

A data file (RECIPE.DAT) contains the individual process details for each batch of product in a production schedule. The first product to make is read in Initialize Actions (Model/Initialize Actions menu) into variables defined in the model which are then used by processor MIXER in its input recipe rule. The MIXER reads the next recipe in when it empties – this can be seen on the warning levels tab of the MIXER detail dialog.

The model runs until a storage tank fills at about time 3500.