Graphical and Display Features

Objectives

- To use display features to your advantage.

- To learn the use of Time series, Histograms and Pie Charts.

- To modify, create and import your own icons.

- To learn how to use associated views, process views, element flows, and meteor trails with your models.

- To import CAD files and use multiple layers in your model.

- To use AVI files, expressions and run controls to enhance the presentation of your models.

## Graphical Features

### Timeseries

The WITNESS Timeseries Element allows you to graphically present simulation results on the screen. A Timeseries plots values over time.

At specified intervals a 'reading' is taken from the model and 'plotted' on a graph. As time passes, a series of values will be plotted from left to right in the graph. Once the space on the screen allocated to the Timeseries has been exhausted, the graph 'scrolls' to allow the new plots to be made.

Timeseries are useful for determining the trends or cycles underlying the model, since they provide a 'history' of the specified value.

WITNESS allows you to create as many Timeseries as you wish; each may be used to plot up to seven values. The detail for a Timeseries specifies the interval at which observations are made and the values to be recorded.

Examples of the use of Timeseries might be to present graphically hourly throughput or the number of parts in a storage area at the end of each shift.

Add a Timeseries to Your Model

- From the Designer Elements Windows, click with the left mouse button on the Element name Timeseries. - A cross arrow appears.

- Position the cross arrow where you wish the distribution to be positioned in the simulation window.

- Click with the left mouse button. - Timeseries001 appears on the screen with a net around it.

- Click and drag inside this net to move Timeseries001 to the required position. - If you have selected the Multiple Designer Elements option from the ModelOptionDisplay menu, you will also need to click the right button of your mouse before dragging the newly created Element.

- Select the detail button from the Standard Action Bar. - The Timeseries detail dialog appears on the screen. (See Figure 5-1)

- Enter the Recording Interval (e.g. how often you want values recorded on the Timeseries).

- For each line on the timeseries plot, enter a plot expression; e.g. NWIP (A) etc.

- Select the OK button on the Timeseries dialog

Figure 5-1: Timeseries Detail Dialog Box

#### Repeat Period Timeseries

It is also possible to display a Timeseries in a slightly different way - e.g. one that reports on the maximum, mean and minimum values over a repeated time period rather than the actual value. For example, if the number of people queuing for a service is known to fluctuate depending on the time of day, a repeat period of a day could be used to show this effect.

#### Display Options

In addition to the common display options of other elements, WITNESS provides you with three new display options:

- Timeseries.

- Max/Min.

- Plot.

The Timeseries display option allows you to make a series of modifications on the actual display. Controlling the number of divisions in the Y-axis can modify the height of the display. The number of operations controls the width of the timeseries before scrolling to the left. The range of values to be displayed in the plot can be modified. The time axis and value axis characteristics such as colors and text sizes can also be changed.

The Max/Min display option allows you to display the actual maximum and minimum values for each of the plots on the screen while the model is running.. The Plot option, on the other hand, allows you to modify the default colors of each plot.

Figure 5-2: Timeseries Display Option Box

### Histograms

The WITNESS Histogram is another Element that allows you to graphically present simulation results on the screen. A Histogram is a frequency distribution of values observed for some parameter in the simulation. One of the advantages of using a Histogram is that it keeps track of both your running average and standard deviation.

WITNESS allows you to create as many histograms as you wish and record values in them as appropriate. Bars in the graph may be drawn vertically or horizontally, allowing Gantt type charts to be produced. Values are recorded in a Histogram using the RECORD statement in a set of actions anywhere in the model:

#### Usage

RECORD <value > in <histogram>

#### Example

RECORD NWIP(A) in WIPHIST

A bar of a specified height may be drawn using the DRAWBAR action command:

#### Usage

DRAWBAR <histogram> <value>, <height>, <color>

#### Example

DRAWBAR UTILHIST 1, PUTIL(OP10,1),3

The height of a bar on the Histogram may be incremented using the ADDBAR action command:

#### Usage

ADDBAR <histogram> <value>, <increment>, <color>

#### Example

ADDBAR UTILHIST 1 PUTIL(OP10,5),1

Examples of the use of Histograms are to graphically present a frequency distribution of repair times or the time Parts take to travel through a system.

| Table 5-1: Color Codes for ADDBAR/DRAWBAR |  |  |  |
| --- | --- | --- | --- |
| Code |  | Code | Color |
| 0 | Black | 8 | Gray |
| 1 | White | 9 | Dark Gray |
| 2 | Red | 10 | Dark Red |
| 3 | Green | 11 | Dark Green |
| 4 | Blue | 12 | Dark Blue |
| 5 | Cyan | 13 | Dark Cyan |
| 6 | Yellow | 14 | Dark Yellow |
| 7 | Magenta | 15 | Dark Magenta |

Add a Histogram to Your Model

- From the Designer Elements Windows, click with the left mouse button on the Element name Histogram. - A cross arrow appears.

- Position the cross arrow where you wish the distribution to be positioned in the simulation window.

- Click the left button of your mouse. - Histogram001 appears on the screen with a net around it.

- Click & drag this net to move Histogram001 to the required position. - If you have selected the Multiple Designer Elements option from the ModelOptionDisplay menu, you will also need to click the right button of your mouse before dragging the newly created Element.

- Click on the Element from which you are going to record the value in the histogram.

- Select the detail button from the Standard Action Bar

- Select the Actions on the detail dialog in which you are going to record the value on the histogram.

- Enter – RECORD value in Histogram001.

- Select the OK buttons from the Actions dialog and the detail dialog of the Element

Figure 5-3: Histogram Detail Dialog Box

#### Display Options

In addition to the common display options of other elements, WITNESS provides you with a couple of new display options:

- Histogram.

- Max/Min.

The Histogram display option allows you to make a series of modifications on the actual display. Controlling the number of divisions in the Y-axis can modify the height of the display. The maximum and minimum display values control the width of the histogram. The number of cells to be displayed on the X-axis can be modified.

You can turn the display of your mean and standard deviation besides the Histogram on or off. You could transpose your X and Y-axis, and auto-re-scale the value axis. The Y-axis and X-axis characteristics such as colors and text sizes can also be changed. You can even select the layer where you want your Histogram to be located.

The Max/Min display option allows you to display the actual maximum and minimum values of your Histogram on the screen while the model is running.

Figure 5-4: Histogram Example

Figure 5-5: Histogram Display Option Box

### Pie Charts

The WITNESS Pie Chart Element allows you to graphically present simulation results on the screen. Like a histogram, a Pie Chart shows the frequency of occurrence of different values in a simulation run, but frequency is expressed as a percentage of the total number of observations.

A Pie Chart is perhaps most frequently used to display the utilization of WITNESS machines, conveyors, labor, and other physical Elements on the screen. They can be used to highlight areas of concern such as the percentage of on-shift time a machine is broken down during a simulation run Another application would be to display the current mix of product currently in stock.

WITNESS allows you to create as many Pie Charts as you wish and record values in them as appropriate. At specified intervals, the Pie Chart is 'refreshed' with the current values of the selected items. The detail for a Pie Chart specifies the 'refresh' interval for values to be re-evaluated. Pie Chart sectors may be given different colors and patterns.

| Add a Pie Chart to Your Model |  |
| --- | --- |
| - From the Designer Elements Windows, click with the left mouse button on the Element name Pie. - A cross arrow appears. |  |
| - Position the cross arrow where you wish the distribution to be positioned in the simulation window. |  |
| - Click with the left mouse button |  |
| - Pie001 appears on the screen with a net around it. |  |
| - Click and drag inside this net you can move Pie001 to the required position. - If you have selected the Multiple Designer Elements option from the ModelOptionDisplay menu, you will also need to click the right button of your mouse before dragging the newly created Element. |  |
| - Select the detail button from the Standard Action Bar. - The Pie Chart detail dialog appears on the screen. (See Figure 5-6) |  |
| - Enter the Refresh Interval (e.g. how often you want values updated on the pie chart). |  |
| - For each sector of the pie chart plot, enter a plot expression; e.g. NWIP (A) etc. using the Add/Remove button. - Click OK when finished. | - Select the Element States property sheet - Enable the display Element states option - Select the Element from the drop-down list. - This generates the expressions for each of the sectors. - Click OK when finished |
| - The Summarize button displays the expressions etc. for all of the sectors. |  |

Figure 5-6: Pie Chart Detail Dialog Box

#### Display Options

In addition to the common display options of other elements, WITNESS provides you with a set of new display options:

- Sectors

- Tilt

- Depth

- Start Angle

- Pull-out Sector

- Key

The Pie Chart display options allow you to make a series of modifications on the actual display. You can change the color and pattern of the different Sectors. The Tilt and Depth of the pie chart are very easy to modify. The Start Angle determines the position where the first sector would be located while the Pull-out Sector option determines the extent to which it is retracted from the other sectors. You could also select to show the names percentages and values of each sector by selecting to display the Key option.

To alter the display of the sectors, tilt, depth, start angle and pull-out sector, you must click on the draw button, after selecting the desired option, so that the cursor changes to a cross hair, then click and drag the pie chart to change its display.

Figure 5-7: Pie Chart Display Options

Figure 5-8: Pie Chart Example

- Do exercise 1 now.

## Display Features

### Icons and the Icon Editor

#### Icons

An icon is a shape or symbol comprised of a rectangular grid of 'pixels'. A pixel is a single square point of color, the smallest unit of display on the computer screen. You may use icons to visually represent various WITNESS Elements in your model such as Parts (Entities), Machines (Activities), Conveyors, Tracks, Vehicles, and Labor (Resources). You may also use them as part of an illustration drawn with the Screen Editor that will be discussed later.

Once an icon has been defined, it may be used repeatedly. Moreover, if you change the definition of the icon after you have used it in the simulation display (to represent a set of Machines, for example), all occurrences of the icon on the virtual screen are changed to match.

WITNESS icons may be color or monochrome. When a monochrome icon is used for a machine display, the icon will change color with the status of the machine. A color icon will not change color in this manner. Similarly, Parts represented by a monochrome icon will use the PEN Attribute to determine their color.

#### The Icon Editor

To create or change icons for use in the model, pull down the VIEW menu and click on Icon Editor. This will bring up the Icon Editor. (See Figure 5-9)

Figure 5-9: The Icon Editor

The Load or Save options allow you to load the selected icon into the editor for editing or save the currently loaded icon respectively. The Fill and Change options enable you to flood fill a specific area or to change a color in the icon to another color.

The Import option enables a Windows bitmap to be loaded into the WITNESS icon editor via a file selector screen. There is no restriction to the size of an imported image.

Creating an Icon

- Decide what the icon is to represent; it is sometimes helpful to sketch the required shape.

- Select VIEWICON EDITOR - The Icon Editor dialog appears.

- If you wish to change an existing icon or base the design of the new one on an existing icon, select the icon and load it into the editor.

- Choose the size of the icon using the size selector.

- Clicking in the monochrome check box switches monochrome on.

- Use the mouse to draw the new icon in the drawing area, using the color selector as necessary.

- Choose the icon number to apply to the new icon using the icon number selector.

- Click on Save to store the icon with the selected reference number.

- Click on Close to leave the icon editor.

#### Saving and Loading Icons into Other Models

Icons created in the model will be saved with the model detail when using the FileSave AsModel command.

If you wish to use a set of icons in another WITNESS model, icons may be saved in an Icon Library file using the FileSave As option and selecting “Icons only Files (*.icn)” from the List Files of Type box. This will save all the icons in your model to the specified file. This icon file may then be loaded into another model using the FileOpen option and selecting “Icons only Files (*.icn)” from the List Files of Type box.

#### Using Icons in the Model

The use of Icons may be divided into three broad categories

- Icons used to represent Machines (Activities), Conveyors, and Tracks.

- Icons used to represent moving Elements such as Parts (entities), Labor Resource and Vehicles.

- Icons used purely to enhance the display with graphics such as company logos.

For the physical Elements, the icon may be set in the Display for the Element. For example when setting the Conveyor style for a Conveyor, select Icon instead of DESC.

For Machines or Activities, Vehicles, Conveyors and Buffers or Queues the icon may be changed while the model is running. This can be done using the SET ICON command in an Action statement.

For example, at a machine OP1O, the Action statement

SET ICON OP1O 22

in Actions on Start for a machine will change the icon of OP1O to 22 each time the machine or activity starts to cycle.

A Part (Entity) icon may be changed by accessing its ICON attribute. For example, Actions on Finish for a Machine or Activity could be ICON = 23, to change the Part (Entity) icon to 23.

### The Screen Editor

To improve the display of your model, the Screen Editor provides you with the ability to create and edit graphics directly on the screen rather than via an Element and the icon editor. It may be useful to display project titles, explanatory text, boxes around a group of Elements on the screen to delineate a functional area in a facility, company logos, etc.

Figure 5-10: The Screen Editor

The screen editor is a tool bar that can be moved around the screen. Its format is the same as the Display dialog box for the Elements. The display dialog box consists of two drop-down lists and a number of buttons. The first of the drop-down lists contains operations that can be carried out on the screen. The second drop-down list contains the item or option that you want to display on the screen. The operations that can be carried out are described below.

| Table 5-2: Screen Editor Operations |  |  |
| --- | --- | --- |
| Operation |  | Description |
| Draw | enables you to draw on the screen the option selected |  |
| Update | enables you to update to display of the item selected |  |
| Delete | deletes selected item |  |
| Stretch | stretches selected item |  |

The item or option that appears in the second drop-down list selects what it is you wish to draw on the screen.

| Table 5-3: Drawing Options |  |  |
| --- | --- | --- |
| Options |  | Description |
| Key | Element keys containing status colors for Elements |  |
| Notes | the notes entered on the title screen |  |
| Expression | any specific information or expression |  |
| Rectangle | a box |  |
| Ellipse | a circle/ellipse |  |
| Line | a line |  |
| Icon | icon |  |
| Text | descriptive text |  |

Buttons on the Element Display Bar provide you with a variety of further display features, the purpose of which is summarized below.

| Table 5-4: Editor Display Bar Buttons |  |  |
| --- | --- | --- |
| Button |  | Description |
|  | Draws selected item |  |
|  | Deletes selected item |  |
|  | Enables you to control the display and movement of graphics on layers |  |
|  | Locks all of the display Elements together so they can be moved as a group |  |
|  | Switches background snap grid on and off |  |
|  | Accesses WITNESS help |  |
|  | Exits the screen editor dialog |  |

The screen editor can be accessed from the View menu.

### Specifying the Clock Display

WITNESS displays the current time in a simulation model run in two ways:

| - A digital clock indicating the number of time units that have elapsed since the run began. This appears in the Time Window (ViewWindowsTime). | Figure 5-11: Time Window |
| --- | --- |
| - An analog clock with hours, minutes and seconds indicating the current time in the simulation run (assuming the simulation starts from 12 midnight). This appears in the Clock Window (ViewWindowsClock). | Figure 5-12: Clock Window |

By default, WITNESS assumes that the time units are minutes. To change this, pull down the Model menu and click on 'Clock' and the clock display form appears.

Figure 5-13: The Clock Display Dialog Box

The digital clock may be changed to reflect weeks, days and time by changing the clock display from Regular to User. You can set up to three multiples of your time units and indicate the number of sub units that each multiple contains.

The analog clock assumes that your time units are minutes. If your time units are seconds, the ratio of analog clock minutes to simulation time units needs to be 1:60; e.g., it takes 60 time units (seconds) to run 1 minute on the analog clock. If your time units are hours, the ratio of analog clock minutes to simulation time units needs to be 60:1; e.g. it takes 1 time unit to run 60 minutes on the analog clock.

### Window Control and Window Association

WITNESS provides the user with eight windows for the display of various parts of a model. The display of graphics and Elements in these windows may be modified using the Window Control command. By selecting the Control option from the ViewWindows menu (or by pressing the '*' on the keypad) the following modifications can be made to the active window:

- Name

- Zoom factor

- Pan to an Element

- Background color

- Layers to be included or excluded in the display

- Setting to enable or disable movement of graphics

Figure 5-14: The Window Control Dialog Box

It is also possible to associate any one of the eight windows with an Element. This associated window could be used to display some graphical information about the Element or to provide a different view of the simulation as it runs.

Element and Window Associations

- Click on the Element that you wish to associate with a window. - The name of the Element will appear in the select box on the Standard Action bar.

- Click on the button on the Standard Action Bar - The Set Associate View dialog appears on the screen.

- Select the Window to associate with the Element.

- Click on the OK button. - The window appears on the screen.

- Size, position and center the window to be associated to the element. - Select the OK button for confirmation.

At any time, even while the model is running this Associate View may be displayed on the screen as follows.

Display Associate View

- Click on the Element that you wish to view the associate window. - The name of the Element appears in the select box on the Standard Action Bar.

- Click on the button on the Standard Action bar. - The window containing the Associate View appears on the screen.

- If the current element is contained in a module (later discussed in Chapter 6), the view parent module button  pans the display back to the parent module (if it is displayed).

### Process Views

WITNESS gives you the opportunity to create a simple process flow diagram (also referred to as a process map) of your model using the Process View button on the Standard Action bar. This button creates a new view of the model in a selected window. The view consists of boxes representing each Element joined by lines indicating the direction of flow. All other display layers in the selected window are automatically turned off.

Create a Process View of Your Model

- Click on the Process View button on the Advanced Action Bar. - The Process View dialog box appears on the screen.

- Select text and box color, font, orientation of the process flow diagram to be created, and the window in which to display the Process View.

- If desired, check the Label Rules button to show the Input and Output Rules being used next to the flow lines between the blocks representing the model Elements.

- Click on the OK button

- Resize the Process View window as needed. Rearrange the Element blocks as required to make the process flow diagram most intelligible by minimizing crossing lines, etc.

Figure 5-15: The Process View Dialog Box

- If you need to close the Process View window, do so by double-clicking the close box at the upper right hand corner of the window. DO NOT close it by clicking the Process View button or you will lose any changes you made to the location of blocks in the window. Then, to re-open the Process View window, select the Windows from the View pull-down menu and click on the Process View window to reactivate it.

Figure 5-16: Process View of the Model

### Element Flow and Meteor Trails

#### Element Flow

The Element flow option, as previously mentioned is accessed via the Element Flow option on the View menu. Activating Element Flow will display lines with arrows on the screen representing the actual flow rules used between the physical Elements in the model. An additional option will also display the type of flow Input or Output Rule being used between the Elements such as “Push,” “Pull”, “Sequence”, etc. Selecting this option again turns off the Element Flow view.

Figure 5-17: Element Flow

#### Meteor Trail

The Meteor Trails button on the Execute action bar also displays flow arrows between model Elements, and records the number of parts flowing between each Element during a simulation run.

To produce a Meteor Trail

- Click on the Meteor Trails button on the Execute Action Bar. - You will be asked if you also wish to log the trail to file.

- Run the model in the usual way.

- Stop the model.

- Select the Meteor Trail option from the Reports menu. - The Meteor Trail dialog box appears on the screen.

- Select the text color, font, indicator *, parts to show and the layer on which to display trail.

- Click on the OK button. - The Meteor Trail appears on the screen.

Figure 5-18: Meteor Trails Dialog Box

- The Indicator field gives you the option of displaying the number of parts that have gone down each route or having the line thickness denote the proportion of parts going down each route.

Figure 5-19: Meteor Trails

### Display Layers and CAD Import

#### Display Layers

Nine predefined layers are available in WITNESS for the display of model Elements, background graphics, and text. Additional layers up to 99 layers can be defined. Independent control of the display and movement of model Elements and graphics drawn on a layer is possible via the Layer Control dialog box (See Figure 5-20). There are different ways to access the Layer Control dialog:

- Select ViewLayers pull-down menu.

- From the Screen Editor by clicking on the  button.

- From an Element Display dialog by clicking on the  button.

Figure 5-20: Layers Dialog

Graphics on a layer can be made visible or invisible, movable or immovable, both or neither.

To Draw on a Layer

- Open the Screen Editor or Display dialog for an Element.

- Click on the Draw button.

- Select the desired layer to draw on from the drop-down list located at the lower left corner of the Display dialog. - All subsequent graphics drawn with the Screen Editor or via the Display dialog for an Element will be drawn on this layer until it is changed again.

- Draw graphics, text, or model Elements as desired.

- Close the Screen Editor or Element Display dialog when finished.

A layer may also contain background graphics created with the Screen Editor, or imported into WITNESS from another graphics application such as Microsoft Paint.

#### CAD Drawing Import

CAD drawings may also be imported into the display of a WITNESS model in DXF file format. All common CAD applications will export files in this format. For example, the layout drawing of the facility being modeled could be loaded into WITNESS as a backdrop serving to add realism to the model. A DXF file may be imported into WITNESS in the following manner.

Import DXF Files

- Select Open from the File pull-down menu.

- Select “Drawing Files (*.dxf)” from the List Files of Type box.

- Go to the directory containing the desired DXF file and select it. Click on OK.

- The Detail CAD Import dialog box will open and the Scale tab will be active.

- Choose an Option from the Scale To list (short descriptions of the different scaling options are given later). - Make any desired changes under the Layers Tab. - In this section of the Detail CAD Import dialog, each layer in the original CAD drawing can be imported into (or “mapped”) into individual layers in WITNESS. It is also possible to exclude selected layers from the original CAD drawing so they will not be imported into WITNESS. - Click OK when finished.

The Allow Distortion check box can also be selected to override the WITNESS default scaling of an imported drawing that maintains its original ratio of X and Y scale. The option may be used with any of the available drawing scaling options. When selected with the “Net” or “Window” scaling options, the drawing will be automatically scaled and re-proportioned to completely fill the rectangular boundary area or window respectively.

Figure 5-21: CAD Import Dialog Box

| Table 5-5: Scaling Options |  |
| --- | --- |
| Option | Description |
| Scaling to Model | Will automatically size the layout drawing to fit the extents of the display of any model on the virtual screen. For more information on the WITNESS virtual screen, see separate section below. |
| Scaling to Window | Will automatically size the layout drawing to fit the visible view area in the current window. |
| Scaling to Value | Allows the layout drawing to be manually scaled. Location of the X and Y coordinates of the drawing and the scale can be specified. For example, if a CAD drawing uses default units of feet and inches and a scale factor of 1.0 is specified when importing it, the drawing units will match the WITNESS coordinate system on the virtual screen. In other words, if the cursor is moved a distance of (5) coordinate units in any direction on the screen in WITNESS, the movement will be 5-feet in relation to the CAD drawing. A WITNESS model can be given physical dimensionality in this way. Through the use of the GETXPOSN and GETYPOSN, physical distances between machines can then be calculated for use in determining travel times. The origin point that the upper left corner of the drawing to be imported will coincide with can also be specified by setting the values for the “Base Point: X” and “Base Point: Y”. Note that the X, Y coordinates of the origin point on the WITNESS virtual screen is (1,1). |
| Scaling to Net | Used turns the cursor into a set of crosshairs. The user can then draw a rectangular boundary area of any size or shape on the screen that will cause the drawing to be automatically scaled to fit inside the boundary area. |

Figure 5-22: CAD Import

When the CAD import process is complete, all graphics from the original CAD DXF format file will be imported per the settings made in the Scale and Layers sections of the Detail CAD Import dialog box. Individual graphical entities from the original CAD drawing will be converted to lines, polylines, and text for the most part in WITNESS. See the WITNESS user manuals for more information on how graphical entities are converted in WITNESS.

### The WITNESS Virtual Screen

Because WITNESS models can become large and complex, WITNESS provides a very large 'canvas' on which to display this animation, and a number of windows through which this 'canvas' may be viewed. We refer to this large canvas as the 'virtual screen’; we refer to the image visible on your computer as the 'physical screen'. The size of the virtual screen varies between different computers and display adapters, but it always has an area which is 1024 (e.g. 32 x 32) times the area of the largest window on the physical screen. On a test performed with a popular laptop computer model with a color palette setting of “High Color (16-bit)”, a virtual screen size of 19,998 units in the X-axis by 9,998 units in the Y-axis was achieved.

- Do exercise 2 now.

## Presentation Enhancements

### Using AVI Files

Sometimes you might need to send progress reports to your supervisors or clients who might be out of town and do not have WITNESS in their computer. To enhance these reports it might be useful to send with them an AVI representation of your model to help them understand what you are doing. WITNESS has an integrated AVI recording facility.

The AVI recording option allows you to save an animated sequence from a portion of a model so that you can play it back later (either within a model or outside WITNESS). You can capture any part of the screen independent of whether it is part of WITNESS.

The AVI recording feature is particularly useful for presentations, where you can recreate the original display of a model and present results without having to run WITNESS. For example, you could show a short sequence of a model to a client so that you can confirm your progress before proceeding with further experimentation.

#### Setting-Up AVI Files

- Click on the FileRecord to AVI command.

#### The AVI File Page

The Record to AVI dialog allows you to record a sequence of frames (or a single ‘snapshot’ frame) to an .avi file and play it back again.  The AVI File tag sets the recording and playback speed of your video file. The location and name of the file you will record to can also be determined in this page.

- Verify the location and name of the AVI file. Click on the File button to display the save as file dialog and enter the name of the .avi file that you want to save the recording to. - The default filename is temp.avi.

- Make sure the Capture on Simulation Events button is checked. The capture on simulation event box captures frames only when simulation events occur. If the box is unchecked, recording takes place at the rate specified in the recording FPS field. - Enter a value for Playback fps, or the rate at which the recording will be played back. - The default is two frames per second.

If the append box is checked, you cannot change the settings for Playback fps, Compression, Region Width or Region Height

| Table 5-6: AVI File Page Options |  |
| --- | --- |
| Option | Description |
| Append | Appends a new recording to an existing file. If the box is unchecked, the new recording overwrites the contents of the current .avi file. |
| Recording FPS | Rate at which the recording is made (in terms of frames per second). The default is 2 frames per second. |
| Compression | Displays the compression dialog. The compression button is disabled if you are appending to an existing file. |
| Recompress | Recompresses an existing file with the new settings described on the compression dialog. Recompressing a file overwrites the contents of the original file. |

#### The Regions Page

This page allows you to select a region (or area) of the screen to record. If you check the capture cursor box, anytime the cursor is displayed in the region during the recording process, the cursor will be included in the frames saved in the .avi file.

Figure 5-23: Record to AVI Region Tag

There are three different ways to specify the region of the screen you want to capture in your AVI file.

- Click on the Select Window button and then click in the window that you want to record.

- Click on the Select Region button then on the top left-hand corner of the area of the screen that you want to capture. Drag the mouse to the bottom right-hand corner of the region and release the mouse button to complete the selection of the region.

- By specifying an absolute position on the screen. You must enter the co-ordinates of the region to be captured. For example, to capture a region of the screen measuring 800 x 600 pixels, starting from the top left-hand corner, enter X = 0; Y = 0; Width = 800; and Height = 600. - If you have selected the append mode on the AVI File page, you cannot modify the height and width co-ordinates (although you can modify the X and Y co-ordinates).

| Table 5-7: AVI Co-ordinates Region Selection Method |  |
| --- | --- |
| Option | Description |
| X | Enters the X co-ordinate of the left-hand edge of the region to be captured (in pixels). The default is 0. |
| Y | Enters the Y co-ordinates of the top edge of the region to be captured in pixels. The default is 0. |
| Width | Enters the width of the region to be captured (in pixels). The default is 320 pixels. |
| Height | Enters the height of the region to be captured (in pixels). The default is 240 pixels. |

#### The Duration Page

This page allows you to set the length of the recording. You have three options:

- The Unlimited mode has no restriction on the length of the recording.  The recording must be stopped manually.

- The Fixed Recording Time mode	lasts for the amount of time specified in the duration field.

- The Fixed Playback Time mode lasts for as long as it is needed to save enough frames that would result in the amount of time specified in the duration field when it is played back.

If you select either a fixed recording time or a fixed playback time, the duration of the recording applies to each individual recording session rather than to the whole file.

- Select the appropriate Duration Mode.

- Select the appropriate Duration (in seconds). The default is 30.

#### The Shortcut Page

This page allows you to set shortcut keys for starting and stopping the recording as well as for recording a single frame (snapshot). It also allows you to specify how the Record to AVI dialog is displayed when a sequence is being recorded to an AVI file. Once you have set up a shortcut key, it takes precedence over all other application’s shortcut keys.

- If you have not set a start/stop recording shortcut key, the only way that you can start or stop the recording is via the record to AVI dialog. Therefore, if you select hidden, the record to AVI dialog will still be visible during the recording process.

Figure 5-24: Record to AVI Shortcut Keys Tag

| Table 5-8: AVI Co-ordinates Region Selection Method |  |
| --- | --- |
| Option | Description |
| Start/Stop Recording Key | Displays the current start/stop recording shortcut key. The default key is CTRL + R. |
| Snapshot Recording Key | Displays the current snapshot shortcut key. The default is CRTL + T. |
| Appearance when Recording | The appearance of the record to AVI dialog could be Visible, Minimize or Hidden and it is only affected when you are recording. It does not apply when you are recording a snapshot. |
| Height | Enters the height of the region to be captured (in pixels). The default is 240 pixels. |

#### Recording and Playing AVI Files

- To record an AVI file, make sure you are comfortable with all the recording options.

- Click on the red record button on the Record to AVI window. - The Record to AVI window is either hidden, minimized or visible depending on the setting you have selected on the Shortcut page.

- Run the model the way you would like it to appear in your AVI file. - The model will stop after a fixed recording time, when the shortcut keys are pressed or when you open the Record to AVI window and press the stop button .

- To view the AVI file you created, click on the play button on the Record to AVI window. - The file will immediately open and begin running. - The button allows you to modify the view and speed of the playback from the AVI file you have recorded.

### Controlling Windows Display During Model Execution

Functions, which are available in WITNESS, allow you to control how windows and views are displayed during model execution. These commands are:

- OPENWIN and CLOSEWIN

- SETVIEW

- SETTITLE

The OPENWIN and CLOSEWIN are WITNESS void functions that allow you to open and close windows during a simulation model run. They may be used in any Actions statement, including the Actions dialog in the body of a WITNESS function.

For example, let us say you have a very large model and you need to display a wide area of it most of the time during a typical model run. However, you want to display a close-up view of a particular operation in the model whenever it occurs. The rest of the time, you just want to display the large-area view of the model. Let us say the operation in question is a machine called LATHE. You can then accomplish the necessary window control as follows:

- Set up a separate window (Window 5) to display just the machine. Set window size and position, zoom factor, title, and window background color as needed.

- Set Actions on Start for machine: OPENWIN (5).

- Set Actions on Finish for machine: CLOSEWIN (5).

- Set Initialize Actions for model: CLOSEWIN (5). This will ensure window 5 is initially closed at the start of a model run.

These functions can also be used in conditional logic statements. For example, you could change the logic in the Actions statements for the above example to open window 5 only if the machine is busy at least 25% of the time:

Set Actions on Start for machine:

IF SUTIL(LATHE,2)>=25

OPENWIN(5)

ENDIF

The SETVIEW is another void function in WITNESS that allows the user to control the position of the upper-left corner and the zoom factor of a specified view window during model execution. For example, SETVIEW(1, 100, 100, 5) would set the position of the upper-left corner of the window in the virtual screen to X=100, Y=100 (remember that origin point of the virtual screen in WITNESS is 1,1 located in the upper left corner).

The SETTITLE is a void function that allows the title of a window to be set or changed during a simulation run. For example, let us take the previous example with the LATHE machine activity that we wanted to highlight. Instead of opening and closing window 5, let’s keep it open and use the window title to display a message indicating the current LATHE activity.

- Change Actions on Start for LATHE to:

IF SUTIL(LATHE,2)>=25

SETTITLE(5,”Lathe is now busy at least 25% of the time”)

ELSE

SETTITLE(5,”Lathe is less than 25% busy at this time”)

ENDIF

- Change Initialize Actions to:

SETTITLE(5,”Lathe is presently inactive”)

### Use of Expression in Element Display and Screen Editor

The use of an Expression in the display of a model Element allows the user to evaluate and display the value of one or more WITNESS functions or an expression containing them without the need to create and display a new variable to do so. Expressions can also be created that contain variables, functions, or both. Note also that the Expression drawn with the display of an Element does not have to pertain to that Element.

Figure 5-25: Display Expression Dialog

An Expression can also be drawn in the background using the Screen Editor without need to attach it to a particular model Element.

### Utilizing Layer Order of Display

Layers in WITNESS appear on screen in a particular order. The Simulation Layer, or Layer 0, is the default layer set when WITNESS is started up. If graphical items are drawn on top of each other in different layers on the screen, they will display as follows:

- Graphics or model Elements drawn on the Simulation Layer will always appear on top of items drawn on those drawn on Layer 1.

- Graphics or model Elements drawn on Layer 1 will appear on top of those drawn on Layer 2, and so on

- Simulation objects drawn on layers other than the simulation layer obeys the same logic except during a simulation run. In this case, any objects that have a state are displayed as if they had been drawn on the simulation layer. This prevents simulation objects (those that can change with the state of an Element such as icons) from ever being obscured by background graphics.

This order of display can best be utilized when drawing graphics that overlap each other on different layers where certain objects need to appear behind others. For a more detailed explanation of layer functionality, refer to the WITNESS online Help System.

### Simulating the Movement of Parts by Labor

Parts (entities) usually require a means of conveyance between operations in a system. Typical means are by conveyor, unmanned (automated) vehicle, or by being carried by a person or in a cart. There are a number of methods for incorporating part transportation time in a WITNESS model, but providing good animation of this activity can be somewhat more difficult. If WITNESS Tracks and Vehicles are used, good animation is already built into the display of these Elements, but it may not be feasible to use them in all cases.

An alternative method involves simply changing the icon of the part just before it leaves an Element and then changing back to its original form as it enters the Element at its destination. For example, assume we have two machines and parts are moved from one to the other by an operator using a cart. Here are the steps:

- Using the Icon Editor, create a new icon showing a person pushing a cart containing the part to be transported between the machines in question. Alternatively, photo-editing software such as Microsoft Paint can be used to create the desired image that can then be imported into WITNESS via the Icon Editor. Save as a new icon and note the icon number.

- In Actions on Finish for first machine enter: ICON = {new icon #}

- In Actions on Start for machine at destination enter: ICON = {original icon #}

### Improving Model Animation by Controlling Model Run Speed

The real power of a graphical simulation tool like WITNESS is the ability to bring life to a simulated system through animation. An effective presentation of a simulation model requires a smooth, fluid animation of parts flowing through machines, buffers, conveyors, and other Elements in the model. It is also essential that Element state changes such as machine breakdowns and other key events in the model do not occur so quickly that they will be missed. Users will notice that some models will appear to run faster on the screen than others. Model run speed is largely determined by the following factors:

- Computer hardware characteristics such as type of processor and amount of RAM memory

- Type of WITNESS installation being used (stand-alone or network, and where WITNESS is actually installed)

- Type of computer operating system and version being used (Windows 95, NT, 3.x, etc.)

- Number of events occurring in the model per unit of time during a model run

- Model size (in bytes) and complexity (amount of conditional logic, external file I/O are big factors)

- Amount of on-screen graphics being displayed while the model is running

WITNESS provides a number of ways to fine tune the speed of model animation as needed:

- Model can be run normally.

- Model can be “walked” at various speeds over a continuous range using the slide bar control.

- The model can also be “stepped” in a continuous fashion by holding down the spacebar on the keyboard. The Interact box can be deactivated so as not to interfere with the on-screen animation if desired simply by closing it while model step mode is active.

- If the model cannot be run or walked at the desired speed, the ModelOptionsRun dialog, allows the defaults for these modes to be adjusted to any desired speed. The Run Time Increment is perhaps the easiest option to use and the setting will be saved with the model. The default value is 1.0. Values less than 1.0 will reduce model run speed while those greater than 1.0 will increase it.

- Do exercises 3 and 4 now.

## Exercises

### Exercise 1

#### Graphical Presentation of Results

##### Monitoring Trends

For an audit, the cost of work in progress must be monitored on an hourly basis. The finance department has provided the following unit costs:

| Part | Unit Cost of WIP ($) |
| --- | --- |
| A | 25.00 |
| B | 30.00 |
| C | 35.00 |

Display this information onscreen using a Timeseries.

##### Monitoring Critical Values

Concern has been shown as to the length of breakdowns on OP10. Display the length of breakdowns using a Histogram.

##### Monitoring Machine Utilization’s

Using a Pie Chart, display the percentage utilization in all states for OP10 onscreen. Save this model as final.mod as we will be returning to it later in the course.

- Use exercise 9 (or PACKING.MOD) from Chapter 3 and save this model as FINAL.MOD. We will be returning to this model later in the course.

### Exercise 2

#### Enhancing the Display of the Model

Your model is reaching the stage where it will be presented to a number of managers in the factory. Therefore, the appearance of the model needs to be enhanced as much as possible to hold the attention of a generally non-technical audience.

To achieve this, do the following:

- Add a model title to the screen.

- Place each of the statistical graphs in separate windows. Rename and change the background color of the windows.

- Add a Process View to Window 8 and adjust its display to reduce the number of crossing lines.

- Create or amend an icon and use it as a company logo. Place it in the upper left corner of the model screen.

- Get OP10 and OP20 to change the icons used to represent them when idle, busy, setting up or being repaired.

- Divide the screen into separate areas by drawing boxes around the storage areas in the model.

- Add a title to each of the storage and production areas in the model. Associate all new graphics with the Elements they represent where applicable.

- Save this model as DISPLAY.MOD.

### Exercise 3

#### Preparing the Model for Presentation to Management - Part 1

Further enhancements must be made to prepare the model for presentation to upper management. Make the following changes to the model:

- When the OP20 machine breaks down, open a separate view window with a magnified view of the machine. Change the title of this window to reflect the activity being represented there.

- When an OP20 machine breaks down, display a message next to the particular machine that broke down indicating this.

- Though part dispatch time does not need to be included in this simulation model, it is still important to show labor moving parts between buffers B1, B2, and B3 and OP10, and from OP10 to buffers B1, B2, and B3. Change the icon of the part as it leaves an Element to one illustrating a person pushing a cart carrying that particular part. Restore the appearance of the part to its original form when it reaches its destination.

### Exercise 4

#### Preparing the Model for Presentation to Management - Part 2

The project team has reviewed the enhancements you just made to the display of the model and have suggested the following changes:

- Only part of the model Elements can be seen in the default view window at any one time. Use the appropriate window control command in Action statements to toggle the magnification of the main model view window between 1X and 2X every 500 minutes.

- Management is no longer interested in emphasizing the display of breakdowns at OP20. Remove the pertinent Action statements and required parts of the display used to open a separate window when the machine broke down, then close it when the machine is repaired.

- Add the appropriate actions to cause the title of window 1 to display the total number of Boxes shipped during a simulation run in the following format:

Number of Boxes Shipped = n,

| Where | n is the current total shipped. |
| --- | --- |

## Case Study

### Integrating a Facility Layout Drawing into Your Model

Production capacity of a machine shop in the facility must be increased to meet new demands. You have been tasked with determining the resource requirements to meet the new demands using simulation. You will be working with managers and engineers from the Facilities Resource Planning department whom work with CAD layout drawings of the operation. The manager of this department has requested that the facility layout for the machine shop be incorporated in the simulation model to be built.

Create a new model as follows:

- Open the startup model titled “STARTLAY.MOD.”

- Load the facility layout drawing provided by the Facilities Resource Planning department titled “LAYOUT.DXF.” They have indicated that travel distances will need to be evaluated so the drawing should be loaded in its original scale. In addition, they need to maintain the layer integrity of the original CAD layout drawing in WITNESS, so map each layer in the CAD drawing to a separate WITNESS layer when importing the drawing.

- After the CAD drawing is loaded, rename the WITNESS layers to match those of the original CAD drawing. Make these layers immovable but still visible.

- Some of the equipment in the layout appears in white and some in cyan. The white equipment is existing, whereas the cyan equipment was added by the Facilities Resource Planning department to allow for the proposed expansion of the machine shop operation. They have added the maximum number of each type of machine and related storage area that will physically fit within the existing walls of the shop. The machine shop cannot be made any larger without affecting adjacent areas in the facility, and management will not allow this.

Build a WITNESS model of the existing machine shop operation using the CAD facility layout as a backdrop based on the following data. Display all WITNESS Elements for this model on the Simulation Layer. Remember, existing machines are white in the layout. Define the following machine quantities.

| Table 5-9: Define Machine Quantities |  |  |
| --- | --- | --- |
| Operation Description | Process Time(min) | Additional Display or Detail Information |
| Bar Stock is delivered to the Incoming Material buffer in batches of (5) every one to two hours. | N/A | When the buffer is full, display a message near the buffer indicating this. There is no need to display any other state of the buffer. |
| Cut Bar Stock into Rods: One of two operators takes a single bar and cuts it into (3) Rods in the bandsaw. The operator places the finished rods in a separate buffer for the Drill Press operation. | 2.0 | Project team wants to track rods, not Bar Stock. Therefore, change part type to Rod upon completion of the operation. Also, change icon to realistically depict part (shorter than original Bar Stock). When the buffer for the Drill Press Operation is full, display a message near the buffer indicating this. |
| Drill and Tap Center Hole: One of two operators takes the Rods from the buffer one at a time and drills and taps a hole through its center using the Drill Press. Finished parts are placed in a separate buffer for the Grinder. | 3.5 | Change icon of the finished Rod to reflect the hole drilled through it. When the buffer for the Grinder is full, display a message near the buffer indicating this. |
| Deburr Rods: An operator takes Rods one at a time and deburrs them on a Grinder. The deburred Rods are then placed in a separate buffer for the Fixture Installation operation. | 2.75 | No need to change icon of part for this operation. When the buffer for the Fixture Installation Operation is full, display a message near the buffer indicating this. |
| Fixture Installation: Two Rods are taken from the parts buffer, one of five assembly fixtures is taken from a separate storage rack, and the Rods are installed in the fixture. The two Rods are then welded together. This process requires both of the operators. When finished, the fixture holding the two Rods is placed in a buffer for the Lathe. | 15.0 | Due to an oversight, the Facilities department neglected to include a dedicated rack for storage of the five assembly fixtures. Create a new rack by copying graphics from an existing one in the layout. Ensure the new rack is created on the Equipment layer. |
| Turn Rods to Correct Diameter: An operator takes an assembly fixture with the welded Rod assembly and installs it in the Lathe. The Rods are then turned down to a diameter of 1.231 plus or minus 0.001 inches. The completed Rods remain in the fixture and are placed in a buffer for the Milling Operation. | 30.0 |  |
| Mill Key Slot in welded Rod assembly: An operator takes a fixture with welded Rod assembly from the buffer and installs it on the mill bed. A key slot is milled the entire length the Rod assembly. The finished welded Rod assembly is then removed from the fixture and placed in a buffer for Inspection. The fixture is returned to its original storage rack for re-use. | 20.0 | Change the icon of the fixture back to its appearance when not in use. Change the icon of the welded Rod assembly to reflect its new condition. |
| Inspection: The inspector takes each welded Rod assembly from the buffer and inspects them for defects. The current yield is 90%; 90% of the parts inspected are shipped while the remaining 10% are sent to a dedicated workstation for rework. | 10.0 | If an assembly is rejected, change its appearance as it leaves the inspection operation. The part should retain this appearance to indicate it has been rejected before. |
| Rework Station: If an assembly is rejected, a rework operation will be performed to repair it. The assemblies are then returned to the Inspection buffer for re-inspection. | 5.0 – 15.0 | The rework operation was added after the Facilities department gave you the layout of the machine shop. Add a workbench and storage rack as background graphics near the aisle opposite Inspection to represent the Rework operation. |

When the base model (model of the existing operation) is complete, make the following changes:

- Increase the quantity of Bandsaws, mills, and lathes in the model to the maximum number shown on the layout. Place the new machines on top of the layout graphics to which they correspond.

- Rename Window 1 to “Machine Shop – View of All Operations,” adjusting pan and zoom in window to view entire machine shop layout.

- Create a timeseries of the current WIP of Rods in Window 2. Set the background color of the timeseries to grey and the axis color to black. Rename the window to “Total Rods Work-in-Process.” Change the background color of the window to blue. Give the graph a 3-D look by adding a black background offset and behind the timeseries itself.

- Create a pie chart showing utilization of the milling machines in Window 3. Rename this window to “Mill Utilization.” Change the background color to gray. To emphasize the statistic for percentage of time busy, add large bold text with the current value of this statistic to the display near the pie chart.

- Provide an easy way for a user to activate the display of the timeseries and pie graphs you created. The user should be able to click on an icon or text on the screen to activate the graphs at their discretion.

Hint: Consider adding descriptive text to the display of the timeseries and pie charts you created. Position the text in a readily accessible location in the main view window and use the Associate View function.

- Display the current number of Rod Assemblies shipped in large lettering font in a separate view window. Have the window open 200 minutes into the simulation run then close 200 minutes later. Have the window open for 200 minutes every 1000 minutes during the simulation run. Hint: consider using “dummy” parts to trigger the event