Continuous Simulation

Objectives

- Summarize the characteristics and uses of continuous Elements.

- Use Fluid routing - input and Output Rules.

- Create control logic using actions and commands.

- The overall aim of this chapter is to help you learn to create WITNESS simulation models that include continuous Elements.

- Various instructional techniques - hands on operation, conventional lecturing, and group discussion - are used to deliver the information and reinforce the class material.

## Continuous Elements

### Fluids

Fluids represent substances or objects that flow continuously through a system. Examples could be liquids, powders, grains, or gases. If discrete (individual) objects are created in sufficiently large-volumes, high production rates, or if they move in a fast-moving environment, they may also be represented as fluids. For example, in the modeling of some production processes, it might be very useful to represent "solid" materials, such as long rolls of steel, wires, chains, and ropes, as continuous Elements. In addition, when modeling a production process involving the mixing of various ingredients or compounds, fluids can be used to represent both the constituents and the final products. WITNESS “creates” fluids and introduces them into the simulation at the time and place dictated by the model. The Fluid Element could be thought of as the continuous counterpart to the discrete Part Element.

Fluids can change type or color as they progress through the model and can be combined to create new Fluids.

Fluids can actively arrive in the model at a given flow rate (e.g. they are “active”) or they may be flowed-in on demand by other Elements (e.g. they are “passive”).

To model fluids in a system, decide which free-flowing products need to be represented. Define each one, choose their Display colors, and then fill in their Details.

| Table 9-1: Fluid Display Characteristics |  |
| --- | --- |
| Option | Description |
| Color | Which color will be used to represent the Fluid |

| Table 9-2: Fluid Detail Characteristics |  |
| --- | --- |
| Option | Description |
| Name | Up to 8 characters allowed. Character choices. Include numbers, letters and underscores. |
| Arrival Mode | Passive or Active arrivals. |
| If “Active Arrival:” |  |
| Maximum Volume | Maximum amount of fluid that will actively flow. Into the model during the run of the model. |
| Shift | Name of shift that governs when fluid can arrive. |
| Arrival Rate | Rate at which Fluid arrives. |
| Rules | Determines where the Fluid enters simulation. |

Figure 9-1: Fluid Detail Characteristics

### Pipes

Pipes are used to transport Fluids between the various Elements of a model. Fluid normally flows through a Pipe in the direction that the Pipe is drawn on the screen, although it may flow in reverse through the Pipe if a negative flow rate is specified. The Pipe Element could be thought of as the continuous counterpart to the discrete Conveyor Element.

The volume capacity and the flow rate of the Pipe determine its size and speed of flowing.

There is the option to have Fluid flow through a Pipe and empty out when there is no input to the Pipe, or to simply flow Fluid when input is present. The ability to flow with no input is useful for modeling gravity feed Pipes or fast moving goods. The option to flow only when input is present is available.

Breakdowns, Cleaning, and Shifts can also be modeled in Pipes.

| Table 9-3: Pipe Display Characteristics |  |
| --- | --- |
| Option | Description |
| Name | Shows the actual name of the Pipe on the screen. |
| Pipe | To draw the Pipe, move the cursor to the point at which you wish the Pipe to start then press the left mouse button. Move the cursor to the point at which you want the Pipe to change direction and click again using the left mouse button, move the cursor to the next point and click again. Repeat this procedure until the Pipe is complete and then click on the right mouse button to end the Pipe. |
| Flow Rate | Rate at which the fluid is to flow, in Units/Time. |
| Labor | Display Labor used at Pipe. |

Figure 9-2: Pipe Display Options

| Table 9-4: Pipe Detail Characteristics |  |
| --- | --- |
| Option | Description |
| Name | Up to 8 characters allowed. Character choices. Include numbers, letters and underscores. |
| Quantity | Number of Pipes with exactly the same display and detail information. |
| Priority | Priority for labor allocation. |
| Capacity | The volume of fluid that can be moving through a Pipe. Initial Fluid Type By selecting a fluid, the Pipe will start the model run filled with the selected fluid type. |
| Fill From | Where the Pipe fills from. |
| Empty To | Where the Pipe empties to. |
| Flow Rate | The speed at which Fluid flows through the Pipe |
| Labor Rule | Labor for Fluid to flow (if required). |
| Flow with No Input | Whether or not Fluid can flow through the Pipe with no input. Select this option if no input is required for the fluid to flow through the pipe (gravity). |
| Fluid Change | Sets Fluid type or color change on input or output. |

Figure 9-3: Pipe Detail Characteristics

### Processors

Processors are physical Elements that operate on Fluids. After spending time in a Processor, the Fluid will be changed in some way. The Processor Element could be thought of as the continuous counterpart to the discrete Machine Element.

- Fill with Fluid(s) up to a specified level.

- Process the Fluid(s) for a specified period of time.

- Empty all of the Fluid(s).

The information needed to properly detail the Processor will be the speed at which the Processor fills and to what level, and from where the Fluid flows in. Then the processing time (cycle time) needs to be entered. Finally the speed and destination to which Fluid flows out of the Processor needs to be entered. The user may also need to detail any Fluid type or color changes.

Examples of Processors are:

- A blending vat, combining a number of fluids to create a mixture.

- A retort, containing a liquid that is heated up and held at a temperature for a specific period.

Breakdowns, Cleaning, Shifts, and Labor requirements can also be modeled.

| Table 9-5: Processor Display Characteristics |  |
| --- | --- |
| Option | Description |
| Name | Shows the actual name of the Processor on the Screen. |
| Processor | Draws a hollow rectangle within which the Fluid can be seen. |
| Icon |  |
| Contents | Text indicating the current volume. |
| Mix | Text indicating proportions of Fluids within the Processor. |
| Labor |  |

Figure 9-4: Processors and Tanks Display Options

| Table 9-6: Processor Detail Characteristics |  |
| --- | --- |
| Option | Description |
| Name | Up to 8 characters allowed. Character choices include numbers, letters and underscores. |
| Quantity | Number of Processors with exactly the same display and detail information. |
| Priority | Priority for labor allocation. |
| Capacity | The volume of Fluid that can be contained by the Processor. |
| Minimum Process Level | The level at which the Processor will start to Process. |
| Initial Fluid Type | If the Processor is to be Initialized containing Fluid, the type of Fluid is set here. |
| Initial Volume | The volume of Fluid that the Processor starts with. |
| Fill From | Where the Processor fills from. |
| Empty To | Where the Processor empties to |
| Labor Rule | Any Labor required for Fluid to flow in or out, or for processing. |
| Actions on Start | Actions statements that are carried out on start of processing. |
| Actions on Finish | Actions statements that are carried out on finish of processing. |
| Fluid Change | Sets any Fluid type or color change on input or output. You could also have actions on Fluid Change. |

Figure 9-5: Processor Detail Characteristics

### Tanks

Tanks are used to store Fluids. They do not carry out any process operations on the Fluid, although Fluid is free to flow in and out of a Tank simultaneously. The Tank Element could be thought of as the continuous counterpart to the discrete Buffer Element.

The capacity of the tank and the rate at which Fluid flows in and out need to be specified. Cleaning and Labor requirements can also be optionally modeled.

| Table 9-7: Tank Display Characteristics |  |
| --- | --- |
| Option | Description |
| Name | Shows the actual name of the Tank on the screen. |
| Tank | Draws a hollow rectangular outline within which Fluid can be seen. |
| Icon |  |
| Contents | Text indicating the current volume. |
| Mix | Text indicating proportions of Fluid in the tank |
| Labor |  |

| Table 9-8: Tank Detail Characteristics |  |
| --- | --- |
| Option | Description |
| Name | Up to 8 characters allowed. Character choices include numbers, letters and underscores. |
| Quantity | Number of Tanks with exactly the same display and detail information. |
| Priority | Priority for labor allocation. |
| Capacity | The volume of Fluid that can be contained by the Processor. |
| Initial Fluid Type | If the Tank is to be Initialized containing Fluid, that type is set here. |
| Initial Volume | The volume of Fluid that the Tank starts with. |
| Fill From | Where the Tank fills from. |
| Empty To | Where the Tank empties to. |
| Labor Rule | Optional Labor requirement for Fluid to flow in and out. |
| Fluid Change | Sets any Fluid type or color change on input or output. Actions on Fluid Change are possible. |

Figure 9-6: Tank Detail Characteristics

## Controlling Fluid Flows

### Basic Continuous Flow Rules

As with the discrete modeling Elements, there is a basic set of rules that are used for input and output portions of the continuous modeling Elements. The default flow rule for Fluid movement is “WAIT” - the same as in modeling discrete part movements.

### Flow Rule

Used to transfer Fluids directly to or from continuous Elements at a specified flow rate, without using Pipes. The format of the rule is:

#### Usage

FLOW to/from Element1 RATE (flow rate), Element2 RATE (flow rate)

#### Example

FLOW to HOLDTANK RATE (10.0)

| causes | the Element with this rule to flow its contents to the Element HOLDTANK at a rate of 10.0 units of volume per time unit. |
| --- | --- |

#### Example

FLOW from TANKA RATE (10.0), TANKB RATE (5.0)

| causes | the Element with this rule to flow from the Element TANKA at a rate of 10.0 AND from the Element TANKB at a rate of 5.0 units of volume per time unit. |
| --- | --- |

### Connect Rule

Used to connect a Pipe to any other continuous Element. The flow rate is determined by the Pipe. The format of the rule is:

#### Usage

CONNECT with Element1,Element2,

#### Example

CONNECT with FEEDPIPE

| causes | the Element with this rule to flow using a connection to the FEEDPIPE at a rate specified by the FEEDPIPE detail. |
| --- | --- |

### Least/Most Rules

The LEAST and MOST rules flow a volume of Fluid to or from an Element with the least/most Fluid or the least/most free space. The format of the rules are:

#### Usage

LEAST <FREE/FLUID> to/from <Elements>VOLUME <volume> RATE <rate>

MOST <FREE/FLUID> to/from <Elements>VOLUME <volume> RATE <rate>

#### Example

LEAST FREE from TANK1,TANK2,TANKVOLUME 50.0 RATE 10.0

| causes | the Element with this rule to take 50.0 units of Fluid from the Tank that has the least free capacity at a rate of 10.0. |
| --- | --- |

The VOLUME part of the rule is optional. If it is not specified, the rule will only be re-evaluated when the flow stops, either because the input Element is empty or the output Element is full.

### If Rule

Both the FLOW and CONNECT rules may be combined with the IF rule. For example:

#### Example

IF N = 1

FLOW to BOILER RATE (10.0)

ELSE

CONNECT with WPIPE

ENDIF

Fluid may flow in from the WORLD or may leave the model by flowing to SHIP or WASTE. If any Fluid actively arrives but cannot get into the model (e.g. an arrival tank is full) it will be rejected and never enter the model.

- Do exercise 1 now.

### Move Fluid Command

The MOVE action command enables the movement of Fluid instantaneously from one Element to another (e.g., instantly dump Fluid from one location to another). This is useful when the time to move Fluids between Elements is relatively small and so has a negligible effect on the simulation result. Using the MOVE command, instead of the flow rules, will improve the model's run speed.

The format of the command is:

#### Usage

MOVE VOLUME <volume> from <Element> to <Element>

Fluids may be moved to and from Tanks and Processors and may be moved from the WORLD or to SHIP and WASTE.

#### Example

MOVE VOLUME 30.0 from TANK to BLENDER

MOVE VOLUME 100.0 from WATER out of WORLD to TANK

The specified volume must be available in the source Element and there must be enough space in the destination Element, otherwise an error will occur when the model runs. WITNESS functions may be useful to determine the contents and free space in a continuous Element. This makes sure, ahead of time, that the source Element has enough fluid to supply and that the destination Element has enough space to receive the Fluid being moved.

There is no limitation on the states of an Element during which Fluid may be moved. For example, Fluid may be moved to a Processor while it is processing, and the process time will be unaffected.

Any warning levels on the Elements concerned will be obeyed after the Fluid has been moved. The actions will be executed in the sequence in which they would have been executed had the Fluid flowed between the Elements (Refer to Section 9.6.3 for more detail on warning levels).

### The Recipe Rule

The RECIPE rule flows specified volumes of one or more Fluids to and from Elements. This is useful where, for example, a Processor works with a specific mixture of Fluids. The RECIPE rule is roughly analogous to the discrete MATCH and SEQUENCE rules, from the standpoint that they enable you to obtain a combination of materials "in hand" before processing.

The mixture of Fluids may be obtained in parallel or in series, where each Fluid will be obtained in a specified order. Fluids delivered in parallel will flow at the same time, each at their specified rate, until the volume of each fluid is transferred. Fluids transferred in series will be transferred at their own flow rate and volume one at a time, based on the order in which they are specified in the RECIPE rule.

The format of the rule is:

#### Usage

RECIPE <SERIAL/PARALLEL>to/from <Element> VOLUME<volume> RATE <rate>, to/from <Element> VOLUME <volume> RATE <rate>, ...

#### Example

RECIPE PARALLEL from TANK1 VOLUME 10.0 RATE 15.0,TANK2 VOLUME 90.0 RATE 50.0

This statement causes the Element with this rule to obtain 10.0 units of Fluid from TANK1 at a rate of 15.0 and at the same time obtain 90.0 units of Fluid from TANK2 at a rate of 50.0.

The RECIPE rule is only evaluated at the start of a new recipe. Therefore, once the rule has started it will not be re-evaluated until the recipe is complete; only the flow rate can vary. RECIPE may also be used within an IF rule.

## Element Details

### Initial Contents and Fluid Changes Initial Contents

The initial contents of any Processor, Pipe, or Tank may be specified on their detail dialog. The type of Fluid and the volume may be specified for Processors and Tanks. If you need a Pipe to be initialized with Fluid in it, only the Fluid type is required since the Pipe is assumed either full or empty when initialized. To initialize a Pipe as partially full, it would be possible to FLOW material from WORLD using a "phantom" Processor, with a zero cycle time.

Alternatively, the MOVE action command could be used in the Initialize Actions by moving Fluid out of the WORLD to the desired Processor or Tank. Note that the MOVE action command will not move Fluid to or from a Pipe.

This would be analogous to partially filling a buffer with parts as soon as the simulation starts.

#### Fluid Changes

Fluids may be changed into different Fluids and/or be given different colors on either entering or leaving any continuous Element.

### Labor and Shift Patterns

#### Labor

Labor requirements may be specified for Processors, Pipes, and Tanks to perform maintenance, operating, and tasks.

For a Processor, Labor may be used for:

- Cleaning

- Processing

- Filling

- Emptying

- Repairing

For a Pipe, Labor may be used for:

- Cleaning

- Repairing

- Flowing Fluid through a Pipe

For a Tank, Labor may be used for:

- Cleaning

- Flowing Fluid through a Pipe

The Labor type and quantity used for each job type must be specified in the Element details. Priorities in using Labor and Labor pre-emption can both be modeled in the same way as for discrete Elements.

#### Shifts

All the continuous flow modeling Elements may be detailed to work according to a shift pattern using specified Shift Elements. The Shift identifies a series of working and non-working periods to be followed in a repeating cycle.

- Do exercise 2 now.

## Stoppages

### Breakdowns

Processors and Pipes can be made to break down in WITNESS based on various conditions and at specified intervals (see Figure 9.7). Specifying the breakdown is performed using parameters similar to those used for discrete Elements such as machines.

| Table 9-9: Breakdown Modes |  |
| --- | --- |
| Mode | Description |
| Available Time | Breakdown occurrence is independent of whether the Element is busy or idle. A breakdown will occur when a specified time interval has elapsed. |
| Fluid In Time | Breakdown occurrence is dependent on the total time interval during which a Fluid is present in the Processor since the previous breakdown event. |
| Process Time | Breakdown occurrence is dependent on the total accumulated processing time since the previous breakdown event. |
| Busy Time | Breakdown occurrence is dependent on the total accumulated busy time (e.g. Fluid passing through) a Pipe since the previous breakdown event. |
| Number of Processes | Breakdown occurrence is dependent on the number of processes that the Processor has completed since the previous breakdown event. |
| Value Change | Breakdown occurrence is dependent on when a specified expression changes value. |

In all cases, the user will be prompted to enter a repair time and to specify whether or any Fluid is to be wasted as a result of the breakdown.

Figure 9-7: Breakdown Details

### Cleaning

Processors, Pipes and Tanks may all be stopped and cleaned by specifying a cleaning mode and a cleaning time in the detail dialog for these Elements. The mode of cleaning can be based on various conditions or time intervals.

| Table 9-10: Cleaning Modes |  |
| --- | --- |
| Modes | Description |
| Empty | Clean the Processor or Tank whenever it becomes empty. |
| Fluid In Change | Clean the Element whenever the Fluid entering it changes. |
| Value Change | Clean the Element whenever a specified expression changes value. |
| Available Time | Clean the Element whenever a specified time has elapsed. |
| Volume In | Clean the Pipe whenever a specified volume of Fluid has passed through it. |

In all cases, a cleaning time must be entered.

Figure 9-8: Cleaning Details

#### Processors and Tanks

When a cleaning event is triggered for a Processor or Tank, the following sequence of events occurs:

- Any Fluid flowing into a Tank or Processor is stopped.

- The current content of the Processor or Tank is wasted immediately.

- The Processor or Tank is cleaned for the specified time interval.

#### Pipes

Pipes can be cleaned two different ways: Clean and Purge. The sequence of events once a cleaning event is triggered is the following:

- Any Fluid currently flowing into a pipe is stopped.

- The current contents of the Pipe are allowed to flow out normally until the Pipe is empty.

- The Pipe is cleaned for the specified time interval.

If the Pipe is Purged, a specified volume of Fluid must pass through the Pipe to fulfil the purging requirements. The purge Fluid comes from the Pipes current Input Rule and is wasted on output.

#### Actions

Cleaning of continuous Elements can also be triggered manually at any time by using the following commands in an Action statement for an Element:

#### Usage

CLEAN Element, clean time

PURGE Pipe, purge volume

If a clean is triggered for an Element that is already in the process of being cleaned, the Actions statement to trigger the second cleaning event will be ignored.

- Do exercise 3 now.

## Further Element Details

### Actions

Actions for a Processor can be executed when it:

- Undergoes a fluid change.

- Starts a cleaning cycle.

- Finishes a cleaning cycle.

- Starts a processing cycle.

- Finishes a processing cycle.

- Breaks down.

- Returns to normal operation after being repaired.

Actions for a Pipe can be executed when it:

- Finishes a cleaning cycle.

- Breaks down.

- Returns to normal operation after being repaired.

Actions for a Tank can be executed when it:

- Undergoes a fluid change.

- Starts a cleaning cycle.

- Finishes a cleaning cycle.

### Built-In Functions

WITNESS provides a comprehensive set of built-in functions to access values indicating the current status of continuous modeling Elements. These values can be manipulated and used in reports, in Rules to control fluid flow through a model, or in Actions to control the value of variables used in the model. Some of the most useful ones are:

| Table 9-11: Built-In Functions |  |
| --- | --- |
| Function | Description |
| CONCENT | Percentage of the specified Fluid in the specified Element. |
| FTYPE | Fluid type in the specified Element. |
| ISTATE | Current state of the specified Element |
| RATEIN | Flow rate of Fluid entering the specified Element. |
| VCONTS | Current volume of the Fluid in the specified Element. |
| VSHIP | Volume of the specified Fluid that has been shipped. |

For a complete list of all available functions and a detailed description of their usage, please see the WITNESS Quick Reference Manual.

### Warning Levels

Figure 9-9: Warning Levels Form

Warning Levels is a feature of Processors and Tanks that allow Actions to be specified when an Element has filled or emptied to a specified volume level. Warning Levels allow controls to be built into a model to represent valves being opened or closed, deciding which Fluid to pump in next, and to perform quality control checks. For example, an Action could be set to occur when a rising volume of fluid in an Element reaches a warning level to slow down a variable flow rate into a Tank when it is almost full.

Rising and Falling Warning Levels are set up in the same way, but will only be detected when the level is changing in the correct direction.

To create a Warning Level, enter a fluid volume value into the “Level” field on the Warning Level detail form and then click on the “Add” button. To enter the Action statements for a particular level, click on that level in the list, then click on the “Edit Actions” button (see Figure 9.9).

- Do exercise 4 now.

## Continuous Links to Discrete Elements

### Continuous Flow Links with Discrete Elements

WITNESS allows you to create links between continuous and discrete modeling Elements by selecting one of two available options on the detail form for Parts and Machines. These options would normally be used to simulate filling and emptying operations. The discrete links can be used for many different applications such as:

- delivery tankers

- bottling plants

- packing lines

- solid raw materials that are pulped

#### Parts

To either fill or empty a Part, an option on the attributes’ form allows the user to set the Part to “Contains Fluids” (see Figure 9.10). If a Part is able to contain Fluid, two system attributes become applicable to the Part:

| Table 9-12: Continuous Attributes for Parts |  |
| --- | --- |
| Attributes | Description |
| FLUID | Indicates the type of Fluid in the Part. |
| CONTENTS | Indicates the volume of Fluid in the Part. |

Figure 9-10: Detail Part Form – Fluid Option

#### Machines

Machines have filling and emptying rules that allow Parts to be filled with Fluid, or allow Parts to be emptied of Fluid.

Filling and emptying rules are available on either single or batch type Machines and may take place before or after the Machines’ cycle. In addition, the volume of Fluid to be filled or emptied is specified along with a flow rule to obtain or direct the Fluid (see Figure 9.11).

Figure 9-11: Detail Machines – Fluid Options

#### Move Command

Fluid can also be moved into and out of Parts in any discrete Element by using the “MOVE” action command.

#### Example

MOVE VOLUME 10.0 from TANK to CONVEYOR at 3

| Would | fill the Part at position 3 on CONVEYOR with 10.0 units of Fluid from TANK. |
| --- | --- |

#### Example

MOVE VOLUME 5.0 from MAC1 at 1 to TANK

| Would | empty 5.0 units of Fluid from the first part in MAC1 into TANK. |
| --- | --- |

However, a Fluid may NOT be moved to or from a Part if:

- It is already in the process of filling or emptying at a machine, or

- It contains a mixture of Fluids, then its contents can only be moved to another Part

- Do exercise 5 and 6 now.

## Modeling Issues

### Monitor Interval

A Monitor Interval may be required if certain functions are used with Input or Output Rules. The monitor interval would determine how frequently expressions are checked during a simulation run (e.g. the contents of a Tank).

If none of the expressions used in the Rules of the model need to be monitored, then the Monitor Interval remains undefined.

Whenever you use functions such as VCONTS in a rule, WITNESS will prompt you for a Monitor Interval. The value would then be stored in the ModelOptionsStatistics form. A Monitor Interval of 10.0 means that expressions in rules will only be checked every 10.0 time units.

Choosing a small Monitor Interval (such as 0.1) will be very accurate but will slow the model down noticeably. A larger Monitor Interval will speed up the model, though the model will be less accurate.

To avoid slowing down a model, put any monitored functions in Actions rather than Rules. This means that they are not being continually checked and the Monitor Interval can be left undefined. The following example illustrates how monitoring can be avoided using this method:

A Tank STORE feeds another Tank INTANK.

The Output Rule of STORE is:

IF VCONTS(STORE) < 500.0

FLOW to INTANK RATE (10.0)

ELSE

FLOW to INTANK RATE (100.0)

ENDIF

Which causes the flow rate to slow if the level of Fluid in STORE gets below 500.0 units.

A better way to model it would be:

- Define a Variable called FLOWRATE - Put Warning Levels on STORE as follows:

Falling level at 500: FLOWRATE = 10.0

Rising level at 500: FLOWRATE = 100.0

- Change the Output Rule of STORE to:

FLOW to INTANK RATE (FLOWRATE)

### Mixture Interval

Whenever you use mixtures of different Fluids in Tanks, WITNESS needs to calculate the proportion of each Fluid in the mixture at regular time intervals. This Mixture Interval works in a similar way to the Monitor Interval.

A Mixture Interval will only be necessary if there is both a flow into and out of a Tank at the same time, resulting in a continuously changing mix of Fluids. WITNESS will prompt you for a Mixture Interval where necessary.

### Flow Allocation

WITNESS allocates flows between model Elements using a complex algorithm. It is not important to know how the algorithm works, but its three basic principles should be kept in mind:

- Flows will not be “shared” between two destinations unless there is enough for both. For example an Output Rule of:

FLOW to TANKA RATE (10.0), TANKB RATE (3.0)

| Where | the total flow available is 3.0, will direct all 3.0 units to TANKA and none to TANKB. |
| --- | --- |

- WITNESS will only reallocate flows if the reallocation results in a greater total flow in the system. In the above example, if TANKA became full, flow would go to TANKB. If later on TANKA has some flow out and therefore has spare capacity, it will not get any more Fluid unless TANKB becomes full or the total flow available becomes greater than 3.0. - In general, the Output Rule shown above would give preference to TANKA. However, the algorithm for allocated flows based on Input Rules is not as straightforward. Therefore, it could not guarantee that TANKA would be given preference.

It is recommended that flows be controlled by variables and actions whenever a preference must be given to particular Elements.

## Examples

### Representing Rolls of Material

This case study shows the modeling of a continuous roll of material. In this case, the materials are tape and padding, which are being fused together. The roll of material might also be steel or another metal. The model listing of this case study could be found at the end of this section and the actual filename of this model is Case_9_1.mod.

A STOCKER machine obtains a discrete part called TAPEROLL (see Figure 9.12). In “Actions on Create,” the roll is assigned a random length in feet, from a uniform distribution between 800 and 1000. The part actually enters the model as a discrete part containing between 800 and 1000 units of fluid. The STOCKER machine empties the fluid and moves the fluid to TAPETANK, which is a processor. Both the flow rate on the Output Rule, and the processing time on the processor combine to represent the time to replenish a roll of material. The material then flows from the processor to a pipe. This flow represents a certain rate in feet per minute, which is our simulation time unit. Another discrete part called PADROLL also arrives, sends fluid to an identical processor and flows to another pipe.

Figure 9-12: Case 9.1 Layout (Time 362)

These two pipes, one containing padding, and the other containing tape, then flow into a third pipe, which represents the process of fusing of two materials together. Because of the way WITNESS displays the mixture of two or more materials, we can actually see the two materials flowing side-by-side through the pipe. By not displaying the outline of the pipe, it actually appears as if the material is moving from one station to another in a continuous strip. The flow rate on this pipe is twice the flow rate of the first two pipes, so that it can accommodate the flow from both of the feeder pipes.

Finally, warning levels on the processors can be used to change an icon on the screen to give the illusion that a roll of material is becoming larger as the processor fills, or smaller as the processor empties. A warning level is also used to set a STOCKOUT flag, to indicate that another roll of material needs to enter the model. By putting an IF condition on the Output Rules of the two processors, we assure that the material only flows to the pipe if both processors contain material and are able to flow.

#### Model Listing

Define

FLUID: TAPE;

FLUID: PADDING;

PIPE : PIPE1,1;

PIPE : PIPE2,1;

PIPE : PIPE3,1;

PROCESSOR : TAPETANK,1;

PROCESSOR : PADTANK,1;

PROCESSOR : FUSER,1;

PART: TAPEROLL,Variable attributes;

PART: PADROLL,Variable attributes;

MACHINE: RESTOCK,1,Single,0,0;

LABOR: STOCKER,3;

VARIABLE: STOCKOUT,1,2,Integer;

Detail

TAPE

NAME OF FLUID : TAPE;

OUTPUT RULE: Wait;

PADDING

NAME OF FLUID : PADDING;

OUTPUT RULE: Wait;

PIPE1

NAME OF PIPE : PIPE1;

QUANTITY: 1;

CAPACITY : 100.000000,Undefined;

FLOW RATE : 2.0;

INPUT RULE: Wait;

OUTPUT RULE: Wait;

PIPE2

NAME OF PIPE : PIPE2;

QUANTITY: 1;

CAPACITY : 100.000000,Undefined;

FLOW RATE : 2.0;

INPUT RULE: Wait;

OUTPUT RULE: Wait;

PIPE3

NAME OF PIPE : PIPE3;

QUANTITY: 1;

CAPACITY : 100.000000,Undefined;

FLOW RATE : 4.0;

INPUT RULE: FLOW from PIPE2 RATE (2.0),

PIPE1 RATE (2.0);

OUTPUT RULE: FLOW to FUSER RATE (4.0);

TAPETANK

NAME OF PROCESSOR : TAPETANK;

QUANTITY: 1;

CAPACITY : 1000.000000,800.0,Undefined,0.000000;

PROCESS TIME : 25.0;

LABOR:

Fill: STOCKER;

END

LABOR:

Process: STOCKER;

END

ACTIONS, Process Start

Add

SET ICON of TAPETANK to 102

End Actions

ACTIONS, Down level

Value : 0.000000;

Add

SET ICON of TAPETANK to 105

STOCKOUT (1) = 0

End Actions

ACTIONS, Down level

Value : 330.000000;

Add

SET ICON of TAPETANK to 104

End Actions

ACTIONS, Down level

Value : 660.000000;

Add

SET ICON of TAPETANK to 103

End Actions

ACTIONS, Up level

Value : 0.000000;

Add

STOCKOUT (1) = 1

End Actions

INPUT RULE: Wait;

OUTPUT RULE: IF ISTATE (PADTANK) = 2

FLOW to PIPE1 RATE (2.0)

ENDIF;

PADTANK

NAME OF PROCESSOR : PADTANK;

QUANTITY: 1;

CAPACITY : 1000.000000,800.0,Undefined,0.000000;

PROCESS TIME : 25.0;

LABOR:

Fill: IF LABORAT (TAPETANK,1,1) <> STOCKER

STOCKER

ENDIF;

END

LABOR:

Process: STOCKER;

END

ACTIONS, Process Start

Add

SET ICON of PADTANK to 106

End Actions

ACTIONS, Down level

Value : 0.000000;

Add

SET ICON of PADTANK to 105

STOCKOUT (2) = 0

End Actions

ACTIONS, Down level

Value : 330.000000;

Add

SET ICON of PADTANK to 108

End Actions

ACTIONS, Down level

Value : 660.000000;

Add

SET ICON of PADTANK to 107

End Actions

ACTIONS, Up level

Value : 0.000000;

Add

STOCKOUT (2) = 1

End Actions

INPUT RULE: Wait;

OUTPUT RULE: IF ISTATE (TAPETANK) = 2

FLOW to PIPE2 RATE (2.0)

ENDIF;

FUSER

NAME OF PROCESSOR : FUSER;

QUANTITY: 1;

CAPACITY : 2000.000000,1600.0,Undefined,0.000000;

PROCESS TIME : 15.0;

LABOR:

Process: STOCKER;

Pre-empt level: None;

END

ACTIONS, Process End

Add

MOVE volume VCONTS (FUSER) from FUSER to SHIP

SET ICON of FUSER to 105

End Actions

ACTIONS, Up level

Value : 300.000000;

Add

SET ICON of FUSER to 109

End Actions

ACTIONS, Up level

Value : 700.000000;

Add

SET ICON of FUSER to 110

End Actions

ACTIONS, Up level

Value : 1200.000000;

Add

SET ICON of FUSER to 111

End Actions

ACTIONS, Up level

Value : 1600.000000;

Add

SET ICON of FUSER to 112

End Actions

ACTIONS, Up level

Value : 2000.000000;

Add

SET ICON of FUSER to 113

End Actions

INPUT RULE: Wait;

OUTPUT RULE: Wait;

TAPEROLL

NAME OF PART: TAPEROLL;

TYPE: Variable attributes;

ACTIONS, Create

Add

FLUID = TAPE

CONTENTS = IUNIFORM (800,1000,385)

End Actions

OUTPUT RULE: Wait;

CONTAINS FLUIDS: Yes;

PADROLL

NAME OF PART: PADROLL;

TYPE: Variable attributes;

ACTIONS, Create

Add

FLUID = PADDING

CONTENTS = IUNIFORM (800,1000,629)

End Actions

OUTPUT RULE: Wait;

CONTAINS FLUIDS: Yes;

RESTOCK

NAME OF MACHINE: RESTOCK;

QUANTITY: 1;

TYPE: Single;

DISCRETE LINKS :

Empty: IF FLUID = TAPE

FLOW to TAPETANK RATE (50.0)

ELSEIF FLUID = PADDING

FLOW to PADTANK RATE (50.0)

ELSE

Wait

ENDIF;

Volume: CONTENTS;

Relation to cycle: Before;

END

CYCLE TIME: 0.0;

INPUT RULE: IF STOCKOUT (1) = 0

PULL from TAPEROLL out of WORLD

ELSEIF STOCKOUT (2) = 0

PULL from PADROLL out of WORLD

ELSE

Wait

ENDIF;

OUTPUT RULE: PUSH to SCRAP;

STOCKER

NAME OF LABOR: STOCKER;

QUANTITY no shift: 3;

STOCKOUT

NAME OF VARIABLE: STOCKOUT;

QUANTITY: 2;

INITIALIZE ACTIONS

SET ICON of TAPETANK to 102

SET ICON of PADTANK to 106

SET ICON of FUSER to 105

### Cola Manufacturing

A major cola beverage manufacturing company needed to model the following key parts of their process.

- Cola manufacture involving the pre-mixing and dissolving of several powder ingredients with different liquids. The liquids are stored in 3 different tanks, and the powders are stored in barrels. The liquids are flowed into the Mixer, and the barrels containing powder are emptied into the Mixer at the specified rate and quantity to make a particular type of soda.

- The processed cola is then piped to a filling machine where cans or bottles are filled.

- The cans or bottles move on conveyors before and after they are filled.  Occasionally, the filling machine fills tanker trucks of the cola for delivery as needed at various restaurant chains where the solution is emptied into bottles.

Figure 9-13: Cola Plant Layout

#### Project Objective

The bottling companies must decide whether or not to fill bottles or tanker truckloads to meet their customers’ product delivery requirements.  Also, they must able to make customer-requested changes in the middle of a run whereby the product is loaded into tanker trucks instead of bottles or vice versa.  In this latter case, they must still maintain statistics gathered on the quantity of product shipped before the change whether by truckload or by the bottle in order to fill their next order.

#### Conclusion/Findings

The bottling companies successfully built a model that was used to accomplish the project objectives.  The model was used thereafter on an ongoing basis to make bottle vs. truckload product delivery decisions.

## Exercises

### Exercise 1

#### Basic Brewery Line

A brewing company makes a range of beers, for sale to grocery stores and restaurants.

Their brewing process begins as follows:

Ale flows into a heated stainless steel agitator kettle at 10.0 gallons per minute. When this kettle is full (at 200 gallons), the Ale is agitated for 7 minutes to achieve a smooth consistency such that fermentation can take place.

The Ale then flows out of this Agitator kettle at 20.0 gallons per minute, down a Pipe to an Ale holding Tank. The Pipe can handle a volume of 50.0 gallons and a maximum flow rate of 25.0 gallons per minute. The Ale holding Tank releases the fermented beer to be canned at a rate of 3 gallons per minute (e.g., to the shipping department), but is capable of storing up to 300 gallons of beer if necessary.

Figure 9-14: Exercise 1 Layout

- Save this model as BREW_EX1.MOD

### Exercise 2

#### Making Stout Beer

In addition to a lighter-colored Ale, a darker Stout is also brewed. A dark, concentrated wort (a slurry of hops, barley, malt, yeast, and water created as part of the brewing process) flows at 1 gallon per minute into a storage tank (capacity 50 gallons) before a blending kettle. The storage tank releases this dark wort to the Blender kettle at a rate of 1 gallon per minute. The relatively slow flow rate of the wort is due to its heavy consistency. Although the wort is a mixture of barley, hops, malt, yeast, and water, for this exercise simply consider it a single wort Fluid type (not a mixture).

The Agitator kettle actually diverts 100 gallons of Ale to the existing Pipe and 100 gallons to the Blender kettle simultaneously, both at a rate of 20 gallons per minute. When the Blender kettle is full (at 100 gallons), the Ale, and concentrated wort are mixed for 10 minutes to produce Stout beer. This newly brewed Stout then flows to another Pipe (capacity 50 gallons) at 15 gallons per minute. This Pipe feeds its own Stout holding Tank (capacity 300 gallons). The fermented Stout beer is then released to be canned at a rate of 3 gallons per minute (e.g., to the shipping department). The wort holding tank stores 30 gallons at the start of the model.

Figure 9-15: Exercise 2 Layout

- Save this model as BREW_EX2.MOD

### Exercise 3

#### No Process is Perfect

The Blender kettle has to be cleaned for 5 minutes whenever it becomes empty, in order to remove any remaining residue from the brewing process.

Being an older piece of equipment, the Ale Agitator kettle's mechanism jams roughly every hour of operation. It is necessary to empty the Agitator kettle before repairing it. The repair takes 10 minutes. Since a jam may occur before the mixing of the current batch of Ale is finished, it may not be totally mixed. Therefore, the unfinished Ale is not suitable for bottling and can be considered a ruined batch whenever the agitator breaks down.

The Pipe between the Ale Agitator kettle and the Ale Holding Tank also needs to be cleaned whenever the Agitator breaks down, since it will likely contain some of the ruined batch of Ale. This cleaning takes 15 minutes.

- Save this model as BREW_EX3.MOD

### Exercise 4

#### Adding Quality of Our Product

If the Wort Storage Tank gets as full as 45 gallons, the valve into the Tank shuts off. Once the level has dropped to 25 gallons, the flow is then restored. In addition, the Wort should only flow out of the Wort Holding Tank to the Blender kettle while the Ale is also flowing to the Blender kettle from the Ale Agitator kettle. This maintains the proper ingredient proportions for brewing the Stout.

After blending, the first 10 gallons of Stout beer is actually siphoned off to a sampling vat. The remainder of the Stout still flows via the Pipe to the Stout Holding Tank. The brewmaster samples the Stout from the sampling vat. The sampling procedure takes 5 minutes, after which the Fluid is wasted, in negligible time. Over time, the brewmaster also becomes wasted.

Figure 9-16: Exercise 4 Layout

### Exercise 5

#### Adding Discrete Elements to Our Continuous Process

Empty beer kegs are delivered in batches of 15 to the warehouse every hour. The warehouse has capacity to store 100 kegs.

A carousel machine takes 1 empty keg at a time from the warehouse and fills it with 15 gallons of beer. It takes 30 seconds to fill the keg, which is filled with whichever variety of beer (Ale or Stout) that is most in stock (e.g. residing the two Holding Tanks). Once full, it takes another 30 seconds to pressurize and seal the keg before it is shipped. Keep track of the number of kegs containing Ale and Stout.

The dark Wort concentrate is actually delivered to the Wort storage tank in canisters of 25 gallons each; 1 canister every 4 hours. The first delivery is after 2 hours. The canisters are emptied into the storage tank, stopping as before if the tank reaches the level of 45 gallons and resuming when the level drops to 25 gallons.

- Save this model as BREWLINE.MOD.

### Exercise 6

#### Analyzing Our Beer Making Process

Predict what will happen to the model in the previous exercise, if you omit the optional VOLUME option on the MOST FLUID rule used to fill kegs on the CAROUSEL machine.

## Case Studies

### Poor’s Brewery

Your boss, P. Ilsner, has informed you that upper-management at the Brewery likes the WITNESS simulation model you have created of the brewing line. You are to be commended for your work. Because of their analyzing your model during a presentation you gave to them, management generally agrees with your assessment that having the Carousel machine pull empty kegs from the Warehouse may be an inefficient way of doing things. An outcome of the resulting discussion is that the Carousel machine should probably be taking empty kegs directly from the machine that is creating them, rather than putting them into a warehouse to be pulled out later and filled by the Carousel. Doing it this way might eliminate the cost of storing empty kegs in a warehouse. Although they see your point, they are not 100% convinced (particularly since one of the managers, who is not an Industrial Engineer, came up with the warehouse idea years ago). However since management has already seen the value of simulation in helping them analyze the brewing line you just did, they have given you the go-ahead to investigate the situation further - via an expanded simulation model.

As such, another Industrial Engineer at the Brewery has been assigned to use WITNESS to create a discrete simulation model of the portion of the plant that provides clean, empty beer kegs. The model the other engineer created is called BASIC.MOD. This model was created in Chapter 3:Exercise 8, Incorporating User and WITNESS Distributions Within A Mode. We will treat BASIC.MOD as the model the other engineer wrote to simulate the creation of the empty beer kegs. You will be using that model to investigate the keg-warehousing situation further.

As a point of reference and review, you should have already created BASIC.MOD earlier in the discrete simulation portion of this class. In other words, YOU are the other engineer. If you don't have BASIC.MOD (or the model you created after Exercise 8 from the Discrete portion of the class as described above), your instructor has been provided with a completed BASIC.MOD which you could use with this exercise if needed. The model BASIC.MOD will look something like Figure 9.17.

Figure 9-17: BASIC.MOD from discrete Exercise 8

Essentially, your existing BASIC.MOD takes parts A, B, and C, and assembles them into a single new part at machine OP20. This single part travels down a conveyor and is shipped. It is at the junction of machine OP20 and conveyor CON1 that we will concentrate most of our effort for this exercise. We will assume at this point that the conveyor CON1 in BASIC.MOD has been simply sending kegs to the warehouse although the Output Rule on CON1 is "PUSH to SHIP.”

Using the sub-model save feature in WITNESS, you will save your BREWLINE.MOD model, created in Exercise 5 above, in its entirety as a sub-model called BREWLINE.SUB.

Your task is to include BREWLINE.SUB into BASIC.MOD, and change the logic and visual display of this new "combined" model such that the Carousel filling machine now resides between OP20 (the quantity 2 machine) and CON1 (the conveyor). This resulting model will be named BREWERY.MOD (e.g., no longer called BASIC.MOD). Watch the placement of things when importing the BREWLINE.SUB sub-model into BASIC.MOD so that you do not have any overlap in the display of the newly combined Elements. Depending on your computer's screen resolution and size, you may have to use the scroll bars in WITNESS to create new blank areas for the sub model, since both models will likely not all fit on the screen. Sometimes it helps to reduce the size of the on-screen zoom to 1/2. It doesn't matter where things are placed on the screen, as long as they are not cluttered, and as long as the Carousel is placed between OP20 and CON1 (you will have to do some shuffling of things on the screen).

It may be possible that the tank level calibration numbers will not appear when the BREWLINE.SUB sub-model is imported into BASIC.MOD. This can be fixed by updating the "Vessel" display for each tank, and clicking on the "Calibration" check box for each one, then clicking the "Update" button. The Maximum, Minimum, and Interval values will be correct, it is just that they may not display when imported into BASIC.MOD.

We will assume that the existing OP20 machine assembles clean kegs from existing parts A, B, and C, which could possibly be the aluminum keg itself, a stopper, and a tamper-proof sealing cap - although for this exercise it does not matter what these three parts are as long as we know OP20 creates kegs. With your inclusion of BREWLINE.SUB, kegs will be pulled from OP20 by the Carousel filling machine and filled as usual. By contrast, the original BASIC.MOD had parts simply going from machine OP20 directly to conveyor CON1.

The kegs created at OP20 are assumed to be ready for filling with Ale or Stout depending on what beer is most available from their respective holding tanks, exactly the same way the Carousel machine currently fills kegs pulled from the Warehouse in Exercise 5. When the kegs are filled, there should be a way of identifying which type of beer each keg contains (Ale or Stout).

The Carousel machine will then place the kegs on the existing conveyor CON1 to be shipped as usual. As the filled kegs leave the conveyor, you must provide some way of identifying, on the model's display, how many kegs of each beer type (Ale or Stout) have been created.

Your ultimate goal is to show management that the warehouse could be considered obsolete from the standpoint of storing empty kegs, and that it could be put to better use storing other things or converted to other uses. The point is that you can test different ideas and scenarios using simulation software, without actually modifying the brewery itself until it is necessary, based on what you derive from your simulation results. Perhaps part of the unused warehouse should be converted to a new, plush office for you, the Brewery's new Vice President of Manufacturing Simulation.

- Save this model as BREWERY.MOD.

### Brewery Line

#### Reading in a Production Schedule from an External File

Create a new model from scratch to start this case study.

The production manager of a brewery that makes two different types of beer, “Light” and “Dark” wants to model a portion of brewing process at her facility. The model needs to include a mixing vat where four different raw ingredients are mixed together, and two tanks connected to the vat by piping.

The amount of each of the four ingredients varies depending on the beer to be made. The time to mix the ingredients also varies depending on the type of beer to be made. The mixed solution is then piped to one of two holding tanks.

Each of the four ingredients flows into the mixing vat in sequence at the rate of 5 gallons per minute. Assume that the ingredients are always available. The capacity of the mixer is 100 gallons. Each pipe can hold 25 gallons of solution and the maximum flow rate through the pipes is 10 gallons per minute. Each tank has a 300-gallon capacity. Solution is then removed from the tanks at the rate of 1 gallon per minute and transported to another area in the facility.

The production manager needs to read a production schedule containing the following types of information about each production lot from an external file:

| Table 9-13: Beer Production Information |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Ingredients |  |  |  |  |
| Type of Beer to Make | Pipe to Flow Processed Solution | Volume of Water | Volume of Barley | Volume of Yeast | Volume of Hops | Process Time (minutes) |
| Dark | Pipe1 | 20 | 30 | 10 | 25 | 25 |
| Light | Pipe2 | 50 | 10 | 20 | 15 | 17 |
| Dark | Pipe1 | 20 | 30 | 10 | 25 | 25 |
| Dark | Pipe1 | 20 | 30 | 10 | 25 | 25 |
| Light | Pipe2 | 50 | 10 | 20 | 15 | 17 |
| Light | Pipe2 | 50 | 10 | 20 | 15 | 17 |
| Dark | Pipe1 | 20 | 30 | 10 | 25 | 25 |
| Dark | Pipe1 | 20 | 30 | 10 | 25 | 25 |
| Dark | Pipe1 | 20 | 30 | 10 | 25 | 25 |

- Save this model as PRODSCHD.MOD.

#### Packaging the Beer for Shipment to Another Site for Further Processing

Based on the previous model, Company management has decided to perform the rest of the beer-making process at an existing plant site located 20 miles away instead of at this site for financial reasons.

Consequently, the beer “solution” will now be pumped from each of the storage tanks into separate 55-gallon drums at a pumping station. The pump is rated at 5 gallon per minute and will always pump from the fullest tank. When the drum is full, another mechanism at the pumping station installs a lid on the drum. This operation takes 30 seconds. The drums are then placed on a 10-position moving conveyor belt with a speed of 5 parts per minute where they are sent to the shipping dock for transport to the other plant site.

The manager also wants to be able to distinguish between drums containing “light” and “dark” beer on the conveyor belt as the model runs.