# Appendix E – Tips and Tricks

When using simulation software, modeling techniques are not always straightforward. There are many times when the user must be aware of different techniques to use at various points in a model. This is why Lanner Group, Inc. has come up with a variety of tips and tricks for modeling in WITNESS. Each tip or trick has been developed to make the modeling process easier for you. Within this appendix, nine different tips and tricks will be explained, some with examples. Keep in mind there may be numerous ways to go about accomplishing the same task when modeling.

## Random Selection

See Model Tnt1.mod

Randomly select between two identical machines with a 50/50 chance of selecting either one. If the machine selected is busy, select the other one (if available). Both machines are utilized equally over time.

#### Define

Part:			Part001
Buffer:			Buff001
Machine:		Mach001, Dummy

#### Display

Do not display the Dummy machine

#### Detail

Part001
Interarrival Time: 1
Output Rule:

Push to Buff001

Mach001
Quantity: 2
Cycle Time: Uniform(1,2,1)
Push to Ship

Dummy
Input Rule:

Pull from Buff001

Cycle Time: 0
Output Rule:

If Random(2) < 0.5

Push to Mach001(1), Mach001(2)

Else

Push to Mach001(2), Mach001(1)

Endif

## Using Tables for Cycle Times

See Model Tnt2.mod

Parts have different cycle times on machines as shown below.

| Table 15-1: Cycle Times |  |  |
| --- | --- | --- |
| Part | Mach001 | Mach002 |
| Part001 | 9.247 | 9.739 |
| Part002 | 8.728 | 8.685 |

Rather than using if statements each time a part enters a machine, use a variable and an attribute to reference the cycle time.

#### Define

Part:				Part001, Part002
Machine:			Mach001, Mach002
Buffer:				Buff001
Variable:			cyctime
Attribute:			partnum

#### Display

Select different icons or colors to differentiate parts.

#### Detail

Part001
Actions on Create: partnum = 1
Interarrival Time: 40
Lot Size: 2
Push to Buff001

Part002
Actions on Create: partnum = 2
Interarrival Time: 60
First Arrival at: 10
Lot Size = 3
Push to Buff001

Mach001
Pull from Buff001
Cycle Time: cyctime(partnum, 1)
Push to Buff002

Mach002
Pull from Buff002
Cycle Time: cyctime(partnum, 2)
Push to Ship

Cyctime
Quantity: 2, 2

#### Initialize Actions

cyctime(1,1) = 9.247
cyctime(1,2) = 9.739
cyctime(2,1) = 8.728
cyctime(2,2) = 8.685

## Special Preventive Maintenance Case

See model Pm2.mod and Downschd.txt

The following is a method for modeling a predetermined preventative and fixed maintenance schedule for a work area with several machines of the same type. This is defined in WITNESS as a machine Element with a quantity > 1. For example, you may have 5 milling machines of the same type on which you want to perform preventative maintenance. The idea is to perform preventative maintenance individually, on a weekly basis, so they will not all be out of service during a busy production period.

In WITNESS, the standard method for modeling a fixed preventative maintenance schedule for a machine involves adding a breakdown activity to the detail of the machine, setting the Breakdown Mode to Available Time, and entering values for Time Between Failures and Repair Time. Unfortunately, if you have a quantity greater than one for this machine, ALL of the machines will breakdown at the same time.

The solution offered here is to use Forced Breakdown and Repair functions, which are new in WITNESS 8.0. This will force each of the machines of the same type to breakdown at a particular system time, then to repair at a later time.

System time values, when each machine is scheduled to start and finish its preventative maintenance period, are read into the variable SCHEDULE from the read file DOWNSCHD in Model/Initialize Actions. The Actual data file is DOWNSCHD.TXT and is shown in the table below. This schedule is repeated every 36 hours. Two dummy machines and a dummy part are used to check the variable SCHEDULE every 2 hours.

| Table 15-2: System Time Values |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Oven |  |  |  |  |  |  |  |  |
| Time | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| Down | 2 | 6 | 10 | 14 | 18 | 22 | 26 | 30 | 34 |
| Up | 4 | 8 | 12 | 16 | 20 | 24 | 28 | 32 | 36 |

#### Define

Part:				A
					Dumpart
Machine:			Oven

Dummy
					Dummy2
Variable:			BrkFlag, integer
					Schedule, real, Qty: 9,2
					X, integer
					Y, integer
					begincyc, real
File:				DownSchd, Read

#### Detail

Dumpart
First Arrival at: 0
Max Arrivals: 1
Lot Size: 1
Push to Dummy

Dummy
Cycle Time: 2
Output Rule

Push to Dummy2

Actions on Start
The following coding is and example for Actions on Start.

! Initiate cleaning cycle for Oven #1

! begincyc is a variable that marks when the current 36-hour cycle began

! BrkFlag is a variable used simply to store the value returned by the functions

!   BREAKDWN and REPAIR, which is required

IF TIME - begincyc >= Schedule (1,1) AND TIME - begincyc < Schedule (1,2)

BrkFlag = BREAKDWN (Oven(1))

PRINT "Oven 1 cleaning began at ";TIME

! Complete cleaning cycle for Oven #1

ELSEIF TIME - begincyc >= Schedule (1,2) AND TIME - begincyc < Schedule (2,1)

BrkFlag = REPAIR (Oven(1))

PRINT "Oven 1 cleaning ended at ";TIME

! Initiate cleaning cycle for Oven #2

ELSEIF TIME - begincyc >= Schedule (2,1) AND TIME - begincyc < Schedule (2,2)

BrkFlag = BREAKDWN (Oven(2))

PRINT "Oven 2 cleaning began at ";TIME

! Complete cleaning cycle for Oven #2

ELSEIF TIME - begincyc >= Schedule (2,2) AND TIME - begincyc < Schedule (3,1)

BrkFlag = REPAIR (Oven(2))

PRINT "Oven 2 cleaning ended at ";TIME

! Initiate cleaning cycle for Oven #3

ELSEIF TIME - begincyc >= Schedule (3,1) AND TIME - begincyc < Schedule (3,2)

BrkFlag = BREAKDWN (Oven(3))

PRINT "Oven 3 cleaning began at ";TIME

! Complete cleaning cycle for Oven #3

ELSEIF TIME - begincyc >= Schedule (3,2) AND TIME - begincyc < Schedule (4,1)

BrkFlag = REPAIR (Oven(3))

PRINT "Oven 3 cleaning ended at ";TIME

! Initiate cleaning cycle for Oven #4

ELSEIF TIME - begincyc >= Schedule (4,1) AND TIME - begincyc < Schedule (4,2)

BrkFlag = BREAKDWN (Oven(4))

PRINT "Oven 4 cleaning began at ";TIME

! Complete cleaning cycle for Oven #4

ELSEIF TIME - begincyc >= Schedule (4,2) AND TIME - begincyc < Schedule (5,1)

BrkFlag = REPAIR (Oven(4))

PRINT "Oven 4 cleaning ended at ";TIME

! Initiate cleaning cycle for Oven #5

ELSEIF TIME - begincyc >= Schedule (5,1) AND TIME - begincyc < Schedule (5,2)

BrkFlag = BREAKDWN (Oven(5))

PRINT "Oven 5 cleaning began at ";TIME

! Complete cleaning cycle for Oven #5

ELSEIF TIME - begincyc >= Schedule (5,2) AND TIME - begincyc < Schedule (6,1)

BrkFlag = REPAIR (Oven(5))

PRINT "Oven 5 cleaning ended at ";TIME

! Initiate cleaning cycle for Oven #6

ELSEIF TIME - begincyc >= Schedule (6,1) AND TIME - begincyc < Schedule (6,2)

BrkFlag = BREAKDWN (Oven(6))

PRINT "Oven 6 cleaning began at ";TIME

! Complete cleaning cycle for Oven #6

ELSEIF TIME - begincyc >= Schedule (6,2) AND TIME - begincyc < Schedule (7,1)

BrkFlag = REPAIR (Oven(6))

PRINT "Oven 6 cleaning ended at ";TIME

! Initiate cleaning cycle for Oven #7

ELSEIF TIME - begincyc >= Schedule (7,1) AND TIME - begincyc < Schedule (7,2)

BrkFlag = BREAKDWN (Oven(7))

PRINT "Oven 7 cleaning began at ";TIME

! Complete cleaning cycle for Oven #7

ELSEIF TIME - begincyc >= Schedule (7,2) AND TIME - begincyc < Schedule (8,1)

BrkFlag = REPAIR (Oven(7))

PRINT "Oven 7 cleaning ended at ";TIME

! Initiate cleaning cycle for Oven #8

ELSEIF TIME - begincyc >= Schedule (8,1) AND TIME - begincyc < Schedule (8,2)

BrkFlag = BREAKDWN (Oven(8))

PRINT "Oven 8 cleaning began at ";TIME

! Complete cleaning cycle for Oven #8

ELSEIF TIME - begincyc >= Schedule (8,2) AND TIME - begincyc < Schedule (9,1)

BrkFlag = REPAIR (Oven(8))

PRINT "Oven 8 cleaning ended at ";TIME

! Initiate cleaning cycle for Oven #9

ELSEIF TIME - begincyc >= Schedule (9,1) AND TIME - begincyc < Schedule (9,2)

BrkFlag = BREAKDWN (Oven(9))

PRINT "Oven 9 cleaning began at ";TIME

! Complete cleaning cycle for Oven #9

ELSEIF TIME - begincyc >= Schedule (9,2) AND TIME - begincyc < Schedule (9,2) + 2

BrkFlag = REPAIR (Oven(9))

PRINT "Oven 9 cleaning ended at ";TIME

ENDIF

IF TIME = begincyc + 36

begincyc = TIME

ENDIF

Dummy2:
Cycle Time: 0
Output Rule:

Push to dummy

Oven:
Quantity: 9
Type: General (Multi-Cycle)
Cycle Detail:
	Cycle number: 1
		Cycle Time: 1.5
		Input Quantity: 1
		Finish Quantity: 1
		Description: 1
		Input Rule

Pull from A out of World

Cycle number: 2
		Cycle Time: 2.5
		Input Quantity: 0
		Finish Quantity: 1
		Description: 2
	Cycle number: 3
		Cycle Time: 0.75
		Input Quantity: 0
		Finish Quantity: 1
		Description: 3
	Cycle number: 4
		Cycle Time: 1.23
		Input Quantity: 0
		Finish Quantity: 1
		Description: 4
		Output Rule

Push to Ship

Actions on Initialize:
	For X = 1 to 2
		For Y = 1 to 9
			Read DownSchd (Y,X)
		Next
	Next
	OPENBOX
	Print “\f”

## Using GETPOSX and GETPOSY

See model Movetime.mod

Modeling move times between operations in a WITNESS Model, and use of new GETPOSX and GETPOSY functions

#### Define

Part:				LockAssy
Machine: 			Trans001
					CNC_Mach
					Trans002
					Assembly
					Trans003
Buffer: 				CNC_OUT
					Receiving
					CNC_IN
					Shipping
Labor:				Operator
					Disptchr
Variable: 			TravTime,Type Real, Qty 3

DISt, Type Real, Qty 3

#### Detail

LockAssy
Name of Part: LockAssy
Type: Variable attributes
Group Number: 1
Maximum Arrivals: Unlimited
Inter Arrival Time: 1.0
First Arrival At: 0.0
Lot Size: 1
Output Rule

PUSH to Receiving(1)

Trans001
Name of Machine: Trans001
Quantity: 1
Type: Batch
* Batch min: 1
* Batch max: 1
Priority: Lowest
Labor:
Cycle: Disptchr#2
Pre-empt level: None
Cycle Time: TravTime (2)
Input Rule:

PULL from CNC_OUT(1)

Output Rule:

Wait

CNC_Mach
Name of Machine: CNC_Mach
Quantity: 1
Type: Single
Priority: Lowest
Labor:
Cycle: Operator#2
Pre-empt level: None
END
Cycle Time: 5.9
Input Rule

PULL from CNC_IN(1)

Output Rule

PUSH to CNC_OUT(1)

Trans002
Name Of Machine: Trans002
Quantity: 3
TYPE: Batch
* Batch min: 5
* Batch max: 5
Priority: Lowest
Labor:
Cycle: Disptchr#1
Pre-empt level: None
Cycle Time: TravTime (1)
Input Rule

PULL from Receiving(1)

Output Rule

PUSH to CNC_IN(1)

Assembly
Name Of Machine: Assembly
Quantity: 1
Type: Single
Priority: Lowest
Labor:
Cycle: Operator#1
Pre-empt level: None
Cycle Time: 1.0
Input Rule

PULL from Trans001(1)

Output Rule

PUSH to Trans003(1)

Trans003
Name of Machine: Trans003
Quantity: 1
Type: Batch
* Batch min: 5
* Batch max: 5
Priority: Lowest
Labor:
Cycle: Disptchr#1
Pre-empt level: None
Cycle Time: TravTime (3)
Input Rule

Wait

Output Rule

PUSH to Shipping(1)

CNC_OUT
Name of Buffer: CNC_OUT
Quantity: 1
Capacity: 10
Delay Mode : None
Input Position: Rear
Output Scan From: Front
* Select: First
Reporting: Individual
Shift: Undefined,0

Receiving
Name of Buffer: Receiving
Quantity: 1
Capacity: 1000
Delay Mode : None
Input Position: Rear
Output Scan From: Front
* Select: First

CNC_IN
Name Of Buffer: CNC_IN
Quantity: 1
Capacity: 10
Delay Mode : None
Input Position: Rear
Output Scan From: Front
* Select: First

Shipping
Name Of Buffer: Shipping
Quantity: 1
Capacity: 1000
Delay Mode : None
Input Position: Rear
Output Scan From: Front
* Select: First

Operator
Name Of Labor: Operator
Quantity no shift: 2

Disptchr
Name Of Labor: Disptchr
Quantity no shift: 3

TravTime
Name of Variable: TravTime
Quantity: 3

Dist
Name of Variable: Dist
Quantity: 3
Reporting: Yes

#### Model / Initialize Actions

PRINT "\f"  -- Clear all previous contents of the Interact Box

The first section calculates the distance between the four functional areas (machines) in the model: Receiving, CNC Machining, Assembly, and Shipping. The GETXPOSN and GETYPOSN functions, new in WITNESS Release 8.0, are used to get the X and Y screen coordinates of each machine. Then the distance between pairs of machines where the movement of material occurs is calculated using the Pythagorean formula. For purposes of this model, all distances are assumed to be in feet.

Distance =  (x2 – x1)2 + (y2 – y1)2

Where

x2 is the X-position of the 2nd machine
x1 is the X-position of the 1st machine
y2 is the Y-position of the 2nd machine
y1 is the y-position of the 1st machine

Calculate distance between Receiving and CNC Machine

Dist (1) = ((GETXPOSN (Receivng,2,1) - GETXPOSN (CNC_Mach,2,1)) ** 2 + (GETYPOSN (Receivng,2,1) - GETYPOSN (CNC_Mach,2,1)) ** 2) ** 0.5

Calculate distance between CNC Machine and Assembly

Dist (2) = ((GETXPOSN (CNC_Mach,2,1) - GETXPOSN (Assembly,2,1)) ** 2 + (GETYPOSN (CNC_Mach,2,1) - GETYPOSN (Assembly,2,1)) ** 2) ** 0.5

Calculate distance between Assembly and Shipping

Dist (3) = ((GETXPOSN (Assembly,2,1) - GETXPOSN (Shipping,2,1)) ** 2 + (GETYPOSN (Assembly,2,1) - GETYPOSN (Shipping,2,1)) ** 2) ** 0.5

The next section calculates the travel time between the four functional areas in the model based on the distances calculated above and assigns them to variables in the array TravTime.

TravTime is then used in the cycle time field for the machines Trans001, Trans002, and Trans003, which represent carts used to transport the parts between the work areas. The cycle time of the machines represents the transport time between two particular work areas, and Disptchr (dispatchers) are assigned to these machines to “perform” the transport. Travel speed is assumed to be 3.5 mph = 5.13 ft/sec = 308 ft/min representative of a normal person walking.

Calculate travel time between Receiving and CNC Machine in minutes.

TravTime (1) = Dist (1) / 308

Calculate travel time between CNC Machine and Assembly in minutes.

TravTime (2) = Dist (2) / 308

Calculate travel time between Assembly and Shipping in minutes.

TravTime (3) = Dist (3) / 308

Load and run the model. Note the display of distance and travel time on the screen calculated during Initialize Actions. Now, try moving Receiving, the CNC Machine, Assembly, or Shipping to different relative positions on the screen. Rewind the model to time zero and run the model again. Note that the distances and travel times have changed.

## Changing the Display Layer of an Existing Element or Background Graphic

This technique is a real time saver if you have a model with a large number of existing Elements that you want to change the layer on which they are displayed. This might be used to prepare a simulation model for presentation to your client or manager when you want to move certain Elements or graphics to separate layers which you can then show or hide as desired.

#### Define & Display

Any WITNESS Element using a Designer Element, or via the Model/Elements dialog. Creating a WITNESS Machine is recommended.

When the Element is defined and displayed on the screen, double-right click on it to edit its display. Set mode to Update if not already done. Examine the layer the different items of the display are currently on. For example, you might see “Name – Simulation Layer”, “Icon – Simulation Layer”, etc.

Edit Display

- Click on Draw, select any item from the list of things to draw.

- Select a layer from the layer pull-down menu in the dialog. All subsequent Elements and graphics drawn in BACKDROP with Screen Editor will be drawn in this layer until it is changed again.

- Click on Cancel.

- Go into Model/Elements dialog.

- Select the desired Element.

- Click on Copy Display, then Paste Display to same Element. Crosshairs will appear, place the Element as desired in the window. Close the Model/Elements Dialog box.

- To move other Elements to this layer, select each Element individually and hit Paste Display button. Close the Model/Elements dialog when finished.

- Double right click on the Elements you changed, note that the display items associated with them have been changed to the layer you selected.

## Cloning a Model Element to a Designer Element

The following is a method for effectively making a copy of a Model (Simulation) Element as a Designer Element. This technique is useful if you have created a Model Element that, you now want to turn into a Designer Element for subsequent model building use. It’s also useful if you have used a Designer Element to create a Model Element, modified the Model Element, and now need to “update” the Designer Element with these new settings.

The Clone function in the Model / Elements dialog will not work because the resulting clone will always be another Model Element. The solution described below describes how to use the Copy Display, Copy Detail, and Copy Display / Detail buttons to transfer the required settings to the Designer Element. You can also copy just the detail OR the display of the Model Element if desired.

Transfer Settings to a Designer Element

- First, open the model containing a Model Element you want to effectively "convert" to a Designer Element.

- Open the Model / Elements dialog.

- Define an Element of the desired type (machine, buffer, or whatever) as a Designer Element. Do not enter display or Detail information for this Element. Skip to Step #4 if you already have a Model Element you want to copy.

- Select the Model Element you want to copy from the list. If you just want to copy the display of the Model Element to the Designer Element, proceed to Step 5, otherwise skip to Step 6.

- Click on Copy Display, select the Designer Element you just created from the list, then Click on Paste Display.

- Click on the Model Element you want to copy from again.

- Click on Copy Detail, select the new Designer Element again, then Click on Paste Detail.

## Maintaining Display Layer Integrity when Cloning an Element

If you use the Clone function to create a copy of a multi-layered Element, you may have noticed that all items in the copy will end up on the same layer (the last layer you drew something on), thereby losing your layer assignments. The following is a method for taking an Element which has items of its display drawn on different layers and creating an exact copy, maintaining the assignment of the items to their respective layers.

Copy an Element with Display Layer Integrity

- Edit the display of the desired Element(s), changing different items to different layers as desired.

- File / Save As, set file type to Sub-model, enter a filename, click on OK.

- Click on the NO button when prompted "Do you want to save a module Element?".

- Select the desired Element(s) from the dialog, hit OK, then click on the message box regarding any unresolved references to close. A sub-model containing the Element(s) you selected has been created.

- From Model / Elements, create a new Module as a Designer Element.

- Do not edit Display of the new Designer module. Instead, edit its Detail, Click on the Browse button, enter name of sub-model file you just saved, click on Open. The sub-model file you created in step #4 will now be linked to this module Element.

You are now ready to make exact copies of the original Element using the new Designer Module. Just click on the Designer Module in your Designer Elements window, move the crosshairs to your model window, and click again to create the new object. Now, double-right click on the Element, and note items in its display are assigned to the same layers as the original Element.

## Utilizing Multi-Dimensional Arrays

See model MD_Array.mod

The following model represents a crane whose cycle time is dependent upon where the parts came from(con001(1),con001(2) or con001(3)), where the parts are going(dock1 or dock2), and how much they weigh.

Multi-dimensional Arrays

- Define a multi-dimensional variable: Dimension one – where the part is going Dimension two – where the part came from Dimension three – part weight

- Assign part attributes accordingly, based upon the parts weight, where it came from, and where it is going.

- Assign values to the multi-dimensional variable (array) in the initialize actions.

- Place the 3-D variable inside the cycle time field of the crane. (e.g. cranecyc(pen, stage, weight/10)

#### Define

Part: 				Widget	
Machine:			Crane
Conveyor: 			Con001
					Dock1
					Dock2
Attribute:			weight – Real
Variable:			cranecyc,2x3x3, Real

#### Detail

Widget

- Three part types: 10, 20 and 30 pound parts come from either Con001(1), Con002(1), or Con003(1).

Interarrival time: Negexp(30,101)
First arrival at: 0.0
Lot Size: 1
Actions, Create
		Pen = Iuniform(1,2,700)
		Weight = Iuniform(1,2,701) * 10
		Desc = weight

Output Rule

Percent /102 Con001(1) at Rear 33.33, Con001(2) at Rear 33.33, Con001(3) at Rear 33.33

Crane
Quantity: 1
Type: Single
Cycle time: cranecyc (pen,stage,weight/10)
Input Rule

Sequence/Next Con001(1) at Front#(1), Con001(2) at Front#(1),Con001(3) at Front#(1)

Output Rule

If pen = 1

Push to dock1 at rear

Else

Push to dock2 at rear

Endif

Con001
Quantity: 3
Type: Queuing
Part Length: 10
Max Capacity: 10
Actions, Front
	Stage=N
Input Rule

Wait

Output Rule

Wait

Cycle time: 1.0

Dock1
Quantity: 1
Type: Queuing
Part Length: 10
Input Rule

Wait

Output Rule

Push to Ship

Cycle time: 1.0

Dock2
Quantity: 1
Type: Queuing
Part Length: 10
Input Rule

Wait

Output Rule

Push to Ship

Cycle time: 1.0

#### Initialize Actions

! 10 pound cycle times

cranecyc(1,1,1) = 3

cranecyc(1,2,1) = 2

cranecyc(1,3,1) = 1

cranecyc(2,1,1) = 1

cranecyc(2,2,1) = 2

cranecyc(2,3,1) = 3

! 20 pound cycle times

cranecyc(1,1,2) = 3.5

cranecyc(1,2,2) = 2.5

cranecyc(1,3,2) = 1.5

cranecyc(2,1,2) = 1.5

cranecyc(2,2,2) = 2.5

cranecyc(2,3,2) = 3.5

! 30 pound cycle times

cranecyc(1,1,3) = 4

cranecyc(1,2,3) = 3

cranecyc(1,3,3) = 2

cranecyc(2,1,3) = 2

cranecyc(2,2,3) = 3

cranecyc(2,3,3) = 4

## Operator Skill Level Affecting the Cycle Time of a Machine

See model Op_Skill.mod

Operator Skill Level

- Create a group 0 real attribute called skill.

- In Initialize Actions, set the skill of the operator (e.g. oper1:skill = .85). For labor whose quantity is greater than one the syntax should be: For x = 1 to nqty(oper1) Member(oper1,x):skill = .85 Next

- Adjust the cycle time according to the operator’s skill level (e.g. Triangle(1,1.5,2,102)/laborat(mill,1,1):skill)

#### Define

Part: 				Part
Machine:			Mill
					Drill
Conveyor: 			Con001
Buffer:				Buff001
Labor:				oper1
Attribute:			skill (group 0)

#### Detail

Part
Interarrival time: Negexp(40,101)
First arrival at: 0.0
Lot Size: Poisson(15,102)
Output Rule

Push to con001 at Rear

Mill
Quantity: 1
Type: Single
Cycle time: Triangle(1,1.5,2,104)/Laborat(mill,1,1):skill
Labor cycle: oper1
Input Rule

Pull from con001 at Front

Output Rule

Push to ship

Drill
Quantity: 1
Type: Single
Cycle time: Triangle(1,1.5,2,103)/Laborat(drill,1,1):skill
Labor cycle: oper1
Input Rule

Pull from part out of buff001

Output Rule

Push to con001 at Rear

Con001
Quantity: 1
Type: Queuing
Part Length: 6
Max Capacity: 6
Input Rule

Wait

Output Rule

Wait

Cycle time: .25

Buff001
Quantity: 1
Capacity: 1000
Name of Labor: oper 1
Quantity: 1

#### Initialize Actions

Oper1:skill = .85