Simulation Projects

Objectives

- To describe the basic steps for a successful simulation project.

- To provide the basic concepts on the determination of warm-up time, simulation running times, and number of replications to be run for each situation.

- To setup experiments in WITNESS.

## Methodology

### Lanner Approach to Simulation Projects

The aim of this section is to show how a simulation project should be performed in order to ensure a successful outcome. Broadly speaking, the stages of a simulation project can be broken down into four major sections: Project Definition, Model Building and Testing, Experimentation, and Project Completion, Documentation and Implementation. The following flowchart and the next few sections investigate these stages in more detail.

Figure 4-1: Simulation Project Flowchart

### Problem Definition

#### Establish Objectives

The first step in any simulation project should be to set the objectives. They should be a statement of what should be achieved. Typical objectives may include:

- Increase plant throughput by 20%

- Reduce WIP by 10%

- How many AGVs are required?

- What schedule achieves best throughput

The objective should never be just to build a simulation model. There may be some general project objectives related to time scale, run speed or visual presentation.

#### Scope and Level of Detail

The scope and level of detail determine what should be included in the model and how much detail should be modeled. The main thing to bear in mind is that to model the minimum amount of detail required in order to achieve the project’s objectives. More detail does not necessarily mean more accuracy.

#### Data Collection

In a simulation project, data is usually required to enable you to build the simulation model, to set the initial level of various factors in the model, and also to validate the model once built. Data usually falls into one of three categories:

- available

- not available, but collectable

- not available and not collectable

In order to deal with the third category, the data may need to be estimated. The data can be estimated by using the experience of people who are familiar with the operation and by referring to manufacturers’ specifications. Data can also be estimated by making the model create the data (e.g., based on knowing the throughput of the area that you are modeling, you could make your model create this throughput by varying the unknown parameters).

Once the project’s objectives, the scope and level of detail, and the data required have been sorted, it is useful to map out a project specification that summarizes these areas and lists any assumptions made. This can be useful in gaining agreement to a project and in validating the approach.

### Model Building and Testing

#### Structuring the Model

Before building the model, it is extremely useful to map out or structure how the model will look on paper. The structure or map of the model can help to get an overview of the whole model and map out any of the complex areas where further investigation is needed.

#### Model Building & Verification

Approach model building by taking small steps, one at a time: coding, documenting, and verifying. Before continuing onto the next step, be confident that the step completed is working as intended. The verification should consist of checking the code, inspecting output reports, and checking that the Elements modeled relate to the real world Elements that they are modeling.

#### Validation of the Model

A validation of the model should be performed on completion of the model. This 'validation' should test the overall accuracy of the model and the ability to meet the project objectives. This phase helps to establish credibility by giving confidence in the results.

The model can be validated in various ways. Firstly, by watching the model run or demonstrating it to people who know the system, an idea of the validity of the model can be obtained. In comparing the model to the real system, the results produced can be compared to historical data. Results form the real system and results from the model could be put in front of an expert to see if they could tell the difference. The model produced could also be compared to other models; e.g., mathematical models, deterministic models, and other simulation models

### Experimentation

Prior to starting the actual experimentation stage, a number of issues need to be addressed; e.g. warm-up period, run length, number of replications, experiments to be performed, etc.

#### Model Classification

A model can be either Deterministic or Stochastic. While a deterministic simulation model ignores randomness, stochastic models include one or more random events. Another way to classify simulation models relates to time: A Dynamic model describes the behavior of the system through time while a Static model portrays the behavior of a system at a single point in time. Simulation models can also be said to be Discrete, Continuous, or Combined. Discrete models describe changes in the status of a system, as occurring at isolated points in time; on the other hand, continuous models treat change like a continuously occurring phenomenon. Finally, a model could be classified as a combined model if it incorporates both discrete and continuous simulation.

Figure 4-2: Model Classification

#### Terminating vs. Non-terminating Systems

A terminating simulation model runs for a predetermined length of time or until a specific event takes place such as the end of the day, the week, or the shift. On the other hand, non-terminating models do not contain such end-points and the model could be theoretically run for an unlimited amount of time.

Given the fact that the duration of a terminating system is fixed, initial conditions can have a serious influence on the system’s performance. Some examples of terminating models could be fast-food restaurants, gas stations, and banks. The goal of such systems is to understand the behavior for a “typical” fixed duration. The overall procedure for analyzing this type of system involves the simulation of a large number of replications. The analysis of the system’s behavior is based upon average data for each replication (e.g.: average daily flow time), and the determination of an appropriate number of replications via output analysis for each what-if scenario.

The duration of Non-terminating systems is not fixed and therefore the effects of the initial conditions must be removed (e.g. Assembly lines, metal processing). The goal of these systems is to understand the steady-state behavior of the model. The overall procedure to analyze such systems involves the following five major steps.

The first three steps involve the running of one replication of the simulation model for an extended period of time, the exclusion of the initial transient phase that is influenced by initial conditions, and the grouping of the remaining data into batches (typically, 10-20). The next two steps involve the analysis of the system’s behavior based upon the average data for each batch (e.g. average flow time for each batch), the determination of an appropriate batch size, and a satisfactory number of batches via output analysis.

A steady-state simulation implies that the system is independent of its startup condition. Transience occurs when the simulation has not reached steady state.

Figure 4-3: Model Classification

#### Warm-up Times and Start-up Conditions

In deciding whether it is necessary to have a warm-up period the question 'does the state of the model represent normal working conditions at time 0?' has to be answered. If the answer is 'No' then the length of the warm-up period needs to be evaluated.

A Warm-up period is the amount of time that a model needs to run before statistical data collection begins. This warm-up period could be estimated through experimentation with moving averages (flow times, throughput) and time series (WIP, buffer sizes). The point at which the model seems real for the first time could be estimated as the warm-up time.

If the level of WIP is plotted on a graph, the plot rises steeply at the start of the model run and gradually reaches a relatively stable level representing the 'typical' condition. Once this level has been reached, you can estimate this as being our warm-up period.

It is always better to have a warm-up period that is too long rather than one that is too short (See Figure 4-3). A warm-up time should be evaluated for each experiment. This period could be the same for every experiment because of time constraints as well as to provide a basis when comparing different what-if scenarios.

Starting conditions could be used as an alternative to warm-up period. This could be achieved by loading the model from the beginning of the simulation run. The disadvantage of this alternative could be found when comparing completely different systems. You could also mix starting conditions with warm-up periods to alleviate this disadvantage.

Terminating systems are usually dependent upon the initial values and quantities used when starting the model; therefore, the start-up conditions should reflect real world situations.

#### Run Length

Once the model has warmed up, we need to decide for how long to run the model. One method for deciding this is by inspecting the random numbers sampled. As a rule of thumb, you should aim to take a minimum of 10 to 20 random numbers from each random number stream used in the model. Typically, to ensure that this takes place for all of the random number streams, you select the least frequent event in your simulation model (usually breakdowns) and run the model until this event has happened 10 to 20 times (e.g. 10 to 20 breakdowns). The values of the random numbers can then be monitored (and plotted on a graph) to ensure that the numbers sampled follow the distribution required.

#### Number of Replications

Due to the very nature of random numbers, it is imprudent to draw conclusions from a model based on the results generated by a single model run. Replication is defined as executing the same model a number of times, but with different random numbers in each run.

As a rule of thumb, you should always perform at least three to five replications for each experiment. It is possible using WITNESS to do this automatically using the EXPERIMENT menu. Using standard statistical analysis, it is then possible to put confidence intervals around the results from the simulation runs, performing further runs if you wish to reduce the confidence intervals.

### Project Completion

On completion of the project, a number of tasks may need to be done, e.g.,

- complete documentation

- communicate results

- produce a report

- implement the recommendations

It is also useful, at the end of a simulation project, to review the overall project process and perhaps refine the process before starting the next project.

## Setting-Up a WITNESS Experiment

The objective of this section is to provide you with the ability to not only setup experiments in WITNESS, but also to introduce you to the use of SIM and WCL files in your experiments.

An experiment is a variation of a ‘base’ model run that you run under closely controlled conditions, collecting results at specified intervals. Each simulation run has a unique number (assigned by WITNESS) called a replication number. Each replication refers to a specific random number stream setting, warm-up period, and run length. Data from all the runs is used to obtain a single set of results, which characterizes the model. This set of results is allocated a number, by you, called a situation number.

To take the greatest advantage of this tool, you need to have a verified and validated model(s) and at least a couple of what-if scenarios to compare. The warm-up time, the length of the simulation run and the number of replications for each run should also be known. If you have any questions regarding the determination of these three values, please refer to the previous section.

- Click on the ModelExperimentDefine command.

### The Situations Page

The Define Experiment window contains different tags or pages. The Situations tag let you define the different scenarios or situations you what to run. For example, if you were trying to evaluate the influence that a new machine may have on your model, Situation No. 1 could be your original model and Situation No. 2 could be the model with the new machine. You could also incorporate situations to evaluate the impact of a new facility layout on your average flow time or even the sensitivity of your model given a different index time for a conveyor.

- Click on the Add/Remove button to define the different scenarios or situations to be run.

- Type the name of the Scenario in the Situation Description input field. - Click on the Add button Do steps 2and 3 to define all the scenarios you want WITNESS to run. You may also add some later if you want. - Click the OK button when done.

If you would like to re-arrange the order in which the situations would be run, you can do so by clicking on the Move Up and Move Down buttons in the Add/Remove Situations dialog. You could also remove any of them by clicking on the Remove button.

At this point, we need to link the actual file names of our models to the situation names we have just defined. If you do not link a file name to a situation name, the experiment will default to the model currently loaded in WITNESS.

- Select the situation name by clicking on the next to the Situation Title display. - Select the Situation File option and click on the Browse button. - Choose the file name of the model. - If you do not link a file name to a situation, the experiment will default to the model currently loaded in WITNESS.

Once you have selected the file to be linked, its name should appear on the left side of your property sheet, right under the display field Name. - Do steps five to seven for all your scenarios. - Click on the Summarize button to see a summary of your situation set-up.

- Click on the Close button when done.

This shows the summary of all the steps we have done in this section. The name of the situations or scenarios can be found under the second column and their respective situation file can be found under the third column.

### The Run Control Page

- Type the Warm-up period and the Replication length in their respective input fields.

At the end of the Warm-up time, WITNESS will reset the statistics of the model. You also need to know that there are two types of execution modes for running experiments. While one mode will restart the model before each replication, the other will run consecutive replications without resetting the time clock

### The Replication Control Page

- Enter the number of the first replication. - Select the Random Number Control Type

Defining the start at replication number could be very useful if you want to run additional replications. For example, if you already have runs 001 to 005, you may want to run 10 additional replications. By setting the first replication number as 6, each subsequent replication would be numbered 006 to 015.

The individual random number control option will let you define each one of the replication’s random streams individually. The all random number control option, on the other hand, will make WITNESS cycle through all the random number streams for a specific replication. For example, stream number 1 would be replaced with stream 2 and stream 1000 with stream 1. Note that depending on the option you choose, WITNESS will either show the replication (all) property sheet or the replication (ind) property sheets.

### The Reporting Control Page

- Select the format type of the reports’ output. - Select a reporting interval or keep the default. - Input a stem of no more than five characters for the reporting filename. (Not available if Printer option has being chosen)

The CSV (comma separated values), multiple CSVs, and DIF formats are compatible with a number of spreadsheets, statistical analysis packages and databases.

WITNESS will save each report under a generic name. The stem you input in the reporting filename field will be combined with a three digits number to save each file in sequence (e.g. Rpt001, Rpt002, …). You could also save the model and the status at the end of each replication for future reference by checking the box next to the Save model & Status field and typing a filename.

### The Replication (All) Page

- Click on the Add/Remove button

- Type the replication number in the Replication Description input field. - Click on the Add button Repeat steps two and three for each replication. - Click OK when you are finished.

If you choose to add five replications in the Add/Remove Replications window, WITNESS would run each scenario five different times.

- Choose between, Regular and Antithetic random number type, for each replication. - Input the number of values to skip before the first the first Pseudo Random Number is generated. - Input the Stream Offset. - Click Ok when finished.

If an offset value were defined, the new stream number would be the old one plus the offset value.

A regular random number is a number between 0 and 1 (e.g. 0.85). The antithetic random number is 1 minus the regular random number (1 - 0.85 = 0.15).

You could click on the Summarize button to get an overview of the step you have completed.

### The Replication (Individual ) Page

- Add the number of replications you want to run (similar to steps 1 through 4 in the previous section) - Select the replication you want to detail by clicking on the next to the Replication Description field.

- Select the stream number you want to modify by clicking on the under the Steam No. display.

- Choose the random number type for the stream: Regular or Antithetic. - Input the number of values to skip before the first Pseudo Random Number is generated. - Type the number of the stream that will replace the current stream number. - Repeat steps three to six for each stream you want to control. - Click OK when you are finished.

### Saving, Opening, and Running Experiment Files

- To save your experiment for future use, click on the ModelExperimentSave As command. - To open an existing experiment file, click on the ModelExperimentOpen command.

- To run your experiment, select either, the Advance or Batch commands.

The Advance mode displays the model on the screen and runs the experiment with normal, active model display. The Batch mode, on the other hand, runs the experiment with the display switch off.

### Using Model & Status (SIM) Files

A way to reduce the running time of an entire experiment could be through the use of SIM files. Model & Status files allow you to save the current simulation model and status of the system at any point in time.

This could be used to generate the initial conditions and/or warm-up time to be used for all your What-if scenarios. Each replication would pickup the simulation run from the time the SIM was generated (>0).

- Run your model up to the end of the warm-up period and reset the reports. (ReportsReset Reports) - Click on FileSave As. - Select the Model & Status File (*.sim) option by clicking on the to the right of the List of files of type. - Type the Filename. - Click OK.

- Open the previous experiment file. - Change the extension of the files linked to each situation from MOD to SIM. OR Choose the SIM filename of the model by clicking on the Browse button.

- Click on the Summarize button to make sure you have selected the right files. - Click Close to close the Experiment Situation Summary dialog. - Click OK when done and run the experiment. - Set the warm-up period option on the Run control page back to 0. - Run the experiment.

### Using Model & Status (SIM) Files and WITNESS Command Language (WCL) Files.

Up to this point, we have being generating different model files depending on the what-if scenario we want to analyze; however, sometimes it is possible to have one single generic model. The trick is to assign variables to the parameters being changed between scenarios, and to let WCL files initialize these variables.

- Create generic model - Generate WCL files. - Open old Experiment file. - Re-assign the same generic model to each situation. - Assign the respective WCL files

- Click on the Summarize button. Notice that we have the same MOD file, but a different WCL for each Situation. - Close and Run the experiment

- Experienced WITNESS users usually use WCL files. For further information on how to create WCL files, please refer to the WITNESS Reference Manuals.

The success of a simulation project depends upon various factors: The definition of the objectives and the scope of your model, data collection and model structure, model building verification and validation, experimentation, documentation, and presentation of the results. Successful experimentation is part of the success of your simulation project. However, once the warm-up time (if any), the number of replications and the length of each replications have being determined, WITNESS allows you to setup your experiments easily. After going through this step-by-step section, you should be able to define and run experiments in WITNESS based on different model files (MODs), model & status files (SIMs) and/or WITNESS Command Language files (WCLs).

## Examples

### The Importance of Selecting a Correct Warm-up Period

(This discussion courtesy of Terry Stockdale of Raytheon TI Systems)

To determine the significance of a simulation warm-up period for a model, the Bells, and Shuttle programs (in the demo directory) was used to measure the impact of statistical data.

As a basis for statistical data collection, a 4-4-4 concept was employed. The 4-4-4 concept requires that the warm-up period be tested 4 times, the experiment would run 4 times the longest statistical distribution or part inter-arrival time used in the model, and 4 experiment replications would be run.

The warm-up significance was determined by varying the length of the warm-up periods to 0%, 25%, 50%, 75%, and 100% of the warm-up period plus a fixed experiment length of 4X the longest statistical distribution or part inter-arrival time. The factor of comparison will be total parts shipped from the time statistics are reset. Resulting data was from the experiment’s mean at a 90% confidence level. The experiment parameters used are detailed in the following table. The execution mode was set at restart.

| Table 4-1: Replication Summary |  |  |  |  |
| --- | --- | --- | --- | --- |
| Replication Number | Replication Description | PRN Selection | Skip | Offset |
| 1 | A | Regular | 0 | 1 |
| 2 | B | Antithetic | 0 | 2 |
| 3 | C | Regular | 0 | 3 |
| 4 | D | Antithetic | 0 | 4 |

The last two rows of the Bells Experiment Table and the last two rows of the Shuttle Experiment Table detail the experiment’s results. The percent error was calculated using warm-up at 100% as the standard.

The replication length for Bells was determined by the packing machine having a cycle time of 50.

| Table 4-2: Bells Experiment |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Warm-ups |  |  |  |  |
|  | 100% | 75% | 50% | 25% | 0% |
| Number of Replications | 4 | 4 | 4 | 4 | 4 |
| 4X Replication Length | 200 | 200 | 200 | 200 | 200 |
| Warm-up Length | 16 | 12 | 8 | 4 | 0 |
| Total Experiment Length | 216 | 212 | 208 | 204 | 200 |
| Experiment Parts Shipped | 6 | 6 | 5 | 4 | 3 |
| % Error (From 100%) | 0 | 0 | 16.7 | 33.3 | 50 |

The impact of running the Bells Experiment in the Bells Warm-up Impact Graph. The graph illustrates that up to a 50% difference may be realized in an experiment’s result by selecting a warm-up period that is too short.

Figure 4-4: Bells Warm-up Impact

The replication length for Shuttle was by the part “head” having an interarrival time of 40.

| Table 4-3: Shuttle Experiment Table |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Warm-ups |  |  |  |  |  |  |  |  |  |
|  | 100% |  | 75% |  | 50% |  | 25% |  | 0% |  |
| Number of Replications |  | 4 |  | 4 |  | 4 |  | 4 |  | 4 |
| 4X Replication Length |  | 160 |  | 160 |  | 160 |  | 160 |  | 160 |
| Warm-up Length |  | 40 |  | 30 |  | 20 |  | 10 |  | 0 |
| Total Experiment Length |  | 200 |  | 190 |  | 180 |  | 170 |  | 160 |
| Experiment Parts Shipped |  | 11.8 |  | 10.8 |  | 9.5 |  | 8.0 |  | 6.8 |
| % Error (From 100%) |  | 0 |  | 8.5 |  | 19.5 |  | 32.2 |  | 42.4 |

The impact of warm-up when running the Shuttle experiment is shown in the following graph. The graph illustrates that up to a 42% difference may be realized in an experiment’s result by selecting a warm-up period that is too short.

Figure 4-5: Shuttle Warm-up Impact

### Experimenting with an Assembly Model

(Model Courtesy of Amana Refrigeration)

A refrigerator door assembly process is shown below (case2.mod). A set number of pallets enter the system initially at pallbuff and then circles the conveyor system. Doors enter at con10 and con11 are then loaded onto the pallets. Every other empty pallet is given a door at the first loading station (con10). The door on the pallet is represented by the part doorpall. Doorpalls must be rotated at the machine represented by the green circle. The remaining empty pallets are loaded at con11. After an assembly station, the doors are unloaded at con12, while the empty pallets continue along the conveyor system. Buff2 is simply a buffer between two conveyors.

Figure 4-6: Assembly Line Layout

If there are too many pallets in the system, con8 will become blocked. If there are too few pallets, con10 and con11 will become blocked. Determine the optimal number of pallets such that the percentage of time blocked is minimized.

Answer: The optimum number of pallets is between 8 and 13. Running the model for 1000 time units yields the following results.

| Table 4-4: Model Results |  |  |  |
| --- | --- | --- | --- |
| Pallets | Con10 % Blocked | Con11 Blocked | Con8 Blocked |
| 5 | 59.30 | 56.81 | 0.00 |
| 7 | 45.32 | 41.65 | 0.00 |
| 8 | 0.00 | 15.09 | 0.00 |
| 9 | 0.00 | 15.09 | 0.00 |
| 10 | 0.00 | 15.09 | 0.00 |
| 13 | 0.00 | 15.09 | 0.00 |
| 14 | 0.00 | 15.09 | 59.46 |
| 15 | 0.00 | 15.09 | 59.46 |

After determining the optimum number of pallets in the system, will increasing the capacity of buff2 increase the performance of the system?