Building WITNESS Models

Objectives

- To provide you with the concepts needed to create a basic model.

- To run a model.

- To generate reports

- To create stoppages.

- To effectively use labor and conveyors.

## Building a Model

### The Three Steps to Model Building in WITNESS

WITNESS provides facilities for building models of great complexity, but the principles of building the model are simple.

Figure 2-1: Model Design Steps

| Table 2-1: Steps to Building WITNESS Models |  |
| --- | --- |
| Steps | Description |
| DEFINE | Define the major Elements that make up the “building blocks” of the simulation. |
| DISPLAY | Display each of the Elements in order to build up a pictorial representation of the facility layout. |
| DETAIL | Specify the timings and routing of parts (entities) as they move through the model. Each Element type has its own characteristics, e.g. machine cycle times or vehicle speeds. The logic for controlling the simulation is specified in this phase. |

In WITNESS, these three steps can be performed simultaneously, using Designer Elements, or in sequence, using the Element Selector dialog box. However, Designer Elements allows you to save time in the creation of Elements by combining these principles into one big step.

When an Element is being defined, displayed and detailed in sequence, the Element needs to be defined first, before it can be displayed or detailed. An Element does not necessarily need to be displayed on the screen; however, it needs to be defined and the minimum amount of detail options provided in order for the model to be able to use the Element.

### The Basic Elements

Models are built with a series of Elements or building blocks. Anything you use in WITNESS is an element: a machine, an attribute, a histogram, etc. This section describes the first three basic elements. The next sections will show you how to introduce them into a model and how to link them together.

| Table 2-2: Basic Elements |  |
| --- | --- |
| Elements | Description |
| PARTS or ENTITIES | Flow through your model. They can represent physical components, projects progressing through a large corporation, telephone calls, or even people moving through a supermarket. |
| BUFFERS or QUEUES | Places where Parts (entities) can be held. People in a queue or line and motors on a shelf are typical examples. |
| MACHINES or ACTIVITIES | Powerful Elements that are used to represent anything that takes parts (entities) from somewhere, processes them and sends them on to their next destination. A drill press, an organization that handles a project then passes it on, or an entire plant can each be represented by this Element. Machines or Activities that handle one or several Parts (Entities) at once can also be modeled. |

### Adding Elements to Your Model Using Designer Elements

Designer Elements are a palette of user-defined Elements with display and detail parameters already defined. When WITNESS first opens, it will open a model called STARTUP.MOD, which contains a basic set of these building blocks. These Designer Elements are used to generate the basic Elements included in your model. Later sections will show you how to create and customize your own Designer Elements.

- Before you save your model for the FIRST time, make sure you change the file name of STARTUP.MOD or START307.MOD. Go to ModelTitle, change the Name and the Title fields, and then save the model.

Figure 2-2: Designer Elements Window (START307.MOD File)

Figure 2-3: Title Window

Adding an Element to your Model

- Click on the Element name in the Designer Element Window using the left mouse button. - A cross arrow appears.

- Move the cross arrow to where you want the Element to be positioned. Click the left mouse button. - The Element appears on the screen with a net around it.

- By clicking and dragging inside this net, you can move the element around the screen.

- Repeat all the above steps to create each of the Elements in your model.

- The name given to the Element in the model defaults to the name of the designer Element plus 001, 002, 003 etc. For example, a designer Element defined as ‘mach’ would, when added to the model for the first time default to ‘mach001’, the second time ‘mach002’ etc.

Multiple Designer Element Option

- Select ModelOptionsDisplayMultiple Designer Element Option and Click OK.

- Click on the Element name in the Designer Element Window using the left mouse button. - A cross arrow appears.

- Move the cross arrow to where you want the Element to be positioned. Click the left mouse button. - The Element appears on the screen with a net around it. The cross arrow remains on the screen.

- Click the left mouse button for an additional copy of your element.

- Click the right button of your mouse when done.

### Moving Parts (Entities) Between Elements

Once the Elements have been placed in the model, use Input and Output Rules to define the movement of Parts (Entities) between Elements. Simple PULL, PUSH, PERCENT, and SEQUENCE rules can be input quickly and easily using the mouse to select from the Model menu:

- ModelVisual OUTPUT Rule

- ModelVisual INPUT Rule

These rules can also be accessed using the Standard Action Bar.

- There must be a currently selected Element before WITNESS will allow a rule to be entered.

Figure 2-4: Input Rule Dialog Box

Entering an Input Rule

- Click with the left mouse button on the Element (e.g. activity MACH001). - The name of the Element appears in the select box of the Standard Action Bar.

- Select the Input Rule icon from the Standard Action bar.

- Click on the Rule from the drop-down list, e.g. PULL. - Click on the Element, from which you wish to take parts (entities), e.g. BUFF001. - Click OK. - The Input Rule is then put in against the Machine (Activity).

Figure 2-5: Output Rule Dialog box.

Entering an Output Rule

- Click with the left mouse button on the Element (e.g. activity MACH001). - The name of the Element appears in the select box of the Standard Action Bar.

- Select the Output Rule icon from the Standard Action Bar. - The Output Rule dialog box appears.

- Click on the Rule for the drop-down list, e.g. PUSH. - Click on the Element you want to feed to, e.g. BUFF002, or on the SHIP or SCRAP buttons to exit the system. - Click OK - The Output Rule is then put in against the Machine (Activity).

As you build up rules, lines appear on the screen denoting the flow of the Elements. To view all of the rules entered, select ViewElement Flow. At this point, if you decide to run your model without entering any details in your model elements, WITNESS will prompt you for any basic parameters required to run the model, e.g. inter-arrival time of any Parts (Entities) and the cycle time duration of Machines (Activities).

### Display Forms

Once the Elements have been added to the model, you can change how they are displayed on the screen. Although the display is extremely valuable for understanding the model, it is not actually required in order to run the model. It is highly recommended that you do display all your Elements.

The Display dialog box consists of two drop-down lists and a number of buttons. The first of the drop-down lists contains operations that can be carried out with the display on the screen of the selected Element. (See Figure 2-6) The second drop-down list contains the item or option that you want to display against the selected Element, e.g. Name of Element. (See Figure 2-7) The operations that can be carried out from the first drop-down list are described below.

Figure 2-6: Sample Display Palette

Amend the DISPLAY of Your Model

- Select ModelElements or press the <F4> key. - Click Left Mouse Button on the required Element. - Click on the DISPLAY button, or click the right button of the mouse and select the Display option. - The Display dialog box appears. (See Figure 2-6)

- Make your modifications and click on the when done.

OR - Click on the Element name on the screen (the name appears in the selector box on the Standard Action Bar). - Select the display icon from the Action Bar - The Display dialog box appears.

- Make your modifications and click on the when done.

OR - Double-click the right hand mouse button on the Element on the screen. - The Display dialog box appears. - Make your modifications and click on the when done.

| Table 2-3: First Drop-down list |  |
| --- | --- |
| List Item | Description |
| Draw | Enables you to draw on the screen the option selected. In some instances, you can draw an option more than once (e.g. two icons, two part queues). |
| Update | Enables you to update the display of the item selected. You cannot use Update to change layers. |
| Delete | Deletes selected item. |
| Stretch | Stretches selected item (icons only). |

The item or option that appears in the second drop-down list depends to some extent on the type of Element being displayed. However, the following options are common to each Element:

Figure 2-7: Display Dialog Box

| Table 2-4: Second Drop-down list |  |
| --- | --- |
| List Item | Description |
| Name | Name of the selected Element |
| Notes | Notes entered on the detail form |
| Expression | Any specific information about Element |
| Rectangle | A box |
| Ellipse | A circle/ellipse |
| Line | A line |
| Icon | An icon |
| Text | Descriptive text |
| Simulation items | Multiple selection of all of the simulation items (e.g. name, icon etc.) |

Buttons on the Element Display Bar provide you with a variety of display features that are common to all Elements.

| Table 2-5: Element Display Buttons |  |
| --- | --- |
| Button | Description |
|  | Displays the Display Graphics dialog so that you can draw a new item or update an item that is already displayed. The contents of the Display Graphics dialog varies according to the type of Element that you have selected. |
|  | Displays a Delete Display dialog box from which you select the display item(s) to be erased the selected |
|  | Displays the Layers dialog box. You can change the layer name and switch the status of the layer to movable and/or visible for all windows. If the move option for a specific layer is selected, you can move items on the layer as long as the window option you are on is also movable. If the visible option is off, the objects on the layer are no longer visible on your screen. |
|  | Locks all of the display Elements together so they can be moved as a group by dragging the Element Name on the screen. You can only move locked display items that are drawn on the same layer. |
|  | Switches background grid on and off. |
|  | Access to WITNESS help |
|  | When you are displaying Elements singularly, the tick button exits the display palette. If you are displaying on a multiple selection, the tick proceeds to the next display palette whereas the cross cancels the display selection by returning to the Element selector. |

In addition to the standard options listed above, the Part (Entity) Display dialog box also gives the option to change the style of the Part (Entity); e.g. whether it is displayed as an icon or description. The Buffer (Queue) Display dialog box gives the option as to how to display the Parts (Entities) at the Buffer. The Machine (Activity) Display dialog allows the user to configure the display of Labor Resource, the Parts (Entities), and the gap style at the Machine (Activity). (See Figure 2-8)

Figure 2-8: Display Options for a Machine

### Detail Forms

Information must be provided which defines how each Element operates and how Parts (Entities) flow through the facility, in order to customize a model so that it reflects the facility being modeled. In other words, how the facility works.

When an Element is first entered into the model, various parameters are assumed. For example, when a Machine (Activity) is defined, it will be assumed that Labor Resource is not required, breakdowns are negligible, and statistics should be collected. However, WITNESS needs to be told how Parts (Entities) are to be routed between Elements. Initially, Elements are independent and do not know where the Parts (Entities) they are handling come from or go to. They must, therefore, be tied together to reflect the actual flow of parts in your facility. Input and Output Rules can be entered using the Visual Push and Pull rules.

These details can be changed through a Detail dialog box. A different Detail dialog is used for each type of Element, since each type of Element is characterized by different information. Every Detail dialog box has its own Notes property sheet which allows you to enter useful information about the Element (e.g. a description of the Element, any modeling assumptions you have made, etc.)

Editing the Detail of an Element

- Select ModelElements or press the <F4> key. - Click Left Mouse Button on the required Element. - Click on the DETAIL button, or click the right button of the mouse and select the Detail option. - The Display dialog box appears. (See Figure 2-9) - Enter the required information and click OK.

OR - Click on the required Element to be detailed. - Click on the Detail Button on the Standard Action Bar. - Enter the required information and click OK.

OR - Double click with the left mouse button on the required Element. - Enter the required information and click OK.

### Parts (Entities)

Parts (Entities) are used to represent those discrete items that move around the model. They can be used to represent small electronic components, large oil tankers, or anything in between. Parts (Entities) have even been used to represent calls in a telephone exchange and other types of information flow. WITNESS creates Parts (Entities) and introduces them into the simulation at the time and place dictated by the model. Parts (Entities) can be handled singly; they can combine into one; one can split into several; or a group can be batched together. Parts (Entities) can change into other Parts (Entities) as they progress through the model.

When detailing Parts (Entities), you need to decide whether they are Passive, Active or Active with Profiles (See Figure 2-9). If parts are passive, it means they are always available. Elements in your simulation model can pull them from an infinite supply location at any point in time. On the other hand, if parts are active, they arrive into your model following a specific pre-determined inter-arrival time and lot size. Parts having the active with profiles option will be covered in another Chapter of this manual.

The information you enter in the detail dialog box of a part indicates when, where and how many parts should appear during the simulation; whether any actions must be taken when they arrive or leave the simulation; and whether statistics should be collected.

Figure 2-9: Part/Entity Detail Characteristics

| Table 2-6: Options for Active Arrivals |  |
| --- | --- |
| Information | Description |
| Inter Arrival Time | Time between arrivals |
| Lot Size | Number that arrive together |
| Maximum Arrivals | Maximum number that can arrive throughout a simulation run |
| First Arrival at | Time first one arrives in the model |

Parts (Entities) must also have a Rule that tells WITNESS where they enter the model (e.g. PUSH to Queue1).

### Buffers (Queues)

Buffers store Parts (Entities). They do not actively pull Parts (Entities) in. Parts (Entities) are pulled out by some other Element or they are pushed out by a buffer exit rule.

Examples of the uses of Buffers (Queues) include:

- A factory floor holding parts awaiting their next operation

- A storage rack holding circuit boards before flow soldering

- The space containing aircraft waiting to land (a stack)

- A hopper at an assembly station containing product components

- People waiting in a line or queue

- Telephone call queues

- Reports in an in-box

- A waiting room at a hospital

Figure 2-10: Buffer (Queue) Detail Characteristics

| Table 2-7: Buffer (Queue) Details |  |
| --- | --- |
| Information | Description |
| Maximum Capacity | Maximum number of Parts (Entities) that can be stored. |
| Input Option | Position at which Part (Entity) enters the Buffer (Queue), rear, front, etc. |
| Output Option | Position from which part/entity will leave buffer/queue (first, etc.) and whether buffer will be searched from the front or rear. |

The ‘delays’ option on a buffer or queue can be altered to truly reflect its behavior. Listed below are the Buffer (Queue) options available.

| Table 2-8: Buffer (Queue) Delay Options |  |
| --- | --- |
| Options | Descriptions |
| None | The Buffer (Queue) behaves as a normal passive Buffer (Queue) and a Part (Entity) can be removed at any time by another Element. |
| min | The Buffer (Queue) behaves as a delay buffer and Parts (Entities) must spend a specified amount of time in the Buffer before another Element can remove them. This allows you to simulate Elements such as ovens and operations such as setting and cooling. |
| max | The Buffer (Queue) behaves as a dwell queue. This means that an Element may remove Parts (Entities) at any time, but if a Part (Entity) remains in the Buffer for longer than the maximum time specified. It will remove itself using the exit rule. If the exit rule fails, the Part (Entity) remains in the Buffer indefinitely, or until another Element removes it. This allows you to simulate callers hanging up the telephone after waiting for a while or to simulate people leaving Queues because the wait is too long. |
| both | The Buffer (Queue) behaves as a delay for Parts (Entities) that have been there for a minimum time, then behaves as a dwell queue for Parts (Entities) which have been there for the maximum time. |
| max repeat | The Buffer (Queue) behaves as a repeating dwell queue. This is the same as the max option, except that if a part or entity fails to output itself via the exit rule, it schedules an event to attempt to output again. It will do this at a time equal to the maximum delay time. The process is repeated indefinitely until either the rule succeeds or another Element removes the Parts (Entities). |
| Both repeat | The Buffer (Queue) behaves as a delay buffer for Parts (Entities) that have been in the Buffer (Queue) for the minimum time. It then behaves as a dwell queue for Parts (Entities) that have been in there for longer than the maximum time. |

### Machines (Activities)

Machines (Activities) are physical Elements that operate on a Part (Entity), often changing it in some way. Machines take time to process Parts (Entities), changing from one state to another as time advances. Examples of Machines and Activities are:

- A lathe

- A press

- A robot welder

- A palletizer

- A supermarket checkout

- An auto teller at a bank

- An ACD at a call center

A Machine or Activity can process one Part (Entity) at a time or many at a time. It can assemble many Parts (Entities) together into one or produce many out of one.

| Table 2-9: Machine (Activity) Basic Information |  |
| --- | --- |
| Information | Description |
| Input Rule | Brings in the part or the entity |
| Cycle Time Duration | Time taken to process the parts (entities) |
| Output Rule | Where the part or entity is sent to after the machine or activity has finished processing. |

Figure 2-11: Machine (Activity) Detail Characteristics

Machines (Activities) can also have Buffers (Queues) directly tied to them, one serving as an input Buffer (Queue) and one as an output Buffer (Queue). These dedicated Buffers may be defined when detailing the rules of a Machine (Activity) and then displayed using the display dialog of the Machine (Activity).

### Adding Elements Directly Into the Model

An Element can be put into the model directly, rather than put first into the Designer Elements Window.

Adding an Element Directly Into your Model

- Choose ModelElements. OR Press the <F4> function key. - The Elements dialog box appears.

- Click the Define button. - The Define dialog box appears. - Click on the “Simulation” option from the Parent field. - Notice Simulation is now highlighted in the Parent field.

- Type in an Element name in the Name input field. (You are limited to 24 characters including letters, numbers, and underscores). - With your left mouse button click on the down arrow to the right of your Element Type list box. - The list opens. - Choose the type of the Element (e.g. Machine/Activity).

- Click the Create button. - Notice you can no longer modify the Element Type of the newly created Element.

- Click the Display button. - Create the display for your new Element.

- Click the Detail button. - Create the detail for your new Element. - Repeat steps 4-9 for each of the new simulation Elements that you require. - Click the Close button once to go back to the Elements dialog box and a second time to go back to your model. - Steps 8 and 9 are interchangeable.

In some cases, you will need to update the Displays and Details characteristics of multiple elements efficiently. There are two ways of obtaining this goal. The first method includes selecting multiple elements from the Element Selector Dialog box and clicking on either the Display or Detail Buttons. In the second case, you create a net around a series of elements in your model. Then, depending on whether you need to update or modify the Display or Detail characteristics, you will either double click on the left or right button of the mouse while keeping the mouse pointer in this net. In both cases, WITNESS will cycle through all the elements you have selected.

Figure 2-12: Element Selector Dialog

Figure 2-13: Element Define Dialog

### Running a Model in WITNESS

The Execute Action Bar is displayed at the bottom of the screen. It contains buttons that run the model in various modes. These buttons resemble the symbols commonly used on video recorder controls).

The Execute Action Bar also contains text boxes to the right of the bar. These input fields allow you to define limits for the button action. See the table below for an explanation of each button and appropriate text box.

| Table 2-10: Execute Action Bar |  |
| --- | --- |
| Button | Description |
| Begin | The Begin button resets the model (that is, it resets the clock to time zero, clears statistics and sets Elements to an idle state). Alternatively, use RunBegin. |
| Autostep Backwards | The Autostep Backwards button allows you to step backwards in time to the previous .sim file in a sequence of saved .sim files. It also takes Run Until instructions into account. |
| Stop | The Stop button stops a run in any of the chosen modes. |
| Record | The Record button records the simulation to a set of .sim files. |
| Step | The Step button lets you examine the run step-by-step and takes Run Until instructions into account. Press the spacebar to advance each step. Alternatively, use RunStep. |
| Run | The Run button runs the model with the simulation visible on all currently open windows and takes Run Until instructions into account. Alternatively, use RunRun. |
| Autostep forwards | The Autostep forwards button allows you to step forwards in time to the next .sim file in a sequence of saved .sim files. It also takes Run Until instructions into account. |
| Batch | The Batch button runs the model in batch mode. Running in batch mode closes the simulation windows and takes Run Until instructions into account. If you have not entered instructions in the Run Until text box, the model runs indefinitely, or until you stop it. Alternatively, use RunBatch. |
| Run Until | The Run Until button is used in conjunction with the text box to its right. - Click on the Run Until button. - In the text box enter a time to batch to, or a time to step forwards or backwards. - The simulation runs until that time is reached. |
|  | OR Enter the name of an Element. - The simulation runs until an event occurs for that Element. If you do not enter anything in the text box, the run continues indefinitely, or until you stop it. |
| Walk speed | The Walk Speed button selects the walk run mode and indicates the speed that the model runs at when you select the walk run mode. The scroll bar sets the walk speed of the animation. Alternatively, use RunWalk. |
| Trace | The Trace button creates an audit of progress. It reports to a file with the extension .trc and provides you with a textual record of each step in much the same way as step does but without the need to press the spacebar after each step. You can edit the .trc file later by using a text editor. In step, batch or run mode all steps are recorded in the trace file until you exit, read in a new model or turn it off by clicking on the button again. |
| Meteor trail | The Meteor Trail button turns the meteor trail on or off. Alternatively, use RunMeteor Trail. |
|  | The current simulation time text box. |

As the model is running, you can display machines, vehicles, buffers, conveyors, processors, pipes and tanks so that they change color in the simulation whenever they change state (e.g. when they become busy or broken down). This is useful for tracing the operation of the model.

You can use the ViewKeys option from the top menu to display color keys on the screen to remind you of the meaning of each color. (See Figure 2-14)

Figure 2-14: Color Keys for Element Status

### Getting Standard Reports from WITNESS

Reports help you identify areas in your model where the operation may be improved. In WITNESS, there are several ways to obtain reports.

Obtaining Reports in WITNESS #1

- Select the ReportsReports option from the top menu. - The Report window appears. (See Figure 2-15) - Select the Elements you want to view the reports on by clicking on the box next to the Element names. OR Click on the Select All button to choose all of them.

- Click on the Statistics button. - Displays report on the screen. - This method allows you to cycle back and forth among the different reports by clicking on either or button

Figure 2-15: Reports Dialog Window

Obtaining Reports in WITNESS #2

- Click on the button. - The Current Element selector window appears. (See Figure 2-16) - Select the Elements to be analyzed by choosing the Selection Method from the drop-down list (Single Element, Type, All). - Click OK - The Element List field on the Standard Action Bar is updated.

- Click on the button. - Displays report on the screen. - WITNESS cycles through the selected reports when you click on the CLOSE button

Figure 2-16: Current Element Selector Window

Obtaining Reports in WITNESS #3

- Click on a specific Element name on your main screen (notice the Element List field is updated with the name of the Element your selected.

OR Type the name of the Element directly into the Element List field. (You could also type All Elements, or a specific group name of Elements such as machines or buffers.

- Click on the button. - Displays report on the screen.

Statistical Reports provide the most detail when they are displayed in numeric format (See Figure 2-17). Alternatively, the display of the report grid can be modified in numerous ways:

- Hide or move individual rows and columns.

- Sort columns and rows in an increasing or decreasing order.

- Swap row and columns.

- Expand the window of the report.

- Cut and Paste date from WITNESS to Excel.

- For additional information on these options go to the “Changing the Display of the Report Grid” topic on the WITNESS Help Facility.

Figure 2-17: Machine Report Numeric Format

You may also view reports in a graphic format as well, by using the Chart and Chart Status buttons of the appropriate statistics report. Graphical reports can sometimes make it easier to identify areas of concern in an Element report (See Figure 2-18).

Figure 2-18: Machine Report Graphic Format

Changing the graphic type of a report is simple. Click on the right button of your mouse on top of your graph and make a selection between vertical, horizontal and stacked bars.

Figure 2-19: Graph Report Choices

In addition to the statistical reports we just mentioned, WITNESS provides you with other types of reports. The following is a list of buttons used to generate various types of reports.

| Table 2-11: Button Descriptions |  |
| --- | --- |
| Button | Description |
| Select Element | The select Element button allows you to select a specific Element, or range of Elements, from a list of all Elements. When you click on this button, WITNESS displays the Element selector dialog, which allows you to select an individual Element, a group of Elements, or a single indexed Element from a group. A description of the selected Element, or range, is displayed in the box to the right of the select Element button. |
| Element list | The Element list contains the 10 most recently selected Elements. You can select an Element from this list by clicking on it, then click on the display or detail button to amend display or detail data for that Element. |
| Statistical Report | The statistical report button generates a statistical report of data collected for the selected Element. |
| Summary Report | The summary report button generates a summary report of data collected for the selected Element. |
| Explode Report | The explode report button generates a report of data collected for the selected Element. |
| Used Report | The used button generates a report of where the selected Element is used. |

- Do exercises 1 and 2 now.

## Modeling Stoppages

### Simulating Stoppages

It is very unlikely that Elements within the model will run continually during a specified period of time. Simulating stoppages gives you the ability to replicate reality. Stoppages are a way to force the Machines or Activities or Conveyors to stop.

In WITNESS, you are able to model two types of stoppages:

- Setups

- Breakdowns

Each of these will be explained in detail, taking you through the stages of setting them up.

#### Setups

Regular stoppages can be modeled using setups. These can be used to model tool changes, maintenance, or similar stoppages that occur after a number of operations or because of a change in the process. Setups can only be applied to Machines (Activities) in WITNESS.

The basis for making a setup may be:

- A number of operations (frequency). In this case an initial life may be specified

- A change of value

- A change of Part or Entity type

A Setup Time must be specified in each case.

Figure 2-20: Setup Property Sheet

Stages to Adding Setups

- Select the detail dialog for the appropriate Machine or Activity. - Click on the Setup tab to take you into the setup property sheet. - Click on the Add/Remove button. - Enter a description for the specific Setup (which is used for reference purposes to describe each setup, e.g. Monthly Maintenance). - Click on ADD. - Repeat steps 4 and 5 for each different type of Setup. - It is possible to specify up to 99 separate Setups on a Machine (Activity). - Once you have added all of the Setups required click on OK. - You return to the main Setup property sheet.

- Select the appropriate option from the Setup Mode drop-down list box on the main Setup property sheet (e.g. No of Operations or Activities, Part (Entity) change and Value Change). - Enter the Setup interval (if required). - Enter the Setup time. - Repeat these steps for each setup added previously.

#### Breakdowns

Machines (Activities) and Conveyors can break down in WITNESS. In order to include this it is necessary to specify that breakdowns will occur in the Element details.

Figure 2-21: Breakdown Property Sheet

| Table 2-12: The Three Breakdown or Stoppages Modes |  |
| --- | --- |
| Mode | Description |
| Available Time | Breakdown occurrence takes into account all states except off-shift time for the machine. |
| Busy Time | Breakdown occurrence is dependent on the amount of busy time that the Element has incurred. |
| Number of Operations | Breakdown occurrence is dependent on the number of operations that the Machine has completed. |

There is also the option to scrap the part or entity when the machine breaks down. If the part is scrapped, then after it has been fixed, a new part or entity is pulled in using the Input Rule. If the Part (Entity) is not scrapped, then when the machine restarts, it continues to process the part or entity present when it stopped.

The final option, Setup on Repair, on the Breakdowns property sheet enables you to specify that when the Machine (Activity) breaks down, any setup which is nearly due is done whilst the machine is being repaired. The % Life Used field enables you to specify how near to being due the Setup needs to be if it is going to be done when the machine breaks down. For example, if a setup is performed every 100 operations and a % Life Used of 75% is specified, then if the Machine breaks down after 75 or more operations the Setup is done; if it breaks down after less than 75 operations, the Setup is not done.

Stages to Adding Breakdowns

- Select the detail dialog for the appropriate Machine or Activity. - Click on the Breakdowns tab to take you into the Breakdowns property sheet. (See Figure 2-21) - Click on the Add/Remove button. - Enter a description for the specific Breakdown (which is used for reference purposes to describe each breakdown, e.g. Motor Jamming). - Click on ADD. - Repeat steps 4 and 5 for each different type of breakdown. - It is possible to specify up to 99 separate breakdowns on a Machine (Activity). - Once you have added all of the breakdowns required click on OK. - You return to the main Breakdowns property sheet.

- Select the appropriate option from the Breakdown Mode drop-down list box on the main Breakdowns property sheet (e.g. Available Time, Busy Time, No of Operations/Activities). - Enter the Breakdown/Stoppage interval - Enter the Repair time e.g. time stopped - Repeat these steps for each Breakdown added.

#### Interactive Breakdowns

To enable you to illustrate the effect of any Breakdown immediately without having to schedule the Breakdown, WITNESS contains an option to breakdown and repair Machines (Activities) interactively.

- These options can also be accessed from the Model menu.

To Interactively Stop Machines (Activities)

- Stop the model running. - Select the appropriate Machine (Activity) by clicking on the Element on the screen. - The name appears in the select box on the Standard Action Bar. - Click on the hammer icon on the Standard Action Bar. - The machine immediately breaks down.

To Interactively Start Machines (Activities)

- Stop the model running - Select the appropriate Machine (Activity) by clicking on the Element on the screen. - The name appears in the select box on the Standard Action Bar. - Click on the spanner icon on the Standard Action Bar. - The Machine or Activity immediately restarts.

- Do exercise 3 now.

## Physical Elements: Labor and Conveyors

### Adding Labor Resource to Your Model

Labor is a resource that may need to be present for an operation or task to take place. For example, operators, inspectors, tools, and fixtures may all be represented by Labor. Labor resource may be used for cycling (e.g. operating), repairing or setting-up a Machine or Activity and for repairing a Conveyor.

Add Labor Resource to Your Model

- From the Designer Elements Window (BUILDING BLOCKS), click with the left mouse button on the Element name LAB (or RES). - A cross arrow appears.

- Position the cross arrow where you wish the labor to be placed in the simulation window. - Click with the left mouse button. - LAB001 (or RES001) appears on the screen with a net around it.

- Click and drag inside this net in order to move LAB001 (or RES001) to the required position. - If you have selected the Multiple Designer Elements option from the ModelOptionDisplay menu, you will also need to click the right button of your mouse before dragging the newly created element.

Displaying the element determines the idle position for the labor; e.g., where the labor will be positioned on the screen when not performing any task. The name of the labor can be changed by entering the detail of the Element and amending the Name field. The display can also be customized.

Figure 2-22: Labor Resource Detail Dialog Box

Once labor has been added to the model, it then needs to be attached to tasks.

Figure 2-23: Labor Resource Rule Dialog

Add Labor Resource to a Machine (Activity)

- Click on required Element to which you wish to assign the labor resource - The name appears in the select box on the Standard Action Bar - Select the labor resource icon from the Standard Action Bar. - The Labor Resource Rule dialog appears on the screen. - From the drop-down list, select the appropriate task to which to assign your resource (e.g. Cycle/Operate, Setup, and Breakdown/Stoppage). - Specify the quantity of the resource required in the quantity box - Click on the resource Element on the screen or type in the name of the Labor resource.

- If another type of resource is going to be assigned to the same task, select AND or OR from the condition box before clicking on the second resource. This will determine whether both types of resource must be present together or if there is a choice between the two resources. - Select the SAVE button to confirm the choice. - If needed, return to step 3 to assign the labor resource to other tasks.

Whenever more than one setup and/or breakdown are defined for a specific Element, an additional field would become visible on the labor rule dialog of that Element. This field would allow you to assign labor to each one of these setups and/or breakdowns. (See Figure 2-24)

Figure 2-24: Labor Rule Dialog for Element Mach001

Examples of the sort of rules you may use include:

| OPERATOR | 1 operator required |
| --- | --- |
| OPERATOR#2 | 2 operators required |
| JANE AND WENDY | 2 labor units required |
| FRED OR JOE | 1 of 2 labor units required |

It is also possible to build up complex rules, e.g. conditional rules - see below, by selecting the Edit button from the Labor Resource Rule dialog.

IF X=1

FRED

ELSE

WAIT

ENDIF

If X=1, use FRED, otherwise wait until X=1.

As soon as an Element requires Labor, its state changes to ‘waiting for Labor’ and will remain so until the specified quantity of Labor is available. If several Elements are waiting when Labor becomes available, the Labor will move to the Element with the highest priority. If more than one has the same priority, then the ‘oldest’ job will be chosen. The priority of the Machine or Activity is set on the detail dialog for the machine.

- The highest priority is 1, second highest is 2 etc. - the default priority is ‘lowest’.

#### Pre-emption

Normally, the Labor Resource will remain with an Element until the process is completed. However, it is possible to pre-empt the Labor resource from its current task in order to perform more critical operations. The labor resource will leave the job it is currently working on, before that job is complete, in order to work on a more important job.

A Pre-empt level can be set that specifies the difference in priority required (between the task the Labor Resource is currently carrying out and the task that is being Pre-empted to) to enable Pre-emption to occur. A sufficient difference in priority must exist for Pre-emption to take place. Look at the following formula, which determines if Pre-emption is to take place.

| Priority of Element which currently has the Labor |  | Priority of Element which requires the Labor |  | Pre-empt level of Element that requires the Labor |
| --- | --- | --- | --- | --- |

See the following figure, Figure 2-25, which illustrates this formula.

Figure 2-25: Pre-empt Level of Element which requires labor

Labor resource will be moved from Task3 to Task1 since the difference in priority is 2 and the Pre-empt level is 2. Labor resource will also be moved to Task2 from Task3. However, Labor resource will not be moved to Task1 from Task2 since there is a difference in priority of 1, but a Pre-empt level of 2. In other words, there is not a sufficient difference in priority to warrant moving labor from Task2 to Task1.

It is also possible to specify an allowance to finish. If there is less than a specified amount of time before the completion of the current operation, the Labor will not be moved, but it will complete its current task. (See Figure 2-26)

In addition, if the Labor resource is pre-empted, it is possible to specify a penalty time. This time represents the fact that when the Labor returns to a job from which it was pre-empted, it may not be able to continue the job and complete it as if the job had not been interrupted. A penalty time may then be entered and this penalty time is incurred when Labor returns to the interrupted task. The penalty time is added to the time remaining on the interrupted task.

Figure 2-26: Pre-Emption on Labor Detail Dialog

In order to pre-empt Labor to a task, the detail of the task to which the labor is to be pre-empted needs to be edited.

Edit the task detail

- Click on the Element to which you wish to pre-empt the Labor. - The name appears in the select box on the Standard Action Bar.

- Select the detail icon from the Standard Action Bar. - The Machine or Activity detail dialog appears on the screen.

- Set the priority of the Element (if not already done so).

- Select the Labor Resource Rule button to open the labor rule editor.

- Click on the Pre-empt Labor check box to obtain the pre-emption fields.

- Enter values for Pre-empt Level, Allowance and Time Penalty

- Select the OK button to confirm the choice.

- Repeat steps 4 - 7 for Setups and Breakdowns, as appropriate, by selecting the appropriate property sheet from the main Machine (Activity) dialog box.

- Do exercises 4 and 5 now.

### Conveyors

Conveyors are one means of satisfying the material movement requirements of your operation. Some examples of Conveyors are:

- A belt transporting luggage in an airport

- A conveyor moving truck bodies along a production line.

- A roller conveyor moving empty cartons to a packing operation.

Figure 2-27: Fixed and Queuing Conveyors

There are two types of Conveyor in WITNESS. ‘Fixed’ Conveyors maintain a constant distance between the Parts (Entities). If the Conveyor stops, the distance between them on the Conveyor remains the same. In contrast, ‘Queuing’ Conveyors allow Parts (Entities) to accumulate until the Conveyor is full. In both cases, Parts (Entities) are said to enter a Conveyor at the rear and move towards the front. (See Figure 2-27)

Conveyors move Parts (Entities) across a facility, generally from one fixed point to another. In addition, the discrete positions that it occupies as it moves along the Conveyor can also be used for loading and unloading. Both fixed and queuing Conveyors can break down and may require Labor Resource for repair.

Figure 2-28: Conveyor Detail Dialog Box

Add a Conveyor to Your Model

- From the Designer Elements Windows, click with the left mouse button on the Element name Conv. - A cross arrow appears.

- Position the cross arrow where you wish the Conveyor to be placed in the simulation window and click with the left mouse button. - Conv001 appears on the screen with a net around it.

- Click and drag inside this net to move Conv001 to the required position. - If you have selected the Multiple Designer Elements option from the ModelOptionDisplay menu, you will also need to click the right button of your mouse before dragging the newly created element

The name of the conveyor can be changed by entering the Detail dialog box for the Element and amending the Name field. (See Figure 2-28) The following details of the Conveyor can also be amended.

| Table 2-13: Conveyor Detail Options |  |
| --- | --- |
| Item | Description |
| Length in Parts (Entities) | Uses the number of Part (Entity) positions on the Conveyor to define the length of the conveyor. If the Conveyor is 30 meters long, and a Part (Entity) is 0.5 meters long, there is a maximum of 60 positions for the Parts (Entities). |
| Max. Capacity | May be less than the Part (Entity) length due to operating constraints e.g. weight restrictions. |
| Index Time | This is the time required to index or move one position on the Conveyor. |
| Restart Delay | Time that a conveyor motor takes to get up to speed if the conveyor has stopped because its destination was not free (that is, the conveyor has become blocked) |

The use of a large number of Conveyors in a model will affect run speed. Conveyor cycles are often small and this creates a large number of events. In order to improve run speed; Queuing Conveyors may be represented by Buffers or Queues with a delay time. This improves run speed.

#### Conveyor Input and Output

Care must be taken when linking Conveyors to Machines (Activities). Conveyors pulling/pushing, is different from the Machines (Activities) pushing/pulling. Conveyors only attempt to input and output on their cycle, while Machines (Activities) will input as soon as a Part (Entity) is available and output when there is a free space.

As soon as Task1 finishes its cycle, it will output to CON1, if a space is available at the back of the Conveyor.

CON1 will only attempt to input when it cycles. When Task1 finishes its cycle, it will block until CON1 cycles and pulls in the Part (Entity) from the Machine (Activity).

The correct Input and Output Rules should be used dependent on the facility being represented. Where Parts must be placed in fixed positions on a Conveyor (e.g. a sling conveyor), a Conveyor Input Rule should be specified. Where Parts may be placed on a Conveyor at any time (e.g. a belt conveyor or roller conveyor) a Machine (Activity) Output Rule should be specified. It is also possible to Pull/Push from Conveyor positions other than the default Front/Back by specifying the position number within parentheses.

PUSH to CON1 at (3)

- Do exercise 6 now.

## Adding Detail to Machines and Activities

### Machines and Activity Types

A Machine (Activity) can process one Part (Entity) at a time or many at a time. It can assemble many together into one or produce many out of one. WITNESS provides six types of Machines (Activities) to cater to these requirements.

Single Machine/Activity - A single Machine or Activity processes one single Part or Entity at a time and outputs that single Part or Entity.

Batch Machine/Activity - A batch Machine (Activity) processes a batch of Parts (Entities) at a time. If the Machine is a BATCH Machine, set the type to BATCH and specify the minimum and maximum number of Parts (Entities), which would constitute a batch.

Assembly Machine or Join Activity - An assembly Machine takes in a number of Parts and outputs a single Part. If the Machine is an ASSEMBLY Machine, set the type to ASSEMBLY and specify the number of Parts that are assembled into one new Part.

Production Machine or Split Activity - A production Machine takes in a single Part and outputs a number of Parts. If the Machine is a Production Machine, set the type to Production and specify the production quantity. Remember that the number of Parts output from such a Machine equals the production quantity PLUS 1.

General Machine/Activity - A general Machine takes in a number of Parts and outputs a different number of Parts. If the Machine is a General Machine, set the type to General and specify the input quantity and the output quantity.

Multi-Station Machine/Activity - A Multi-Station Machine processes one or more parts through a number of stations. Parts move in unison through the Machine and only advance if an input Part is available. This prevents ‘gaps’ in the Machine.

If the Machine is a Multi-Station machine, set the type to Multi and specify the number of stations and the number of Parts per station. If the number of Parts is more than 1, the Machine is effectively a Multi-Station Batch machine.

A Machine can have dedicated input and output buffers where Parts are stored adjacent to it. The time it takes for a Machine to process a Part, the time between failures, the time to repair the Machine after a breakdown or the time to be setup after a changeover, can all be easily modeled. If Labor is required to run, repair, or setup the Machine, this can also be modeled.

Figure 2-29: Machine Detail Form and Types of Machines

Set the Machine (Activity) Type

- Click with the left mouse button on the Machine (Activity) Element name on the screen. - The name appears in the box on the Standard Action Bar.

- Select the detail icon from the Standard Action Bar - The Machine (Activity) Detail dialog box appears on the screen.

- Click on the drop-down box for the Type.

- Select the appropriate type (e.g. Single, Batch etc.).

- Set the Input and Output Quantity as required - If this field is grayed out, it cannot be changed.

- Select OK to save changes.

### Input and Output Rules

Input and Output Rules are used to describe how parts should flow between modeling Elements. An Input Rule is used to get the Parts into an Element and an Output Rule is used to get the Parts out of an Element. If an Element does not actively Pull in Parts, it has the Input Rule WAIT. Such Elements never receive Parts unless other Elements Push Parts to them. Similarly, an Element, which does not actively Push out parts, has the Output Rule WAIT. These Elements may become permanently blocked unless other Elements Pull from them. WAIT is the default rule - it is attached to each Element when it is put in the model. Parts will not move if you attempt to run a model without first specifying Input or Output Rules. The simplest Input and Output Rules are PULL and PUSH.

PULL from INTRAY

| causes | the Element with this rule to Input Parts from the Element called INTRAY. |
| --- | --- |

PUSH to DRILL

| causes | the Element with this rule to Output its parts to the Element called DRILL |
| --- | --- |

The PULL and PUSH Rules allow you to specify a list of Elements in preferred order.

PUSH to LATHE, DRILL

| causes | the Element with this Rule to attempt to Push its Parts (Entities) to the Element named LATHE; if this fails it will attempt to Push to the Element named DRILL. |
| --- | --- |

PUSH A to LATHE, B to DRILL

| will | push all A Parts to the LATHE and all B Parts to the DRILL. |
| --- | --- |

An element can PULL a specific Part (Entity) from an imaginary location, outside your model, where an infinite raw material is available at any point in time. This location in pre-defined in WITNESS as WORLD.

PULL A out of WORLD

| will | pull Parts As from WORLD into your model. |
| --- | --- |

Elements such as Machines (Activities) may either Pull or Push, both, or neither. You should choose whether each Element is ‘active’ or ‘passive’ according to the role of that Element in real life. For example, Machines may be said to Pull Parts from a bin, process them, and Push the resultant Parts on to the next process.

You should not define any Element where it both Pushes and is Pulled from; or both Pulls and is Pushed to. These definitions would make it difficult to predict which Element has precedence, the Pusher, or the Puller.

Figure 2-30: Input Rule Dialog

| Enter an Input Rule for a Machine (Activity) |  |
| --- | --- |
| - Click on the Element into which you wish to bring Parts (Entities). - The name of this Element appears in the select box on the Standard Action Bar. |  |
| - Select the Input Rule icon from the Standard Action bar. - The Input Rule dialog box appears on the screen. |  |
| - For a simple rule (e.g. PULL, SEQUENCE, PERCENT), select the Rule you wish to use from the drop-down box. |  |
| - Click on the Element or Elements from which you wish to take Parts (Entities). | - Click on the Edit button to enter the full Input Rule dialog box. (See Figure 2-30) |
| - Click on OK. | - Enter the appropriate Input Rule. |
|  | - Click on OK. |

- It is also possible to enter the rule editor from the main detail dialog of a machine or activity by clicking on the From... or To… buttons on the main detail dialog for the Input or Output Rule respectively.

Figure 2-31: The Rule Editor

A list of the available Input or Output Rules may be obtained while in the Rule Editor in two ways.

Select EditInsert Rule...

OR

Select ViewAction Bars

Select Rules from the floating Selector Action Bar

In either case, a dialog box, (See Figure 2-32), will be displayed from which the appropriate Input or Output Rule may be chosen. There is also a Prompt facility, which helps to build up the rule in the correct format.

Figure 2-32: The Rule Select Dialog

The following two tables summarize the most frequently used Input and Output Rules.

| Table 2-14: Summary of Input Rules |  |
| --- | --- |
| Rule | Description |
| WAIT | Until Part is pushed to station. |
| PULL | From list of stations in a preferred-order list. PULL from B1, B2 First Pull from B1 and if this fails Pull from B2 PULL A from B1, B from B2 Pull Part A from B1 and if this fails, pull Part B from B2. |
| MOST | Parts/Free space in a list of stations. MOST PARTS B1, B2 Take a Part from B1 or B2, whichever has most Parts. |
| LEAST | Parts/Free space in a list of stations. LEAST FREE B1, B2. Take a Part from B1 or B2 whichever has least space. |
| PERCENT | Sample from list of stations at random. PERCENT /1 B1 70.00, B2 30.00 Parts are pulled randomly using random number stream 1, 70% from B1, and 30% from B2. |
| SEQUENCE | From list of stations, with Failure Rule, which tells the system what to do, if, no Part is available from the next place in the sequence. SEQUENCE/WAIT B1#(1), B2#(2), B3#(1) Take 1 Part from B1 then from B2, then 1 from B3. If a Part is not available, wait. SEQUENCE/NEXT B1#(1),B2#(2),B3#(1) As above, but if a Part is not available, go on to try the next stage of the sequence. SEQUENCE/RESET B1#(1), B2#(2), B3#(1) As above, but if a Part is not available, reset the sequence and try from the beginning. |
| IF | An expression is true. IF NPARTS (B1) > 5 PULL from B1 ELSE PULL from B2 ENDIF If there are more than 5 Parts in B1 take a Part from B1, otherwise take a Part from B2. |
| SELECT | According to expression SELECT on X B1, B2 If X = 1 take a Part from B1 otherwise if X = 2 take a Part from B2. |
| BUFFER: | Create Input Buffer, for Machines (Activities) only. BUFFER(5) Machine has an Input Buffer of capacity 5. |
| MATCH | Allows Machine or Activity to input matching sets of Parts for processing. MATCH/ANY (B1#1 AND B2#2) Allows the Machine (Activity) to match any Part (Entity). The above example will select one Part from B1 and two Parts from B2. MATCH/ATTRIBUTE COLOR CHASSIS#1 AND DOORBUFF#4 Selects Parts with the same value of a specified attribute. The example only takes Parts when WITNESS finds one item from the CHASSIS buffer and 4 items from the door buffer with the same color. MATCH/CONDITION (ATT2=5 AND ATT3=1) BUF1#1 AND BUF2#1 OR BUF3#2 Selects parts which meet certain conditions, which are normally, attribute based. The machine can obtain a Part from BUF1 and BUF2 or both from BUF3. The condition on the selection is that the attribute ATT2 is equal to 5 and ATT3 is equal to 1. |

| Table 2-15: Summary of Output Rules |  |
| --- | --- |
| Rule | Description |
| WAIT | Until Part is pulled from station |
| PUSH | To list of stations in priority order |
| MOST | Parts/Free space in a list of stations |
| LEAST | Parts/Free space in a list of stations |
| PERCENT | Sample from list of stations at random |
| SEQUENCE | To a list of stations with Failure rule |
| IF | An expression is true |
| SELECT | According to expression |
| BUFFER | Create output Buffer (Machines only) |
| DESTINATION | According to destination (Tracks only) |

MATCH can not be used for Output Rules.

All rules may be combined with the IF rule with the exception of PERCENT and SEQUENCE. Parts may be Pulled in from the WORLD or leave the model by being Pushed to SHIP, SCRAP, or ASSEMBLE. If Parts are actively arriving but cannot get into the model (e.g. an arrival Buffer or Queue is full), they will be rejected and never enter the model.

- Do exercise 7 now.

## Examples

You could find the actual models used in this section in your Student Demo Disk, provided to you with this manual. Look for CASE_2_1.MOD and CASE_2_2.MOD for the TV and Automated Sensor Assembly Line case studies, respectively. You could also obtain a listing of the entire model in an ASCII format by saving any model as an LST file.”

### TV Manufacturer

A small manufacturer of television sets produces three different varieties of sets. They are doing a brisk business, and they cannot keep up with customer demand. The company must cut their lead times to maintain their market share, while at the same time trying to gain a larger percentage. The problem area in the manufacturing process has been determined to be the inspection and test. This is the last step in the process before the televisions are shipped, so this area must be improved to increase throughput.

The three types of television sets arrive to the inspection area from final assembly. The load sequences pulling in one TV1, then one TV2, and one TV3 from a pool of TVs outside the model (final assembly area). The TV’s must be loaded into the calibration station. An operator (L1) loads the TV’s and then calibrates them using a special (and highly costly) gage. After the TV is calibrated, it is unloaded to a belt conveyor.

The belt conveyor transports TV’s to the inspection station. An inspector (L2) performs various tests to determine quality. 75% of the TV’s are good and these are passed to a burn-in station. The 25% that are rejected are sent to a rework station where a technician (using the gage) must trouble shoot the TV. After trouble shooting, 90% are sent back to the belt for re-inspection, while 10% are scrapped.

After the burn-in is performed, 90% are ready to ship, while 10% must be adjusted and then shipped.

The existing system must be simulated in order to establish a correct baseline before experimentation. Several alternatives have been proposed. These must be evaluated to determine the best use of finite capital. The budget, along with the cost of alternatives is given below. Your assignment is to maximize the number of TVs produced while staying within the budget.

Budget: $40,000

| Table 2-16: Capital Expenditure Alternatives |  |
| --- | --- |
| Alternatives: | Cost |
| - Add a second gage | $10,000 |
| - Cut set-up time on load by 50% | $5,000 |
| - Reduce reject rate at inspect to 5% | $10,000 |
| - Change belt conveyor to queuing type | $10,000 |
| - New Burn-in station that can handle 5 TVs | $20,000 |
| - Decrease cycle time at any station by 25% (may be used multiple times) | $10,000 |

Figure 2-33: Post Assembly and Inspection Area

| Table 2-17: Case Study Data |  |  |
| --- | --- | --- |
| Calibration Unit | Cycle Times | Labor |
| Load | 15.0 | L1 |
| Calibrate | 4.5 | L1 and Gage |
| Unload | 1.5 | L1 |
| Belt* | 1.0 | None |
| Rework | 15.0 | Tech and Gage |
| Inspect | 5.5 | L2 |
| Burn_in | 15.0 | None |
| Adjust | 7.5 | None |

- * The length of the Belt Conveyor in Parts is 5 units and the Cycle Time column is used for the Belt Index Time.

| Time Frame: |  |
| --- | --- |
| Hours of Production | 38 hours |
| Warm-up Period | 8 hours |
| Total Run Length | 46 hours |

#### Simplified Model Listing

##### Define

PART: tv1,Variable attributes;

PART: tv2,Variable attributes;

PART: tv3,Variable attributes;

BUFFER: burn_in,1,1;

MACHINE: calib,1,General,0,0,1,0;

MACHINE: inspect,1,Single,0,0;

MACHINE: adjust,1,Single,0,0;

MACHINE: rework,1,Single,0,0;

MACHINE: outburn,1,Single,0,0;

LABOR: L1,1;

LABOR: L2,1;

LABOR: gage,1;

LABOR: Tech,1;

CONVEYOR: belt,1,Fixed,5;

##### Detail

tv1

NAME OF PART: tv1;

OUTPUT RULE: Wait;

tv2

NAME OF PART: tv2;

OUTPUT RULE: Wait;

tv3

NAME OF PART: tv3;

OUTPUT RULE: Wait;

burn_in

NAME OF BUFFER: burn_in;

QUANTITY: 1;

CAPACITY: 1;

DELAY MODE : Min;

* Minimum Time : 15.0;

INPUT POSITION: Rear;

calib

NAME OF MACHINE: calib;

QUANTITY: 1;

TYPE: General(Multi-Cycle);

SETUP_DETAIL

Setup number: 1

* Mode: Part change;

* Setup time: 10.0;

* Description: Change over

* Station number: 1;

LABOR:

Setup: L1;

END

END SETUP_DETAIL

CYCLE_DETAIL

Cycle number: 1

* Cycle time: 1.5;

* Input quantity: 1;

* Finish quantity: 1;

* Description: load TV

LABOR:

Cycle: L1;

END

Input Rule: SEQUENCE /Next tv1 out of WORLD#(1),

tv2 out of WORLD#(1), tv3 out of WORLD#(1);

Cycle number: 2

* Cycle time: 4.5;

* Input quantity: 0;

* Finish quantity: 1;

* Description: Calibrate

LABOR:

Cycle: L1 AND gage;

END

Input Rule: Wait;

Cycle number: 3

* Cycle time: 1.5;

* Input quantity: 0;

* Finish quantity: 1;

* Description: Unload

LABOR:

Cycle: L1;

END

Input Rule: Wait;

END CYCLE_DETAIL

OUTPUT RULE: PUSH to belt at Rear;

Inspect

NAME OF MACHINE: inspect;

QUANTITY: 1;

TYPE: Single;

LABOR:

Cycle: L2;

Pre-empt level: None;

END

CYCLE TIME: 5.5;

INPUT RULE: PULL from belt at Front;

OUTPUT RULE: PERCENT /75 burn_in 75.00 ,rework 25.00 ;

adjust

NAME OF MACHINE: adjust;

QUANTITY: 1;

TYPE: Single;

CYCLE TIME: 7.5;

INPUT RULE: Wait;

OUTPUT RULE: PUSH to SHIP;

rework

NAME OF MACHINE: rework;

QUANTITY: 1;

TYPE: Single;

LABOR:

Cycle: Tech AND gage;

END

CYCLE TIME: 15.0;

INPUT RULE: Wait;

OUTPUT RULE: PERCENT/600 belt at Rear 90.00 ,SCRAP 10.00;

outburn

NAME OF MACHINE: outburn;

QUANTITY: 1;

TYPE: Single;

LABOR:

Cycle: L2;

END

CYCLE TIME: 0.5;

INPUT RULE: PULL from burn_in;

OUTPUT RULE: PERCENT /700 SHIP 90.00 ,adjust 10.00 ;

L1

NAME OF LABOR: L1;

QUANTITY no shift: 1;

L2

NAME OF LABOR: L2;

QUANTITY no shift: 1;

gage

NAME OF LABOR: gage;

QUANTITY no shift: 1;

Tech

NAME OF LABOR: Tech;

QUANTITY no shift: 1;

Belt

NAME OF CONVEYOR: belt;

QUANTITY: 1;

TYPE: Fixed;

PART LENGTH: 5;

MAX CAPACITY: 5;

INPUT RULE: Wait;

OUTPUT RULE: IF NPARTS(Inspect)+NPARTS(rework)=2

PUSH to SCRAP

ENDIF;

CYCLE TIME: 1.0;

### A High-Volume Assembly Line

#### Model Courtesy of Honeywell MICRO SWITCH

The Automated Sensor Assembly Line (shown below) is a multi-million dollar, high-volume assembly line. The end-product sensor is a new, high precision engine control sensor used in the automotive industry.

Figure 2-34: High Volume Assembly Line Layout

The assembly line uses conveyors to move pallets of eight sensors. These sensors are moved from the start of the assembly process (labeled ASSM1 in the model micro.mod) through multiple assembly stations, process checks, and cure cycles, until the pallets are unloaded at the Visual Inspect and Package station (VISPACK1). All conveyors are “fixed” except the two cure oven conveyors, which are queuing.

A simulation project was initiated with the following two goals:

- Increase the throughput of the line in the short term, and

- Have a model that can be used in the future to test alternative manufacturing processes.

The model was built first with estimated cycle times and no set-ups or breakdowns. The purpose of this phase was to correctly model the assembly line logic (to “verify” the model logic.) Next, actual cycle time data was collected and modeled. Then, non-cyclic elements were added to the model -- set-ups, changeovers, breakdowns, process checks, etc. Finally, the model was validated by the manufacturing engineering team.

The simulation project was successful by showing ways to increase throughput by 48 percent over the current situation. The simulation model identified the bottleneck operation. Then the model was used to test alternatives to remove this bottleneck.

The WIREBOND machine has eight separate breakdowns described below. All breakdowns are based on the available time and required laborer, WB_OPER, who is solely responsible for the WIREBOND machine. The time unit is minutes.

| Table 2-18: WIREBOUND Machine Breakdowns |  |  |
| --- | --- | --- |
| Description | Time Between Failures | Repair Time |
| Flameoff Breakdown | 60 | 0.1 |
| Wirepull | 240 | 20 |
| Handler Jam | 360 | 3 |
| Breakdown | 1440 | 74 |
| Change Wire | 1440 | 20 |
| Change Capillary | 1440 | 10 |
| Shift Start | 480 | 5 |
| Visual Check | 60 | 4 |

A.

Now focus on the Wirepull breakdown. The repair time is altered as shown below. Throughput (number shipped), busy time of the WIREBOND machine, and downtime of the WIREBOND machine varies for each repair time. The model is run for 960 minutes with a warm-up of 240 minutes.

| Table 2-19: Repair time data |  |  |  |
| --- | --- | --- | --- |
| Wirepull Repair Time | Throughput | WIREBOND % Busy | WIREBOND % Down |
| 20 | 271 | 58.68 | 29.02 |
| 10 | 318 | 68.37 | 24.83 |
| 1 | 350 | 72.95 | 21.42 |
| 0.1 | 332 | 68.41 | 19.34 |

The results are a bit counter-intuitive. One would expect that as the Wirepull repair time is decreased, throughput and busy time will increase, but this is not the case. Explain.

B.

Running the model for 1920 minutes with a warmup of 480 minutes yields the following results. Explain why the results are no longer counter-intuitive.

| Table 2-20: Repair time data |  |  |  |
| --- | --- | --- | --- |
| Wirepull Repair Time | Throughput | WIREBOND % Busy | WIREBOND % Down |
| 20 | 572 | 61.72 | 29.48 |
| 10 | 619 | 65.33 | 25.05 |
| 1 | 683 | 73.86 | 21.56 |
| 0.1 | 683 | 73.97 | 21.19 |

C.

Taking the model from part B, changing the basis of all the WIREBOND breakdowns to busy time from available time yields the following results. Explain.

| Table 2-21: Repair time data |  |  |  |
| --- | --- | --- | --- |
| Wirepull Repair Time | Throughput | WIREBOND % Busy | WIREBOND % Down |
| 20 | 679 | 74.46 | 17.26 |
| 10 | 708 | 77.13 | 15.29 |
| 1 | 731 | 79.25 | 12.47 |
| 0.1 | 738 | 79.99 | 12.19 |

D.

Taking the model from part B, removing the labor requirement from the WIREBOND breakdowns yields the following results. Explain.

| Table 2-22: Repair time data |  |  |  |
| --- | --- | --- | --- |
| Wirepull Repair Time | Throughput | WIREBOND % Busy | WIREBOND % Down |
| 20 | 646 | 70.00 | 24.01 |
| 10 | 694 | 75.19 | 20.00 |
| 1 | 700 | 75.78 | 16.09 |
| 0.1 | 700 | 75.85 | 15.72 |

E.

Again, taking the model from part B, removing the requirement “Check Only at Start of Cycle” for the WIREBOND breakdowns, yields the following results. Explain.

| Table 2-23: Repair time data |  |  |  |
| --- | --- | --- | --- |
| Wirepull Repair Time | Throughput | WIREBOND % Busy | WIREBOND % Down |
| 20 | 599 | 64.87 | 27.98 |
| 10 | 663 | 71.68 | 23.91 |
| 1 | 684 | 73.97 | 20.06 |
| 0.1 | 684 | 74.10 | 19.69 |

F.

Notice that one of the WIREBOND breakdowns is termed “Shift Start.” How might one model this as a setup? What other WIREBOND breakdowns could be modeled as setups?

| Table 2-24: Explanation for High-Volume Assembly Line Problems |  |
| --- | --- |
| Question | Explanation |
| A | The main point to understand here is that the breakdowns on a certain machine are not independent of one another. For instance, decreasing the downtime for one breakdown may increase the occurrence of other breakdowns. More answers are illustrated in the next parts of this case study. |
| B | In the previous part, the model was run for too short of a time. Decreasing the repair time for one breakdown increased the occurrence of other breakdowns. With longer run times, this is less likely to occur. For instance, if the model is run for one minute, the WIREBOND machine may be either 100% busy or 100% down for that minute. Running this model for 1000 minutes, there is no possible way that that the machine would be either 100% busy or 100% idle. |
| C | Throughput and busy time increases and down time decreases because available time is always greater than or equal to busy time. Therefore, breakdowns occur less frequently. One needs to be careful though. Decreasing the repair time for one breakdown may increase busy time and thus increase the occurrence of other breakdowns. |
| D | Again, throughput and busy time increases and down time decreases. Without labor, breakdowns could occur simultaneously instead of having the total repair time equal to the sum of the repair time for each breakdown. In this case, the total repair time is equal to the longest repair time of all simultaneous breakdowns. Having no labor for breakdowns would be the same as having breakdowns requiring labor that has infinite capacity. |
| E | Not much, change in results. The explanation of this feature given below is taken from the WITNESS on-line help. A WITNESS machine can be forced to break down only when there are one or more parts in it, through checking the Check only at start of cycle option. In this mode, you should be careful when modeling an infrequently used machine that breaks down because of available time as the machine may break down each time a part goes to that machine. You should also consider Labor availability, as if the machine waits a long time to be repaired, it may be due to break down again immediately after repair. In addition, when the Check only at start of cycle option is chosen, WITNESS machines will not break down more than once per cycle. The breakdown will occur only at the start of the cycle. (In this mode, WITNESS tests for breakdowns at the beginning of each of the sub-cycles on a Multiple Cycle machine). In this mode, WITNESS represents machine failure by causing machines to break down at the start of a cycle if a failure is due to occur during that cycle. Any 'life' remaining when the WITNESS machine breaks down is added to the following inter-breakdown period. |
| F | One could define dummy machines and a dummy part to keep track of time. When a shift begins, one could alter a variable, e.g. numshift, and have the setup occur on the value change of that variable. Other breakdowns could be modeled in the same way if they occur at given periods of time, like the beginning of the week, end of the day, etc. Variables have not been discussed as of yet, but the students should be able to explain in general terms how one might go about doing this. |

## Exercises

### Exercise 1

#### Building a Basic Model

Three part types are delivered to buffers BUFF 1, 2, and 3 respectively. A machine takes a single part from any one of these buffers, processes it, and places it in any one of three other buffers: BUFF 4 to 6. A second operation removes a part from these buffers, processes it, and dispatches it. The capacity of each buffer is ten units.

Figure 2-35: Exercise 1 Layout

| Table 2-25: Data for Exercise 1 |  |  |  |
| --- | --- | --- | --- |
| Delivery Schedule: | part | 1: | 1 every 7.5 minutes |
|  |  | 2: | 1 every 20 minutes |
|  |  | 3: | 1 every 20 minutes |
| Machine Cycle Times: |  | mc 1 | 2.5 minutes |
|  |  | mc 2 | 4 minutes |

- The capacity of all Buffers is 10 units.

### Exercise 2

#### Display and Detail

Amend the previous model with the following information.

Figure 2-36: Exercise 2 Layout

Color-code the parts so that they look different on the screen. Change the display of buffer so that the parts are displayed and not just a count of how many parts there are at the buffer.

Change the name of the Elements to something more meaningful:

Parts A, B and C

Buffers B1 to B6

Machines OP10 and OP20

Change the arrival of part B to 2 every 20 minutes

Increase the number of OP20 machines to 2

### Exercise 3

#### Modeling Stoppages

Machine OP10 has three separate tools, each one being used for each operation. The following data has been supplied on the replacement frequency for each of these three tools.

| Table 2-26: Tool Replacement Data |  |  |  |
| --- | --- | --- | --- |
| Tool | Ops between tool change | Change Time | Initial Life (no of ops) |
| 1 | 25 | 1.2 | 10 |
| 2 | 40 | 1.7 | 20 |
| 3 | 35 | 0.9 | 25 |

There are no breakdowns on OP10.

Whenever the part type being processed at OP20 changes (e.g., from A to B, A to C etc.) the machine needs retooling. These tool changes take 1 minute.

Data is also known on the frequency of breakdowns at OP20. After 2 hours of processing on machine OP20, the belt snaps, and this takes 45 minutes to repair.

### Exercise 4

#### Modeling Manpower and Resources

Two machinists, FRED and JOE, are responsible for working on the production line. The jobs FRED and JOE are capable of performing are listed below.

FRED or JOE can operate OP10. However, for retooling OP10 the following labor is required:

| Table 2-27: Labor Retooling Requirements for OP10 |  |
| --- | --- |
| Tool | Labor |
| 1 | FRED |
| 2 | JOE |
| 3 | FRED and JOE |

Either one of them can retool OP20, but only FRED is qualified to repair OP20.

Since there is only one OP10 machine and there are two OP20 machines, FRED and JOE will always give OP10 jobs a higher priority than jobs at OP20. Therefore, if there are a number of jobs waiting to be done, all of the jobs at OP10 will be done first.

- No labor is required to repair OP10 or operate OP20.

### Exercise 5

#### Pre-empting Labor

Due to the importance of the OP10 machine, if a setup is required at OP10, then the Labor will stop what they are doing elsewhere to come and complete the setup. However, this will cost the Laborer 2 minutes of walk time.

### Exercise 6

#### Modeling Conveyors

Build this exercise on top of Exercise 4. After OP20 has finished processing Parts, the Parts are output to a conveyor CON1. The Parts travel along this conveyor. Then they are shipped. CON1 is a 36 feet long queuing conveyor with a speed of 1.2 feet per minute. (Assume that the Parts are three feet long).

### Exercise 7

#### Assembling Parts into Sub-components

The model in Exercise 6 will now be modified into an assembly cell. OP10 now processes parts one at a time, and sorts parts A, B and C into the buffers B4, B5 and B6 respectively. OP20 then takes one A, one B and one C and assembles them into a single part.

The set-ups can now be removed from OP20.