# Appendix C - Actions

## Introduction

WITNESS provides a simple programming language called actions. Actions share similarities with the BASIC programming language, but it is easier to read and it also incorporates help facilities.

By using actions, you can give WITNESS specific instructions on the logic of your model, allowing you to model the calculations and formulae that underpin decisions in the real-life situation under study.

You may use actions to:

- Manipulate variables and attributes

- Control parts, tracks and vehicles

- Read and write data from disk files

- Produce your own statistical reports

- Create your own dialogue between the model and its user.

If you use actions extensively in the model, they can make the model too complicated and difficult to modify. You might find that using expressions simplifies the construction of the model.

If you find you are repeatedly using the same actions, but on behalf of different Elements, you should define and use a function. In long sets of actions the use of functions make for easier reading due to a reduction in size and clearer layout.

You can copy or cut text data from an action and paste it to another action, or to a file, using the standard Windows keyboard and mouse editing features.

## When Actions Take Place

You may specify actions to be executed:

- When the simulation run begins at time 0.0. These actions are known as Actions on Initialize.

- When you select the run/user actions option.

- When you select the run/immediate actions option.

- When all WITNESS statistical reports are reset or output to file or printer. These actions are typically used to reset variables that are used to collect information and to output a report of your own design.

- At those points when the status of the model changes in some way. For example, actions on start for a machine.

- Certain actions apply to a 'current part', that is, the part which a part-carrying Element is currently moving, attempting to move, or has just moved. Only this part may be acted upon. That is, only the attributes of this part may be manipulated in such actions.

Actions can be triggered for the following Elements when certain events occur:

#### Parts

- When a part enters the model

- When a part leaves the model

#### Buffers

- When a part enters a buffer

- When a part leaves a buffer

- When a part has completed the delay time in a buffer

#### Machines

- When a machine starts its cycle

- When a machine ends its cycle

- When a machine breaks down

- When a machine finishes being repaired

- When a machine starts its setup

- When a machine finishes its setup

#### Conveyors

- When a part arrives on a conveyor

- When a part reaches the front of a conveyor

- Vehicles

- When a vehicle enters the model

#### Tracks

- When a vehicle gets onto the track

- When a vehicle reaches the front of the track

- When a vehicle unloads from the track

- When a vehicle loads to the track

#### Shifts

- When a working period begins

- When a working period ends

#### Processors

- When the fluid flowing into a processor changes to another fluid type

- When cleaning begins

- When cleaning finishes

- When processing begins

- When processing finishes

- When a processor breaks down

- When a processor is repaired

#### Pipes

- When the cleaning begins

- When the cleaning ends

- When a pipe breaks down

- When a pipe is repaired

#### Tanks

- When the fluid flowing into the tank changes to another fluid type

- When the cleaning begins

- When the cleaning finishes

#### Files (not part files)

- When a file is opened (e.g., first accessed)

- When an end-of-file is found during a READ

- When a file is closed by Begin, New, or Close

- When a file error occurs

#### Timeseries

- Before observations are plotted

- After observations are plotted

#### Pie Charts

- Before refresh interval

- After refresh interval

It is very important to remember that Actions take place in zero simulation time. That is, the WITNESS simulation clock does not advance while actions are being executed.

An excellent source of information about the order of execution of events (including Actions) for a given WITNESS Element can be found in the online help, under "Order of Execution.” To access it from a WITNESS session, go to "Help…Control Logic…Order of Execution". Here you will find detailed information about exactly what happens, when, and in what order, as parts, fluids and labor move to, from, and through the various WITNESS Elements.

## Stand Alone Actions

WITNESS invokes Actions at a number of points that are independent of the execution of the model and the changing status of Elements in it.

#### Actions on Initialize

You can use the Model…Initialize Actions option (or the advanced action bar) to set the initial conditions of the model. For example, you may set the initial value of variables which are involved in routing decisions or in your own reports, read tables of values from disk files and instruct vehicles as to their first pickup. You may engage the user of the simulation in a dialogue in order to obtain many of these values.

Beginning Simulation Actions

- Actions on Initialize.

- WITNESS schedules Initial events (for example, first arrivals of parts) and values actions for each part type with fixed attributes, in order of definition.

- WITNESS causes each idle Element to attempt to pull in parts.

#### Immediate Actions

You can use the Run…Immediate Actions option (or the advanced action bar) to execute a single line of actions at any time. For example, you could:

- Print the value of a variable in the interact box

- Set the value of a variable

- Print the value returned from a status function (for example, to determine the current state of an Element)

- Issue a CALL statement for a vehicle

- Rewind a data file

#### User Actions

You can use the Run…User Actions option (or the advanced action bar) to invoke these actions at the time of simulation. There are many possible uses for user actions. For example, you could:

- Output reports to a disk file

- Prompt the user to enter the value of a critical variable

- Create a model which allows a user to experiment without having to use the usual WITNESS menu options

#### Reports - Actions on Output

You can use the Statistics property sheet on the Model…Options dialog to enter actions that are invoked whenever you output reports to a file. WITNESS executes these actions before writing the report to the file.

#### Reports - Actions on Reset

You can use the Statistics property sheet on the Model…Options dialog to enter actions which reset your own statistics whenever WITNESS resets the statistical reports.

#### Values - Actions on Initialize

For each type of part which has fixed attributes, WITNESS provides the opportunity to set these attributes once at the start of the run (this is not possible during the run). The attributes may be set in the 'Values' Actions, which may be entered by clicking on the "Values" button on the Detail form of a part with fixed attributes.

## Building Actions

WITNESS actions are built using a simple English-like 'programming language' which will be familiar to BASIC users. Most of the features of BASIC are present, plus a number of powerful extra facilities appropriate to simulation control.

Each 'action' is a statement giving WITNESS a task to perform, like a line of code in most other programming languages. Unless you give instructions to alter the flow of the actions (using GOTO, IF, or looping), the action statements are executed in a straight sequence, starting with the first and ending with the last. Each action statement consists of a 'keyword' followed by a list of 'parameters'.

- Keywords are analogous to verbs (telling WITNESS what to do).

- Parameters are analogous to adjectives and nouns (telling WITNESS how to do it and what to do it to). WITNESS expressions may be used as parameters. Some statements are so simple that no parameters are required - only the keyword need be entered.

#### Terminating Actions

You can terminate actions by pressing the <Ctrl C> keys during execution. This allows looping actions commands to be terminated, returning control to you.

#### Limitations of Actions

WITNESS actions have the following restrictions:

- The maximum size of a single action block is 32K bytes. Given an average line length of 25 characters, this would allow up to about 1300 lines in any one action block. Space required for carriage return characters is taken from this space.

- Each line of actions is limited to 256 characters in length.

## Using Actions Within Functions

WITNESS provides a number of built-in functions that may be used in any expression. Alternatively, you can define your own functions. These user-defined functions can invoke a number of action statements simply by using the function's name in an expression anywhere in your model. Functions make the programming aspects of your model much more modular and easier to maintain and understand. Conceptually, functions in WITNESS are identical to the concept of subroutines, subprograms, procedures, and functions in other programming languages such as BASIC, FORTRAN, COBOL, Pascal, LISP, C, C++, etc. Programming experience is helpful, but not required, to use functions and actions in WITNESS.

Functions are useful if the same sequence of actions is used in several different contexts.

Functions may return a value that is determined by the actions within the function. It is also possible to pass parameters into functions, exactly as parameters are passed to the built-in functions.

You should not use functions 'recursively', that is, a function must not invoke (call) itself, either directly or indirectly. For example, a function called FUNC1 may not contain expressions involving FUNC1. Attempting to do this will result in unpredictable behavior.

Before you reference a function you must check that you have entered its details correctly, since you cannot change its type or parameters once it is used by other Elements of the model. You may change the actions within a function at any time.

You can use any action statement in a function. You can use the RETURN action statement to return values from functions, to the 'caller'. If the value to be returned does not match the type of the function, it is coerced (e.g., promoted or demoted) to the proper type.

## Assignment - In Actions

### = (equal sign character)

#### Usage

v_or_a = expression

| v_ or _a | is an individual variable or attribute name |
| --- | --- |
| expression | is a WITNESS expression of the appropriate type |

This assigns the value to the right of the equal sign to the variable or attribute on the left of the equal sign. WITNESS evaluates all other operators before the assignment takes place (that is, ‘=‘ has the lowest algebraic precedence).

If the variable or attribute has a quantity greater than 1 (e.g., it is an array or matrix of values), you must specify an index (or indices if multidimensional) by using an integer constant or expression.

The part currently being processed is the one whose attributes are accessible in actions. To override this, you may use an attribute modifier to access attributes of a part not currently being processed (e.g., the desired part and it associated attribute(s) are currently residing in some other Element in the model).

Every part is characterized by the attributes PEN, DESC, ICON and TYPE. When a value is assigned to the string attribute DESC, the value is automatically coerced to type string.

#### Examples

PEN = 7

PEN = 2 * MYFN(3)

DESC = @VAR

DESC = "NEW.”

DESC = DESC + "*"

DESC = ">" + TYPE

ICON = 43

## Part Control Actions

### Change

#### Usage

CHANGE old_part_type, new_part_type

CHANGE ALL, new_part_type

| old_part_type | is the name of a WITNESS part |
| --- | --- |
| new_part_type | is the name of a WITNESS part |

This command is used to change an individual part from one type to another. It is typically used in the actions on finish cycle for a machine, to indicate that the process has changed the part in some way. If only certain types of part experience the change, you can indicate the part affected in old_part_type. Specify ALL as old_part_type if all parts should be changed, regardless of type.

CHANGE Sequence of Actions

- WITNESS executes actions on leave for the part being changed.

- The assembly count is incremented by one for the part being changed.

- If the new part has variable attributes and is of the same group as the old part, WITNESS copies all attribute values across to the new part. Note that WITNESS only copies system attributes for part routing and fluids if the new part uses these features. If the new part has variable attributes but is of a different group, WITNESS only copies system attributes. If the new part has fixed attributes, WITNESS does not copy any attributes.

- The ‘create’ count is incremented by one for the new part type.

- WITNESS executes ‘actions on create’ for the new part.

#### Examples

CHANGE ALL, PAINTED

CHANGE FAULTY, OK

## Track and Vehicle Control Actions

These actions are used exclusively when modeling Track and Vehicle systems. WITNESS provides various functions for detecting the status of Tracks and Vehicles.

Unless an index is explicitly specified, Tracks with a quantity greater than one are considered to be equally acceptable by these statements, just as they are to Input and Output Rules (e.g., all tracks of greater than one are affected as well).

### CALL

#### Usage

CALL vehicle, load_track, unload_track,priority

| vehicle | is a WITNESS vehicle name |
| --- | --- |
| load_track | is a WITNESS track name |
| unload_track | is a WITNESS track name |
| priority | is a number indicating the priority of the CALL |

This is used to place a demand for a vehicle in the demand list. Two destinations are stored with each vehicle; one is the track to which the vehicle will travel in order to load parts, the other is the track to which it will then travel to unload the parts. These destinations, together with the state of the vehicle, fully indicate the disposition of a vehicle.

#### Example

CALL AGV, TLOAD, TULOAD, 1

### DEST

#### Usage

DEST track

| track | is a WITNESS track name |
| --- | --- |

This assigns a destination to the current vehicle, overriding its current destination.

#### Example

DEST T1

### DESTV

#### Usage

DESTV vehicle, track

| vehicle | is a WITNESS vehicle name |
| --- | --- |
| track | is a WITNESS track name |

This assigns a destination to the specified vehicle, overriding its current destination.

#### Example

DESTV AGV(2), T1

### FINDV

#### Usage

FINDV search_track {,search_track}

| search_track | is a WITNESS track name |
| --- | --- |

Tries to satisfy a vehicle request immediately following a CALL. FINDV is usually used with the work search method of track and vehicle control.

#### Example

FINDV TS1, TS2

### VSEARCH

#### Usage

VSEARCH track {, track}

| track | is a WITNESS track name |
| --- | --- |

This attaches a list of tracks to be searched in order to find an idle vehicle to each entry in a vehicle’s demand list (that is, attached to each CALL).

#### Example

VSEARCH TS1, TS2, TS3

### WAKEUP

#### Usage

WAKEUP vehicle, track {, track}

| vehicle | is a WITNESS vehicle name |
| --- | --- |
| track | is a WITNESS track name |

Reactivates a parked vehicle by making it check its demand list for CALLs that require it to load onto a track that is in the domain of the work search on the track where the vehicle currently resides. You may need to read this paragraph twice.

#### Example

WAKEUP AGV, PARKA, PARKB

## Interact Box Input and Output Actions

WITNESS provides facilities for displaying information on the screen as the run progresses. You can also input information from the keyboard during the model's run. Using these facilities, it is possible to create a dialog between WITNESS and the user.

Interact Box input and output is very useful for tracing the exact behavior of some aspect of the model one event at a time, and for customizing a WITNESS model so that inexperienced users need not know about the usual WITNESS mechanisms.

The Interact Box input and output statements are similar to those provided by standard BASIC.

### PRINT

#### Usage

PRINT {expression {separator} ...}

| expression | is any integer, real, name or string expression |
| --- | --- |
| separator | is a symbol used to separate the expressions and format the output. |

| Table 13-1: Format of Output Expressions |  |
| --- | --- |
| Description | Example |
| Integers are displayed as a one to five digit number plus a sign if appropriate. | 25000 |
| Reals are displayed in floating point format. Very large or small numbers are displayed in exponential format. | 1.2345 or -1.2345E24. |
| Names (e.g., Element names) are displayed as entered in WITNESS. A name variable which contains MILL(3). | MILL(3) |
| Strings are displayed as entered. The backslash character (\) may be used to achieve special effects such as tab and formfeed. | "Enter cycle time:” |

Variables or attributes which have a quantity greater than one (e.g., they are arrays or matrices) MUST be specified with an index (or indices if multidimensional) when used in expressions.

Valid separators are:

| , | (comma) moves the output which follows the comma to the next tab stop (that is, the next multiple of eight character positions). |
| --- | --- |

PRINT "one","two","three"

…produces:

one  two  three

col.  	1   9    17

| ; | (semicolon) may be used to cause subsequent PRINTed or INPUT expressions to follow the previous expression without intervening spaces or new lines. |
| --- | --- |

If the length of a line output by PRINT or INPUT exceeds the current width of the Interact Box, the text will be hidden but will 'wrap around' onto the next line after 132 characters.

#### Examples

PRINT "The value of X is ", X

PRINT "1 + 2 = "; 1+2

PRINT

PRINT DRILL; " has "; NPARTS(DRILL); " parts.”

PRINT "Enter batch size for "; OVEN; "> ";

### INPUT

#### Usage

INPUT {prompt, } v_or_a {, prompt, } v_or_a...}

| Prompt | is an integer, real, name, or string constant used as a prompt |
| --- | --- |
| v_or_a | is a variable or attribute |

This statement is used to obtain responses from the keyboard. All the data types that are valid for PRINT (above) are also valid for INPUT.

If the response to an INPUT prompt is a carriage return, the variable associated with the input is not changed. You may therefore specify that carriage return can be used as a default reply by setting the variable to the default value before the INPUT statement is obeyed.

#### Examples

INPUT VAR1

INPUT "Enter lot size: "; LSIZE

INPUT "New description for WIDGET: "; DESC

INPUT "Width? ", WIDTH, "Height? ", HEIGHT

### OPENBOX

#### Usage

OPENBOX

The PRINT and INPUT statements manifest themselves through the WITNESS Interact Box. If this is currently closed, PRINT has no effect, but the INPUT statement forces the box to open. You may also force the interact box to open using the OPENBOX statement.

Since WITNESS allows you to arrange windows and the interact box at will on the screen, OPENBOX obeys the following rules:

If the interact box is already open, no action is taken. The interact box is opened at its previous size at its previous position.

### CLOSEBOX

#### Usage

CLOSEBOX

This statement closes the interact box. If windows were rearranged on the screen by the OPENBOX statement (see above), the CLOSEBOX statement does not attempt to 'undo' this rearrangement.

### INTERACT COLOR

#### Usage

INTERACT COLOR number

| number | the PEN number used to specify the color |
| --- | --- |

Allows you to set the color of text in the interact box. Once the command is issued, all text subsequently displayed in the box will be in the color specified. The numbers used to specify the colors are the same as for the PEN options. The cursor is shown in the current color. If the interact box is redrawn. The text is shown in the correct colors.

#### Example

INTERACT COLOR 12

produces white text on a blue background

### INTERACT CURSOR

#### Usage

INTERACT CURSOR x-coord, y-coord

| x-coord | with respect to origin located at the top left-hand corner of the interact box. The x-axis runs from left to right. |
| --- | --- |
| y-coord | with respect to origin located at the top left-hand corner of the interact box. The y-axis runs from top to bottom. |

This allows you to set the starting point for the output of text.

The character at the extreme left of the top line of the Interact Box is at position (1,1).

The x-coordinate specified must be in the range 1-132 and the y-coordinate must be in the range 1-50.

### INTERACT CLEAR

#### Usage

INTERACT CLEAR point, point

| point | The diagonally opposite points (top left, bottom right) of the line to be cleared. |
| --- | --- |

Allows you to clear a selected rectangle within the interact box. Each point is specified by an x coordinate followed by a y coordinate with the coordinates separated by spaces. The area is cleared to black.

Limits for x and y coordinates are 1-132 and 1-50 respectively.

When selecting the area to be cleared, the interact box will scroll, allowing you to highlight the area required.

After clearing the selected area, the cursor will be reset to the top left coordinate of the cleared area.

#### Example

INTERACT CLEAR 20, 20, 80, 40

## File Input and Output Actions

WITNESS has facilities for reading and writing disk files. File input and output has many uses, for example:

- Specifying values to be used in detail dialogs (for example, the cycle time of machines)

- Attaching attribute values to parts as they arrive (for example, to indicate the process route required for each part type)

- Producing reports tailored to your requirements, with your own layout and contents.

Like all other WITNESS Elements, you must define files before using them. You must define your files with the correct type, e.g. you may only READ from a READ file, and you may only WRITE to a WRITE file.

These files are text files, which you may edit using your favorite word processor or editor, or print on your printer. The values to be read by the READ statement should be valid constants of one of the four WITNESS data types: integer, real, name or string. Entries in the file may be separated by:

- Any number of spaces

- A comma (,)

- A semicolon (;)

- Being placed on separate lines (e.g., separated by a carriage return)

There is no practical limit to the length of each line in a file, and the size of files is limited only by the space available on your computer.

The actions for file Input and Output are:

### READ

#### Usage

READ file v_or_a {, v_or_a ...}

| FILE | is the WITNESS Element name of the file concerned |
| --- | --- |
| v_or_a | is the name of any integer, real, name or string variable or attribute |

The READ file is opened as soon as a WRITE or READ statement references it. If a READ file does not exist, an error message is displayed and the simulation run is stopped.

The list of variables or attributes must match the data in the file. For example, trying to read a real variable from a string containing no digits will cause an error message to be displayed.

It is good practice to have one line per READ statement. If a READ statement encounters too few entries on a line, it continues reading on the next line. If there are too many entries on a line, the surplus text is ignored and can be used for annotation.

#### Example

READ FP, NEXTAR, STEP1, STEP2, STEP3, STEP4

If you need many values on one line, the command can be inconveniently long or exceed the allowed line length. If this is likely to occur use a semicolon line terminator to leave the read open - a subsequent READ will continue from the same point in the data file where the previous read finished. The subsequent READ need not be in the same set of actions.

#### Example

READ R4DEMO NEXTTYPE, PACKQTY;

A semicolon may also be used to separate entries within a READ command in which case it acts the same as a comma separator.

### WRITE

#### Usage

WRITE file expression {{separator} expression {separator} ...}

| file | is the WITNESS Element name of the file concerned. |
| --- | --- |
| expression | is any integer, real, name or string expression |
| separator | is a symbol used to separate the expression and format the output. |

The WITNESS Element name of the file should NOT be confused with the actual name of the file in your computer. This is specified in the file detail. In fact, the WITNESS Element name for your file can be completely different than the name of the actual disk file. Out of convenience and by default, WITNESS gives the disk file the same name as the WITNESS file Element name.

All expressions and separators, which are valid for PRINT, are also valid for WRITE. Lines in WRITE or READ will wrap around after 256 characters, thus you should bear in mind the width of the paper on your printer when producing reports using WRITE.

A file is opened as soon as a WRITE or READ statement references it from somewhere within your model. If an attempt is made to open a WRITE file which already exists, the option to overwrite or append is offered via a popup alert.

Separators:

| ; | (semicolon) – expressions are written to the file with no spaces inserted. |
| --- | --- |

A semicolon at the end of the statement suppresses the carriage return character that is normally output

| , | (comma) – expressions are written with spaces inserted to the next ‘tab’ position (usually every 8 characters). |
| --- | --- |

#### Example

WRITE FILEA, “report at time “; TIME

WRITE FILEA, MILL; “ completed “; NOPS(MILL); “cycles”

### REWIND

#### Usage

REWIND file

| file | is the WITNESS Element name of the file concerned |
| --- | --- |

This statement is used to reset a READ file so that the next READ will be performed at the start of the file. You may not REWIND a WRITE file.

REWIND is commonly used in the actions on end-of-file, to implement a 'circular' or 'endless' file from which variables are read. For example, the variable ARRVTIME may be read from file CIRCULAR as follows:

READ CIRCULAR, ARRVTIME

When the end of the file associated with CIRCULAR is encountered, the following actions may be executed:

REWIND CIRCULAR

READ CIRCULAR, ARRVTIME

#### Example

REWIND TIMINGS

## Modifying Simulation Elements

The following actions allow you to modify characteristics of simulation Elements directly:

### SET ICON

#### Usage

SET parameter of Element to value

| parameter | is the ICON |
| --- | --- |
| element | is the name of the machine, vehicle, buffer, conveyor, processor or tank to be modified. |
| value | is the new value to be assigned to the parameter |

Typically used in machine actions to arrange for the machine to change shape instead of (or in addition to) changing color when the machine changes state. This may also be used to change the icon used to represent a vehicle when the vehicle 'turns a corner'.

- If the vehicle is not already displayed as an icon, this command will change its display to icon mode.

#### Examples

SET ICON of OP20 to 13

SET ICON of CRANE(N) to ICONARRY(ISTATE(CRANE(N)))

SET ICON of AGV(1) to 3

WARNING: It is recommended that when SET is used in Actions within the model itself (for example, changing machine icon when a machine breaks down), that Actions on Initialize are used to reset values to their normal state when you start the model again.

### SET LIFE

#### Usage

SET parameter of Element setup number to value

| parameter | is the LIFE |
| --- | --- |
| element | is the name of the machine for which setup life is to be set |
| setup number | is the reference number of the setup on the specified Element |
| value | is the new value to be assigned to the parameter |

This is used to change the remaining life of a machine setup, that is, the number of operations to that setup. This would typically be used to trigger a setup before it is due according to the life remaining. This could be used to model tool changes out of shift, or during a breakdown.

#### Example

SET LIFE MACH(1) 5 500

### SET QUANTITY

#### Usage

SET QUANTITY of Element name to new quantity

| element name | specifies the name of Element whose quantity is to be changed |
| --- | --- |
| new quantity | is the new quantity required |

This command has the same effect as changing the quantity of an Element detail. It can only be used before starting a simulation run in actions on initialize, user actions, immediate actions and functions called in these actions. It is recommended that you SET the initial quantity at actions on initialize if it is used in user actions. This protects against possible problems when the file is saved. When the file is saved the changed quantity is saved with it.

This command cannot be used with name variables.

#### Example

SET QUANTITY MACH 3

SET QUANTITY LABOR

#### Usage

SET QUANTITY LABOR laborname on shiftname to new quantity.

| laborname | is the name of labor to be changed |
| --- | --- |
| shiftname | specifies the name of the shift for which the labor quantity is to be changed. If set to NONE the new labor quantity is always available |
| new quantity | is the new labor quantity required |

This command has the same effect as changing the shift labor quantity detail. It can only be used before starting a simulation run in actions on initialize, user actions, immediate actions and functions called in these actions. It is recommended that you SET the initial labor quantity at actions on initialize if it is used in user actions. This protects against possible problems when the file is saved. When the file is saved the changed labor quantity is saved with it.

#### Example

SET QUANTITY LABOR of FRED on Shift to 5

### SET CAPACITY

#### Usage

SET CAPACITY of machine to new capacity

| Element name | specifies the name of the Element for which the capacity is to be changed |
| --- | --- |
| new capacity | is the new capacity required |

This command has the same effect as changing the capacity of an Element detail. It can only be used before starting a simulation run in actions on initialize, user actions, immediate actions and functions called in these actions.

It is recommended that you SET the initial capacity at actions on initialize if it is used in user actions. This protects against possible problems when the file is saved. When the file is saved the changed capacity is saved with it.

#### Example

SET CAPACITY of TANK to 1000

- It is possible here to set the capacity of a tank to less than a warning level. If you do, the warning level action cannot take effect since the fluid never reaches it. If you set the capacity to less than a warning level, then you enter the tank's detail dialog, you are compelled to set a warning level within the capacity of the tank before you can exit from the dialog.

## Comments in Actions

### ! (exclamation mark)

#### Usage

! text

| text | is a textual string of any form, typically used to explain the purpose of the following statement. |
| --- | --- |

Comments are permitted on the end of any line and should be preceded by an exclamation mark (!). To continue a comment on a new line, a new exclamation mark is required. A line of pure comment is permitted by starting the line with an exclamation mark. If an exclamation mark is used in the middle of a line of active action code, any text to the right of the exclamation mark is considered a comment and thus ignored.

Actions containing comments use more memory on your computer than those without, but execution speed is completely unaffected (e.g., WITNESS 'compiles' your action code at runtime, without comments).

It is strongly recommended that comments be used to clarify the reasoning behind Actions of more than a few lines. This way, complex models may more easily be used and understood by more than one person.

#### Example

! This is a comment.

NUMBOLTS = NPARTS(BUFF001) ! This is also a comment.

!

The last line, with just the "!" character, will effectively insert a blank space line. This can help visually separate functionally different sections of action statements.

Another very convenient use of the comment character is to selectively disable active lines of action code while you are testing or debugging a certain section of actions statements. Disabling lines of active code by simply putting the exclamation point to the left of the line will allow the action to run with less complexity (e.g., without that particular line). This allows you to concentrate on finding the problem by eliminating portions of code little by little.

#### Example

NUMBOLTS = NPARTS(BUFF001) ! + NPARTS(BUFF002)

The last line above shows an example of using the comment character to disable an active portion of an action statement (perhaps for testing or debugging purposes). In this case, the partial command "+ NPARTS(BUFF002)" is ignored by WITNESS until the "!" is removed.

It is also recommended that any 'old' unused action code be similarly commented out, rather than being deleted entirely. This allows you to go back and reconstruct a previous programming idea by simply uncommenting the code, rather than having to re-write the code from memory. This is good for trying different ideas and picking the one that works the best. Having the 'old code' commented out lets you go back and see previous attempts at something.

## Statistical Recording

The following actions allow control the recording of statistical data:

### RECORD

#### Usage

RECORD value in histogram

| value | is a numeric expression |
| --- | --- |
| histogram | is a WITNESS histogram name |

This is used to record a value in a WITNESS histogram. It may be used at any time in the simulation.

#### Example

RECORD NPARTS(LINE) in WAIT_H

### DRAWBAR

#### Usage

DRAWBAR histogram, value, height {, color}

| histogram | is a valid WITNESS histogram name |
| --- | --- |
| value | is the value to be recorded |
| height | is the number of observations to remain in the appropriate cell after the observations have been recorded |
| color | is the color that this bar is drawn in. It is optional and is specified using the values in the table below, NOT the pen colors. If you do not indicate a color here, the bar is drawn in the color selected on the histogram display form. |

This allows you to draw the bars of the histogram in different colors.

If the height specified is less (or greater) than the number of observations recorded, values will be added (or deleted), at the value given in the DRAWBAR command, to make the number in the appropriate cell as requested.

#### Example

DRAWBAR LOG_B NPARTS(LINE), 10, 5

### ADDBAR

#### Usage

ADDBAR histogram, value, increment {, color}

| histogram | is a valid WITNESS histogram name. |
| --- | --- |
| Value | is the value to be recorded. |
| Increment | is the number of observations to be added to the bar. |
| Color | is the color in which the addition to the bar is drawn. It is optional and is specified using the values in the table below, NOT the pen colors. If you do not indicate a color here, the bar is drawn in the color selected on the histogram display form. |

ADDBAR is used to extend a bar that has already been drawn, using a different color.

#### Example

ADDBAR

OG_B NPARTS(LINE), 2

Color-codes for ADDBAR and DRAWBAR:

0: Black			4: Blue			8: Gray				12: Dark blue

1: White		5: Cyan			9: Dark gray		13: Dark cyan

2: Red			6: Yellow		10: Dark red		14: Dark yellow

3: Green		7: Magenta		11: Dark green		15: Dark magenta

## Continuous Processing Elements Actions

### CLEAN

#### Usage

CLEAN Element_name, clean_duration

| Element_name | is a WITNESS continuous processing Element name |
| --- | --- |
| clean_duration | is a number indicating the duration of the cleaning cycle |

This executes a cleaning cycle on the specified continuous processing Element.

#### Example

CLEAN PROC1, 20

- If a clean interrupts another clean, the second clean is ignored (as there is no point in cleaning something that is already clean).

If a breakdown interrupts a clean, the breakdown is processed and then the clean is completed. If the contents are scrapped on breaking down then any purged fluid included in this is treated the same as if it had been pushed out of the far end and sent to WASTE.

If a clean interrupts a breakdown, the first clean to interrupt is queued to the end of all breakdowns, unless the Element was cleaning immediately before breaking down, in which case the clean is ignored.

### PURGE

#### Usage

PURGE pipe_name, purge_volume

| pipe_name | is a WITNESS pipe name |
| --- | --- |
| purge_volume | is a number defining the volume of the purge fluid |

This executes a purge cycle of a specified volume through the specified pipe.

#### Example

PURGE P1, 150

- If a purge interrupts another purge, the second purge is ignored (as there is no point in purging something that is already purged).

If a breakdown interrupts a purge, the breakdown is processed and then the purge is completed.

If the contents are scrapped on breaking down then any purged fluid included in this is treated the same as if it had been pushed out of the far end and sent to WASTE.

If a purge interrupts a breakdown, the first purge to interrupt is queued to the end of all breakdowns, unless the Element was purging immediately before breaking down, in which case the purge is ignored.

### MOVE

#### Usage

MOVE volume, Element-name, Element-name

| volume | is the volume to be moved |
| --- | --- |
| element_name | is Element from which the volume of fluid is to be moved |
| element_name | is Element to which the volume of fluid is to be moved |

MOVE is used to transfer fluid instantaneously between tanks and/or processors. It may also be used to move fluid into the system from WORLD, to move fluid to SHIP or WASTE or to move fluid to or from parts held in discrete Elements, in which case the destination may be qualified by AT <position> to specify which part in the Element is to be updated.

The specified volume of fluid must be available in the source Element when the command is issued and there must be space for it in the destination Element.

#### Example

MOVE 30.0 TANK(3) PROC(5)

Any warning levels in the Elements concerned will be obeyed after the fluid has been moved between the Elements and the levels will be obeyed in the sequence they would have been obeyed if the fluid had flowed between the Elements. As the levels obeyed as the result of a MOVE command may themselves include MOVE commands (maximum depth = 5) you will have to be careful to avoid recursion.

If the fluid type is MIXTURE, fluid may not be moved out of a part unless it is to another part.

Fluid may not be moved in to, or out of, parts in a machine that is currently filling or emptying those parts.

## Media Control Interface (MCI) Actions

The Media Control Interface controls a variety of multimedia devices within the Microsoft Windows environment. For example: sound (WAV), Midi music (MID), CD audio, digital video (AVI), QuickTime (MOV), animation’s (FLI, FLC), external videodisk players, and so on. You can check the MCI devices that are currently available on your machine by selecting the Windows media player and examining the options under the devices menu.

Some MCI devices (such as sound (WAV)) can only process one command at a time. Others (such as AVI) can process multiple files. Therefore, you could, for example, play a WAV, MID, CD AUDIO, two AVI and an FLC file concurrently.

File names containing the '\' character should use '\\' as shown in the examples.

Action statements available are:

### MCIOPEN

#### Usage

MCIOPEN mci_id, window_no, start_x, start_y, filename

| mci_id | is a string used to identify this file by later MCI commands. |
| --- | --- |
| window_no | is the WITNESS window number. |
| start_x, start_y | is the start position within the window (in pixels). |
| filename | is the full path and name of the file to open. |

Opens files (such as AVI files) within a WITNESS window instead of a pop-up window. Sets the identifier of a file for use by other MCI commands.

- The coordinates (start_x, start_y) indicate the position on the virtual screen. However, in order to optimize the performance of the graphics, it will only be visible in the specified window, and then only if the specified window is viewing that portion of the virtual screen.

#### Example

MCIOPEN “test”, 1, 1, 1, “c:\\mm\\sample.avi”

This command opens the file sample.avi in the top left-hand corner of Window 1 and sets the identifier of that file to be test.

### MCICLOSE

#### Usage

MCICLOSE mci_id

| mci_id | a string used to identify this file by later MCI commands. |
| --- | --- |

This command closes the identified file that has been opened by the MCIOPEN command.

- MCICLOSE will not give an error if the identified file is already closed. You can therefore use this command as a ‘safety net’.

#### Example

MCICLOSE “test”

### MCIPLAY

#### Usage

MCIPLAY mci_id [,REPEAT | WAIT]

| mci_id | a string used to identify this file by later MCI commands. |
| --- | --- |
| REPEAT ¦ WAIT | is an optional parameter which controls the manner in which the file is played. |

Plays the opened file once through. (In the case of an AVI file, the video is played and then remains on the screen at the last frame).

The second parameter gives you the option to either pause the WITNESS model while the file is played (WAIT) or to play the file repeatedly for the duration of the time that you have the model open (REPEAT).

- If you are already playing a file and you run another play command for the same file, the new command will apply to the file that is already running.

#### Examples

MCIPLAY “test”, WAIT

MCIPLAY “test2”

MCIPLAY “test3”, REPEAT

This series of commands will play the file identified in the open command as test, halting the WITNESS model while it plays. The file test2 is played regardless of whether the model is running or not and the file test3 is played repeatedly for as long as the model is open. All the files are played concurrently (except WAV files).

MCI OPEN “test”, 1, 10, 10, “c:\\mm\\sample.avi”

MCIPLAY “test”

MCICLOSE “test”

This series of commands opens and plays an AVI file within window 1, then closes the file when it has finished.

MCIOPEN “test”, 1, 10, 10, “c:\\mm\\sample.avi”

MCIPLAY “test”, REPEAT

This series of commands opens and plays a video repeatedly until it is stopped using MCISTOP or MCICLOSE.

### MCIBEGIN

#### Usage

MCIBEGIN mci_id

Sets the file back to the start.

#### Example

MCIBEGIN

### MCISTEP

#### Usage

MCISTEP mci_id

Advances the play of a video or animation file forward by one frame.

#### Example

MCISTEP “test”

### MCI

#### Usage

MCI command_string

Sends the command string command_string to the MCI command processor. When you view the following examples, note that MCI “play... etc.” is not the same as the MCIPLAY statement.

#### Example

MCI “play c:\\videos\\sample.avi repeat”

Repeatedly plays an AVI file in a pop-up window.

MCI “play c:\\windows\\ringin.wav”

Plays a WAV file in the background and continues to run WITNESS. The file is opened, played and then closed when it has finished.

MCI “stop c:\\videos\\sample.avi”

The above command stops a file playing.

MCI “play c:\\mm\\sample.avi”

The above command plays the video for windows file in a pop-up window and continues to run WITNESS. The file is opened, played and then closed when it has finished.

MCI “set cdaudio time format TMSF”

MCI “play CDAUDIO from 3 to 3”

This series of commands sets the positioning mode of cdaudio to access via tracks and plays track 3.

## Logical Expressions

The IF and WHILE conditions make use of logical expressions (that is, expressions which have a value of either true or false). Such expressions are only permitted within statements that require them, such as IF and WHILE. Type coercion is supported between real and integer expressions, defaulting to type REAL.

Logical expressions may contain any of the following operators. In order of evaluation (algebraic precedence) they are:

| () | An expression in parentheses |
| --- | --- |
| - | Unary minus (negation, e.g. swaps sign, not the algebraic subtract) |
| NOT | Logically NOT a following sub-expression |
| & | Convert number to letter e.g. 1 to "A", 2 to "B" |
| @ | Convert number to string e.g. 1 to "1", 1234 to "1234" |
| ** | Raise to power (exponentiation) |
| * | Multiply |
| / | Divide |
| + | Numerical add or string concatenate |
| - | Subtract (not the unary minus) |
| = | Compare for equality (not the algebraic assignment statement) |
| <> | Compare for not equal |
| < | Less than |
| > | Greater than |
| <= | Less than or equal to |
| >= | Greater than or equal to |
| AND | Logically AND two sub-expressions |
| OR | Logically OR two sub-expressions |

#### Note on comparison

Because of the finite precision of real numbers on all computers, comparing reals (floating point values) for absolute equality or inequality should be avoided. For example, a calculation may introduce rounding errors such that a value of 10 is actually evaluated as 9.99999. WITNESS would correctly regard this as not equal to 10. It is safer to use the operators <, <=, > and >= with real numbers.

Logical AND and OR must not be confused with the labor rules

AND and OR  AND is evaluated before OR in labor rules.

AND and OR are evaluated left-to-right in logical expressions. Expressions of type name may be compared very much like numeric expressions. Care is needed, since a name may optionally be followed by an index.

Consider the following cases involving a name type variable called NAME and a vehicle called AGV with quantity 3:

NAME = AGV

IF NAME = AGV

PRINT "Match"

ELSE

PRINT "No match"

ENDIF

results in the Interact Box output: Match

NAME = AGV(1)

IF NAME = AGV

PRINT "Match"

ELSE

PRINT "No match"

ENDIF

results in the Interact Box output: No match

## Conditional Execution

### IF…ELSE…ELSEIF…ENDIF

#### Usage

IF condition

action

{ELSEIF condition

action}

{ELSE

action}

ENDIF

| Condition | is a logical expression that evaluates to TRUE or FALSE action is one or more action statements, potentially including more (“nested”) IF constructs. |
| --- | --- |

The ELSE statement and the associated statements may be omitted if not required.

#### Example

IF (I >= 0) AND (I <= 5)

PRINT "I is between 0 and 5"

ELSEIF (I >= 6) AND (I <= 10)

PRINT "I is between 6 and 10"

ELSE

PRINT "I is negative or exceeds 10"

ENDIF

## Branching

### GOTO and LABEL

#### Usage

GOTO label

LABEL label

| Label | is a unique name used to identify the line at which execution of Actions should continue |
| --- | --- |

The use of the GOTO statement is slightly different to that in BASIC since WITNESS does not use line numbers. A label is declared at the jump point, and the GOTO must refer to this label.

#### Example

LABEL a_label

PRINT "Forever"

GOTO a_label

The only restriction on the use of GOTO is that you must NOT jump in a FOR/ NEXT loop. If you do not wish to loop a fixed number of times, it is recommended that you use the WHILE/ENDWHILE loop construct (below).

The label mentioned above is subject to the same restrictions as variable names, e.g. its length is restricted to eight characters. The label must be by itself on a line (which makes the labels easy to see).

## Repeating (Looping Constructs)

### FOR…NEXT

#### Usage

FOR var = exp TO exp {STEP constant}

<Action statements>

NEXT {var}

| var | is a numeric variable |
| --- | --- |
| exp | is a numeric expression |
| constant | is a constant value |
| Action statements | is one or more Action statements, potentially including more ('nested') FOR/ NEXT constructs |

This allows a block of Actions to be executed a fixed number of times, while a variable increases or decreases in value. It functions in a similar fashion to its namesake in BASIC.

- The variable is compared with the TO expression before each pass through the loop. Consequently, the intervening Actions may not be executed at all. For example:

#### Example

FOR II = 10 TO JJ

<action statements>

NEXT II

Here, <action statements> will not be executed if JJ is less than 10.

The variable used in the loop must not have an index, that is, it must be a variable with a quantity of one. This is so as not to degrade execution speed. The expression used in the FOR statement can be any numerical expression permitted in an assignment, including array and non-void function references.

The STEP constant is optional and defaults to one. Both positive and negative STEP values are permitted. Expressions (e.g. variables) may NOT be used for STEP values.

#### Example

FOR II= 	1 TO 5 STEP 2

PRINT "II is ";II

NEXT II

### WHILE…ENDWHILE…END

#### Usage

WHILE condition

action

ENDWHILE

| Condition | is a logical expression that evaluates to TRUE or FALSE |
| --- | --- |
| Action | is one or more Action statements, potentially including more ('nested') WHILE/ENDWHILE constructs |

This construct may be used to cause a block of Actions to be executed while some condition remains true. Of course, if the condition is always true, the loop will never end! You may choose to do this deliberately, and use the GOTO or RETURN statements to exit in the middle of a WHILE loop. As a form of shorthand, the ENDWHILE statement may be reduced to just END.

#### Example

II = 0

WHILE I < 5

II = II + 1

PRINT "II is ";II

ENDWHILE

## Terminating Actions

### RETURN

#### Usage

RETURN {value}

| value | is an expression whose value is to be returned as the value of the function |
| --- | --- |

Actions are automatically terminated when the last line is reached. If you wish to terminate before this, use the RETURN statement. If you are using Actions to create a new function which returns a value, entering RETURN followed by an expression will return the expression's value to the point at which the function was called, and cease execution of the function at that point.

#### Example

RETURN SQUARE**0.5

### STOP

##### Usage

STOP

The STOP statement will cause the simulation run to abort at the current simulated time. It has the same effect as selecting the menu option Run/Stop or pressing the VCR-style stop button at the bottom.

STOP is typically used when an 'emergency' occurs (for example, a file error from which it is impossible to recover). It is helpful to use PRINT to indicate the reason for aborting the run before using this statement.

STOP does not immediately terminate the current Actions. If this is the required effect, you should follow the STOP statement by a suitable RETURN statement.

#### Example

STOP

RETURN 0

Another use for the STOP command is to find out how much simulation time it takes the first part to arrive or depart from an Element. You can also use STOP in the very last Element in the model as a way of determining how long it takes the first part to move through the model from beginning to end. This is useful for determining what the model's warm-up period might need to be (see Model…Options…Statistics)