Tracks and Vehicles

Objectives

- To understand the basic characteristics and uses of Tracks and Vehicles.

- To create logic to load and unload Vehicles with parts as required.

- To understand the various methods for allocating jobs to Vehicles.

- To use Actions, Functions etc. to build complex Tracks and Vehicles models.

## Introduction

Vehicles are used to represent devices that transport parts around a facility. Examples of these devices are forklift trucks, cranes, automated guided Vehicles (AGV’s) or, any other flexible means of transport. Vehicles can also be used to model an operator who carries a part from machine to machine in a work cell where information about time and travel distance need to be included in the model.

Vehicles are characterized by their quantity (the number of Vehicles of a given type), capacity (the number of parts each Vehicle can carry), speed, acceleration and the location they occupy at the start of the model run.

Although it is the Vehicles that actually move, it is the Tracks that the Vehicles travel along that define the physical layout and contain the logic needed to make the system work.

It is recommended that any model that includes Tracks and Vehicles be built in two steps:

- Design the Track layout and Vehicle routing so that the Vehicles move around the Track appropriately. This entails Adding the required Tracks and Vehicles and then Displaying and Detailing them as necessary.

- Specify how and where the Vehicles will load and unload parts in the system.

This approach will tend to shorten your model building time.

### Vehicles

A Vehicle is an Element that can carry a number of parts. Vehicles can only travel along Tracks (e.g. they cannot go into a buffer or machine for example). Each Vehicle defined needs to be assigned a capacity (default 1) and a speed. The quantity can also be changed, allowing you to change the number of Vehicles of a particular type quickly.

| Table 8-1: Vehicle Display Characteristics |  |
| --- | --- |
| Characteristics | Description |
| Vehicle name | Displays name of Vehicle on screen. |
| Vehicle style | Vehicle displayed as index of Vehicle, no of parts loaded, description of first part or as a chosen icon. |
| Demand List | Shows a list of the jobs waiting to be done. |

Figure 8-1: Vehicle Detail Characteristics

| Table 8-2: Vehicle Detail Characteristics |  |
| --- | --- |
| Characteristics | Description |
| Name | up to 8 characters allowed |
| Quantity | Number of Vehicles of this type in model (may be identified as AGV(1), AGV(2) etc. |
| Capacity | number of parts each Vehicle can carry |
| Speed | speed of Vehicle |
| Time delay | models acceleration and deceleration of Vehicle |
| Rules | determines where Vehicle enters the simulation |
| Shift | name of shift on which Vehicle is working |
| Actions, Entry | Actions can be executed when Vehicle first enters the simulation. |

### Tracks

The path that a Vehicle follows in a model is made up of a series of one-way Tracks. Vehicles join at the rear of the Track and automatically travel to the front of the Track. At the front of the Track, the Output To Rule specifies the next Track where the Vehicle will move.

Tracks have a 'physical length' which is used to control how long it takes to travel the length of a Track and a 'display length' which determines how many Vehicle positions are shown on the screen. This enables you to fit the display into a relatively small space although your Tracks may be long.

The 'maximum speed' feature allows you to slow the Vehicle down on a particular Track to reflect operating constraints such as area congestion, presence of pedestrians, etc.

| Table 8-3: Track Display Characteristics |  |
| --- | --- |
| Characteristics | Description |
| Track name | displays name of Track on screen |
| Track | displays the empty Track (using form similar to buffers) |
| Track style | allows you to choose the style for the Track (icons, text, foreground/background colors etc.) |
| Display length | no. of positions for Vehicle displayed on screen |

Figure 8-2: Track Detail Characteristics

| Table 8-4: Track Detail Characteristics |  |
| --- | --- |
| Characteristics | Description |
| Name | name of Track |
| Quantity | number of Tracks with exactly the same display and detail information |
| Physical length | actual length of Track |
| Capacity | maximum number of Vehicles allowed on Track at one time |
| Zone | allows Tracks to be grouped into areas |
| Max. Speed | speed limit on Track |
| Work Search | mechanism by which Vehicle looks for job in Demand List |
| Stop time | amount of time Vehicle should stop and wait at front of Track before progressing to the next Track |
| Unloading |  |
| Loading |  |
| Rules | Output To Rule moves Vehicle to the next Track |
| Busy time | indicates how long Track will remain 'busy' after Vehicle has left |
| Actions On | actions are executed when Vehicle first gets onto Track |
| Actions On Front | actions are executed when Vehicle reaches front of Track. |

### Loading and Unloading

A Vehicle can only load or unload parts at the front of a Track. In addition to the Vehicle routing logic, each Track can contain details for loading and/or unloading Vehicles. These details include input and/or Output Rules that tell visiting Vehicles where to load and/or unload parts respectively.

Figure 8-3: Track Unloading Characteristics

| Table 8-5: Track Unloading Characteristics |  |
| --- | --- |
| Characteristics | Description |
| Transfer mode | decision logic which determines whether or not Vehicle unloads at this Track |
| Quantity to unload | number of parts to be unloaded at this Track |
| Time to unload | total time to unload parts specified above |
| Unloading rule | moves the parts from the Vehicle to their next location |
| Park position | Track on which Vehicle will park when idle |
| Actions On Unload | actions executed after Vehicle has unloaded |

Figure 8-4: Track Loading Characteristics

| Table 8-6: Track Loading Characteristics |  |
| --- | --- |
| Characteristics | Description |
| Transfer mode | decision logic which determines whether or not Vehicle loads at this Track |
| Quantity to load | number of parts to be loaded at this Track |
| Time to load | total time taken to load above number of parts |
| Loading Rule | moves parts onto Vehicle |
| Actions On Load | actions executed for the first part on Vehicle |

- Do exercise 1 now.

### Transfer Modes

As previously mentioned, the transfer mode for loading and unloading is used to determine whether a Vehicle loads or unloads at a specified Track. Once enabled, the three modes of loading and unloading are as follows:

- ALWAYS

- IF

- CALL

The ALWAYS and IF modes may be used to model systems where Vehicles pass loading or unloading points at intervals and load and unload if required. This type of system is often referred to as a 'merry-go-round' type system.

The CALL mode is used with demand driven Vehicle systems, where a Vehicle is called when it is required.

- A machine or other Element may not load a Vehicle by pushing to a Track or Vehicle.

- A machine or other Element may not unload a Vehicle by pulling from a Track or Vehicle.

- Vehicles must only be pushed to Tracks.

- Only Vehicles may be pushed to tracks.

- Do exercise 2 now.

## Demand Driven Vehicle Systems

### Introduction

The previous chapter described how Tracks and Vehicles could be used to model systems where Vehicles travel around a facility, loading and unloading parts as they proceed; e.g. the 'merry-go-round' type system. This chapter describes how to model a system where there is an explicit mechanism for requesting transport and a method by which Vehicles are allocated to meet this demand. An example of this type of system is a taxi service. A control center receives calls from customers that then dispatches the taxis to the customers’ pick-up locations. In fact, it can be useful to draw a comparison between demand driven Vehicles such as taxis versus non-demand driven Vehicles such as buses.

There are two distinct steps to modeling such a system:

- Generate a demand

- Allocate a Vehicle.

The following chapters will describe how to generate a demand for a Vehicle and three different methods for allocating a Vehicle to this demand.

### Generating Demands

When the need for transport arises in your model, you may use the CALL action statement to generate a demand for a Vehicle. This statement may be invoked in the finish cycle actions for a machine (e.g. the Vehicle comes to take away the part when the machine has finished processing it) or the actions on out for a buffer when this buffer requires replenishing (e.g. when buffer has run out of parts). A list of all of the unsatisfied CALLs is maintained for each type of Vehicle in the model. This list is known as the Demand List and the Demand List for each type of Vehicle can be displayed on the screen.

The CALL statement has the following form.

#### Usage

CALL vehicle, load_track, unload_track   {, priority}

| vehicle | is name of Vehicle required |
| --- | --- |
| load_track | is name of Track to which Vehicle should come to load |
| unload_track | is name of Track to which Vehicle should come to unload |
| priority | is used to prioritize CALLs (1 highest, 2...., 0 lowest - default) |

#### Example

CALL AGV, T6, T1, 1

| means | Call Vehicle AGV to load at Track T6, then once loaded send it to Track T1 to unload. Place the call to perform this pickup at the top of the Demand List. |
| --- | --- |

Each time a CALL action statement is executed, the following sequence of events occur:

- An entry is added to the Demand List in the appropriate place according to the priority of the CALL.

- Once the Vehicle has been assigned to the call, the entry is removed from the Demand List, and the state of the Vehicle is set to 'demanded'.

- The first destination of the Vehicle is set to the load Track specified in the CALL statement.

- The second destination of the Vehicle (the Track it will attempt to move to once it is loaded) is set to the unload Track specified in the CALL statement.

### Using VSEARCH to Allocate Vehicles

This approach is used to model situations where all Vehicles are controlled by a central controller or computer much like a radio-dispatched taxi company. The customer calls a dispatch center that is in constant radio contact with all the taxis and is therefore aware of their locations. The dispatch center calls a taxi to pick up this customer. When the driver drops off the customer, the controller assigns a new job to the taxi.

Using this approach, the central controller continuously tries to assign work to idle Vehicles. To model this situation, the action statement VSEARCH is used to tell the central controller which Tracks to search in order to find an idle Vehicle to satisfy the CALL. The VSEARCH statement immediately follows the CALL statement and the list of Tracks specified in the VSEARCH statement (called the domain) is stored with the CALL in the Demand List.

The VSEARCH statement has the following form.

#### Usage

VSEARCH  track  {, track...}

#### Example

CALL AGV T6, T1, 1

VSEARCH T2, T3, T5

| means | Call Vehicle AGV to load at Track T6, then once loaded send it to Track T1 to unload. Place the call to perform this pickup at the top of the Demand List. Look for the vehicle to satisfy this demand on tracks T2, T3, or T5 only. |
| --- | --- |

If there is an idle Vehicle anywhere within the domain specified in the VSEARCH statement, the demand is immediately assigned to the Vehicle. If no idle Vehicle is found, the domain is stored in the Demand List with the CALL statement and as soon as an idle Vehicle enters one of these Tracks, it assigned to the task in the Demand List.

To use the VSEARCH method:

- Set all loading Tracks to have a Transfer mode of CALL

- Set all unloading Tracks to have a Transfer mode of CALL

Follow each CALL by VSEARCH, list the Tracks on which you wish to look for idle Vehicles that can be assigned the job.

### Destination Rule

As previously mentioned, there are two destinations stored with each Vehicle: the Track to which the Vehicle will travel to load parts, and the Track to which the Vehicle will travel to unload the parts. There are times when based on the destination of the Vehicle you will want to route the Vehicle accordingly. This is achieved using the DESTINATION rule.

The DESTINATION rule has the following form.

#### Usage

DESTINATION dest {, dest...}rule

{DESTINATION dest {, dest}	rule}

{DEFAULT	rule}

| dest | is the Track name, and |
| --- | --- |
| rule | is the Output Rule (e.g. PUSH) |

#### Example

DESTINATION T6, T5

PUSH to T2

DEFAULT

PUSH to T1

| means | If the destination of the current Vehicle is to load at Track T6, then unload at Track T5, push the Vehicle to Track T2. Otherwise, push the Vehicle to Track T1. |
| --- | --- |

- Do exercise 3 now.

### Using Work Search to Allocate Vehicles

This method is sometimes called the 'Vehicle Control' method since the Vehicle attempts to satisfy the job that is most appropriate from the viewpoint of the Track on which the idle Vehicle currently resides. This method may be compared to a conventional independent taxi service; taxi drivers operate in ignorance of one another and pick up the first customer they see after dropping off the previous one.

To use this method, a list of Tracks where parts may be waiting to be loaded is specified for a Track. This list is known as the 'Work Search List' for that Track. When an idle Vehicle reaches the front of a Track, it scans down its Demand List and if it encounters a demand to load at a Track which appears in the Work Search List of the current Track it will be assigned to that CALL.

Figure 8-5: Work Search Form

- The order of the Work Search List DOES NOT determine the order in which demands are satisfied. The order that jobs are done depends on their order in the Demand List (e.g. job priority).

To use the Work Search List method:

- Set the transfer mode of all loading Tracks to CALL

- Set the transfer mode of all unloading Tracks to CALL

- Add a Work Search List to the appropriate Tracks

- Do exercise 4 now.

### The FINDV Statement

We know from the previous section that the Work Search List is only searched when an idle Vehicle is at the front of the Track. Consider the following example:

All the Tracks in the system being modeled have the same Work Search List and a machine has just generated a demand for a Vehicle (CALL statement). The Vehicle on the Track closest to the calling machine is 10 minutes from reaching the end of its Track. The Vehicle on the Track farthest from the calling machine is only 1 minute from reaching the end of its Track. In the model, the Vehicle farthest from the machine will be given the job although in practice the Vehicle closest to the job would be given the job.

To model this situation correctly, the FINDV action command could be used, as follows:

CALL AGV T6, T1, 1

FINDV T1, T2, T3

If an idle Vehicle is found in any of the Tracks specified in the FINDV statement, it is given the job immediately. If no idle Vehicle is found, the FINDV is ignored and the CALL statement remains on the Demand List in the normal manner.

## Zones and Capacities for tracks

### Capacity

By default, WITNESS allows only one vehicle to be on a track at any time. This may be necessary in order to maintain spacing between vehicles. However, it may also cause inefficiency in operation because a vehicle will not be able to enter a track until the previous vehicle has finished loading or unloading, and then cannot load or unload itself until it has traveled the entire length of the track. It may be more efficient to allow the vehicles to queue up at the front of the track to wait to load or unload.

In order to allow this, the “capacity” field on the track detail form should be changed from the default (1) to reflect the maximum number of vehicles, which can be on the track simultaneously. You will then see vehicles queuing at the front of the tracks and they will only have to move one position before they can start to load or unload.

### Zones

Several tracks may be grouped together in one zone using a common zone number. There are 32,767 zones available. Zones are linked to the capacities of the tracks. The maximum number of vehicles allowed in a zone is equal to the lowest capacity of any track in the zone.

This feature is useful for modeling AGVs, which often operate in confined spaces where there may not be enough room to accommodate more than one vehicle at a time.

This feature is also useful for modeling a two-way track. You can model a two-way track by placing one track on top of another of equal length, but traveling in the opposite direction. By assigning both tracks to the same zone, only one vehicle at a time will be allowed to travel on the track, thus avoiding collisions.

- Do exercise 5 now.

## Parking Vehicles

### Parking a Vehicle

A vehicle may only be parked when it is idle. If a vehicle unloads a part and becomes idle, it will park.

There are two ways to achieve this:

- After unloading you may specify a “park position”

- The DEST or DESTV commands can be used in any Actions.

We will discuss the first option in this chapter and the second, the use of the DEST and DESTV action commands later on in this Chapter.

In the unloading detail of a track, there is a field entitled Park Position. The name of the track on which you wish to park the vehicle may be specified in this field. The sequence of events will be as follows:

- The vehicle will complete unloading and become idle.

- If there is a work search on this track, it will be performed and, if successful, the vehicle will be demanded and will begin its new job (e.g. the park position will be ignored.)

- If there is no Work Search, or if the Work Search is unsuccessful, the vehicle will be given the park position as its new destination and will move according to the track Output Rules.

- As long as the vehicle is not given a new destination by a Work Search, VSEARCH, FINDV, DEST or DESTV command, it will park at the specified track.

### Waking Up a Parked Vehicle

Once a vehicle is parked, it will ignore any new calls coming into the demand list - it has effectively gone to sleep.

There are three commands that can be used to “wake-up” the vehicle:

- If either FINDV or VSEARCH is used, the vehicle will be woken up, allocated to the job, and given a new destination, provided the parking track is in the search list of these statements.

- The WAKEUP command may be used in conjunction with each CALL statement in order to wakeup the vehicle and cause it to search the parking tracks Work Search list. For example,

CALL AGV, T6, T1, 1

WAKEUP AGV, TPARK

If no job is found, the vehicle will go back to sleep. You must also have Work Search enabled on the parking track, in order to wakeup the vehicle.

- Do exercise 6 now.

## Action Commands and Built-in Functions

### Indices for Tracks and Vehicles

As with other WITNESS Elements, you can specify a quantity greater than one for both tracks and vehicles. WITNESS then provides you with two system variables, which allow you to identify an individual track or vehicle:

| N | index of the current track (compare with use for machines and other Elements) |
| --- | --- |
| I | index of current vehicle |

These indices may be used in conjunction with any logic. For example, in a CALL statement, you may specify a particular track or a particular vehicle.

#### Example

CALL AGV(3), T6(2), T1,1

### Stop Time

On the track detail form, you can specify a stop time for the vehicle on the track. This stop time is the amount of time a vehicle should stop and wait at the front of this track before continuing onto the next track.

The Stop Time specified can be set as a conditional expression, if desired. This is very useful in modeling the recharging of the batteries of a vehicle. By specifying a conditional expression for the Stop Time, involving the distance traveled, the vehicle can be forced to stop for a specified duration once this distance has been reached or exceeded.

### Sequence of Events

Actions may be executed when a vehicle first gets onto a track and when it reaches the end of the track. Since the 'Actions On' are executed as soon as the vehicle arrives at the track, they can be used to calculate the speed of the vehicle, if required.

- There are also actions that can be executed on loading or unloading the vehicle.

The order of execution of these actions is as follows:

As a vehicle joins a track:

- “Actions On” are executed.

- The speed of the vehicle is calculated.

- The vehicle checks its demand list for a job with a domain including the current track.

As a vehicle reaches the front of the track:

- “Actions On Front” are executed.

- If appropriate, the vehicle checks its demand list for jobs matching the work search list of the current track.

- If required, the vehicle stops.

- The vehicle unloads, if appropriate.*

- The vehicle loads, if appropriate.

- The vehicle attempts to push to the next track.

After unloading, the vehicle will again check its demand list for jobs matching the work search list of the current track. If nothing is found and a park position is specified for the vehicle, the appropriate destination will be set for the vehicle.

- Do exercise 7 now.

### Summaries

#### Action Commands

There are several action commands in WITNESS which are specific to tracks and vehicles. The major ones are summarized below:

| Table 8-7: Action Commands |  |
| --- | --- |
| Command | Description |
| CALL | Places a job in the demand list for the specific vehicle |
| FINDV | Looks for a vehicle to allocate to a job |
| VSEARCH | Looks for a vehicle to allocate to a job. If a vehicle is not currently available, the next idle vehicle to enter the search domain will be allocated. |
| WAKEUP | Makes a parked vehicle search the current track’s work search list. |
| DEST | Sets the immediate destination of the current vehicle |
| DESTV | Sets the immediate destination of a specified vehicle. |

#### Functions

There are several system functions WITNESS that are specific to tracks and vehicles. The major ones are summarized below:

| Table 8-8: System Functions |  |
| --- | --- |
| Functions | Description |
| DESTOF1, DESTOF2 | Return the primary and secondary destinations of a vehicle respectively. |
| DISTANCE | Returns the physical distance traveled by the specified vehicle. |
| NLOADS | Returns the number of times a specified vehicle has loaded. |
| NVEHICLE | Returns the number of vehicles currently on a specified track. |
| TRACK | Returns the name of the track currently occupied by the specified vehicle. |
| VEHICLE | Returns the name of the vehicle currently on the specified track. |

- Do exercise 8 now.

## Advanced Tracks and Vehicles

### Busy Time

The Busy Time of a track indicates the length of time the track will remain busy after a vehicle has left the track. The Busy Time is entered on the track detail form.

The Busy Time can be used where it is necessary to prevent another vehicle from using the track immediately after the first one leaves. This might be necessary to avoid a collision with a long train of the vehicle. The spare capacity of the track is not incremented until the Busy Time has elapsed.

The Busy Time can be an expression based on the number of parts on the vehicle or an expression including variables, attributes, or other functions.

### Shifts for Vehicles

Occasionally, it will be necessary to model vehicles going on and off shift. This can be done by entering the name of the shift pattern on the vehicle detail form. In the same way as with other WITNESS Elements, you can only refer to main shifts in this field.

At the end of a working period on the shift, one of the following alternatives will decide what happens to the vehicle.

| Table 8-9: Vehicle Options |  |
| --- | --- |
| Option | Description |
| Continue current job | Any job in progress when the shift ends will be completed, e.g. it will continue until the vehicle has parked or is idle. |
| Stop immediately | Any job in progress when the shift ends will be halted immediately. |

Figure 8-6: Vehicle Detail Dialog Box

#### Vehicle Speed, Acceleration and Deceleration

The speed of the vehicle when loaded and unloaded can be entered on the vehicle detail form. Speed is defined as the distance traveled in physical lengths of track in one unit of simulation time. For example, if the physical length of the track is expressed in feet (e.g. the section of track is 40 feet long) and the simulation time unit is minutes, vehicle speed would be expressed in feet per minute.

The speed can be an expression based on the number of parts on the vehicle or an expression including variables, attributes, or other functions.

When a vehicle starts or stops, it usually takes some time to accelerate up to or decelerate down from the specified speed. To model this in WITNESS, you can set a “time delay” which is added to the time that the vehicle would normally take to move one position on the track. The “time delay” is entered on the vehicle detail form.

For example, a vehicle cruises at the speed of 60 mph (e.g. one mile per minute) and has an acceleration time from 0-60mph of 6 seconds. To work out the time delay we first need to work out the distance covered during the acceleration. Assuming linear acceleration from 0-60mph, average speed during acceleration is 30mph (e.g. 0.5 miles per minute)

|  | Distance covered during acceleration | = | (6/60) minutes * 0.5 miles/minute (time * speed) |
| --- | --- | --- | --- |
|  |  | = | 0.05 miles |
| Therefore, |  |  |  |
|  | Distance remaining after acceleration | = | 1 - 0.05 |
|  |  | = | 0.95 miles |
|  | Time to cover remaining distance | = | 0.95 miles * 1.0 miles/minute |
|  |  | = | 0.95 minutes (e.g. 57 seconds) |
|  | Total time to cover total distance | = | 6 + 57 |
|  |  | = | 63 seconds |
| Therefore, |  |  |  |
|  | Time Delay | = | 3 seconds |

An expression based on the vehicle’s current load may be used for the time delay.

- Do exercises 9-12 now.

## Exercises

### Exercise 1

#### Vehicle Always Loading and Unloading

Parts are produced and loaded into a pallet that is placed into a buffer B1 to await collection by an AGV (Automatic Guided Vehicle). These pallets arrive in the buffer one at a time every 2 to 5 minutes.

The AGV picks up the pallet and takes it onto the next process following the route shown in the diagram below. The AGV unloads the pallet onto a roller conveyor that takes the pallet to the next process.

Figure 8-7: Exercise 1 Layout.

| Table 8-10: Exercise 1 Data |  |  |
| --- | --- | --- |
| Element | Characteristics |  |
| AGV | Speed: | 50 meters/min |
|  | Capacity: | 1 pallet |
|  | Loading time: | 1.0 min |
|  | Unload time: | 1.8 min |
| Buffer | Capacity: | 50 parts |
| Conveyor | Cycle time: | 0.25 min |
|  | Part Length: | 10 |

Display a count of the number of pallets shipped on the screen.

### Exercise 2

#### Vehicle Loading/Unloading when IF Condition Satisfied

A crate of parts is delivered to a gate by an outside supplier in batches of 10 every hour. The AGV has been re-routed so that it always visits the gate to pick up any crates that have been left there before proceeding to buffer B1. It takes 1 minute to load the crates. The crates are unloaded at the roller conveyor in the same way as the pallets. If there are no parts at the gate or at B1, the AGV will not wait, but will continue around the system. (Gate capacity is 50)

Figure 8-8: Exercise 2 Layout

##### Balancing the System

How many Vehicles are required to cope with the demand?

### Exercise 3

#### Allocating Vehicles to Demands Using VSEARCH

It has been decided to implement a control system so that the AGVs are requested when a pallet arrives at B1, or a crate arrives at the gate respectively. The AGV will then be allocated to go and satisfy this demand (e.g. jobs look for free Vehicle).

The AGV will take the shortest route to the assigned destination following the existing layout.

### Exercise 4

#### Allocating Vehicles to Demands Using Work Search

Due to a change in policy, management has decided not to implement an expensive system that would cause a part awaiting Vehicle pickup to request any free vehicle in the system. They have decided to implement a more inexpensive system such that once a Vehicle has completed unloading a part, it will search the Demand List and take the next job from this Demand List. Change your model to reflect this new functionality.

### Exercise 5

#### Improving the Flow

Having decided to implement the work search approach to allocate the AGVs, management is now looking for ways to improve the flow of the AGVs around the system.

After a study of the route that the AGVs take to the gate, it was decided, for safety reasons and due to the narrow aisle widths en route to the gate, that only one AGV should be allowed between the front of T4 and the gate at any one time.

It was also decided to allow the queuing of vehicles on the loading and unloading tracks.

Finally, the management authorized the building of a shortcut track, parallel to T4, to allow AGVs which had collected crates from the gate to take a shortcut back from the gate rather than continuing around T1, T2 and T3. However, only one vehicle would be allowed to unload onto the roller conveyor at any time. To facilitate this a one-position unloading track was constructed. This track takes AGVs from both T3 and the new shortcut track.

### Exercise 6

#### Parking Vehicles

In order to continue the improvements to the system, when an AGV becomes idle it will be sent to a parking track rather than circling around the system empty. This parking track, TPARK, is going to be constructed at the end of the track T4. Once an AGV becomes empty, it will park on this track until it is required again.

### Exercise 7

#### Recharging Batteries

The AGVs need to recharge their batteries after traveling a distance of about 5000 meters. The recharging takes place at the front of T4 and takes 10 minutes.

### Exercise 8

#### Using Action Commands and Built-In Functions

Add the following to your model:

- Assign the AGVs a parking destination when they first enter the model, so that they do not run around the system empty.

- If there are a number of vehicles queuing on T3 and T9 (the shortcut parallel track), then preference is given to the vehicle at the front of the track with the most vehicles on it for moving onto the unloading track.

- In order to prevent an AGV that is recharging its batteries from blocking a track, a small bypass is to be built alongside the front of T4. When an AGV needs to recharge its batteries, it will now be diverted from T4 to this recharging track, thus allowing other AGVs to continue around the system.

### Exercise 9

#### Variable Speed

It is now necessary to model the speed of the AGVs more accurately. The speed of an AGV depends on whether it is loaded or unloaded and which type of part it is carrying. Incorporate the following data in you model.

| Table 8-11: Exercise 9 Data |  |  |
| --- | --- | --- |
|  | Part | AGV Speed |
| Unloaded | none | 50 meters/min |
| Loaded | pallet from B1 | 40 meters/min |
|  | crate from gate | 30 meters/min |

### Exercise 10

#### Representing Labor with Tracks and Vehicles

Design a simple model that uses tracks & vehicles to the physical transfer of a part from one machine to another. The vehicle will represent the laborer who actually carries the part, and the track will represent the path the laborer takes from one location to another. Use only one set of tracks that can allow travel in a left-to-right fashion.

### Exercise 11

#### Loading and Unloading Multiple Parts at Different Locations

##### Part 1

Using Exercise 9. Management would like to expand the operation to include a new assembly and packaging area. The crates and pallets coming off the roller conveyor will now be picked up by an AGV (AGV_2) operating on a separate track and vehicle system per the layout below.

Figure 8-9: Exercise 11 Layout

The system must be designed to operate as follows:

- Starting from track T11, AGV_2 will move to the front of this track and will pick up (1) part A and (1) part B from buffer B4.

- AGV_2 continues down track T12, then loads (1) pallet and (1) crate from the conveyor.

- AGV_2 then travels down track T9 and drops off all of the parts in buffer B3.

- Machine M1 must remove the parts from the pallet and the crate, place the parts in buffer B3, and then scrap the empty pallet and crate. Each pallet contains (1) part D and (1) part E. Each crate contains (1) part F and (1) part G. The operation to remove parts from a pallet or crate takes 2 minutes.

- AGV_2 then picks up all the parts in buffer B3 and travels up track T10.

- At the end of track T10, AGV_2 unloads all the parts into the Packager machine for final preparation for shipment..

- After dropping off the parts, Vehicle AGV_2 then moves to track T11 to start the process again.

Table 8-12: Exercise 11 Data

Information

Parts A and B each arrive into Buffer B4 at the rate of 2 every hour.

Buffers B3 and B4 each have a capacity of 10, and parts can be removed from any position.

Vehicle AGV_2 travel speed is 25 meters/minute.

The Packager operation takes 5 minutes per batch.

Machine M1 process time is 1 minute.

##### Part 2

During a presentation of the model of the proposed system, management points out that the batches of parts that the Packager is shrink wrapping and shipping contain different parts from batch to batch. Each batch must contain only one of each of the six parts: A, B, D, E, F, and G. A short examination reveals the cause of the problem: crates and pallets come off the roller conveyor at unequal rates.

To solve the problem, management will only allow changes to the control logic (affecting how the vehicles move on the tracks and load/unload parts) for the original track and vehicle system because parts to build it are already on order.

| HINT: | Change the model so the crates and pallets are made available to the 2nd track and vehicle system via the conveyor in the required sequence. |
| --- | --- |

- Save this model as MULTPAER.MOD

### Exercise 12

#### Loading Vehicles with Capacity Expressed in Terms of Weight

In WITNESS, the capacity of a vehicle is specified in terms of the quantity of parts. In practice, a vehicle such as an AGV may need to transport parts of varying types, quantities, sizes, and weights. Therefore, it may be necessary to specify vehicle capacity in other terms. Create a new model per the layout below:

Figure 8-10: Exercise 12 Layout

A single AGV travels around a system of four tracks as shown above. When the AGV reaches the front of track T1, it will begin loading parts of varying types and weights from a buffer. If no parts are present, the AGV will continue around the track. Loading will continue until its weight capacity of 200-lbs is reached, or there are no more parts in the buffer, whichever occurs first. At this time, a sensor will automatically stop the loading cycle. The AGV will then travel to track T2, then continue to track T3 where it will unload all parts into a tumbler for de-burring. The AGV then returns to the front of track T1 to pick up another load of parts. When the tumbler receives a batch of parts, it begins processing immediately. When finished, the de-burred parts are sent to shipping.

Table 8-13: Exercise 12 Data

Information

Vehicle AGV speed is 1 meter/minute.

Set vehicle capacity to 500 parts, though total part weight will determine actual capacity for this exercise.

Buffer has a capacity is 500 parts and operates on a FIFO basis.

Tumbler machine processing time is 10 minutes per batch.

| Table 8-14:Part Data |  |  |
| --- | --- | --- |
| Part | Arrival Rate | Weight |
| Light | 2 every 25 to 60 minutes | 1 to 10-kg |
| Medium | 2 every 4 to 100 minutes | 11 to 30-kg |
| Heavy | 2 every 12 to 44 minutes | 31 to 60-kg |

- Save this model as WEIGHT.MOD.

## Case Studies

### International Airport Parking Lot Courtesy Bus Service

The Get-U-There Courtesy Bus Service would like to build a simulation model of their operation at the Crash and Burn International Airport. The bus service uses electric-powered buses to shuttle passengers between two different airport terminals and five different parking structures.

Figure 8-11: Airport Parking Layout

Passengers arrive at each of the terminal and parking structure bus stops for pick-up at the rates shown in Table 8-15. Each passenger arriving at a terminal on a flight from another airport will need a ride to one of the parking structures as explained in Table 8-15. Each passenger arriving at a parking structure to catch an outbound flight will need transport to one of two terminals as explained in Table 8-15.

Buses travel continuously around the airport via a one-way road system. As the driver passes the designated shuttle bus stop located at teach terminal or parking structure, he will stop only if he has passengers to drop off there, or if any passengers are awaiting pick-up. Otherwise, the driver will continue around the track until he finds a terminal or parking structure where he can pick up or drop off. If passengers are waiting for pick-up at a particular stop, the driver will attempt to pick up all of them.

As passengers are dropped off at their destinations, it is assumed that they immediately leave the airport, e.g., leave the model via SHIP.Data

| Table 8-15:Passenger Arrival Rates at Terminals & Parking Structures |  |  |  |
| --- | --- | --- | --- |
| Pick Up or Drop Off Location | Time Between Arrivals (minutes) | Number of Passengers Arriving | Comments |
| Terminal #1 | Normal mean 10 minutes, std. dev. 1.5 minutes | Uniform Min 1 Max 5 | Passengers from inbound flights that need transport to either Parking Structure A, B, or C on a random basis |
| Terminal #2 | Normal mean 10 minutes, std. dev. 1.5 minutes | Uniform Min 1 Max 5 | Passengers from inbound flights that need transport to either Parking Structure D or E on a random basis |
| Parking Structure A | Normal mean 10 minutes, std. dev. 1.5 minutes | Uniform Min 1 Max 5 | Passengers destined for outbound flights that need transport to either Terminal #1 or #2 on a random basis |
| Parking Structure B | Normal mean 10 minutes, std. dev. 1.5 minutes | Uniform Min 1 Max 5 | Passengers destined for outbound flights that need transport to either Terminal #1 or #2 on a random basis |
| Parking Structure C | Normal mean 10 minutes, std. dev. 1.5 minutes | Uniform Min 1 Max 5 | Passengers destined for outbound flights that need transport to either Terminal #1 or #2 on a random basis |
| Parking Structure D | Normal mean 10 minutes, std. dev. 1.5 minutes | Uniform Min 1 Max 5 | Passengers destined for outbound flights that need transport to either Terminal #1 or #2 on a random basis |
| Parking Structure E | Normal mean 10 minutes, std. dev. 1.5 minutes | Uniform Min 1 Max 5 | Passengers destined for outbound flights that need transport to either Terminal #1 or #2 on a random basis |

| Table 8-16: Other Data |  |  |
| --- | --- | --- |
| Description |  | Value |
| Total number of buses available | 3 |  |
| Bus capacity | 10 passengers |  |
| Bus loading/unloading time | 30 seconds/passenger |  |
| Bus speed limit (posted on road) | 25 miles/hour |  |
| Bus maximum speed | 40 miles/hour |  |
| Capacity of tracks | (1) bus maximum on a track at any time |  |
| Maximum number of passengers that may wait at any location | 50 |  |

#### Rules & Assumptions

When passengers are dropped off at terminals or parking structures, they immediately depart for their destinations (exit the model).

Since parts (the passengers) can only be unloaded from a vehicle in WITNESS in the same order they are loaded (e.g.; like a FIFO buffer). This presents a problem because it may be necessary for passengers in the “middle” or “back” of the bus to get off before the ones in the “front” (first ones on). Therefore, it is acceptable to add “dummy” Elements to the model to allow passengers to be “temporarily” unloaded from the bus to let off those who need to get off at a particular stop. However, since this does not actually occur in the real system, calculation of the passenger load and unload times must ONLY include the time to load or unload passengers actually getting off at a particular stop.

#### Base Model Validation

The total number of passengers carried to their destinations by the bus system in the first 24 hrs of operation should be approximately 450.

##### Simulation Project Objective

Make changes to the model within the constraints indicated in Table 8-17 to minimize the total number of passengers waiting for service and the total number who left because a line was too long, at all terminals and parking structures.

| Table 8-17:Variables and Constraints |  |  |
| --- | --- | --- |
| Parameter | Minimum Value | Maximum Value |
| Number of buses | 1 | 10 |
| Speed of bus on any track | 10 miles/hour | 40 miles/hour |
| Increase capacity of tracks | 1 bus | 3 buses |

- Save this model as BUSSERVC.MOD

### Automated Courtesy Bus System Feasibility Study

The Get-U-There Courtesy Bus Service would now like to determine the feasibility of replacing their current bus service with an automated vehicle system. The vehicles would travel the same route as the current buses, but would be guided by special tape applied to the existing roadways. A central computer would control operation of the vehicles. The vehicles would be programmed to pick up only passengers going to the same destination. Only after these passengers are dropped off at their destination would the vehicle become available to pick up more passengers.

Using the existing case study model as a starting point, build a model of this proposed system.

#### Layout

Same as Case Study #1.

#### Data

Same passenger arrival rates as Case Study #1.

#### Rules & Assumptions

Passengers will await pick-up in separate lines according to their destination at each parking structure or terminal. The total capacity of all lines (in number of passengers) at each location cannot exceed 50. For example, if there are two lines at a location, the capacity of each must be 25. When passengers are dropped off at terminals or parking structures, they immediately depart for their destinations (exit the model).

#### Base Model Validation

The total number of passengers carried to their destinations by the bus system in the first 24 hrs of operation should be approximately 450.

Simulation Project Objective

Make changes to the model using the same variables and constraints as the first case study model (see Table 8-18), and with the same objective.

| Table 8-18:Variables and Constraints |  |  |
| --- | --- | --- |
| Parameter | Minimum Value | Maximum Value |
| Number of buses | 1 | 10 |
| Speed of bus on any track | 10 miles/hour | 40 miles/hour |
| Increase capacity of tracks | 1 bus | 3 buses |