Progress Review

Objectives

- To observe a step-by-step example of a how a typical manufacturing simulation model is constructed.

- To use your current WITNESS knowledge to create your own simulation models.

## Example: Fan Blade Manufacturing

### Introduction

This case study model is designed for students and instructors as an exercise to walk-through and learn how a typical manufacturing simulation model is constructed. With the notes below, this case study should make a good topic for group discussion, using the completed model provided, FANBLADE.MOD. This completed model is referred to throughout this document.

At the end of this document, there is an abbreviated Model Library listing of FANBLADE.MOD.

The model shows an injection molding machine line producing plastic polymer cooling fanblades for five types of engines: 4 cylinder, 5 cylinder, 3-liter V6, 4-liter V6, and V8. The fanblades are produced as replacement parts, which are subsequently shipped to car dealership service departments and automotive parts retailers.

### Basic Process Steps

Each molded fanblade goes through a variety of steps before being shipped.

- Five injection molding machines, which create the original fanblades

- A Trim station, which trims any flashing or scrap plastic from the fanblades

- An Inspect station, which inspects fanblades for flaws, cracks, or cold-shuts. The ones that do not pass inspection are sent to SCRAP (currently at 10%)

- Balancer station, which spins each fanblade up-to-speed to make sure it spins true, and does not come apart in service. 10% of the fanblades fail this test as well, and go to scrap.

- Packer station, which packs each fanblade into a cardboard box and seals it

- Labeler station, which puts a label on each packed box. The label contains information such as manufacturer, serial number, model number, SKU, retail price, etc.

- After the Labeler station, the fanblades are shipped out

### Detailed Description of Model

#### Molding Machines

The five Molding Machines are collectively a quantity-5 machine called "Molder". Each of the five molding machines produces a different style of fanblade, indicated by the legend near the top of the simulation window. The different styles of fanblades are: 4 cylinder (blue), 5 cylinder (red), 3-liter V6 (white), 4-liter V6 (cyan), and V8 (yellow).

The Molder machines are single-type machines that pull fanblade parts out of WORLD using the following Input Rule:

SELECT on N Fan_4CYL out of WORLD,Fan_5CYL out of WORLD, Fan_3LV6 out of WORLD,Fan_4LV6 out of WORLD,Fan_V8 out of WORLD

This SELECT Input Rule pulls a single fanblade out of WORLD (assumed to be an infinite stock), based on which is the current machine pulling a part. The current machine pulls the proper part into the proper machine by using the system variable "N" in the SELECT rule. The value of N in this case can vary between 1 and 5, since Molder is a quantity-5 machine. The value of N indicates which of the 5 fanblade types specified in the SELECT rule are to be pulled from WORLD. So for example, if N happens to equal 3, then machine Molder (3) will pull a Fan_3LV6 part from WORLD.

The cycle time for all five Molder machines is UNIFORM (5,10,N). This generates a uniform floating-point random number between 5.0 and 10.0. N is used again as the random number stream designator, which will range from 1 to 5, depending on which machine it is.

The Actions on Finish accumulates some statistical information about the model as fanblades are being produced:

NoIn = NoIn + 1

Nmold (N) = Nmold (N) + 1

WIP = NoIn - NoOut - NSCRAP (Fan_4CYL) - NSCRAP (Fan_5CYL) - NSCRAP (Fan_3LV6) - NSCRAP (Fan_4LV6) - NSCRAP (Fan_V8)

The variable NoIn is tallying how many fanblades have entered the system so far. This variable is displayed in the lower left-hand corner of the model as "No. Entered:.”

The Nmold variable is a quantity 5 variable that is used to keep track of how many fanblades EACH molding machine is creating. This information is displayed on the screen directly below each molding machine, as red text on a cyan background. This helps the viewer see at a glance how many of each fanblade has been produced by each molding machine.

The WIP variable calculates the amount of Work in Process currently in the model. The NoOut variable is tracked at the front (departing end) of the shipping conveyor (shipconv). The WIP is displayed on the model screen above the Inspect station.

The Output Rule for Molder is:

PUSH to OutMold(N) at Rear

Again, N is used to select which of five "chutes" each fanblade dumps to after it has been molded. In our case, these chutes are represented by a quantity 5 conveyor called OutMold.

The Molder's themselves are displayed using a lathe icon bitmap file from the Bitmap directory in the WITNESS installation. In addition, a small status icon is attached to the front of each Molder icon (near the headstock of each lathe, actually). This status icon changes color based on the current state of each molding machine (Idle = yellow, Busy = green, Blocked = purple).

Labels for each molding machine have been placed above each one to indicate the type of fanblade it produces.

#### Molding Machine Chutes

There are five vertical conveyors called "OutMold" at the bottom of each molding machine. OutMold is a quantity 5 conveyor.  They direct each fanblade produced to a long horizontal moving conveyor (MoldConv) which moves from left to right.

The Length In Parts is 4, and the Index Time is 0.5, and the Input Rule is Wait. It is a queuing conveyor, which will allow fanblades to stack up on the OutMold conveyors even if the conveyors are not moving. The Output Rule is:

SELECT on N MoldConv at (6), MoldConv at (12), MoldConv at (18), MoldConv at (24), MoldConv at (30)

This SELECT Output Rule operates similarly to the Input Rule in the Molder multiple quantity machine. That is, N is again used to select one of five different options. In this case, each of the five conveyors is directed to push their parts to a particular location on the long horizontal conveyor (MoldConv). There are five locations on the MoldConv that fanblades are being deposited: 6, 12, 18, 24, and 30 (you can see this in the Output Rule above). By doing this, we simulate a situation where fanblades will push to a specific spot on the conveyor, as long as there is an empty slot on the long horizontal conveyor. If there is no open slot then fanblades will start to back-up on the vertical OutMold conveyors. As an example, OutMold (3) will push its fanblades to location 18 on MoldConv.

The OutMold conveyors have been outlined with colored lines using the Screen Editor, for effect.

#### Horizontal Molding Conveyor

This conveyor is a queuing conveyor that moves from left to right, carrying accumulated fanblades pushed from the vertical OutMold conveyors to the Trim station.

The Input Rule is Wait, Length in Parts is 30, the Index Time is 1.0, and the Output Rule is:

PUSH to ToTrim

#### Trim Station

The Trim station removes any flashing or scrap plastic from the fanblade because of the molding operation. It is preceded by a ToTrim buffer that has a capacity of 10. ToTrim holds 10 fanblades until Trim is ready to process them, one at a time.

The Trim machine's Input Rule is:

PULL from ToTrim

The Cycle Time is NORMAL (2,0.1,11) and it's Output Rule is:

PUSH to ToInspec

ToInspec is the buffer just before the Inspect station.

#### Inspect Station

The Inspector inspects each fanblade for cracks, inclusions, and cold-shuts, or any other molding flaws. He/she currently rejects about 10% of the fanblades as scrap. Although the Inspector is considered a person, Labor is not required.

The Inspector machine's Input Rule is:

PULL from ToInspec

The Cycle Time is UNIFORM (2,3,12) and it's Output Rule is:

PERCENT /41 ToBalanc 90.00 ,SCRAP 10.00

This Output Rule simulates the inspection scrap rate. The "/41" is the random number stream chosen for the PERCENT rule. It was arbitrarily chosen as 41 - as long as 41 is not used anywhere else in the model. Anything not scrapped is sent to the ToBalanc buffer.

The Actions on Start, SET ICON of Inspect to 48, simply picks one of two icons as the machine runs. It gives the effect of a fanblade showing up in the Inspector's icon magnifying glass.

The Actions on Finish is:

Scrapped = NSCRAP (Fan_4CYL) + NSCRAP (Fan_5CYL) + NSCRAP (Fan_3LV6) + NSCRAP (Fan_4LV6) + NSCRAP (Fan_V8)

SET ICON of Inspect to 36

This code calculates the total number of fanblades scrapped by Inspect. It also sets the icon back to an inspector with an empty magnifying glass.

#### Balance Station

The Balance station spins each fanblade up to speed to make sure it will not come apart under true working conditions. It is also balanced as required, by adding small weights around the circumference of the hub (the weights are not modeled in this simulation). The Balance station is preceded by a ToBalanc buffer that has a capacity of 10. ToBalanc holds 10 fanblades until Balance is ready to process them one at a time. The Balance station also rejects about 10% of the fanblades.

The Balance machine's Input Rule is:

PULL from ToBalanc

Its Cycle Time is UNIFORM (0.5,1.0,13) and it is Output Rule is:

PERCENT /20 ToPacker 90.00 ,SCRAP 10.00

ToPacker is the buffer just before the Packer station.

The Actions on Start, SET ICON of Balance to 46, and the Actions on Finish, SET ICON of Balance to 47, create an effect that gives the appearance of the fanblade spinning in the Balancer icon.

#### Packer Machine

The Packer takes a fanblade and packs it into a box. For simplicity, the combining of fanblades and boxes is not modeled. Rather, we simply change the icon of each fanblade to a box of the same color.

The Packer machine is preceded by a ToPacker buffer that has a capacity of 10.

The Packer machine is a single machine, with a cycle time of UNIFORM (1,3,14) and an Input Rule of:

PULL from ToPacker

Its Output Rule is:

PUSH to ToLabel

ToLabel is a buffer just before the Labeler machine. It has a capacity of 5 parts.

The Actions on Start:

SET ICON of Packer to 44

Actions on Finish:

ICON = ICON + 5

SET ICON of Packer to 38

These simply set the robot icon to animate up and down, to indicate that something is being done. The ICON = ICON + 5 was a simple way to call up the properly colored icon for the packed fanblade (colored squares). In FANBLADE.MOD, look at the icon library (from the Icon Editor under View). You will see that the packed colored boxes are each 5 icon slots away from the original colored fanblade icons. Thus referring to them with "+ 5" is a convenient way to change their icons appropriately.

#### Labeler Machine

The Labeler puts a label on each boxed fanblade. The label has the model number, retail price, and manufacturer. The Labeler pulls the unlabeled boxed fanblades from the ToLabel buffer. ToLabel is a buffer with a capacity of 10.

The Labeler is a single machine who's Cycle Time is 1.0, and its Input Rule is:

PULL from ToLabel

Its Output Rule is:

PUSH to ToShip

The ToShip buffer comes right after the Labeler, just before the "shipconv" conveyor. It stores packaged and labeled fanblades.

#### Shipping Conveyor

The "shipconv" conveyor moves packaged fanblades from the ToShip buffer to the shipping area to be shipped. It is a 20-length queuing conveyor with an Index Time of 1.0. Its Input Rule is:

PULL from ToShip

Its Output Rule is:

PUSH to SHIP

In the Actions on Finish, the expression NoOut = NoOut + 1 tallies the number of fanblades that have gone out of the model. This variable is used in the Actions on Finish for the Molder machines to calculate the amount of WIP in the model. The NoOut variable is displayed inside a rectangle, with a title "Shipping.”

#### Pie Charts

There are five pie charts being displayed, one each for the Trim, Inspect, Balance, Packer, and Labeler stations. These pie charts are configured to show the relative duration of the Idle (yellow), Busy (green), and Blocked (purple) states. The model warm-up period is 30, so you will see the pie charts reset to three equal sectors at that time. The pie charts are very convenient to see how much relative time each of these five machines are spending in these three states. Having too much showing in any one state usually indicates some sort of problem.

If a machine is spending a lot of time Idle (yellow), that means that it is not receiving enough parts to satisfy the amount of capacity it has to do productive work, e.g., it is "starved". This can mean that there is a blockage of some sort upstream.

If a machine is spending a lot of time Busy (green), that is good, as long as it's not so busy as to create a blockage upstream, and starvation downstream. If blockages upstream are occurring, and there is too much starvation downstream, then it means the Element you are looking at is likely the cause of the blockage. The blockage-causing Element is sometimes referred to as the "pacesetter,” and if it is on your process's critical path, it could indicate a serious problem.

If a machine is spending a lot of time in the Blocked state (purple/magenta), that means the machine is finished doing what it needs to do, but cannot get rid of it's part(s) because something else downstream from it cannot receive parts fast enough. More times than not, the one or two buffers or machines ahead of a blocked machine are the ones causing the blockage (and are probably busy or full most of the time). It is important to note that a blocked machine is usually NOT the cause of the blockage. Rather, it should be interpreted as an indication of a blockage (or blockages) somewhere downstream from it.

### Problems Inherent in the Model

- The mix of different fanblades produced (4 cylinder, 5 cylinder, 3-liter V6, 4-liter V6, and V8) is not consistent.

- The production line is producing too many of some fanblade types and not enough of others, as evidenced by running the model and watching the 4 and 5 cylinder fanblade molding machines clog up (become blocked) over time.

- The manufacturer is having trouble filling orders for some fanblades, and at the same time, they are producing an inventory surplus for others. For example, we are creating too many V8 and V6 fanblades, but hardly any 4 and 5 cylinder fanblades.

- There is a 10% scrap rate at the Inspect and Balance stations.

- Things are apparently clogging up at the Inspector as well.

- The Balancer, Packer, and Labeler stations are "starved". In other words, they are idle more of the time than they ought to be. It is costing the company money to pay people and have them be idle.

Management wants something done to improve the production line's throughput.

### Possible Solutions

#### Suggestion # 1

If you run the model long enough, you will see that things are clogged at Inspect. We can perhaps assume that either the Inspector needs to be trained more thoroughly, or they need newer, faster inspection equipment. Perhaps you should even consider hiring another inspector to see if that helps things. Add another one. Go to inspect detail, set Quantity to 2. Two inspectors now are able to flush parts downstream, lessening the blockage. Also, note that you will see another inspector icon show up to the right of the original one, indicating that you now have two inspectors running in tandem. You will see that the fanblades in the ToInspec buffer will slowly reduce, allowing the Trim station to work more efficiently.

A bright, young Industrial Engineering graduate from Ohio State was recently hired by the company to help diagnose this production line. He took one look at the molding machine conveyors and came to a brilliant conclusion. He said that it would make much more sense from a product mix standpoint, to have the short vertical conveyor chutes (OutMold) direct their contents to the REAR of the longer horizontal conveyor (MoldConv), instead of trying to drop fanblades onto MoldConv wherever there is room. Realizing that this is a very costly proposition (to bust up concrete and redo the conveyors), and the fact that the student is relatively inexperienced, management has told the new IE to check out his idea first using WITNESS rather than reconfiguring the shop floor and hoping for the best.

To do this, change the Output Rule of OutMold from:

SELECT on N MoldConv at (6),MoldConv at (12),MoldConv at (18), MoldConv at (24),MoldConv at (30)

…to…

PUSH to MoldConv at Rear

This simply causes ALL fanblades to start their journey down MoldConv at the left end (Rear) of the conveyor. This gives each one an equal chance of landing on the MoldConv, regardless of the blockages created in Trim, Inspect, etc. Now, whenever there happens to be blockage, the Blocked state winds up being distributed between all five molding machines rather than just the 4 and 5 cylinder fanblade molding machines. This greatly reduces the inventory stocking problems, and just about all of the customer complaints about backordered fanblades no longer occur. Once you change the OutMold Output Rule as indicated above, you will indeed see a very consistent mix of fanblades, regardless of any blockage in the system. This is clearly indicated by the number of fanblades being produced by each machine, as shown by the red-on-cyan counters being displayed near the bottom of each machine (at the intersection of each vertical OutMold conveyor with the horizontal MoldConv). The IE was promoted.

#### Suggestion # 2

Now that the Inspect station is moving more freely, we see that the Trim station is working to keep up. Let us say that we have had our Mechanical Engineers look at the problem, and they have determined that the scrap rate (10%) is too high, indicating that the quality of fanblades being produced is inferior. Therefore, management decides to throw some money at the problem and have the injection molding machine dies redone to produce a better quality fanblade. As a result we find that the Trim person is able to do their job in an average of 1.5 minutes instead of 2.0 (e.g., we shaved 30 seconds off the time it takes them to do their job). Go to the Trim detail and change NORMAL(2.0,0.1,11) to NORMAL(1.5,0.1,11). This changes the average cycle time of Trim from 2.0 to 1.0. The model runs with things a bit more balanced now, and there are more fanblades being produced. We will assume that there is not much more we can do to improve the quality of the fanblades at this point, so there is not much sense in reducing the Trim machine's cycle time without it being impossible to do in real life.

#### Suggestion # 3

Since we improved the quality of fanblades, we have also reduced our scrap rate by 5%, thus instead of 10% scrap from both the Inspect and Balance stations, we are getting only 5% scrap, which is great news. Go to the Output Rules of both Inspect and Balance and change the proportions for both from 90.00/10.00 to 95.00/5.00 (e.g.,. SCRAP gets only 5% of the parts). Well, by reducing the scrap rate to 5% because of an increase in the quality of the fanblades, we have unknowingly caused another problem. Since there are now more fanblades going from Inspect to Packer than there were before (because we reduced the scrap rate), the Packer now becomes very busy. In fact, the Packer is now the blockage in the system. Note that we are also successfully flushing the "blockage" downstream. At this point, you could do any of a number of things, such as hire another Packer (relatively inexpensive labor), or perhaps you could modify the packing process by packing fanblades into cases (12 to a case) rather than one per box. This would likely make the packing area more efficient, as well as save you the cost of boxes over the long run. The price per fanblade would also be cheaper since the customer is buying them in bulk instead of one at a time.

### Other Items

The Screen Editor was used to draw a cyan outline of the counterclockwise route the parts take. It was also used to draw the white arrows indicating the direction of flow.

Two legends were created from scratch, using the Screen Editor. One is at the upper left and shows larger versions of the fanblade icons, showing them what they are. The other shows the three possible Element states (Idle, Busy, and Blocked).

Make use of the Report menu to show the statistics of all the Elements after the model has run for a while. The Pie Charts are nice to show relative times spent idle, busy, and blocked, but it’s only when you look at the statistical reports that you get the big picture.

The animated icons for Trim, Inspect, Balance, Packer, and Labeler were drawn as custom icons, using the Icon Editor. The lathe icon (used by the Molder machines) was imported from the file Lathe64.bmp from the Bitmap directory in the WITNESS installation area.

Figure 7-1: FANBLADE.MOD at 5000 Time Units, Before Suggested Solutions

Figure 7-2: FANBLADE.MOD at 5000 Time Units, After Suggested Solutions

What follows is an abbreviated Model Library listing of FANBLADE.MOD.

#### Define

PART: Fan_4CYL,Variable attributes;

PART: Fan_5CYL,Variable attributes;

PART: Fan_3LV6,Variable attributes;

PART: Fan_4LV6,Variable attributes;

PART: Fan_V8,Variable attributes;

BUFFER: ToTrim,1,10;

BUFFER: ToInspec,1,10;

BUFFER: ToBalanc,1,10;

BUFFER: ToPacker,1,5;

BUFFER: ToLabel,1,10;

BUFFER: ToShip,1,10;

MACHINE: Molder,5,Single,0,0;

MACHINE: Trim,1,Single,0,0;

MACHINE: Balance,1,Single,0,0;

MACHINE: Inspect,1,Single,0,0;

MACHINE: Packer,1,Single,0,0;

MACHINE: Labeler,1,Single,0,0;

CONVEYOR: OutMold,5,Queuing,4;

CONVEYOR: MoldConv,1,Queuing,30;

CONVEYOR: shipconv,1,Queuing,20;

VARIABLE: NoIn,1,1,Integer;

VARIABLE: NoOut,1,1,Integer;

VARIABLE: Nmold,1,5,Integer;

VARIABLE: WIP,1,1,Integer;

VARIABLE: Scrapped,1,1,Integer;

PIE_CHART: TrimPie,1;

PIE_CHART: InspPie,1;

PIE_CHART: BalPie,1;

PIE_CHART: PackPie,1;

PIE_CHART: LabelPie,1;

#### Detail

Parts

Fan_4CYL

NAME OF PART: Fan_4CYL;

MAXIMUM ARRIVALS: 0;

ACTIONS, Create

Add

ICON = 23 ! Set icon to blue fanblade.

OUTPUT RULE: Wait;

Fan_5CYL

NAME OF PART: Fan_5CYL;

MAXIMUM ARRIVALS: 0;

ACTIONS, Create

Add

ICON = 24 ! Set icon to red fanblade.

OUTPUT RULE: Wait;

Fan_3LV6

NAME OF PART: Fan_3LV6;

MAXIMUM ARRIVALS: 0;

ACTIONS, Create

Add

ICON = 25 ! Set icon to white fanblade.

OUTPUT RULE: Wait;

Fan_4LV6

NAME OF PART: Fan_4LV6;

MAXIMUM ARRIVALS: 0;

ACTIONS, Create

Add

ICON = 26 ! Set icon to cyan fanblade.

OUTPUT RULE: Wait;

Fan_V8

NAME OF PART: Fan_V8;

MAXIMUM ARRIVALS: 0;

ACTIONS, Create

Add

ICON = 27 ! Set icon to yellow fanblade.

OUTPUT RULE: Wait;

Buffers

ToTrim

NAME OF BUFFER: ToTrim;

QUANTITY: 1;

CAPACITY: 10;

ToInspec

NAME OF BUFFER: ToInspec;

QUANTITY: 1;

CAPACITY: 10;

ToBalanc

NAME OF BUFFER: ToBalanc;

QUANTITY: 1;

CAPACITY: 10;

ToPacker

NAME OF BUFFER: ToPacker;

QUANTITY: 1;

CAPACITY: 5;

ToLabel

NAME OF BUFFER: ToLabel;

QUANTITY: 1;

CAPACITY: 10;

ToShip

NAME OF BUFFER: ToShip;

QUANTITY: 1;

CAPACITY: 10;

Machines

Molder

NAME OF MACHINE: Molder;

QUANTITY: 5;

TYPE: Single;

CYCLE TIME: UNIFORM (5,10,N);

ACTIONS, Finish

Add

NoIn = NoIn + 1

Nmold (N) = Nmold (N) + 1

WIP = NoIn - NoOut - NSCRAP(Fan_4CYL) - NSCRAP(Fan_5CYL) - NSCRAP(Fan_3LV6) - NSCRAP(Fan_4LV6) - NSCRAP(Fan_V8)

INPUT RULE: SELECT on N Fan_4CYL out of WORLD,Fan_5CYL out of WORLD, Fan_3LV6 out of WORLD,Fan_4LV6 out of WORLD,Fan_V8 out of WORLD;

OUTPUT RULE: PUSH to OutMold(N) at Rear;

Trim

NAME OF MACHINE: Trim;

QUANTITY: 1;

TYPE: Single;

PRIORITY: Lowest;

CYCLE TIME: NORMAL (2,0.1,11);

INPUT RULE: PULL from ToTrim;

OUTPUT RULE: PUSH to ToInspec;

Inspect

NAME OF MACHINE: Inspect;

QUANTITY: 1;

TYPE: Single;

CYCLE TIME: UNIFORM (2,3,12);

ACTIONS, Start

Add

SET ICON of Inspect to 48

ACTIONS, Finish

Add

Scrapped = NSCRAP(Fan_4CYL) + NSCRAP(Fan_5CYL) + NSCRAP(Fan_3LV6) + NSCRAP(Fan_4LV6) + NSCRAP(Fan_V8)

SET ICON of Inspect to 36

INPUT RULE: PULL from ToInspec;

OUTPUT RULE: PERCENT /41 ToBalanc 90.00 ,SCRAP 10.00 ;

Balance

NAME OF MACHINE: Balance;

QUANTITY: 1;

TYPE: Single;

CYCLE TIME: UNIFORM (0.5,1.0,13);

ACTIONS, Start

Add

SET ICON of Balance to 46

ACTIONS, Finish

Add

SET ICON of Balance to 47

INPUT RULE: PULL from ToBalanc;

OUTPUT RULE: PERCENT /20 ToPacker 90.00 ,SCRAP 10.00 ;

Packer

NAME OF MACHINE: Packer;

QUANTITY: 1;

TYPE: Single;

CYCLE TIME: UNIFORM (1,3,14);

ACTIONS, Start

Add

SET ICON of Packer to 44

ACTIONS, Finish

Add

ICON = ICON + 5

SET ICON of Packer to 38

INPUT RULE: PULL from ToPacker;

OUTPUT RULE: PUSH to ToLabel;

Labeler

NAME OF MACHINE: Labeler;

QUANTITY: 1;

TYPE: Single;

CYCLE TIME: 1.0;

ACTIONS, Start

Add

SET ICON of Labeler to 45

ACTIONS, Finish

Add

SET ICON of Labeler to 43

INPUT RULE: PULL from ToLabel;

OUTPUT RULE: PUSH to ToShip;

Conveyors

OutMold

NAME OF CONVEYOR: OutMold;

NOTES:

This is the SELECT text from the Output Rule. Copy and paste as needed.

SELECT on N MoldConv at (6),MoldConv at (12),MoldConv at (18), MoldConv at (24),MoldConv at (30)

QUANTITY: 5;

TYPE: Queuing;

PART LENGTH: 4;

MAX CAPACITY: 4;

INPUT RULE: Wait;

OUTPUT RULE: SELECT on N MoldConv at (6),MoldConv at (12),MoldConv at (18), MoldConv at (24),MoldConv at (30);

CYCLE TIME: 0.5;

MoldConv

NAME OF CONVEYOR: MoldConv;

QUANTITY: 1;

TYPE: Queuing;

PART LENGTH: 30;

MAX CAPACITY: 30;

INPUT RULE: Wait;

OUTPUT RULE: PUSH to ToTrim;

CYCLE TIME: 1.0;

Shipconv

NAME OF CONVEYOR: shipconv;

QUANTITY: 1;

TYPE: Queuing;

PART LENGTH: 20;

MAX CAPACITY: 20;

ACTIONS, Front

Add

NoOut = NoOut + 1

INPUT RULE: PULL from ToShip;

OUTPUT RULE: PUSH to SHIP;

CYCLE TIME: 1.0;

Variables

NoIn

NAME OF VARIABLE: NoIn;

QUANTITY: 1;

NoOut

NAME OF VARIABLE: NoOut;

QUANTITY: 1;

Nmold

NAME OF VARIABLE: Nmold;

QUANTITY: 5;

WIP

NAME OF VARIABLE: WIP;

QUANTITY: 1;

Scrapped

NAME OF VARIABLE: Scrapped;

QUANTITY: 1;

Pie Charts

TrimPie

NAME OF PIE_CHART: TrimPie;

QUANTITY: 1;

* Sector: 1;

* Value: SUTIL (Trim,1);

* Description: Waiting Parts;

* Sector: 2;

* Value: SUTIL (Trim,2) + SUTIL (Trim,9) + SUTIL (Trim,10);

* Description: Busy;

* Sector: 3;

* Value: SUTIL (Trim,3);

* Description: Blocked;

REFRESH INTERVAL: 5;

InspPie

NAME OF PIE_CHART: InspPie;

QUANTITY: 1;

* Sector: 1;

* Value: SUTIL (Inspect,1);

* Description: Waiting Parts;

* Sector: 2;

* Value: SUTIL (Inspect,2) + SUTIL (Inspect,9) + SUTIL (Inspect,10);

* Description: Busy;

* Sector: 3;

* Value: SUTIL (Inspect,3);

* Description: Blocked;

REFRESH INTERVAL: 5;

BalPie

NAME OF PIE_CHART: BalPie;

QUANTITY: 1;

* Sector: 1;

* Value: SUTIL (Balance,1);

* Description: Waiting Parts;

* Sector: 2;

* Value: SUTIL (Balance,2) + SUTIL (Balance,9) + SUTIL (Balance,10);

* Description: Busy;

* Sector: 3;

* Value: SUTIL (Balance,3);

* Description: Blocked;

REFRESH INTERVAL: 5;

PackPie

NAME OF PIE_CHART: PackPie;

QUANTITY: 1;

* Sector: 1;

* Value: SUTIL (Packer,1);

* Description: Waiting Parts;

* Sector: 2;

* Value: SUTIL (Packer,2) + SUTIL (Packer,9) + SUTIL (Packer,10);

* Description: Busy;

* Sector: 3;

* Value: SUTIL (Packer,3);

* Description: Blocked;

REFRESH INTERVAL: 5;

LabelPie

NAME OF PIE_CHART: LabelPie;

QUANTITY: 1;

* Sector: 1;

* Value: SUTIL (Labeler,1);

* Description: Waiting Parts;

* Sector: 2;

* Value: SUTIL (Labeler,2) + SUTIL (Labeler,9) + SUTIL (Labeler,10);

* Description: Busy;

* Sector: 3;

* Value: SUTIL (Labeler,3);

* Description: Blocked;

REFRESH INTERVAL: 5;

#### Initialize Actions

! Set packer and labeler to initial icons.

SET ICON of Packer to 43

SET ICON of Labeler to 38

## Case Studies

### ACME Manufacturing Co.

You have been employed as a member of a production improvement task force for the ACME Valve Manufacturing Company. ACME has been experiencing a severe backlog in sales orders due to antiquated machinery and poor production planning. The Manager of the company, I. Firem, will not tolerate the situation any longer. By the end of the day, Mr. Firem and his brother and major investor, Mr. W.E. Firem, would like to hear your proposal to get ACME into the 21st century. Output is currently between 120 and 160 valves every 75 hours. You will be provided with a cost list of potential capital improvements.

#### Part 1: Creating The Model

##### Acme Plant Layout

The following diagram shows the layout of the main factory at the ACME Valve Manufacturing Company.

Figure 7-3: Acme Plant Layout

The following sections cover each of the different areas of the plant in more detail.

##### The Saw Area

ACME produces valves. The first step in the manufacturing process starts with the stock of bars (length 3 feet) which come in to the saw area stock buffer. The time between arrivals is a fixed interval of time based upon a vendor’s supply schedule. The bars are then cut, producing 3 blocks (length 1 foot) from each bar.

After sawing, the blocks go to a belt Conveyor, which transfers the cut bars to the coating operation.

| Table 7-1: Saw Area Data |  |  |
| --- | --- | --- |
| Items |  | Description |
| Bar |  | Arrive in lots of 3, every half-hour (first arrival at time 0.0 minutes). Bar stock Buffer can hold only 10 bars. |
| Saw Machine |  | Time to cut bar into 3 pieces (or blocks) = 6 minutes |
|  | Setups | Blade A needs replacing every 40 operations Blade B needs replacing every 30 operations Set-up time: Uniformly distributed between 10 and 15 minutes |
|  | Breakdown | Time between failures = Negative Exponential with mean of 100 Repair time = Triangular Distribution: min = 10, mode =25, max = 30 |
| Belt Conveyor |  | Length = 20 feet Speed = 4 feet/minute |

##### The Coating Area

Once sawed, the blocks must be coated for protection. The coating machine coats 6 blocks at a time. Once the blocks are coated, they are placed in a one-position staging area adjacent to the inspection station. The inspector draws from his/her staging area one block at a time and determines the quality of the block’s coating. If it is bad, the inspector will put it into a rework Buffer for the coating Machine to re-coat. The coating machine MUST take blocks from the rework Buffer first. If the block passes inspection, the inspector passes it on to the hardening process.

| Table 7-2: Coating Area Data |  |
| --- | --- |
| Items | Description |
| Coating Machine | Cycle time = 60 minutes Set-up time = 5 minutes every 10th cycle |
| Inspector | 1 Inspector Current rejection rate = 20% Cycle time = 20.0 minutes Set-up time = 1.0 minutes per part |
| Staging Area | Capacity is 1 part |
| Rework Area | Capacity is 10 parts |

##### The Hardening Process Area

Parts are cured on an individual basis travelling through an open furnace. Parts cure for at least one hour in the furnace, which holds a maximum of 10 blocks. Parts are removed from the furnace on a “first in, first out” basis.

| Table 7-3: Hardening Process Data |  |
| --- | --- |
| Items | Description |
| Furnace | Curing time is 60 minutes. Capacity is 10 parts. |

##### The Grinding Area

The hardened blocks must be ground to meet customer specifications. There are two grinders, with no priorities between them. The grinders, however, require a special fixture so that 4 blocks can enter the Machine at once.

There are 10 fixtures in the system, and when not being used, they are stored in a fixture Buffer. The blocks are loaded onto the fixture at a loading station and then sent to the grinders. As far as possible, the fixtures are evenly loaded between the 2 grinders.

Once finished, the grinders automatically place the fixture and the 4 blocks into an unloading station. The unloading station will send the ground blocks (valves) into finished goods stock areas (on a random 50/50 split between the valve stock buffers) and place the fixtures onto an overhead Conveyor. The Conveyor puts the fixtures back into the fixture Buffer for reuse by the loading Machine.

| Table 7-4: Grinding Area Data |  |  |
| --- | --- | --- |
| Items |  | Description |
| Fixture |  | 10 available in the system from the start Length = 5 feet |
| Loader |  | Cycle time = 4 minutes |
| Grinder |  |  |
|  | Cycle time | 35 minutes for grinder 1 40 minutes for grinder 2 |
|  | Machine Breakdowns | Time Between Failures is Normal with a mean of 75 minutes and Standard Deviation of 15.0 minutes. Repair Time is a Negative Exponential with a mean of 15 minutes. |
| Fixture Return Conveyor |  | Length = 30 feet (two 15 foot length at right angles) Speed = 5 feet/minute |
| Unloader |  | Cycle time = 4 minutes |
| Valve Stock |  | Capacity = 10 parts per buffer |

##### The Cleaning Area

Valves are taken from the valve stocking area and cleaned before shipping. The cleaning Machine is a multi-station machine consisting of 5 single stations through which valves pass. The next valve to be cleaned depends on which one of the valve stock Buffers is the fullest. The cleaning head requires a change over every 100 cycles.

| Table 7-5: Cleaning Area Data |  |
| --- | --- |
| Items | Description |
| Cleaning | Cycle time = 15 minutes per station Number of stations = 5 Time Between Failures = Negative Exponential: Mean = 120 minutes Repair Time = Erlang: Mean = 20 minutes, K = 3 Set-up: every 100 operations Set-up time = 5 minutes |

#### Part 2: Experimenting with the Model

Once you are making between 120 and 160 valves every 75 hours, and you have verified and validated your model, you can begin the experimentation process. From now on, the only changes you can make are listed in the cost table below. You will have an objective to meet a specific budget. If you go over budget, you might need to be very well prepared to explain your decisions to Mr. Firem. You know what I mean!

##### Objective

Increase total throughput.

##### Budget

$100,000

##### Time Frame

| Hours of Production | 4500 minutes (75 hours) |
| --- | --- |
| Warm-up Period | 500 minutes |
| Total run length | 5000 minutes |
| Number of runs | 1 |

- To introduce a warm-up time in a single simulation run, type 500 in the Warm-up Period field from the ModelOptionStatistics window. Then, run the model for 5000 minutes. If you were using the Experimenting tool from WITNESS, you would not have to do this.

| Table 7-6: Acme Manufacturing Cost Data |  |
| --- | --- |
| Item | Cost |
| BUFFERS: |  |
| Per Part capacity increase in any Buffer | $ 100 |
| SAW: |  |
| Identical new saw | $ 40,000 |
| Cycle time decrease per 10% | $ 5,000 |
| Set-up decrease per 10% | $ 8,500 |
| Increase Time Between Failures per 10% | $ 5,000 |
| Decrease Repair Time per 20 % | $ 7,500 |
| BELT CONVEYOR: |  |
| Convert belt to roller | $ 10,000 |
| New conveyor of either type | $ 20,000 |
| Cycle time decrease per 10% | $ 5,000 |
| COATING MACHINE: |  |
| Increase batch capacity per Part | $ 10,000 |
| Cycle time decrease per 10% | $ 6,500 |
| Set-up decrease to double number of operations | $ 5,000 |
| INSPECTOR |  |
| New Inspector and Machine | $ 30,000 |
| Cycle time decrease per 10% | $ 10,000 |
| Set-up decrease per 10% | $ 5,000 |
| Decrease rejection rate by 50% | $ 20,000 |
| HARDEN PROCESS |  |
| Cut process time 10 minutes | $ 20,000 |
| Increase capacity per part | $ 10,000 |
| LOADER/UNLOADER |  |
| New loader or unloader | $ 25,000 |
| Decrease cycle time per 10% | $ 15,000 |
| New Fixture | $ 5,000 |
| GRINDER |  |
| New grinder | $ 80,000 |
| Cycle time decrease per 10% | $ 12,500 |
| Double time between failures | $ 30,000 |
| CLEANER |  |
| Additional cleaner | $ 16,500 |
| Cycle time decrease per 10% | $ 10,000 |

#### Part 3: Additional Studies

What would happen, if you were to include the following changes into your original model? Would the end result in Part two remain the same? If not, what would be the best strategy for this modified model to meet the company’s objective under budget?

##### Changes

There is one Operator at the saw and (s)he sets up, repairs and operates the Saw. This Operator is also responsible for fixing the Grinders and the Cleaning machine. Additional Operators would cost you $32,000 each.

Mr. Firem is a conservative person. Therefore, in order for him to make a final decision based upon a simulation model, he needs the following points covered.

He expects you to provide him with the following:

- A verified and validated model (all assumption should be stated)

- A well documented report on both the model and the experimentation analysis.

- Average values and 95% confidence intervals for any final numeric results.

This Confidence Interval should be a half-width inferior to the 10% of the average value. For example, if you were producing an average of 300 units over a period of 75 hours, a 95% Confidence Interval of (285,315) would be acceptable while a (265, 335) Confidence Interval would not.

How many runs would you have to make to satisfy Mr. Firem? Explain. Would you be able to say that this is the best solution to your Company’s problems? If yes, why? If not, why not?

- Note: Use the original model to answer these questions.

### The ACME Boxmaker Model

A simple model needs to be created to reflect the current manufacturing process of The ACME Box Company (see Figure 1.1).  Once this model is modeled, you will be required to take this model and update it so that it shows the proposed new operating environment of the factory.

Figure 1. 1: The ACME Box Company Layout

#### Part 1: Creating The Model

The main elements of the model are as follows.

There are three types of parts, which are used a top, a side and a bottom.  A box, not surprisingly, is made from 1 top, 1 bottom and four sides. Each part has a thickness between 1 and 3 inches. These numbers are obtained from integer uniform distributions.

A forklift truck (FLT) is used to collect these parts and put them into a pre-processing Buffer. Each type of part is put into its own Buffer before the appropriate Drilling Machine (One for tops, one for sides and one for bottoms).  Each one of these Buffers has a maximum capacity of 90 units. The FLT can carry 6 Parts at once, and the driver picks up the Parts at random from the untidy goods-receiving area.  It is assumed that they are picked up in the proportions of 1 top to 4 sides to 1 bottom, since this is the ratio in which they are purchased.

The first process is a drilling operation where fixture holes are placed into the panels in preparation for assembly.  The drills just take the Parts from the appropriate dedicated Buffer, drill one panel at a time, and place the Parts into a panel store. There is one drill for bottoms, and one for tops, and two drills for sides.

The drilling machines have various setup and breakdown information defined, and this is believed to be realistic. The number of operations between setups is 75 units and the setup time is 10 minutes for all the drills. However, it is assumed that the number of operations to the first setup are 25, 50 and 75 for the top drill, side drills, and bottom drills, respectively. In addition, each drilling machine has a time between breakdowns that follows a Negative Exponential distribution with a mean of 100 minutes, and a repair time that follows an Erlang distribution with mean of 5 minutes and a K value of 2.

There is a single panel store (Panels) into which all drilled panels are placed with a 500 units capacity.

The Assembly machine takes a bottom, 4 sides and a top from the panel store and assembles them to produce a box. These new boxes have the panel thicknesses described in three attributes called:

- Thk_Top	(the thickness of the top panel, either 1 2 or 3)

- Thk_Side	(the thickness of the thickest side panel, either 1 2 or 3)

- Thk_Bot	(the thickness of the bottom panel, either 1 2 or 3)

The assembled box finally travels along a 10 parts-queuing Conveyor (CON001) and into an Oven.

There is a timeseries defined to show the number of Parts in the panel store, and how this varies over time (it is updated every 30 minutes).  There is also a histogram to show how long Parts spend in the panel store. The timeseries and histogram each have their own display window.

The factory operates an 8-hour day. For the sake of reducing the run-time required for the exercises, a 2-day model run will be used. (I.e. run experiments up to 960).

| Table 7-7: Cycle times for the Acme Boxmaker Case Study |  |  |  |  |
| --- | --- | --- | --- | --- |
| CYCLE TIMES |  |  |  |  |
| Machine |  | Distribution | Parameters |  |
| Fork Lift | ERLANG |  |  | Mean = 5, K = 2 |
| Top Drill | ERLANG |  |  | Mean = 4, K = 3 |
| Side Drills | ERLANG |  |  | Mean = 2, K = 3 |
| Bottom Drill | ERLANG |  |  | Mean = 4, K = 3 |
| Assembly | ERLANG |  |  | Mean = 5, K = 3 |
| Conveyor | N/A |  |  | Index Time = 1.0 |
| Oven | UNIFORM |  |  | Lower = 2, Upper = 6 Limits |

- Note: At this time, it would be a good idea to give a labor priority to the following elements: 4 for the fork lift, 3 for all the drills, and 2 for the Assembly machine.

#### Part 2: Bringing Order to the Goods-Receiving Area

Mainly due to the large number of industrial injuries suffered because of falling panels, the ACME Box Company has decided to introduce some semblance of order into their goods receiving area. They have taken the stylish approach of introducing shelving, and attaching notices to indicate which shelves are for tops which for bottoms and which for sides.

ACME’s chairman, I Firem, is not convinced of the benefits of this expenditure. However, he has decided that since the panels are stored in order, the FLT driver should be able to go to a location and load a single type of panel. This will reduce the average FLT cycle time from 5 minutes to 4. This means that the FLT will fetch panels in a fixed sequence as follows: -

- 1 load of tops (6 tops)

- 2 loads of sides (12 sides)

- 1 load of bottoms (6 bottoms)

- 2 loads of sides (12 sides).

As well as reducing the time for the transport operation, ACME also hopes that this will give a smoother production flow of Parts through the factory.  Incorporate these changes to your model, and note any change in the daily throughput, which occurs.

#### Part 3: We Don’t Need All These Men

ACME’S range of products is constantly changing, and this means those requirements for the box in question is being reduced to 75 per day. They are very interested in reducing the number of staff working in the box making area, and want to know how many staff will be needed.

ACME currently employs two grades of staff in the box making area. The first grade of staff is an unskilled laborer, who is allowed to carry out set-ups and repairs to the drills, and to drive the FLT. ACME refers to this grade of employee as a “Runner”. They employ three Runners.

The second grade of employee is somewhat less skilled than the Runner, and is known as a “Thumper”. ACME employs three Thumpers, who operate the Drills and Assembly machines. If there is no Thumper available, a Runner may also operate a machine.

In reviewing the model, it has been observed that the Oven times do not seem particularly realistic. It had been assumed that the Oven time was random, and evenly distributed between 2 and 6 minutes. Following extensive discussions with ACME Engineer, Dr. Dolittle, a complex formula for the Oven time has been devised based upon the thicknesses of the panels.

The boxes have the panel thicknesses described in three attributes called:.

- Thk_Top	(the thickness of the top panel, either 1 2 or 3)

- Thk_Side	(the thickness of the thickest side panel, either 1 2 or 3)

- Thk_Bot	(the thickness of the bottom panel, either 1 2 or 3)

Because nobody else can understand Doctor Dolittle’s formula, he has converted it to a series of look-up tables that allow us to calculate the oven time based on the three thicknesses.

Figure 7-4: Panel Thickness Data

Create a three dimensional table of numbers in WITNESS to allow the precise oven time to be modeled based upon the panel thicknesses. You should also display the table of numbers in their own window in the model so that you can check that the correct numbers have been input.

Use the revised model to decide how many Runners and Thumpers are needed to achieve the required output of 75 boxes per day.

#### Part 4: Somebody Invented Quality

The cycle time for the assembly machine does not in fact have an average of 5 minutes as is modeled now. The average is certainly 5 minutes if a Thumper carries out the task, but on average, a Runner will perform the task in 4.5 minutes.

ACME is accustomed to receiving complaints about the quality of their boxes.  Recently their major customer has threatened to take his business elsewhere unless the tops of the boxes reach the specified standard. The chairman is understandably worried, and has decided to rectify the situation. Following a lengthy discussion with an acknowledged quality expert who he met in the pub one evening, he has decided to introduce a new inspection process before the boxes are shipped.

The inspection process will be extremely thorough, and is expected to take 30 seconds to complete. 10% of boxes are expected to need some rework, which is carried out at the inspection station by the operator, and takes two minutes. (In other words, 90% of inspections take 30 seconds, and 10% take 2½ minutes.) There will be storage space for 5 completed boxes between the Oven and the Inspect station.

To give some level of accountability and ensure that inspection is carried out, it has been decided that each box will be stamped with the name of the person who assembled it, and this same person must also inspect it. Inspection will be viewed as the highest priority task for the operators.

Incorporate the revised assembly time and the inspection procedure into your model and see what effect this has on the throughput of the factory. Can you still manage with 3 Runners and 2 Thumpers?

#### Part 5: War Breaks Out

The Landlord of the local pub, The Dragon Drop, has complained that too much damage is being caused by the gang-fights, which occur between the Runners and the Thumpers. Being a considerate individual (and also a regular at the Dragon Drop), the foreman of the ACME Box Company has decided to implement a staggered lunch break.

In the future, the Runners and Thumpers will both work from 8 AM to 5 PM. However, the Runners will go to lunch between noon and 1 PM, while the Thumpers will go between 1 PM and 2 PM. Incorporate this into your model.

- Note: Create two files, RUNNER.SFT and THUMPER.SFT that would contain the appropriate shift patterns. You must now run the model for 9 hours per day (i.e. up to 1080) to get comparable results.

See if this change affects the throughput of the model, and use the histogram and timeseries to see what effect it has on the panel store.  Tidy up the displays for the panel store histogram and timeseries to make them easier to use.

##### Part 6: Make It Look Good

The model is now complete.  The time has come to present it, so naturally you want it to be easy to understand.  Improve the on-screen presentation of your model.  In particular you should do the following: -

Introduce a display for the part Box.  This should at the least include a rectangle, on top of which is displayed the expression NSHIP(Box).

For the components (Top, Side and Bottom) include statistics which show the average time each type of part remains in the simulation, but only include those parts which have been assembled.

Arrange for windows 2 and 3 (the panel store statistics) to be opened at the start of the lunch period (i.e. 12 noon) and then closed at the end of the lunch period (i.e. 2 PM).

Improve some of the machine displays, including some elaborate configurations of display items, and using the grouping option so that they can be more easily moved.  Read in some icons from the library called LookGood.ICN to help you.