# Industry Studies

## Assembling Electronic Components

### An Example on how Simulation can be Used

#### PROBLEM DESCRIPTION

This case study analyzes a portion of a portable, consumer electronics, high volume manufacturing assembly line, shown in Figure 1.  Data used for this model is representative of an actual manufacturing line, but not exactly the same.

Figure 10-1: A Portion of a Portable, Consumer Electronics, High Volume Manufacturing Line.

#### Machines/Stations

The numbered squares in Figure 10-1 represent machines. 2a and 2b, and 6a and 6b, are identical, parallel machines. Machines are connected to one another by conveyor, which is represented by lines in the drawing. The conveyor runs at a speed of one foot per second and each section of the conveyor is five feet long (not drawn to scale). Although each printed circuit board (pcb) is one foot long, only one pcb is allowed for each section of the conveyor.

The capacity in front of the first machine is infinite, e.g., there is always sufficient raw material available to begin production. Each machine/station performs work on one pcb at a time.

There is a 1% chance that a pcb will have a major defect at each machine.  Defects are additive, so a pcb can have as many as eight defects.  Any minor rework necessary is already considered in the cycle time of the visual inspection/minor rework station.

Each machine experiences similar downtimes. The mean time before a machine experiences an error or failure (MTTF) varies from 60 to 120 minutes (this variance is described later in this section) and emanates from a negative exponential distribution. A sample negative exponential distribution is shown in Figure 10-2.

Figure 10-2: A Sample Negative Exponential Distribution.

The mean time to repair or correct a machine error (MTTR) comes from a gamma distribution with shape = 2 and scale = 3 or 5, depending on if the error occurred during the night shift (3) or the day shift (5).  Thus, the average time to correct an error is either 6 or 10 minutes. The gamma distribution is often used to model the time to complete a task, for example, machine repair.  A sample gamma distribution with alpha equal to 2 is shown in Figure 10-3.

Figure 10-3: A Sample Gamma Distribution with Alpha = 2.

99.9% of the time there is enough material available for production. When material is unavailable, there is a delay whose value is taken from a uniform distribution that varies from 10 to 20 minutes.

Finally, the production rates for each machine and station are determined from a uniform distribution and are shown in Table 10-1. The bottleneck machine shifts depending on the model of the pcb that is being produced, which is discussed later.

| Table 10-1: Range of Values for the Cycle Time (Min.) of all Machines/Stations. |  |  |
| --- | --- | --- |
| Machine/Station |  | Cycle Time (Min.) |
| Rework, 1 defect | 4 – 6 |  |
| Rework, 2 defects | 9 – 11 |  |
| Rework, 3+ defects | 14 – 16 |  |
| Bottleneck machine | 0.95 – 1.05 |  |
| Parallel machines | 1.45 – 1.55 |  |
| All other machines/stations | 0.7 – 0.8 |  |

#### Products/Changeover

Three models are produced: A, B, and C. The bottleneck machine varies depending on the model being produced (Table 10-2).

| Table 10-2: The Bottleneck Machine for Each Model Produced. |  |
| --- | --- |
| Model | Bottleneck |
| A | Machine 8 |
| B | Machine 4 |
| C | Machine 1 |

The time to setup a machine for producing a new model emanates from a uniform distribution that varies between one and three minutes. The mean time until a machine experiences a failure or error is 60 minutes for the first three hours after a changeover. This value increases to 80 minutes for the next three hours after changeover, then 100 minutes for the three hours after that, and finally 120 minutes for any additional time after that until another changeover occurs.

At the beginning of every three hour interval, there is a 20% chance that there will be a changeover to another model. When a changeover occurs, the two remaining models that are currently not being produced have an equal chance of being selected for production.

Lastly, a pcb becomes scrap if it spends more than an hour on the assembly line between machine 1 and machine 4, or more than an hour on the assembly line between machine 5 and machine 8.

#### Workers

The number of workers assigned to each area of the process is shown in Table 10-3. Machines only require workers during downtime. When a downtime occurs, it takes six seconds for the worker to react. When a setup is necessary, one minute of travel time for the worker is assumed.

| Table 10-3: Worker Assignment. |  |  |
| --- | --- | --- |
| Number of Workers |  | Assignment |
| 2 | Machines 1, 2a, and 2b |  |
| 2 | Machines 3 and 4 |  |
| 2 | Inspection/Minor Rework 1 |  |
| 2 | Machines 5, 6a, and 6b |  |
| 2 | Machines 7 and 8 |  |
| 2 | Inspection/Minor Rework 2 |  |
| 2 | Major Rework |  |
| 2 | Machine Setup After Model Changeover |  |

There are two shifts for workers. One is from 4:30 a.m. to 4:30 p.m. and the other is from 4:30 p.m. to 4:30 a.m. Each shift includes a half hour lunch and two 15 minute breaks.

#### Other System Parameters

80 independent random distributions are used in a model of the current system, so several uncertainties have been taken into account in the model as previously discussed. The model is replicated ten times with each run 24 simulated hours long with three hours of warm-up. These values are deemed sufficient since larger values do not yield results that are significantly different.

### Experimentation

The following table lists the parameters of the system considered in the experimentation and the levels of each. These parameters and levels are potential future modifications to the actual production line or are characteristic of other lines within the factory.

| Table 10-4: The Five Factors Considered in the Experimentation. |  |  |
| --- | --- | --- |
| Parameter | Level 1 | Level 2 |
| 1. Workers for Each Assignment | 2 | 1 |
| 2. Allowable WIP for Each Section of the Conveyor | 1 | 2 |
| 3. Day and Night Shift Difference in MTTR | 4 min. | 0 |
| 4. Average MTTR | 8 min. | 4 min. |
| 5. Potential Changeover Frequency | Every 3 hr. | Every 12 hr. |

Experimentation is performed using OPTIMIZER within WITNESS.  The objective function maximizes throughput (nship(a) + nship(b) + nship(c)).  There are over 2 billion possible combinations.  Using the Hill Climb Method in Optimizer., a maximum of 100 evaluations are performed.

#### Results

The top 5 evaluations can be viewed in “case4.xls”.  As expected, higher values for workers, allowable WIP, and potential changeover frequency seem to be preferred, while lower values are preferred for mean time to repair.

Better values for the objective function may be obtained by re-optimizing within Optimizer using the values generated from the first optimization.  Also, one could add in costs for the additional labor, additional WIP, etc.  Lastly, even greater values for labor, allowable WIP, etc., could be considered.

## Andover International Airport

### Overview Of The Case

Andover International Airport is one of the largest airport authorities in the U.S. As Planning Manager for the Federal Airport Authority, you have the task of improving the service to customers. Angry passengers have recently complained of excessive waiting times. You have been tasked with evaluating a number of alternatives to improve customer service. You have decided to build a WITNESS model and test the alternatives.

### Creating The Model

Figure 10-4: Andover International Airport Layout

The following sections cover each of the different areas of the airport in more detail.

#### Passenger Arrival at the Check-In Area

Passengers arrive at the airport and join one of the check-in queues. There are six check-in desks, two of which are dedicated to club passengers. All passengers will join the shortest of the six passengers’ queues; however, only Club members are allowed in the dedicated queues.

| Table 10-5: Passenger Inter-arrival Times |  |  |
| --- | --- | --- |
| Passenger Type |  | Inter-arrival time (mean) |
| Club International | 6 minutes |  |
| Club Domestic | 5 minutes |  |
| Economy International | 3 minutes |  |
| Economy Domestic | 2 minutes |  |

- Note: The inter-arrival time for passenger arrivals follow a negative exponential distribution with the mean specified.

On average, check-in for international flights is 4.5 minutes and for domestic flights is 3.5 minutes. Every 50 passengers, a check-in desk runs out of baggage tags. It takes 3 minutes to restock them.

#### Baggage Check-In

When Passengers arrive at the check-in, their bags are tagged and placed on a conveyor. Data has been collected on the number of bags a passenger usually checks-in. This data is summarized in the following table.

| Table 10-6: Passenger Baggage Percentages |  |  |
| --- | --- | --- |
| Number of bags |  | Percentage of passengers |
| 0 | 25% |  |
| 1 | 40% |  |
| 2 | 25% |  |
| 3 | 8% |  |
| 4 | 1% |  |
| 5 | 1% |  |

Each check-in desk has a dedicated 10 position fixed conveyor with a transit time of 30 seconds. These conveyors break down on average every hour (Busy Time) according to a negative exponential distribution and take 3 minutes to repair. All these conveyors feed onto the same baggage conveyor every fifth position starting from the back (the last conveyor feeds on position 30, the next on position 25, all the way down to the Club conveyor feeding on position 5) and take the bags to baggage handling. This baggage conveyor has the same speed and is 30 positions long.

#### Security and Passport Control

After checking in their baggage, International passengers go to a passport control stage, which takes on average 30 seconds. Domestic passengers do not need to go through passport control. They proceed through different departure lounges on another floor of the airport. The domestic departure procedure does not need to be modeled at this point.

International passengers then proceed through a security check after passing through passport control. Passengers are equally likely to have 1, 2, or 3 pieces of hand luggage. It takes, on average, 30 seconds to place each item on the x-ray infeed, which is one position. The bags are x-rayed in 30 seconds and deposited in a 5 position output tray.

The x-ray machine breaks down on average every 30 minutes according to a negative exponential distribution. It takes 2 minutes to repair. Passengers then walk through the metal detector, which takes 15 seconds. Out of 100 passengers, 20 of them tend to set off the alarm and get frisked, which takes 2 minutes.

Before leaving the security area, the Passengers collect their bags, which takes 30 seconds per item. Due to security regulations, no more than 3 passengers can be can be picking up hand luggage or waiting to pick up hand luggage. Passengers then go to the departure lounge.

### Experimenting with Your Model

#### Objective

How could you achieve the standards laid down by the Aviation Authority given a specific set of options? Explain all your recommendations.

Would a single simulation run of the model for 16 hours be enough to test the changes? Explain?

The following two sections provide you with the service standards to be met as well as with the options, which can be tested in the model.

#### Service Standards

There are a number of service level standards laid down by Aviation Authority to which all airports should strive to adhere. The targets for the service level related with passengers are as follows:

- 80% of club passengers check in within 8 minutes

- 80% of economy passengers check in within 12 minutes

- 80% of club passengers complete check in and security within 14 minutes

- 80% of economy passengers complete check in and security within 18 minutes

#### Options

| Table 10-7: Options |  |
| --- | --- |
| Options | Cost |
| Automatic restocking of baggage tags removing delays | $ 15,000 |
| Adapt conveyor system to allow baggage to accumulate | $ 35,000 |
| New conveyor motor, doubling speed | $ 20,000 |
| New check in desk and staffing | $ 150,000 |
| Maintenance work to half breakdown frequency of x-ray machines | $ 7,500 |
| Increase input capacity to x-ray by 2 | $ 5,000 |
| Increase output capacity to x-ray by 2 | $ 5,000 |
| Change security procedures by allowing passengers through security first, followed by passport control. The security channels would allow both international and domestic passengers. | $ 200,000 |

## Dewey, Cheetam & Howe

### Introduction

Due to the changing competitive nature of the personal, car, and fire insurance business, the Dewey, Cheetam & Howe Insurance Co. is beginning to lose business to the competition. They have approached your Business Process Re-engineering consulting firm because of your reputation for rapidly finding the right solutions to these types of problems. They are relying on you to help them resolve their current difficulties.

They have concluded that a lot of potential business is being lost due to incoming telephone calls not being answered quickly enough. There is also a problem in the back office with the speed at which quotes are being issued to the customer, as well as how long it takes customers to receive the finished policy documents.

Your BPR consulting company has been asked to help resolve the above issues. They have given you a schematic layout of the existing system, as well as relevant data they have gathered for you to examine. If any data is not available, make assumptions. These assumptions can be clarified by D.C. & H. Insurance Co. at your next meeting with them (your instructor is the Chief Executive Officer).

Using the General Terminology version of WITNESS, Dewey, Cheetam & Howe Insurance has asked that you model their current process to provide a base from which to start looking at solving their problems. When you have an operational model, you will then be asked to report to D. C. & H with your proposed changes to their process.

Figure 10-5: Dewey, Cheetam & Howe Insurance Co. Model Layout

### The Call Center

#### Incoming Calls

Dewey, Cheetam & Howe Insurance currently deals with three types of insurance policies:

- Personal (life and health insurance)

- Car (auto, boat, RV)

- Home (fire, flood, theft)

Inquiries for all three types of policies are received via telephone, with all calls entering the company via a receptionist (Reception). By monitoring the incoming telephone activity over a period of days, it has been determined that each of the three call types (e.g. personal, car, and home) arrives with an inter-arrival time of 3 minutes, on average.

After picking up the next call from the switchboard, the receptionist decides which one of the telesales operators (personal, car, or home) should handle the call. The receptionist then routes the calls accordingly. This process takes an average of 30 seconds per call. The switchboard can queue up to 3 calls in addition to the one being answered. If the switchboard circuits are busy, the caller will get a busy signal and thus hang up.

Once the caller has been routed through to the relevant telesales operator (personal, car, or home), it has been found that callers will generally wait on the phone for between two and five minutes before hanging up if the call has not been answered. There is a capacity for up to 10 calls to be queued in each of the three areas.

The office is open from 8:30 a.m. to 5:30 p.m. every day of the week.

#### Dealing with Calls

There are a total of six telesales operators - two dedicated to each of the insurance policy types (personal, car, and home). These operators process calls waiting in their queues on a first-come first-served basis.

The Call Center has an experienced supervisor who is available to deal with any specific queries or problems that might arise during a call with a customer. To help get a handle on things, the supervisor was asked to keep track of where and how often their assistance was required during a typical week. It was found that an average of one in five calls requires the supervisor’s assistance. If the supervisor is requested, the caller will be put on hold while the supervisor works with the telesales operator. (This will be modeled as a set up)

Only sixty percent of the calls will result in a draft insurance quote being generated by the telesales operators (the rest of the calls need no further action). The draft quotes are eventually reviewed and finalized by the sales support team in the Back Office (see diagram above). All the draft quotes are delivered to sales support via the internal mail system. The internal mail is collected every three hours during the day and delivered to the sales team in the back office.

| Table 10-8: Data |  |  |
| --- | --- | --- |
| Description |  | Value |
| Telesales Operators |  |  |
|  | Time to process the call | ERLANG - Mean = 5 min., K = 3 |
|  | Supervisor required | Every 5 calls on average |
|  | Assistance time required | Between 3 and 5 minutes |
| Internal Mail System |  |  |
|  | Process Time (time to deliver all mail) | 5 minutes |

### The Back Office

#### Processing Quotes

All draft quotes are processed by the sales support team. The team is comprised of two people who produce final quotes for each customer. Each customer then needs to visit the office, usually next day, to sign the documents and make the necessary payment.

The payment will be either an electronic funds transfer debit, or the necessary amount such as a written check or money order. One of the sales support team will be responsible for getting the customer to sign all of the documentation. This sales person will also sort the finalized paperwork, sending the signed quotes to the policy issue department and the monetary payment (either an electronic debit approval or a check/money order) to the finance department.

The finance department will process the payments. There are two people doing this job. Of the payments that are received, thirty percent are electronic debits and the remaining are direct payments (e.g. checks/money orders). If the payment is by electronic debit, the finance department will send the request for authorization to the bank via a computer link that will take, on average, 2 hours. If the payment is by check/money order, the bank notifies the policy issue department immediately and they can continue processing their paperwork immediately.

| Table 10-9: Data |  |
| --- | --- |
| Action | Process Time |
| Sales support | 2 minutes |
| Customer signing and sorting documents | 30 seconds |
| Finance | 10 minutes |

#### Policy Issue Department

The policy issue department needs to match a completed quote from the customer with a payment notification from the finance department to enable them to create a finished policy document.

These policy documents are then issued to the customer via mail. This completes the scope of the simulation.

| Table 10-10: Data |  |
| --- | --- |
| Action | Process Time |
| Policy creation process time | Average 5 minutes |

### Experimentation

The broad objectives of the simulation should be to:

1.	 Reduce the number of calls lost and hence the amount of business lost

2. 	 Reduce the time taken to issue policy documents to the customer

The following information should be used to run the model and perform the experimentation:

| Table 10-11: Data |  |
| --- | --- |
| Item | Time |
| Warm-up Period | 1 day |
| Run Length | 10 days (including the 1-day warm-up period) |

A "day" is considered to be from 8:30 a.m. to 5:30 p.m., including the lunch hour, since we will assume that calls are still being taken even during lunch.

Use the 1-day warm-up period as a starting point, since it may vary. For the purpose of this model, set the warm-up period to be a time shortly after the first completed policy leaves the model. In fact, see if you can figure out a way to have the model stop exactly as the first policy leaves, and then jot down that time. Use that time as your warm-up period if it's longer than a day. If it takes less than a day for the first policy to leave the model, then just use a whole day as your warm-up period, e.g., do not use less than a day as your warm-up period.

Include in the model any graphs or statistics that will help you to achieve the above objectives.

## Auto Components Inc.

### Summary

Auto Components Inc. has nine production facilities within the U.S. together with two plants in Europe. Each plant produces a variety of components for various truck and auto makers. The types of product produced in a given plant appear to vary for historical reasons. Each plant has been given a considerable amount of autonomy and has grown to supply the needs of a couple plants in its vicinity.

The Thistown facility has traditionally concentrated on producing small, relatively simple components. Over recent years, much of this work has been put out to competition with Auto Components Inc. frequently losing business to specialized, more modern suppliers.

One of the few areas where the Thistown facility is still competitive is the production of valves. A variety of such devices are built for a range of applications.

The ZA valve lines produce valves coated with tetra-methyl diglucene. The coating makes the valves impervious to damage by water, grit, or mud. It also makes the valves expensive. They are therefore only used in luxury sport utility vehicles.

The ZA810 line produces valves of both 8 and 10 inches. Other than the length, these valves are identical in every respect.

Your job is to build the ZA810 line incrementally through each example. When the examples are finished, you will perform many ‘what-if’ analyses to determine how to run the facility more efficiently within the assigned budget.

#### Part 1

Raw material (bar stock) for both valve types is delivered to buffer STORE1. They are delivered via a forklift from the Parts Crib with a particular delivery schedule. The supplier of the bar stock has promised to never let the Part Crib run empty.

The Saw takes a single bar from STORE1, processes it, and places it in STORE2. The DRILL takes a single bar out of STORE2, processes it, and aces it in STORE3. Finally, the COATER takes a single bar out of STORE3, processes it, and dispatches it to a ROLLER.

Figure 10-6: Layout Part 1

| Table 10-12:Delivery Schedule |  |  |
| --- | --- | --- |
| Of Forklift: | Bar Stock | 50 Bars every 240 minutes |
| Buffer Capacities | STORE2 and STORE3 | 10 parts |
|  | STORE1 | 1000 parts |
| Machine Cycle Times: | SAW | 3.7 Minutes |
|  | DRILL | 5.2 Minutes |
|  | COATER | 2 Minutes |

#### Part 2

The SAW and DRILL each have 2 separate tools that are used for each operation. The following data has been supplied on the replacement frequency of these tools:

| Table 10-13:Tool Replacement Frequency |  |  |  |
| --- | --- | --- | --- |
| Machine | Tool | Ops Between Tool Change | Change Times |
| SAW | 1 | 50 | 1.5 |
| SAW | 2 | 75 | 2.1 |
| DRILL | 1 | 45 | 1.2 |
| DRILL | 2 | 35 | 2.8 |

In addition, the SAW has a tendency to breakdown every hour of usage; while the COATER breaksdown every 2 hours of usage. Both of these repairs take 20 minutes to complete. There are no breakdowns on the DRILL.

#### Part 3

More data collection has revealed that the SAW’s process time is usually 3.7 minutes, but could be as short as 2.9 minutes, or as long as 4.3 minutes.

There has also been more data collection on the breakdown characteristics of the SAW and the COATER. Breakdowns occur on average every 60 and 120 minutes for the SAW and COATER respectively and both follow a negative exponential curve. Their repair times both follow an Erlang distribution with a mean of 20 minutes and a K value of 3.

There is now data on the breakdown characteristics of the DRILL. On average, it breaks down every 100 operations. The following is what has been observed for the repair time of the DRILL:

| Table 10-14: Drill Repair Times |  |
| --- | --- |
| Range (Minutes) | Frequency |
| 0 – 10 | 12 |
| 10 – 20 | 8 |
| 20 – 30 | 18 |
| 30 – 40 | 4 |
| 40 – 50 | 10 |
| 50 – 60 | 16 |

#### Part 4

There are three machinists, LARRY, CURLY and MOE. All three are responsible for staffing the production line. Their respective tasks are listed below.

| Table 10-15: Machinists |  |  |  |
| --- | --- | --- | --- |
|  | Saw | Drill | Coater |
| Cycle Repair Setup | Larry or Curly | Curly or Larry | Moe |
|  | Larry and Curly | Curly and Larry | Moe |
|  | 1) Larry | 1) Curly |  |
|  | 2) Larry | 2) Curly |  |

#### Part 5

After the COATER has finished machining its parts – they are output to the ROLLER and then dispatched.

Figure 10-7: Layout Part 5

| Table 10-16: Data |  |
| --- | --- |
| Item | Value |
| Bar Length | 1 ft |
| Length of ROLLER | 8 feet |
| Speed of ROLLER | 10 ft/min |

#### Part 6

The fixed conveyors, STAT_8 and STAT_10 are both a series of ten manual workstations. The workstations at STAT_8 will produce 8” valves and the workstations at STAT_10 will produce 10” valves. Both conveyors need to have a carrier or pallet to position the bars as they move from station to station. To accomplish this, the LOAD operation takes a pallet from FIX_STORE and places a bar from the ROLLER in it. Fifty percent of the fixtures go to STAT_8 and the other fifty percent go to STAT_10.

The UNLOAD operation will disassemble the pallets from STAT_8 and STAT_10. It will send pallets down the RETCONV and the RETCONV will deposit the pallets back into the FIX_STORE.

Figure 10-8: Layout Part 6

| Table 10-17: Data |  |
| --- | --- |
| Item | Value |
| Pallet Length | 2 feet |
| Length of STAT_8 | 10 feet |
| Length of STAT_10 | 10 feet |
| Length of RETCONV | 20 feet |
| Speed of Conveyors | 10 ft/min |
| Number of Pallets | 10 |
| Capacity of FIX_STORE | 10 |
| LOAD Process Time | 4 Minutes |
| UNLOAD Process Time | 4 Minutes |

#### Part 7

The valves will now move into an INSPECT station. Quality has been a big issue in the past with only 85% of the valves passing INSPECTION. The plant manager would like to see the rejection rate be improved to 5%.

If a valve is rejected, it will go into the REWORK buffer for repairs. The repair time depends on what valve needs to be repaired. An 8” valve takes 16 minutes to repair, while a 10” valve takes 20 minutes to repair.

If there is one or more valves in the REWORK buffer, INSPECT will take one to re-inspect it. Otherwise, INSPECT has to wait for UNLOAD to be ready to give it a valve to inspect.

Figure 10-9: Layout for Part 7

| Table 10-18: Data |  |  |
| --- | --- | --- |
| Element |  | Value |
| INSPECTCycle Time |  | Uniformly Distributed 5-10 Minutes |
| REWORK |  |  |
|  | Cycle Time: | 8” – 16 minutes |
|  |  | 10” – 20 minutes |
| REWORK |  |  |
|  | Capacity: | 10 valves |

#### Part 8

Information on the total number of 8” and 10” valves that have been reworked should be displayed on the screen as the model runs. Calculate and display separately the total number of 8” and 10” valves that have been reworked.

Calculate and display the average downtime of the DRILL, taking all failures since Time 0.0 into account.

#### Part 9

The facility works three shifts per day, 7 days per week. Their patterns are detailed below:

| Table 10-19: Shift Patterns |  |  |  |
| --- | --- | --- | --- |
|  | Shift 2 | Shift 3 | Shift1 |
| Start | 8:00 AM | 4:00 PM | 12:00 AM |
| Lunch | 11:00 AM | 8:30 PM | 4:00 AM |
| Finish | 4:00 PM | 12:00 AM | 8:00 AM |

This shift applies to part arrivals and labor.

#### Part 10

The INSPECTION station actually works in more detail than we have previously modeled. The operation is actually a series of 4 distinct cycles. They are as follows:

| Table 10-20: |  |  |
| --- | --- | --- |
| Description | Cycle Time | Labor Requirement |
| Position Valve | 1 min | Larry or Curly |
| 1st Automatic Test | 3 min | No |
| Calibrate | 1 min | Curly or Larry |
| 2nd Automatic Test | 2 min | No |

#### Part 11

#### Objective

Increase total throughput. (Around 100% improvement.)

#### Budget

$75,000

#### Time Frame

Hours of production	50 hours (3000 minutes)

Warm-up period	10 hours (600 minutes)

Total Run Length	60 hours (3600)

| Table 10-21: Cost Data |  |  |
| --- | --- | --- |
| Item | Cost |  |
| BUFFERS: |  |  |
| Per Part capacity increase in any Buffer | $ 100 |  |
| SAW or DRILL: |  |  |
| Identical new SAW or DRILL |  | $ 30,000 |
| Cycle Time decrease per 10% |  | $ 5,000 |
| Setup Time decrease per 10% |  | $ 8,500 |
| Setup decrease to double number of operations |  | $ 5,000 |
| Increase time between failures per 10% |  | $ 5,000 |
| Decrease repair time per 20% |  | $ 7,500 |
| COATER: |  |  |
| Identical new COATER |  | $ 25,000 |
| Cycle Time decrease per 10% |  | $ 6,500 |
| Increase time between failures per 10% |  | $ 7,000 |
| Decrease repair time per 15% |  | $ 6,500 |
| CONVEYORS: |  |  |
| Convert previously fixed to queuing |  | $ 10,000 |
| New conveyor of either type |  | $ 20,000 |
| Cycle time decrease per 10% |  | $ 6,000 |
| LOAD or UNLOAD: |  |  |
| New LOAD or UNLOAD |  | $ 15,000 |
| Decrease cycle time per 10% |  | $ 17,000 |
| New Fixture |  | $ 5,000 |
| INSPECT: |  |  |
| New INSPECT Machine |  | $ 15,000 |
| New Labor to relieve Curly and Larry |  | $ 30,000 |
| Cycle time decrease per 10% |  | $ 10,000 |
| Decrease rejection rate by 50% |  | $ 20,000 |
| Rework time decrease by 10% |  | $ 8,000 |

## Ink Jet Cartridge Manufacture & Distribution

### Introduction

A manufacturer of inkjet printer ink cartridges wishes to model three major parts of its supply chain:

- Inkjet cartridge Manufacturing Center

- Inkjet cartridge Packaging & Shipping Center

- Shipping carton manufacturing

- Distribution center

The system they want to model operates as follows:

- Ink Solution and four different colors of Ink Solid are mixed together to make the ink. Cartridges are then filled with the different colored inks and are packed into boxes manufactured in a separate facility. - A separate facility manufactures shipping cartons for the ink cartridges. Cartons undergo cutting, printing, gluing, folding, and bundling operations. - Trucks are used to transport bundles of 10 shipping cartons to the ink cartridge manufacturing facility. - Once the ink cartridges have been packed into shipping cartons at that facility, trucks transport them to a distribution center.

Figure 10-10: Process Flow Diagram

Figure 10-11: Site Map

### Creating the Model

#### Inkjet Cartridge Manufacturing

- Four different colors of Ink Solids arrive in plastic containers, per the following repeating sequence, and are placed on a roller conveyor. The roller conveyor can hold a maximum of 10 ink solids bottles and has an index time of 1 minute.

| Table 10-22: |  |  |
| --- | --- | --- |
| Ink Solids Color | Bottle Size (liters) | Quantity |
| Black | 1.001 | 10 |
| Cyan | 0.501 | 4 |
| Yellow | 0.501 | 4 |
| Magenta | 0.501 | 4 |

- An emptying machine then takes one ink solids bottle at a time from the roller conveyor. A bar code reader on the machine scans the bottle to determine its color number. If the color number is different than the last bottle processed at this machine, the machine will not empty it into the Ink Solids Holding Tank until the Mixer has completed mixing and dispensing the current ink color. When the Mixer has completed its processing cycle, the emptying machine then removes the cap and other packaging material from the bottle, and empties its contents into the Ink Solids Holding Tank. The cap removal and bottle de-packaging process takes 45 seconds. The ink solids are then emptied into the Solids Holding Tank at the rate of 0.5 liters/minute. The Ink Solids Holding Tank has a capacity of 125 liters. - Ink Solution, a solvent-like liquid, is pulled from a source that is assumed to have an unlimited supply at the rate of 0.25 liters/minute into a Solution Holding Tank with a capacity of 125 liters.

Different volumes of Ink Solution and Ink Solids are flowed into the Mixing Vat at the rate of 1 gallon/minute depending on the color of the Ink Solids per the mixing table below:

| Table 10-23: |  |  |  |
| --- | --- | --- | --- |
| Ink Solids Color | Ink Solids Volume | Ink Solution Volume | Total Quantity Ink Produced |
| Black | 10 | 15 | 25 |
| Cyan | 2 | 8 | 10 |
| Yellow | 2 | 8 | 10 |
| Magenta | 2 | 8 | 10 |

- When the Mixing Vat has finished its processing cycle, an ink cartridge filling machine then takes an empty ink cartridge from a buffer and draws the required volume of ink from the Mixing Vat, filling the cartridge. Black ink cartridges hold 0.5 liters while the colored cartridges hold 0.25 liters. Empty ink cartridges are delivered to the buffer in batches of 5 every 2 hours. This buffer can hold a total of 20 cartridges.

#### Packaging & Shipping the Cartridges

- The Cartridge Packaging station packs a quantity of each color of cartridge into individual shipping cartons. Different colored cartridges may not be mixed in the cartons. Black cartridges are packed 25 per box, while all other colors are packed 40 per box. Packing time is 1 minute per cartridge. - The shipping cartons are manufactured in a separate facility, and shrink wrapped, into 10-carton bundles. The bundles of cartons are then shipped one at a time to and shipped to an unpacking station where they are separated again and unfolded from their flat condition into individual cartons ready to accept product. The cartons are then stored in a buffer for use by the Cartridge Packaging station. - The loaded cartons of ink cartridges are then moved to a shipping dock where they await transport to the Distribution. The shipping dock can hold a maximum of 10 cartons.

#### Shipping Carton Manufacturing

- Sheets of raw corrugated cardboard are cut into 6 separate pieces in a die-cutting machine. It is assumed for purposes of modeling that an unlimited supply of the raw material is available. The die cutting operation takes 10 minutes. - Each die cut piece of cardboard is then printed with product name, etc. one at a time in a printing machine. This operation takes 3 minutes. - The printed, die cut pieces then undergo a final folding and gluing operation which takes 1 minute per box. The result is a finished shipping carton. - A shrink wrapping machine then bundles 10 cartons in their flat condition together. This operation takes 7 minutes. - The 10-carton bundles are then placed on a shipping dock awaiting transport by truck to the ink jet cartridge packaging facility. The shipping dock can hold a maximum of 10 bundles.

- Shipping cartons will only be manufactured if the number of completed carton bundles falls below 3.

#### Distribution Center

- The truck will deliver 1 carton of inkjet cartridges at a time to a Receiving Dock. The Receiving Dock can hold a maximum of 10 cartons. - A processing center then adds routing and other information to each carton and ships them retail outlets as demanded. The processing operation takes 10 minutes. For purposes of modeling, the cartons can leave the model (be shipped) from the processing center.

#### Truck Transport & Roadway System

- A single truck travels a road system connecting all of the plant sites in the model per Figure 2 (Site Map) to pick up or drop off carton bundles or shipping cartons containing inkjet cartridges. The truck is radio dispatched from a control center and will handle calls to pick up or drop off in the order that it receives them. The truck travels at a speed of 45 miles/hour (0.75 miles/minute) whether it is loaded or unloaded. All loading operations take 5 minutes. The truck should always take the shortest route to its destination to load or unload parts. Note that the truck must return to the depot whenever it becomes idle. Note also that some roads are one-way while others are two-way as illustrated by the directional arrows in the Site Map. - The truck, originating from a depot, is initially dispatched to pick up a bundle of completed shipping cartons at the Shipping Carton Manufacturing Center. The truck then travels to the receiving dock at the Inkjet Cartridge Packaging Center. It then drops off the carton into the Unpacking station. After this initial shipment of cartons, the truck will only deliver another bundle of cartons when only one carton remains in the buffer for the Cartridge Packaging station, assuming that cartons are ready for pick up at the Carton Manufacturing Center. - When the truck is not transporting bundles of shipping cartons, it will travel to one of two shipping docks at the Inkjet Cartridge packaging facility to pick up any cartons containing product one at a time and deliver them to the receiving dock at the Distribution Center. Two shipping docks have been installed at the Inkjet Cartridge Packaging Center to accommodate future increases in production demand from this facility. Create the appropriate logic in your model to allow a truck to move to whichever dock is available.

### Experimenting with Your Model

Maximize the total number of cartons of ink cartridges processed and shipped from the Distribution Center by making improvements to the operation whose total cost does not exceed $100,000. Management will allow only the following changes.

| Table 10-24: Change Options |  |
| --- | --- |
| Description of Change | Cost (or credit) per Unit |
| Increase flow rate from Ink Solids and Ink Fluid tanks | $5,000 per 1 liter/minute increase in flow rate |
| Reduce Ink Solids Bottle emptying machine cycle time | $1,000 per 10% reduction in cycle time |
| Increase flow rate of bottles being emptied into Ink Solids Holding Tank. | $1,000 per 1 liter/minute increase in flow rate |
| Reduce Ink Cartridge Filling machine cycle time | $2,000 per 10% reduction in cycle time |
| Increase ink cartridge filling rate | $1,000 per 1 liter/minute increase in flow rate |
| Increase empty inkjet cartridge delivery rate to Inkjet Cartridge Manufacturing Center | $8,000 per 20% reduction in delivery interval (time between deliveries), or $6,000 per 20% increase in delivery lot size |
| Decrease Cartridge Packaging station batch cycle time | $10,000 per 10% reduction in cycle time |
| Order truck with larger payload capacity from trucking company | $5,000 per additional unit of capacity |
| Reduce cycle time at Processing Center | $3,000 per 10% reduction in cycle time |
| Increase maximum number of shipping carton bundles permitted to be stored in the shipping area of the Carton Manufacturing Center | <$1,000> credit for each additional bundle. This decreases frequency of disruptions to production cycle at this facility so they are willing to reduce their selling price per bundle accordingly. |