# Lab 7.5.3: Troubleshooting Wireless Configuration

## Topology Diagram

## Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| R1 | Fa0/0.5 | 5.5.5.10 | 255.255.255.0 | N/A |
|  | Fa0/0.10 | 192.168.10.1 | 255.255.255.0 | N/A |
|  | Fa0/0.11 | 11.11.11.1 | 255.255.255.0 | N/A |
|  | Fa0/0.18 | 18.18.18.1 | 255.255.255.0 | N/A |
|  | Lo0 | 10.1.1.1 | 255.255.255.252 | N/A |
| WRS2 | WAN | 192.168.10.2 | 255.255.255.0 | 192.168.10.1 |
|  | LAN/Wireless | 192.168.20.1 | 255.255.255.0 | N/A |
| WRS3 | WAN | 192.168.10.3 | 255.255.255.0 | 192.168.10.1 |
|  | LAN/Wireless | 192.168.30.1 | 255.255.255.0 | N/A |
| PC1 | NIC | 11.11.11.10 | 255.255.255.0 | 11.11.11.1 |
| PC4 | NIC | 18.18.18.10 | 255.255.255.0 | 18.18.18.1 |
| S1 | VLAN 5 | 5.5.5.1 | 255.255.255.0 | N/A |
| S2 | VLAN 5 | 5.5.5.2 | 255.255.255.0 | N/A |
| S3 | VLAN 5 | 5.5.5.3 | 255.255.255.0 | N/A |

## Scenario

In this lab, a basic network and wireless network have been configured improperly. You must find and correct the misconfigurations based on the minimum network specifications provided by your company.

Here are the configurations to load into your router and switches.

R1 Configuration

hostname R1

!

interface Loopback0

ip address 10.1.1.1 255.255.255.0

!

interface FastEthernet0/0

no ip address

duplex auto

speed auto

no shutdown

!

interface FastEthernet0/0.5

encapsulation dot1Q 5

ip address 5.5.5.10 255.255.255.0

!

interface FastEthernet0/0.10

encapsulation dot1Q 10

ip address 192.168.11.1 255.255.255.0

!

!

interface FastEthernet0/0.18

encapsulation dot1Q 18

ip address 18.18.18.1 255.255.255.0

!

ip route 192.168.20.0 255.255.255.0 192.168.10.2

ip route 192.168.30.0 255.255.255.0 192.168.10.3

!

line con 0

exec-timeout 0 0

logging synchronous

!

end

Switch 1 Configuration

hostname S1

!

vtp mode transparent

!

vlan 5,10-11

!

interface FastEthernet0/1

switchport trunk allowed vlan 5,10,11

switchport mode trunk

switchport trunk native vlan 5

!

interface FastEthernet0/2

switchport trunk allowed vlan 5,10,11

switchport mode trunk

switchport trunk native vlan 5

!

interface FastEthernet0/3

switchport trunk allowed vlan 5,10,11

switchport mode trunk

switchport trunk native vlan 5

!

interface FastEthernet0/4

switchport trunk allowed vlan 5,10,11

switchport mode trunk

switchport trunk native vlan 5

!

interface FastEthernet0/5

switchport mode trunk

switchport trunk native vlan 5

!

interface Vlan5

ip address 5.5.5.1 255.255.255.0

no shutdown

!

line con 0

exec-timeout 0 0

logging synchronous

!

End

Switch 2 Configuration

hostname S2

!

vtp mode transparent

ip subnet-zero

!

vlan 5,10-11,18

!

interface FastEthernet0/1

switchport trunk allowed vlan 5,10,11,18

switchport mode access

!

interface FastEthernet0/2

switchport trunk allowed vlan 5,10,11,18

switchport mode access

!

interface FastEthernet0/3

switchport trunk allowed vlan 5,10,11,18

switchport mode access

!

interface FastEthernet0/4

switchport trunk allowed vlan 5,10,11,18

switchport mode access

!

interface FastEthernet0/7

switchport access vlan 10

!

interface FastEthernet0/11

switchport access vlan 11

switchport mode access

switchport port-security mac-address sticky

switchport port-security mac-address sticky 0336.5b1e.33fa

!

interface FastEthernet0/18

switchport access vlan 18

switchport mode access

switchport port-security

switchport port-security mac-address sticky

switchport port-security mac-address sticky 022c.ab13.22fb

!

interface Vlan1

no ip address

shutdown

!

interface Vlan5

ip address 5.5.5.2 255.255.255.0

no shutdown

!

line con 0

exec-timeout 0 0

logging synchronous

!

End

Switch 3 Configuration

hostname S3

!

vtp mode transparent

!

vlan 5,10-11,18

!

interface FastEthernet0/1

switchport trunk allowed vlan 5,10,11,18

switchport mode trunk

switchport trunk native vlan 5

!

interface FastEthernet0/2

switchport trunk allowed vlan 5,10,11,18

switchport mode trunk

switchport trunk native vlan 5

!

interface FastEthernet0/3

switchport trunk allowed vlan 5,10,11,18

switchport mode trunk

switchport trunk native vlan 5

!

interface FastEthernet0/4

switchport trunk allowed vlan 5,10,11,18

switchport mode trunk

switchport trunk native vlan 5

!

interface FastEthernet0/7

!

interface Vlan1

no ip address

no ip route-cache

!

interface Vlan5

ip address 6.6.6.3 255.255.255.0

no shutdown

!

line con 0

exec-timeout 0 0

logging synchronous

!

end

## Wireless Router Network Requirements

While troubleshooting WRS2 and WRS3, ensure that at least the following capabilities exist:

- Connections via the IP addresses shown in the topology diagram.

- More than 30 clients can get an IP address through DHCP at a single time.

- A client can have a DHCP address for at least 2 hours.

- Clients using both B and G wireless network modes can connect, but N clients cannot.

- Wireless clients must be authenticated using WEP with a key of 5655545251.

- Traffic between PC2 and PC3 must take the most efficient route possible.

- Ping requests coming from outside WAN ports of the Linksys routers to their inside LAN/wireless IP addresses (192.168.30.1) must be successful.

- DHCP must not give out IP addresses in a range that includes the addresses for PC2 and PC3.

- The two wireless networks must not interfere with each other.

## Wireless Network Solution

Record your solution below.