# Lab 4.4.3 Troubleshooting VTP Configuration

## Topology Diagram

## Addressing Table

| Device (Hostname) | Interface | IP Address | Subnet Mask |
| --- | --- | --- | --- |
| S1 | VLAN 99 | 172.17.99.11 | 255.255.255.0 |
| S2 | VLAN 99 | 172.17.99.12 | 255.255.255.0 |
| S3 | VLAN 99 | 172.17.99.13 | 255.255.255.0 |
| PC1 | NIC | 172.17.10.21 | 255.255.255.0 |
| PC2 | NIC | 172.17.20.22 | 255.255.255.0 |
| PC3 | NIC | 172.17.30.23 | 255.255.255.0 |
| PC4 | NIC | 172.17.10.24 | 255.255.255.0 |
| PC5 | NIC | 172.17.20.25 | 255.255.255.0 |
| PC6 | NIC | 172.17.30.26 | 255.255.255.0 |

## Port Assignments (Switches 2 and 3)

| Ports | Assignment | Network |
| --- | --- | --- |
| Fa0/1 – 0/5 | 802.1q Trunks (Native VLAN 99) | 172.17.99.0 /24 |
| Fa0/6 – 0/10 | VLAN 30 – Guest (Default) | 172.17.30.0 /24 |
| Fa0/11 – 0/17 | VLAN 10 – Faculty/Staff | 172.17.10.0 /24 |
| Fa0/18 – 0/24 | VLAN 20 – Students | 172.17.20.0 /24 |

## Objectives

Upon completion of this lab, you will be able to:

- Cable a network according to the topology diagram

- Erase the startup configuration and vlan.dat files and reload switches to the default state

- Load the switches with supplied scripts

- Find and correct all configuration errors

- Document the corrected network

## Scenario

The VLAN Trunking Protocol (VTP) helps ensure uniform VLAN configurations on your switched network, but it must be configured correctly. In this lab, you will use the supplied scripts to configure S1 as a VTP server, and S2 and S3 as VTP clients. The VTP domain name is Lab4_3, and the VTP password is cisco. However, there are a number of errors in this configuration that you must troubleshoot and correct before end-to-end connectivity within the VLAN is restored.

You will have successfully resolved all errors when the same VLANs are configured on all three switches, and you can ping between any two hosts in the same VLAN or between any two switches.

Task 1: Prepare the Network

Step 1: Cable a network that is similar to the one in the topology diagram.

You can use any current switch in your lab as long as it has the required interfaces shown in the topology diagram. The output shown in this lab is based on 2960 switches. Other switch types may produce different output. If you are using older switches, then some commands may be different or unavailable.

Set up console connections to all three switches.

Step 2: Clear any existing configurations on the switches.

Clear switch configurations and VLANs on all three switches and reload them to restore the default state. Use the show vlan command to confirm that only default VLANs exist and that all ports are assigned to VLAN 1.

Step 3: Configure the Ethernet interfaces on the host PCs.

Configure the Ethernet interfaces of PC1, PC2, PC3, PC4, PC5, and PC6 with the IP addresses indicated in the addressing table at the beginning of the lab. There is no need to configure the default gateways for this lab.

Task 2: Load Switches with Supplied Scripts

S1 Configuration

enable

!

config term

hostname S1

enable secret class

no ip domain-lookup

!

vtp mode server

vtp domain Lab4_3

vtp password Cisco

!

vlan 99

name management

exit

!

vlan 10

name Faculty/Staff

exit

!

vlan 20

name Students

exit

!

vlan 30

name Guest

exit

!

interface FastEthernet0/1

switchport trunk native vlan 99

switchport mode trunk

!

interface FastEthernet0/2

switchport trunk native vlan 99

switchport mode access

!

interface FastEthernet0/3

switchport trunk native vlan 99

switchport mode access

!

interface FastEthernet0/4

switchport trunk native vlan 99

switchport mode trunk

!

interface FastEthernet0/5

switchport trunk native vlan 99

switchport mode trunk

!

interface range FastEthernet0/6-24

shutdown

!

interface GigabitEthernet0/1

shutdown

!

interface GigabitEthernet0/2

shutdown

!

interface Vlan99

ip address 179.17.99.11 255.255.255.0

no shutdown

!

line con 0

logging synchronous

password cisco

login

line vty 0

no login

line vty 1 4

password cisco

login

line vty 5 15

password cisco

login

!

end

S2 Configuration

hostname S2

!

enable secret class

no ip domain-lookup

!

vtp mode client

vtp domain Lab4

!

!

interface FastEthernet0/1

switchport trunk native vlan 99

switchport mode access

!

interface FastEthernet0/2

switchport trunk native vlan 99

switchport mode access

!

interface FastEthernet0/3

switchport trunk native vlan 99

switchport mode trunk

!

interface FastEthernet0/4

switchport trunk native vlan 99

switchport mode trunk

!

interface FastEthernet0/5

switchport trunk native vlan 99

switchport mode trunk

!

interface range FastEthernet0/6 - 10

switchport access vlan 10

!

switchport mode access

!

interface range FastEthernet0/11 - 17

switchport access vlan 20

switchport mode access

!

interface range FastEthernet0/18 - 24

switchport access vlan 30

switchport mode access

!

interface Vlan99

ip address 172.17.99.12 255.255.255.0

no shutdown

!

ip http server

!

line con 0

password cisco

logging synchronous

login

line vty 0 4

password cisco

login

line vty 5 15

password cisco

S3 Configuration

hostname S3

!

enable secret class

no ip domain-lookup

!

vtp mode client

vtp domain Lab4

!

!

interface FastEthernet0/1

switchport trunk native vlan 99

switchport mode trunk

!

interface FastEthernet0/2

switchport trunk native vlan 99

switchport mode trunk

!

interface FastEthernet0/3

switchport trunk native vlan 99

switchport mode trunk

!

interface FastEthernet0/4

switchport trunk native vlan 99

switchport mode trunk

!

interface FastEthernet0/5

switchport trunk native vlan 99

switchport mode trunk

!

interface range FastEthernet0/6 - 10

switchport access vlan 30

switchport mode access

!

interface range FastEthernet0/11 - 17

switchport access vlan 10

switchport mode access

!

interface range FastEthernet0/18 - 24

switchport access vlan 20

switchport mode access

!

interface Vlan99

ip address 172.17.99.12 255.255.255.0

no shutdown

!

line con 0

password cisco

login

line vty 0 4

password cisco

login

line vty 5 15

password cisco

login

end

Task 3: Troubleshoot and Correct VTP and Configuration Errors

When all errors are corrected, you should be able to ping PC4 from PC1, PC5 from PC2, and PC6 from PC3. You should also be able to ping the management interfaces on both S2 and S3 from S1.

Task 4: Document the Switch Configuration

When you have completed your troubleshooting, capture the output of the show run command and save it to a text document for each switch.

Task 5: Clean Up

Erase the configurations and reload the switches. Disconnect and store the cabling. For PC hosts that are normally connected to other networks (such as the school LAN or to the Internet), reconnect the appropriate cabling and restore the TCP/IP settings.