# Lab 5.5.3: Troubleshooting Spanning Tree Protocol

## Topology Diagram

## Addressing Table

| Device (Hostname) | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| S1 | VLAN 99 | 172.17.99.11 | 255.255.255.0 | N/A |
| S2 | VLAN 99 | 172.17.99.12 | 255.255.255.0 | N/A |
| S3 | VLAN 99 | 172.17.99.13 | 255.255.255.0 | N/A |
| PC1 | NIC | 172.17.10.21 | 255.255.255.0 | 172.17.10.1 |
| PC2 | NIC | 172.17.20.22 | 255.255.255.0 | 172.17.20.1 |
| PC3 | NIC | 172.17.30.23 | 255.255.255.0 | 172.17.30.1 |

## Port Assignments – Switch 2

| Ports | Assignment | Network |
| --- | --- | --- |
| Fa0/1 – 0/4 | 802.1q Trunks (Native VLAN 99) | 172.17.99.0 /24 |
| Fa0/5 – 0/10 | VLAN 30 – Guest (Default) | 172.17.30.0 /24 |
| Fa0/11 – 0/17 | VLAN 10 – Faculty/Staff | 172.17.10.0 /24 |
| Fa0/18 – 0/24 | VLAN 20 – Students | 172.17.20.0 /24 |

## Learning Objectives

Upon completion of this lab, you will be able to:

- Analyze a congestion problem in a redundant, switched LAN network.

- Recognize the capabilities for per-VLAN load balancing with PVST.

- Modify the default STP configuration to optimize available bandwidth.

- Verify that modifications have had the intended effect.

Scenario

You are responsible for the operation of the redundant switched LAN shown in the topology diagram. You and your users have been observing increased latency during peak usage times, and your analysis points to congested trunks. You recognize that of the six trunks configured, only two are forwarding packets in the default STP configuration currently running. The solution to this problem requires more effective use of the available trunks. The PVST+ feature of Cisco switches provides the required flexibility to distribute the inter-switch traffic using all six trunks.

This lab is complete when all wired trunks are carrying traffic, and all three switches are participating in per-VLAN load balancing for the three user VLANs.

Task 1: Prepare the Network

Step 1: Cable a network that is similar to the one in the topology diagram.

You can use any current switch in your lab as long as it has the required interfaces shown in the topology diagram. The output shown in this lab is based on Cisco 2960 switches. Other switch models may produce different output.

Set up console connections to all three switches.

Step 2: Clear any existing configurations on the switches.

Clear NVRAM, delete the vlan.dat file, and reload the switches.

Step 3: Load the switches with the following script:

S1 Configuration

hostname S1

enable secret class

no ip domain-lookup

!

vtp mode server

vtp domain Lab5

vtp password cisco

!

vlan 99

name Management

exit

!

vlan 10

name Faculty/Staff

exit

!

vlan 20

name Students

exit

!

vlan 30

name Guest

exit

!

interface FastEthernet0/1

switchport trunk native vlan 99

switchport mode trunk

no shutdown

!

interface FastEthernet0/2

switchport trunk native vlan 99

switchport mode trunk

no shutdown

!

interface FastEthernet0/3

switchport trunk native vlan 99

switchport mode trunk

no shutdown

!

interface FastEthernet0/4

switchport trunk native vlan 99

switchport mode trunk

no shutdown

!

interface range FastEthernet0/5-24

shutdown

!

interface GigabitEthernet0/1

shutdown

!

interface GigabitEthernet0/2

shutdown

!

interface Vlan99

ip address 172.17.99.11 255.255.255.0

no shutdown

!

line con 0

logging synchronous

password cisco

login

line vty 0

no login

line vty 1 4

password cisco

login

line vty 5 15

password cisco

login

!

end

S2 Configuration

hostname S2

!

enable secret class

no ip domain-lookup

!

vtp mode client

vtp domain Lab5

vtp password cisco

!

interface FastEthernet0/1

switchport trunk native vlan 99

switchport mode trunk

no shutdown

!

interface FastEthernet0/2

switchport trunk native vlan 99

switchport mode trunk

no shutdown

!

interface FastEthernet0/3

switchport trunk native vlan 99

switchport mode trunk

no shutdown

!

interface FastEthernet0/4

switchport trunk native vlan 99

switchport mode trunk

no shutdown

!

interface range FastEthernet0/5 - 10

switchport access vlan 30

switchport mode access

!

interface range FastEthernet0/11 - 17

switchport access vlan 10

switchport mode access

!

interface range FastEthernet0/18 - 24

switchport access vlan 20

switchport mode access

!

interface fa0/6

no shutdown

interface fa0/11

no shutdown

interface fa0/18

no shutdown

!

interface Vlan99

ip address 172.17.99.12 255.255.255.0

no shutdown

!

line con 0

password cisco

logging synchronous

login

line vty 0 4

password cisco

login

line vty 5 15

password cisco

S3 Configuration

hostname S3

!

enable secret class

no ip domain-lookup

!

vtp mode client

vtp domain Lab5

vtp password cisco

!

interface FastEthernet0/1

switchport trunk native vlan 99

switchport mode trunk

no shutdown

!

interface FastEthernet0/2

switchport trunk native vlan 99

switchport mode trunk

no shutdown

!

interface FastEthernet0/3

switchport trunk native vlan 99

switchport mode trunk

no shutdown

!

interface FastEthernet0/4

switchport trunk native vlan 99

switchport mode trunk

no shutdown

!

interface range FastEthernet0/5 - 10

switchport access vlan 30

switchport mode access

!

interface range FastEthernet0/11 - 17

switchport access vlan 10

switchport mode access

!

interface range FastEthernet0/18 - 24

switchport access vlan 20

switchport mode access

!

interface Vlan99

ip address 172.17.99.13 255.255.255.0

no shutdown

!

line con 0

password cisco

login

line vty 0 4

password cisco

login

line vty 5 15

password cisco

login

end

Task 2: Configure Host PCs

Configure the Ethernet interfaces of PC1, PC2, and PC3 with the IP address, subnet mask, and gateway indicated in the addressing table.

Task 3: Identify the Initial State of All Trunks

On each of the switches, display the spanning tree table with the show spanning-tree command. Note which ports are forwarding on each switch, and identify which trunks are not being used in the default configuration. You can use your network topology drawing to document the initial state of all trunk ports.

Task 4: Modify Spanning Tree to Achieve Load Balancing

Modify the spanning tree configuration so that all six trunks are in use. Assume that the three user LANs (10, 20, and 30) carry an equal amount of traffic. Aim for a solution that will have a different set of ports forwarding for each of the three user VLANs. At a minimum, each of the three user VLANs should have a different switch as the root of the spanning tree.

Task 5: Document the Switch Configuration

When you have completed your solution, capture the output of the show run command and save it to a text file for each switch.

Task 6: Clean Up

Erase the configurations and reload the switches. Disconnect and store the cabling. For PC hosts that are normally connected to other networks (such as the school LAN or to the Internet), reconnect the appropriate cabling and restore the TCP/IP settings.