# Lab 3.5.3: Troubleshooting VLAN Configurations

## Topology Diagram

## Addressing Table

| Device (Hostname) | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| S1 | VLAN 56 | 192.168.56.11 | 255.255.255.0 | N/A |
| S2 | VLAN 56 | 192.168.56.12 | 255.255.255.0 | N/A |
| S3 | VLAN 56 | 192.168.56.13 | 255.255.255.0 | N/A |
| PC1 | NIC | 192.168.10.21 | 255.255.255.0 | 192.168.10.1 |
| PC2 | NIC | 192.168.20.22 | 255.255.255.0 | 192.168.20.1 |
| PC3 | NIC | 192.168.30.23 | 255.255.255.0 | 192.168.30.1 |
| PC4 | NIC | 192.168.10.24 | 255.255.255.0 | 192.168.10.1 |
| PC5 | NIC | 192.168.20.25 | 255.255.255.0 | 192.168.20.1 |
| PC6 | NIC | 192.168.30.26 | 255.255.255.0 | 192.168.30.1 |

## Initial Port Assignments (Switches 2 and 3)

| Ports | Assignment | Network |
| --- | --- | --- |
| Fa0/1 – 0/5 | 802.1q Trunks (Native VLAN 56) | 192.168.56.0 /24 |
| Fa0/6 – 0/10 | VLAN 30 – Guest (Default) | 192.168.30.0 /24 |
| Fa0/11 – 0/17 | VLAN 10 – Faculty/Staff | 192.168.10.0 /24 |
| Fa0/18 – 0/24 | VLAN 20 – Students | 192.168.20.0 /24 |

## Learning Objective

Practice basic VLAN troubleshooting skills.

## Scenario

In this lab, you will practice troubleshooting a misconfigured VLAN environment. Load or have your instructor load the configurations below into your lab gear. Your objective is to locate and correct any and all errors in the configurations and establish end-to-end connectivity. Your final configuration should match the topology diagram and addressing table. All passwords are set to cisco, except the enable secret password, which is set to class.

Task 1: Prepare the Network

Step 1: Cable a network that is similar to the one in the topology diagram.

Step 2: Clear any existing configurations on the switches, and initialize all ports in the shutdown state.

Step 3: Import the configurations below.

Switch 1

hostname S1

no ip domain-lookup

enable secret class

!

!

interface range FastEthernet0/1-5

switchport mode trunk

!

interface range FastEthernet0/6-24

shutdown

!

interface Vlan1

no ip address

no ip route-cache

!

interface Vlan56

ip address 192.168.56.11 255.255.255.0

no ip route-cache

!

line con 0

logging synchronous

line vty 0 4

no login

line vty 5 15

password cisco

login

!

end

Switch 2

hostname S2

no ip domain-lookup

enable secret class

!

vlan 10,20,30,56

!

interface range FastEthernet0/1-5

switchport trunk native vlan 56

switchport mode access

!

interface range FastEthernet0/6-10

switchport access vlan 30

switchport mode access

!

interface range FastEthernet0/11-17

switchport access vlan 10

switchport mode access

!

interface range FastEthernet0/18-24

switchport access vlan 20

switchport mode access

!

interface GigabitEthernet0/1

!

interface GigabitEthernet0/2

!

interface Vlan1

ip address 192.168.56.12 255.255.255.0

no ip route-cache

shutdown

!

line con 0

password cisco

login

line vty 0 4

password cisco

login

line vty 5 15

password cisco

login

!

end

Switch 3

hostname S3

no ip domain-lookup

enable secret class

!

vlan 10,20,30

!

interface range FastEthernet0/1-5

switchport trunk native vlan 56

switchport mode trunk

!

interface range FastEthernet0/6-10

switchport mode access

!

interface range FastEthernet0/11-17

switchport mode access

!

interface range FastEthernet0/18-24

switchport mode access

!

interface GigabitEthernet0/1

!

interface GigabitEthernet0/2

!

interface Vlan1

no ip address

no ip route-cache

shutdown

!

interface Vlan56

no ip route-cache

!

line con 0

password cisco

login

line vty 0 4

password cisco

login

line vty 5 15

password cisco

login

!

end

Task 2: Troubleshoot and Repair the VLAN Configuration

Task 3: Document the Switch Configurations

On each switch, capture the running configuration to a text file and save for future reference:

Task 4: Clean Up

Erase the configurations and reload the switches. Disconnect and store the cabling. For PC hosts that are normally connected to other networks (such as the school LAN or to the Internet), reconnect the appropriate cabling and restore the TCP/IP settings.