CCNPv6 SWITCH

Chapter 4 Lab 4-1, Inter-VLAN Routing with an External Router

Topology

Objective

- Configure inter-VLAN routing using an external router, also known as a router on a stick.

- Background

Inter-VLAN routing using an external router can be a cost-effective solution when it is necessary to segment a network into multiple broadcast domains. In this lab, you split an existing network into two separate VLANs on the access layer switches, and use an external router to route between the VLANs. An 802.1Q trunk connects the switch and the Fast Ethernet interface of the router for routing and management. Static routes are used between the gateway router and the ISP router. The switches are connected via an 802.1Q EtherChannel link.

Note: This lab uses Cisco 1841 routers with Cisco IOS Release 12.4(24)T1 and the Advanced IP Services image c1841-advipservicesk9-mz.124-24.T1.bin. The switches are Cisco WS-C2960-24TT-L with the Cisco IOS image c2960-lanbasek9-mz.122-46.SE.bin. You can use other routers (such as 2801 or 2811), switches (such as 2950), and Cisco IOS Software versions if they have comparable capabilities and features. Depending on the router or switch model and Cisco IOS Software version, the commands available and output produced might vary from what is shown in this lab.

Required Resources

- 2 routers (Cisco 1841 with Cisco IOS Release 12.4(24)T1 Advanced IP Services or comparable)

- 2 switches (Cisco 2960 with the Cisco IOS Release 12.2(46)SE C2960-LANBASEK9-M image or comparable)

- Serial and Ethernet cables

Step 1: Prepare the switches and routers for the lab.

- Cable the network as shown in the topology diagram. On each switch, erase the startup configuration, delete the vlan.dat file, and reload the switches. Refer to Lab 1-1, “Clearing a Switch” and Lab 1-2, “Clearing a Switch Connected to a Larger Network” to prepare the switches for this lab.

- Erase the startup configuration and reload the routers.

Step 2: Configure the hosts.

Configure PC hosts A and B with the IP address, subnet mask (/24), and default gateway shown in the topology.

Step 3: Configure the routers.

- Configure the ISP router for communication with your gateway router. The static route used for the internal networks provides a path for the local network from the ISP. In addition, configure a loopback interface on the ISP router to simulate an external network.

Router(config)# hostname ISP

ISP(config)# interface Loopback0

ISP(config-if)# ip address 200.200.200.1 255.255.255.0

ISP(config-if)# interface Serial0/0/0

ISP(config-if)# ip address 192.168.1.2 255.255.255.0

ISP(config-if)# no shutdown

ISP(config-if)# exit

ISP(config)# ip route 172.16.0.0 255.255.0.0 192.168.1.1

- Configure the Gateway router to communicate with the ISP router. Notice the use of a static default route. The default route tells the router to send any traffic with an unknown destination network to the ISP router.

Router(config)# hostname Gateway

Gateway(config)# interface Serial0/0/0

Gateway(config-if)# ip address 192.168.1.1 255.255.255.0

Gateway(config-if)# clockrate 64000

Gateway(config-if)# no shutdown

Gateway(config-if)# exit

Gateway(config)# ip route 0.0.0.0 0.0.0.0 192.168.1.2

- Verify connectivity from the Gateway router using the ping command.

Was this ping successful?

Step 4: Configure the switches.

- Configure the switch hostnames and IP addresses on the management VLAN according to the diagram. By default, VLAN 1 is used as the management VLAN. Create a default gateway on both access layer switches using the ip default-gateway ip_address command.

The following is a sample configuration for switch ALS1.

Switch(config)# hostname ALS1

ALS1(config)# interface vlan 1

ALS1(config-if)# ip address 172.16.1.101 255.255.255.0

ALS1(config-if)# no shutdown

ALS1(config-if)# exit

ALS1(config)# ip default-gateway 172.16.1.1

The following is a sample configuration for switch ALS2.

Switch(config)# hostname ALS2

ALS2(config)# interface vlan 1

ALS2(config-if)# ip address 172.16.1.102 255.255.255.0

ALS2(config-if)# no shutdown

ALS2(config-if)# exit

ALS2(config)# ip default-gateway 172.16.1.1

- (Optional) Set an enable secret password and configure the vty lines for Telnet access to the switch.

ALS1(config)# enable secret cisco

ALS1(config)# line vty 0 15

ALS1(config-line)# password cisco

ALS1(config-line)# login

ALS1(config-line)# end

ALS2(config)# enable secret cisco

ALS2(config)# line vty 0 15

ALS2(config-line)# password cisco

ALS2(config-line)# login

ALS2(config-line)# end

- By default, how many lines are available for Telnet on the access switches?

Step 5: Confirm the VLANs.

- Verify that the only existing VLANs are the built-in VLANs. Issue the show vlan command from privileged mode on both access layer switches.

ALS1# show vlan

VLAN Name                             Status    Ports

---- -------------------------------- --------- -----------------------------

1    default                          active    Fa0/1, Fa0/2, Fa0/3, Fa0/4

Fa0/5, Fa0/6, Fa0/7, Fa0/8

Fa0/9, Fa0/10, Fa0/11, Fa0/12

Fa0/13, Fa0/14, Fa0/15, Fa0/16

Fa0/17, Fa0/18, Fa0/19, Fa0/20

Fa0/21, Fa0/22, Fa0/23, Fa0/24

Gi0/1, Gi0/2

1002 fddi-default                     act/unsup

1003 token-ring-default               act/unsup

1004 fddinet-default                  act/unsup

1005 trnet-default                    act/unsup

VLAN Type  SAID       MTU   Parent RingNo BridgeNo Stp  BrdgMode Trans1 Trans2

---- ----- ---------- ----- ------ ------ -------- ---- -------- ------ -----

1    enet  100001     1500  -      -      -        -    -        0      0

1002 fddi  101002     1500  -      -      -        -    -        0      0

1003 tr    101003     1500  -      -      -        -    -        0      0

1004 fdnet 101004     1500  -      -      -        ieee -        0      0

1005 trnet 101005     1500  -      -      -        ibm  -        0      0

Remote SPAN VLANs

-----------------------------------------------------------------------------

Primary Secondary Type              Ports

------- --------- ----------------- -----------------------------------------

Which VLAN is the default management VLAN for Ethernet? What types of traffic are carried on this VLAN?

Step 6: Configure trunk links and EtherChannel on switches.

- Use the Fast Ethernet 0/11 and 0/12 ports of ALS1 and ALS2 to create an EtherChannel trunk between the switches.

ALS1# configure terminal

Enter configuration commands, one per line.  End with CNTL/Z.

ALS1(config)# interface range fastEthernet 0/11 - 12

ALS1(config-if-range)# switchport mode trunk

ALS1(config-if-range)# channel-group 1 mode desirable

ALS1(config-if-range)# end

ALS2# configure terminal

Enter configuration commands, one per line.  End with CNTL/Z.

ALS2(config)# interface range fastEthernet 0/11 - 12

ALS2(config-if-range)# switchport mode trunk

ALS2(config-if-range)# channel-group 1 mode desirable

ALS2(config-if-range)# end

- Verify the EtherChannel configuration using the show etherchannel command.

ALS1# show etherchannel 1 summary

Flags:  D - down        P - in port-channel

I - stand-alone s - suspended

H - Hot-standby (LACP only)

R - Layer3      S - Layer2

U - in use      f - failed to allocate aggregator

u - unsuitable for bundling

w - waiting to be aggregated

d - default port

Number of channel-groups in use: 1

Number of aggregators:           1

Group  Port-channel  Protocol    Ports

------+-------------+-----------+--------------------------------------------

1      Po1(SU)         PAgP      Fa0/11(P)   Fa0/12(P)

Step 7: Configure VTP.

- Set up the VTP domain for the access layer switches in global configuration mode. The default VTP mode is server for both switches. Configure ALS2 as a VTP client, and leave ALS1 as a server. Configure the VTP domain name and version on VTP server ALS1.

ALS2(config)# vtp mode client

Setting device to VTP CLIENT mode.

ALS1(config)# vtp domain SWLAB

Changing VTP domain name from NULL to SWLAB

%SW_VLAN-6-VTP_DOMAIN_NAME_CHG: VTP domain name changed to SWLAB.

ALS1(config)# vtp version 2

- Use the show vtp status command to verify the ALS1 VTP configuration and that client ALS2 has learned the new VTP domain information from ALS1.

ALS1# show vtp status

VTP Version                     : running VTP2

Configuration Revision          : 1

Maximum VLANs supported locally : 255

Number of existing VLANs        : 5

VTP Operating Mode              : Server

VTP Domain Name                 : SWLAB

VTP Pruning Mode                : Disabled

VTP V2 Mode                     : Enabled

VTP Traps Generation            : Disabled

MD5 digest                      : 0x6A 0x1A 0x90 0xA3 0x10 0xCE 0x86 0xFA

Configuration last modified by 172.16.1.101 at 2-28-10 00:36:24

Local updater ID is 172.16.1.101 on interface Vl1 (lowest numbered VLAN interface found)

ALS2# show vtp status

VTP Version                     : running VTP2

Configuration Revision          : 1

Maximum VLANs supported locally : 255

Number of existing VLANs        : 5

VTP Operating Mode              : Client

VTP Domain Name                 : SWLAB

VTP Pruning Mode                : Disabled

VTP V2 Mode                     : Enabled

VTP Traps Generation            : Disabled

MD5 digest                      : 0x6A 0x1A 0x90 0xA3 0x10 0xCE 0x86 0xFA

Configuration last modified by 172.16.1.101 at 2-28-10 00:36:24

Step 8: Configure VLANs and switch access ports.

- Configure the VLAN 100 named Payroll and VLAN 200 named Engineering on VTP server ALS1.

ALS1(config)# vlan 100

ALS1(config-vlan)# name Payroll

ALS1(config-vlan)# vlan 200

ALS1(config-vlan)# name Engineering

- Use the show vlan brief command on ALS2 to verify that ALS2 has learned the new VLANs from ALS1.

ALS2# show vlan brief

VLAN Name                             Status    Ports

---- -------------------------------- --------- -----------------------------

1    default                          active    Fa0/1, Fa0/2, Fa0/3, Fa0/4

Fa0/5, Fa0/6, Fa0/7, Fa0/8

Fa0/9, Fa0/10, Fa0/13, Fa0/14

Fa0/15, Fa0/16, Fa0/17, Fa0/18

Fa0/19, Fa0/20, Fa0/21, Fa0/22

Fa0/23, Fa0/24, Gi0/1, Gi0/2

100  Payroll                          active

200  Engineering                      active

1002 fddi-default                     act/unsup

1003 trcrf-default                    act/unsup

1004 fddinet-default                  act/unsup

1005 trbrf-default                    act/unsup

- Configure the switch access ports for the hosts according to the diagram. Statically set the switch port mode to access, and use Spanning Tree PortFast on the interfaces. Assign the host attached to ALS1 Fast Ethernet 0/6 to VLAN 100, and the host attached to ALS2 Fast Ethernet 0/6 to VLAN 200.

ALS1(config)# interface fastEthernet 0/6

ALS1(config-if)# switchport mode access

ALS1(config-if)# switchport access vlan 100

ALS1(config-if)# spanning-tree portfast

%Warning: portfast should only be enabled on ports connected to a single

host. Connecting hubs, concentrators, switches, bridges, etc... to this

interface when portfast is enabled, can cause temporary bridging loops.

Use with CAUTION

%Portfast has been configured on FastEthernet0/6 but will only

have effect when the interface is in a non-trunking mode.

ALS2(config)# interface fastEthernet 0/6

ALS2(config-if)# switchport mode access

ALS2(config-if)# switchport access vlan 200

ALS2(config-if)# spanning-tree portfast

%Warning: portfast should only be enabled on ports connected to a single

host. Connecting hubs, concentrators, switches, bridges, etc... to this

interface when portfast is enabled, can cause temporary bridging loops.

Use with CAUTION

%Portfast has been configured on FastEthernet0/6 but will only

have effect when the interface is in a non-trunking mode.

- Use the show vlan brief command to verify that Fa0/6 is in VLAN 100 on ALS1 and in VLAN 200 on ALS2.

ALS1# show vlan brief

VLAN Name                             Status    Ports

---- -------------------------------- --------- -----------------------------

1    default                          active    Fa0/1, Fa0/2, Fa0/3, Fa0/4

Fa0/5, Fa0/7, Fa0/8, Fa0/9

Fa0/10, Fa0/13, Fa0/14, Fa0/15

Fa0/16, Fa0/17, Fa0/18, Fa0/19

Fa0/20, Fa0/21, Fa0/22, Fa0/23

Fa0/24, Gi0/1, Gi0/2

100  Payroll                          active    Fa0/6

200  Engineering                      active

1002 fddi-default                     act/unsup

1003 trcrf-default                    act/unsup

1004 fddinet-default                  act/unsup

1005 trbrf-default                    act/unsup

ALS2# show vlan brief

VLAN Name                             Status    Ports

---- -------------------------------- --------- -----------------------------

1    default                          active    Fa0/1, Fa0/2, Fa0/3, Fa0/4

Fa0/5, Fa0/7, Fa0/8, Fa0/9

Fa0/10, Fa0/13, Fa0/14, Fa0/15

Fa0/16, Fa0/17, Fa0/18, Fa0/19

Fa0/20, Fa0/21, Fa0/22, Fa0/23

Fa0/24, Gi0/1, Gi0/2

100  Payroll                          active

200  Engineering                      active    Fa0/6

1002 fddi-default                     act/unsup

1003 trcrf-default                    act/unsup

1004 fddinet-default                  act/unsup

1005 trbrf-default                    act/unsup

Step 9: Configure ALS1 trunking to the Gateway router.

- Configure switch ALS1 interface Fast Ethernet 0/1 for trunking with the Gateway router Fast Ethernet interface, according to the topology diagram.

ALS1(config)# interface fastEthernet 0/1

ALS1(config-if)# switchport mode trunk

ALS1(config-if)# end

Note: Optionally, you can apply the spanning-tree portfast trunk command to interface Fa0/1 of switch ALS1. This allows the link to the router to rapidly transition to the forwarding state despite being a trunk.

Step 10: Configure the Gateway router Fast Ethernet interface for VLAN trunking.

The native VLAN cannot be configured on a subinterface for Cisco IOS releases earlier than 12.1(3)T. The native VLAN IP address must be configured on the physical interface. Other VLAN traffic is configured on subinterfaces. Cisco IOS release 12.1(3)T and later support native VLAN configuration on a subinterface with the encapsulation dot1q native command. If a subinterface is configured using the encapsulation dot1q native command, the configuration on the physical interface is ignored. This technique is used in the lab configuration.

- Create a subinterface for each VLAN. Enable each subinterface with the proper trunking protocol, and configure it for a particular VLAN with the encapsulation command. Assign an IP address to each subinterface, which hosts on the VLAN can use as their default gateway.

The following is a sample configuration for the Fast Ethernet 0/0 interface.

Gateway(config)# interface fastEthernet 0/0

Gateway(config-if)# no shut

The following is a sample configuration for the VLAN 1 subinterface.

Gateway(config)# interface fastEthernet 0/0.1

Gateway(config-subif)# description Management VLAN 1

Gateway(config-subif)# encapsulation dot1q 1 native

Gateway(config-subif)# ip address 172.16.1.1 255.255.255.0

Note: For enhanced switch security, it is considered best practice to use independent unused VLANs for native and management VLANs.

The following is a sample configuration for the VLAN 100 subinterface.

Gateway(config-subif)# interface fastEthernet 0/0.100

Gateway(config-subif)# description Payroll VLAN 100

Gateway(config-subif)# encapsulation dot1q 100

Gateway(config-subif)# ip address 172.16.100.1 255.255.255.0

The following is a sample configuration for the VLAN 200 subinterface.

Gateway(config-subif)# interface fastEthernet 0/0.200

Gateway(config-subif)# description Engineering VLAN 200

Gateway(config-subif)# encapsulation dot1q 200

Gateway(config-subif)# ip address 172.16.200.1 255.255.255.0

Gateway(config-subif)# end

- Use the show ip interface brief command to verify the interface configuration and status.

Gateway# show ip interface brief

Interface            IP-Address      OK? Method Status                Protocol

FastEthernet0/0      unassigned      YES unset  up                    up

FastEthernet0/1.1    172.16.1.1      YES manual up                    up

FastEthernet0/1.100  172.16.100.1    YES manual up                    up

FastEthernet0/1.200  172.16.200.1    YES manual up                    up

FastEthernet0/1      unassigned      YES unset  administratively down down

Serial0/0/0          192.168.1.1     YES manual up                    up

Serial0/0/1          unassigned      YES unset  administratively down down

- Use the show interfaces description command to verify the interface status and description assigned.

Gateway# show interfaces description

Interface                      Status         Protocol Description

Fa0/0                          up             up

Fa0/0.1                        up             up       Management VLAN 1

Fa0/0.100                      up             up       Payroll VLAN 100

Fa0/0.200                      up             up       Engineering VLAN 200

Fa0/1                          admin down     down

Se0/0/0                        up             up

Se0/0/1                        admin down     down

- Use the show vlans command on the Gateway router.

Gateway# show vlans

Virtual LAN ID:  1 (IEEE 802.1Q Encapsulation)

vLAN Trunk Interface:   FastEthernet0/1.1

This is configured as native Vlan for the following interface(s) :

FastEthernet0/1

Protocols Configured:   Address:             Received:        Transmitted:

IP              172.16.1.1                 198                  54

Other                                           0                  29

277 packets, 91551 bytes input

83 packets, 15446 bytes output

Virtual LAN ID:  100 (IEEE 802.1Q Encapsulation)

vLAN Trunk Interface:   FastEthernet0/1.100

Protocols Configured:   Address:             Received:        Transmitted:

IP              172.16.100.1                 1                  25

0 packets, 0 bytes input

25 packets, 2350 bytes output

Virtual LAN ID:  200 (IEEE 802.1Q Encapsulation)

vLAN Trunk Interface:   FastEthernet0/1.200

Protocols Configured:   Address:             Received:        Transmitted:

IP              172.16.200.1                 1                  25

0 packets, 0 bytes input

25 packets, 2350 bytes output

- Use the show cdp neighbor detail command on the Gateway router to verify that ALS1 is a neighbor. Telnet to the IP address given in the CDP information.

Gateway# show cdp neighbor detail

-------------------------

Device ID: ISP

Entry address(es):

IP address: 192.168.1.2

Platform: Cisco 1841,  Capabilities: Router Switch IGMP

Interface: Serial0/0/0,  Port ID (outgoing port): Serial0/0/0

Holdtime : 174 sec

Version :

Cisco IOS Software, 1841 Software (C1841-ADVIPSERVICESK9-M), Version 12.4(24)T1,

RELEASE SOFTWARE (fc3)

Technical Support: http://www.cisco.com/techsupport

Copyright (c) 1986-2009 by Cisco Systems, Inc.

Compiled Fri 19-Jun-09 13:56 by prod_rel_team

advertisement version: 2

VTP Management Domain: ''

-------------------------

Device ID: ALS1

Entry address(es):

IP address: 172.16.1.101

Platform: cisco WS-C2960-24TT-L,  Capabilities: Switch IGMP

Interface: FastEthernet0/0.1,  Port ID (outgoing port): FastEthernet0/1

Holdtime : 118 sec

Version :

Cisco IOS Software, C2960 Software (C2960-LANBASEK9-M), Version 12.2(46)SE, RELE

ASE SOFTWARE (fc2)

Copyright (c) 1986-2008 by Cisco Systems, Inc.

Compiled Thu 21-Aug-08 15:59 by nachen

advertisement version: 2

Protocol Hello:  OUI=0x00000C, Protocol ID=0x0112; payload len=27, value=0000000

0FFFFFFFF010221FF000000000000001D46350C80FF0000

VTP Management Domain: 'SWLAB'

Native VLAN: 1

Duplex: full

Was the Telnet successful?

Step 11: Verify inter-VLAN routing on the Gateway router and the host devices.

- Ping to the 200.200.200.1 ISP loopback interface from either host. Was this ping successful?

- Ping from Host A to Host B. Was this ping successful?

- Telnet to the ALS2 VLAN 1 management IP address from the Engineering host. Was this Telnet successful?

If any of the tests failed, make the necessary corrections to the configurations for the router and switches.

Router Interface Summary Table

| Router Interface Summary |  |  |  |  |
| --- | --- | --- | --- | --- |
| Router Model | Ethernet Interface #1 | Ethernet Interface #2 | Serial Interface #1 | Serial Interface #2 |
| 1700 | Fast Ethernet 0 (FA0) | Fast Ethernet 1 (FA1) | Serial 0 (S0) | Serial 1 (S1) |
| 1800 | Fast Ethernet 0/0 (FA0/0) | Fast Ethernet 0/1 (FA0/1) | Serial 0/0/0 (S0/0/0) | Serial 0/0/1 (S0/0/1) |
| 2600 | Fast Ethernet 0/0 (FA0/0) | Fast Ethernet 0/1 (FA0/1) | Serial 0/0 (S0/0) | Serial 0/1 (S0/1) |
| 2800 | Fast Ethernet 0/0 (FA0/0) | Fast Ethernet 0/1 (FA0/1) | Serial 0/0/0 (S0/0/0) | Serial 0/0/1 (S0/0/1) |
| Note: To find out how the router is configured, look at the interfaces to identify the type of router and how many interfaces the router has. Rather than list all combinations of configurations for each router class, this table includes identifiers for the possible combinations of Ethernet and serial interfaces in the device. The table does not include any other type of interface, even though a specific router might contain one. For example, for an ISDN BRI interface, the string in parenthesis is the legal abbreviation that can be used in Cisco IOS commands to represent the interface. |  |  |  |  |