CCNPv6 SWITCH

Chapter 5 Lab 5-2, IP Service Level Agreements in a Campus Environment

Topology

Objectives

- Configure trunking, VTP, and SVIs.

- Implement IP SLAs to monitor various network performance characteristics.

- Background

Cisco IOS IP service level agreements (SLAs) allow users to monitor network performance between Cisco devices (switches or routers) or from a Cisco device to a remote IP device. Cisco IOS IP SLAs can be applied to VoIP and video applications as well as monitoring end-to-end IP network performance.

In this lab, you configure trunking, VTP, and SVIs. You configure IP SLA monitors to test ICMP echo network performance between DLS1 and each host. You also configure IP SLA monitors to test jitter between DLS1 and the access layer switches ALS1 and ALS2.

Note: This lab uses Cisco WS-C2960-24TT-L switches with the Cisco IOS image c2960-lanbasek9-mz.122-46.SE.bin, and Catalyst 3560-24PS with the Cisco IOS image c3560-advipservicesk9-mz.122-46.SE.bin. You can use other switches (such as 2950 or 3550) and Cisco IOS Software versions if they have comparable capabilities and features. Depending on the switch model and Cisco IOS Software version, the commands available and output produced might vary from what is shown in this lab.

Required Resources

- 2 switches (Cisco 2960 with the Cisco IOS Release 12.2(46)SE C2960-LANBASEK9-M image or comparable)

- 1 switch (Cisco 3560 with the Cisco IOS Release 12.2(46)SE C3560-advipservicesk9-mz image or comparable)

- Ethernet and console cables

Step 1: Prepare the switches for the lab.

Erase the startup configuration, delete the vlan.dat file, and reload the switches. Refer to Lab 1-1 “Clearing a Switch” and Lab 1-2 “Clearing a Switch Connected to a Larger Network” to prepare the switches for this lab. Cable the equipment as shown.

Step 2: Configure host PCs.

Configure PCs Host A and Host B with the IP address and subnet mask shown in the topology. Host A is in VLAN 100 with a default gateway of 172.16.100.1. Host B is in VLAN 200 with a default gateway of 172.16.200.1.

Step 3: Configure basic switch parameters.

Configure the hostname, password, and, optionally, remote access on each switch.

Switch(config)# hostname ALS1

ALS1(config)# enable secret cisco

ALS1(config)# line vty 0 15

ALS1(config-line)# password cisco

ALS1(config-line)# login

Switch(config)# hostname ALS2

ALS2(config)# enable secret cisco

ALS2(config)# line vty 0 15

ALS2(config-line)# password cisco

ALS2(config-line)# login

Switch(config)# hostname DLS1

DLS1(config)# enable secret cisco

DLS1(config)# line vty 0 15

DLS1(config-line)#password cisco

DLS1(config-line)# login

Configure a management IP address on VLAN 1 for each of the three switches according to the diagram.

ALS1(config)# interface vlan 1

ALS1(config-if)# ip address 172.16.1.101 255.255.255.0

ALS1(config-if)# no shutdown

ALS2(config)# interface vlan 1

ALS2(config-if)# ip address 172.16.1.102 255.255.255.0

ALS2(config-if)# no shutdown

DLS1(config)# interface vlan 1

DLS1(config-if)# ip address 172.16.1.1 255.255.255.0

DLS1(config-if)# no shutdown

Configure default gateways on the access layer switches. The distribution layer switch will not use a default gateway because it acts as a Layer 3 device. The access layer switches act as Layer 2 devices and need a default gateway to send traffic off of the local subnet for the management VLAN.

ALS1(config)# ip default-gateway 172.16.1.1

ALS2(config)# ip default-gateway 172.16.1.1

Step 4: Configure trunks and EtherChannels between switches.

To distribute VLAN and VTP information, trunks are needed between the three switches. Configure these trunks according to the diagram. EtherChannel is used for these trunks.

Note: It is good practice to shut down the interfaces on both sides of the link before a port channel is created and then reenable them after the port channel is configured.

Configure the trunks and EtherChannel from DLS1 to ALS1.

DLS1(config)# interface range fastEthernet 0/7 - 8

DLS1(config-if-range)# switchport trunk encapsulation dot1q

DLS1(config-if-range)# switchport mode trunk

DLS1(config-if-range)# channel-group 1 mode desirable

Creating a port-channel interface Port-channel 1

Configure the trunks and EtherChannel from DLS1 to ALS2.

DLS1(config)# interface range fastEthernet 0/9 - 10

DLS1(config-if-range)# switchport trunk encapsulation dot1q

DLS1(config-if-range)# switchport mode trunk

DLS1(config-if-range)# channel-group 2 mode desirable

Creating a port-channel interface Port-channel 2

Configure the trunks and EtherChannel between ALS1 and DLS1 and between ALS1 and ALS2.

ALS1(config)# interface range fastEthernet 0/11 - 12

ALS1(config-if-range)# switchport mode trunk

ALS1(config-if-range)# channel-group 1 mode desirable

Creating a port-channel interface Port-channel 1

ALS1(config-if-range)# exit

ALS1(config)# interface range fastEthernet 0/7 - 8

ALS1(config-if-range)# switchport mode trunk

ALS1(config-if-range)# channel-group 2 mode desirable

Creating a port-channel interface Port-channel 2

Configure the trunks and EtherChannel between ALS2 and DLS1 and between ALS2 and ALS1.

ALS2(config)# interface range fastEthernet 0/11 - 12

ALS2(config-if-range)# switchport mode trunk

ALS2(config-if-range)# channel-group 1 mode desirable

Creating a port-channel interface Port-channel 1

ALS2(config-if-range)# exit

ALS2(config)# interface range fastEthernet 0/9 - 10

ALS2(config-if-range)# switchport mode trunk

ALS2(config-if-range)# channel-group 2 mode desirable

Creating a port-channel interface Port-channel 2

Step 5: Configure VTP on ALS1 and ALS2.

Change the VTP mode of ALS1 and ALS2 to client.

ALS1(config)# vtp mode client

Setting device to VTP CLIENT mode.

ALS2(config)# vtp mode client

Setting device to VTP CLIENT mode.

Step 6: Configure VTP on DLS1.

Create the VTP domain on DLS1, and create VLANs 100 and 200 for the domain.

DLS1(config)# vtp domain SWPOD

DLS1(config)# vtp version 2

DLS1(config)# vlan 100

DLS1(config-vlan)# name Finance

DLS1(config-vlan)# vlan 200

DLS1(config-vlan)# name Engineering

Step 7: Configure access ports.

Configure the host ports for the appropriate VLANs according to the diagram.

ALS1(config)# interface fastEthernet 0/6

ALS1(config-if)# switchport mode access

ALS1(config-if)# switchport access vlan 100

ALS2(config)# interface fastEthernet 0/6

ALS2(config-if)# switchport mode access

ALS2(config-if)# switchport access vlan 200

Step 8: Configure VLAN interfaces and enable routing.

On DLS1, create the SVIs for VLANs 100 and 200. Note that the corresponding Layer 2 VLANs must be configured for the Layer 3 SVIs to activate. This was done in Step 6.

DLS1(config)# interface vlan 100

DLS1(config-if)# ip address 172.16.100.1 255.255.255.0

DLS1(config-if)# interface vlan 200

DLS1(config-if)# ip address 172.16.200.1 255.255.255.0

The ip routing command is also needed to allow the DLS1 switch to act as a Layer 3 device to route between these VLANs. Because the VLANs are all considered directly connected, a routing protocol is not needed at this time. The default configuration on 3560 switches is no ip routing.

DLS1(config)# ip routing

Verify the configuration using the show ip route command on DLS1.

DLS1# show ip route

Codes: C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2, E - EGP

i - IS-IS, su - IS-IS summary, L1 - IS-IS level-1, L2 - IS-IS level-2

ia - IS-IS inter area, * - candidate default, U - per-user static route

o - ODR, P - periodic downloaded static route

Gateway of last resort is not set

172.16.0.0/24 is subnetted, 3 subnets

C       172.16.200.0 is directly connected, Vlan200

C       172.16.1.0 is directly connected, Vlan1

C       172.16.100.0 is directly connected, Vlan100

Run the following Tcl script on DLS1 to verify full connectivity. If these pings are not successful, troubleshoot.

Note: Tcl is only supported on DLS1.

DLS1# tclsh

foreach address {

172.16.1.1

172.16.1.101

172.16.1.102

172.16.100.1

172.16.200.1

172.16.100.101

172.16.200.101

} {

ping $address }

Step 9: Configure Cisco IOS IP SLA responders.

IP SLA responders are Cisco IOS devices that support the IP SLA control protocol. An IP SLA responder uses the Cisco IOS IP SLA Control Protocol for notification configuration and on which port to listen and respond. Some operations, such as ICMP echo, do not require a dedicated IP SLA responder.

Use the ip sla responder command on ALS1 and ALS2 to enable sending and receiving IP SLAs control packets.

Note: This command replaces the ip sla monitor responder command. All commands that used to begin with “ip sla monitor” now begin with “ip sla” (without “monitor”).

ALS1(config)# ip sla responder

ALS2(config)# ip sla responder

Configure ALS1 and ALS2 as IP SLA responders for UDP jitter using the ip sla responder udp-echo ipaddress command. Specify the IP address of DLS1 VLAN 1 to act as the destination IP address for the reflected UDP traffic on both ALS1 and ALS2.

ALS1(config)# ip sla responder udp-echo ipaddress 172.16.1.1 port 5000

ALS2(config)# ip sla responder udp-echo ipaddress 172.16.1.1 port 5000

Step 10: Configure the Cisco IOS IP SLA source to measure network performance.

IP SLA uses generated traffic to measure network performance between two networking devices.

On DLS1, create an IP SLA operation and enter IP SLA configuration mode with the ip sla operation-number command.

DLS1(config)# ip sla 1

DLS1(config-ip-sla)#

Configure an IP SLA ICMP echo operation using the icmp-echo command in IP SLA configuration mode. The IP SLA ICMP echo operation does not require a dedicated Cisco IOS IP SLA responder (the destination device can be a non-Cisco device, such as a PC). By default, the ICMP operation repeats  every 60 seconds. On DLS1, for ICMP echo operation 1, specify the IP address of Host A as the target. For ICMP echo operation 2, specify the IP address of Host B as the target.

DLS1(config-ip-sla)# icmp-echo 172.16.100.101

DLS1(config-ip-sla-echo)# exit

DLS1(config)# ip sla 2

DLS1(config-ip-sla)# icmp-echo 172.16.200.101

DLS1(config-ip-sla-echo)# exit

Jitter means inter-packet delay variance. UDP-based voice traffic associated with IP phone and PC softphone applications at the access layer require strict adherence to delay and jitter thresholds. To configure an IP SLA UDP jitter operation, use the udp-jitter command in IP SLA configuration mode. By default, the UDP jitter operation repeats every 60 seconds. For UDP jitter operation 3, specify the destination IP address of the ALS1 VLAN 1 interface as the target. For operation 4, specify the destination IP address of the ALS2 VLAN 1 interface as the target. The IP SLA communication port is 5000 for both operations.

DLS1(config)# ip sla 3

DLS1(config-ip-sla)# udp-jitter 172.16.1.101 5000

DLS1(config-ip-sla-jitter)# exit

DLS1(config)# ip sla 4

DLS1(config-ip-sla)# udp-jitter 172.16.1.102 5000

DLS1(config-ip-sla-jitter)# exit

Schedule the IP SLAs operations to run indefinitely beginning immediately using the ip sla schedule global configuration mode command.

DLS1(config)# ip sla schedule 1 life forever start-time now

DLS1(config)# ip sla schedule 2 life forever start-time now

DLS1(config)# ip sla schedule 3 life forever start-time now

DLS1(config)# ip sla schedule 4 life forever start-time now

Step 11: Monitor IP SLAs operations.

View the IP SLA configuration for IP SLA 1 on DLS1. The output for IP SLA 2 is similar.

DLS1# show ip sla configuration 1

IP SLAs, Infrastructure Engine-II.

Entry number: 1

Owner:

Tag:

Type of operation to perform: echo

Target address/Source address: 172.16.100.101/0.0.0.0

Type Of Service parameter: 0x0

Request size (ARR data portion): 28

Operation timeout (milliseconds): 5000

Verify data: No

Vrf Name:

Schedule:

Operation frequency (seconds): 60

Next Scheduled Start Time: Start Time already passed

Group Scheduled : FALSE

Randomly Scheduled : FALSE

Life (seconds): Forever

Entry Ageout (seconds): never

Recurring (Starting Everyday): FALSE

Status of entry (SNMP RowStatus): Active

Threshold (milliseconds): 5000

Distribution Statistics:

Number of statistic hours kept: 2

Number of statistic distribution buckets kept: 1

Statistic distribution interval (milliseconds): 20

History Statistics:

Number of history Lives kept: 0

Number of history Buckets kept: 15

History Filter Type: None

Enhanced History:

What type of operation is being performed with IP SLA 1?

View the IP SLA configuration for IP SLA 3 on DLS1. The output for IP SLA 4 is similar.

DLS1# show ip sla configuration 3

IP SLAs, Infrastructure Engine-II.

Entry number: 3

Owner:

Tag:

Type of operation to perform: udp-jitter

Target address/Source address: 172.16.1.101/0.0.0.0

Target port/Source port: 5000/0

Type Of Service parameter: 0x0

Request size (ARR data portion): 32

Operation timeout (milliseconds): 5000

Packet Interval (milliseconds)/Number of packets: 20/10

Verify data: No

Vrf Name:

Control Packets: enabled

Schedule:

Operation frequency (seconds): 60

Next Scheduled Start Time: Start Time already passed

Group Scheduled : FALSE

Randomly Scheduled : FALSE

Life (seconds): Forever

Entry Ageout (seconds): never

Recurring (Starting Everyday): FALSE

Status of entry (SNMP RowStatus): Active

Threshold (milliseconds): 5000

Distribution Statistics:

Number of statistic hours kept: 2

Number of statistic distribution buckets kept: 1

Statistic distribution interval (milliseconds): 20

Enhanced History:

What type of operation is being performed with IP SLA 3?

Display global information about Cisco IOS IP SLAs on DLS1.

DLS1# show ip sla application

Version: 2.2.0 Round Trip Time MIB, Infrastructure Engine-II

Time of last change in whole IP SLAs: *13:16:30.493 UTC Fri Mar 5 2010

Estimated system max number of entries: 11928

Estimated number of configurable operations: 11924

Number of Entries configured  : 4

Number of active Entries      : 4

Number of pending Entries     : 0

Number of inactive Entries    : 0

Type of Operation to Perform: dhcp

Type of Operation to Perform: dns

Type of Operation to Perform: echo

Type of Operation to Perform: ftp

Type of Operation to Perform: http

Type of Operation to Perform: jitter

Type of Operation to Perform: pathEcho

Type of Operation to Perform: pathJitter

Type of Operation to Perform: tcpConnect

Type of Operation to Perform: udpEcho

IP SLAs low memory water mark: 16273927

Display information about Cisco IOS IP SLA responders on ALS1. The ALS2 output is similar.

ALS1# show ip sla responder

IP SLAs Responder is: Enabled

Number of control message received: 38 Number of errors: 0

Recent sources:

Recent error sources:

udpEcho Responder:

IPv6/IP Address        Port

172.16.1.1             5000

Display IP SLA statistics on DLS1 for IP SLA 1. The IP SLA 2 output is similar.

DLS1# show ip sla statistics 1

Round Trip Time (RTT) for       Index 1

Latest RTT: 1 ms

Latest operation start time: *13:17:21.231 UTC Fri Mar 5 2010

Latest operation return code: OK

Number of successes: 15

Number of failures: 1

Operation time to live: Forever

From this output, you can see that the latest round-trip time (RTT) for SLA operation Index 1 (icmp-echo) is 1 millisecond (ms). The number of packets sent successfully from DLS1 to PC Host A was 15, and there was one failure.

Display IP SLA statistics on DLS1 for IP SLA 3. The IP SLA 4 output is similar.

DLS1# show ip sla statistics 3

Round Trip Time (RTT) for       Index 3

Latest RTT: 3 ms

Latest operation start time: *13:19:45.322 UTC Fri Mar 5 2010

Latest operation return code: OK

RTT Values

Number Of RTT: 10

RTT Min/Avg/Max: 2/3/5 ms

Latency one-way time milliseconds

Number of Latency one-way Samples: 0

Source to Destination Latency one way Min/Avg/Max: 0/0/0 ms

Destination to Source Latency one way Min/Avg/Max: 0/0/0 ms

Jitter time milliseconds

Number of SD Jitter Samples: 9

Number of DS Jitter Samples: 9

Source to Destination Jitter Min/Avg/Max: 0/1/2 ms

Destination to Source Jitter Min/Avg/Max: 0/1/1 ms

Packet Loss Values

Loss Source to Destination: 0           Loss Destination to Source: 0

Out Of Sequence: 0      Tail Drop: 0    Packet Late Arrival: 0

Voice Score Values

Calculated Planning Impairment Factor (ICPIF): 0

Mean Opinion Score (MOS): 0

Number of successes: 14

Number of failures: 0

Operation time to live: Forever

From this output, you can see that the latest RTT for SLA operation Index 3 (udp-jitter) is 3 ms. Jitter time from source to destination and from destination to source is averaging 1 ms, which is acceptable for voice applications. The number of packets sent successfully from DLS1 to ALS1 was 14, and there were no failures.

Disable interface VLAN 1 on ALS1 using the shutdown command.

ALS1(config)# interface vlan 1

ALS1(config-if)# shutdown

Allow a few minutes to pass and then issue the show ip sla statistics 3 command on DLS1. The output should look similar to the following.

DLS1# show ip sla statistics 3

Round Trip Time (RTT) for       Index 3

Latest RTT: NoConnection/Busy/Timeout

Latest operation start time: *13:19:45.322 UTC Fri Mar 5 2010

Latest operation return code: Timeout

RTT Values

Number Of RTT: 0

RTT Min/Avg/Max: 0/0/0 ms

Latency one-way time milliseconds

Number of Latency one-way Samples: 0

Source to Destination Latency one way Min/Avg/Max: 0/0/0 ms

Destination to Source Latency one way Min/Avg/Max: 0/0/0 ms

Jitter time milliseconds

Number of SD Jitter Samples: 0

Number of DS Jitter Samples: 0

Source to Destination Jitter Min/Avg/Max: 0/0/0 ms

Destination to Source Jitter Min/Avg/Max: 0/0/0 ms

Packet Loss Values

Loss Source to Destination: 0           Loss Destination to Source: 0

Out Of Sequence: 0      Tail Drop: 0    Packet Late Arrival: 0

Voice Score Values

Calculated Planning Impairment Factor (ICPIF): 0

Mean Opinion Score (MOS): 0

Number of successes: 14

Number of failures: 2

Operation time to live: Forever

If there is a connectivity problem between IP SLA source DLS1 and responder ALS1 or ALS2, the communication to the responder will be lost and statistics will cease to be collected, except for the number of failed tests.

Note: The IP SLA itself is an additional task that must be performed by the switch CPU. A large number of intensive SLAs could create a significant burden on the CPU, possibly interfering with other switch functions and having detrimental impact on the overall device performance. Therefore, you should carefully evaluate the benefits of running IP SLAs. The CPU load should be monitored after the SLAs are deployed to verify that they do not stress the device’s CPU above safe limits.