CCNPv6 SWITCH

Chapter 4 Lab 4-2, Inter-VLAN Routing with an Internal Route Processor and Monitoring CEF Functions

Topology

Objective

- Route between VLANs using a 3560 switch with an internal route processor using Cisco Express Forwarding (CEF).

- Background

The current network equipment includes a 3560 distribution layer switch and two 2960 access layer switches. The network is segmented into three functional subnets using VLANs for better network management. The VLANs include Finance, Engineering, and a subnet for equipment management, which is the default management VLAN, VLAN 1. After VTP and trunking have been configured for the switches, switched virtual interfaces (SVI) are configured on the distribution layer switch to route between these VLANs, providing full connectivity to the internal network.

Note: This lab uses Cisco WS-C2960-24TT-L switches with the Cisco IOS image c2960-lanbasek9-mz.122-46.SE.bin and Catalyst 3560-24PS with the Cisco IOS image c3560-advipservicesk9-mz.122-46.SE.bin. You can use other switches (such as 2950 or 3550) and Cisco IOS Software versions if they have comparable capabilities and features. Depending on the switch model and Cisco IOS Software version, the commands available and output produced might vary from what is shown in this lab.

Required Resources

- 2 switches (Cisco 2960 with the Cisco IOS Release 12.2(46)SE C2960-LANBASEK9-M image or comparable)

- 1 switch (Cisco 3560 with the Cisco IOS Release 12.2(46)SE C3560-advipservicesk9-mz image or comparable)

- Ethernet and console cables

Step 1: Prepare the switches for the lab.

Erase the startup configuration, delete the vlan.dat file, and reload the switches. Refer to Lab 1-1, “Clearing a Switch” and Lab 1-2, “Clearing a Switch Connected to a Larger Network” to prepare the switches for this lab. Cable the equipment as shown.

Step 2: Configure basic switch parameters.

- Configure the hostname, password, and optionally, Telnet access on each switch.

Switch(config)# hostname ALS1

ALS1(config)# enable secret cisco

ALS1(config)# line vty 0 15

ALS1(config-line)# password cisco

ALS1(config-line)# login

Switch(config)# hostname ALS2

ALS2(config)# enable secret cisco

ALS2(config)# line vty 0 15

ALS2(config-line)# password cisco

ALS2(config-line)# login

Switch(config)# hostname DLS1

DLS1(config)# enable secret cisco

DLS1(config)# line vty 0 15

DLS1(config-line)#password cisco

DLS1(config-line)# login

- Configure management IP addresses on VLAN 1 for all three switches according to the diagram.

ALS1(config)# interface vlan 1

ALS1(config-if)# ip address 172.16.1.101 255.255.255.0

ALS1(config-if)# no shutdown

ALS2(config)# interface vlan 1

ALS2(config-if)# ip address 172.16.1.102 255.255.255.0

ALS2(config-if)# no shutdown

DLS1(config)# interface vlan 1

DLS1(config-if)# ip address 172.16.1.1 255.255.255.0

DLS1(config-if)# no shutdown

- Configure default gateways on the access layer switches. The distribution layer switch will not use a default gateway, because it acts as a Layer 3 device. The access layer switches act as Layer 2 devices and need a default gateway to send management VLAN traffic off of the local subnet.

ALS1(config)# ip default-gateway 172.16.1.1

ALS2(config)# ip default-gateway 172.16.1.1

Step 3: Configure trunks and EtherChannels between switches.

To distribute VLAN and VTP information between the switches, trunks are needed between the three switches. Configure these trunks according to the diagram. EtherChannel is used for these trunks. EtherChannel allows you to utilize both Fast Ethernet interfaces that are available between each device, thereby doubling the bandwidth.

- Configure the trunks and EtherChannel from DLS1 to ALS1. The switchport trunk encapsulation [isl | dot1q] command is used because this switch also supports ISL encapsulation.

DLS1(config)# interface range fastEthernet 0/7 - 8

DLS1(config-if-range)# switchport trunk encapsulation dot1q

DLS1(config-if-range)# switchport mode trunk

DLS1(config-if-range)# channel-group 1 mode desirable

Creating a port-channel interface Port-channel 1

- Configure the trunks and EtherChannel from DLS1 to ALS2.

DLS1(config)# interface range fastEthernet 0/9 - 10

DLS1(config-if-range)# switchport trunk encapsulation dot1q

DLS1(config-if-range)# switchport mode trunk

DLS1(config-if-range)# channel-group 2 mode desirable

Creating a port-channel interface Port-channel 2

- Configure the trunks and EtherChannel between ALS1 and DLS1, and for the trunks and EtherChannel between ALS1 and ALS2.

ALS1(config)# interface range fastEthernet 0/11 - 12

ALS1(config-if-range)# switchport mode trunk

ALS1(config-if-range)# channel-group 1 mode desirable

Creating a port-channel interface Port-channel 1

ALS1(config-if-range)# exit

ALS1(config)# interface range fastEthernet 0/7 - 8

ALS1(config-if-range)# switchport mode trunk

ALS1(config-if-range)# channel-group 2 mode desirable

Creating a port-channel interface Port-channel 2

- Configure the trunks and EtherChannel between ALS2 and DLS1, and for the trunks and EtherChannel between ALS2 and ALS1.

ALS2(config)# interface range fastEthernet 0/11 - 12

ALS2(config-if-range)# switchport mode trunk

ALS2(config-if-range)# channel-group 1 mode desirable

Creating a port-channel interface Port-channel 1

ALS2(config-if-range)# exit

ALS2(config)# interface range fastEthernet 0/9 - 10

ALS2(config-if-range)# switchport mode trunk

ALS2(config-if-range)# channel-group 2 mode desirable

Creating a port-channel interface Port-channel 2

- Verify trunking between DLS1, ALS1, and ALS2 using the show interface trunk command on all switches.

DLS1# show interface trunk

Port        Mode         Encapsulation  Status        Native vlan

Po1         on           802.1q         trunking      1

Po2         on           802.1q         trunking      1

Port      Vlans allowed on trunk

Po1         1-4094

Po2         1-4094

Port        Vlans allowed and active in management domain

Po1         1

Po2         1

Port        Vlans in spanning tree forwarding state and not pruned

Po1         1

Po2         1

- Use the show etherchannel summary command on each switch to verify the EtherChannels.

The following is sample output from ALS1. Notice the two EtherChannels on the access layer switches.

ALS1# show etherchannel summary

Flags:  D - down        P - in port-channel

I - stand-alone s - suspended

H - Hot-standby (LACP only)

R - Layer3      S - Layer2

U - in use      f - failed to allocate aggregator

u - unsuitable for bundling

w - waiting to be aggregated

d - default port

Number of channel-groups in use: 2

Number of aggregators:           2

Group  Port-channel  Protocol    Ports

------+-------------+-----------+---------------------------------------------

1      Po1(SU)         PAgP      Fa0/11(P)   Fa0/12(P)

2      Po2(SU)         PAgP      Fa0/7(P)    Fa0/8(P)

On ALS1, which ports are used for channel group 2?

Step 4: Configure VTP on ALS1 and ALS2.

- Change the VTP mode of ALS1 and ALS2 to client.

ALS1(config)# vtp mode client

Setting device to VTP CLIENT mode.

ALS2(config)# vtp mode client

Setting device to VTP CLIENT mode.

- Verify the VTP changes with the show vtp status command. The output on ALS2 is similar to that of ALS1.

ALS2# show vtp status

VTP Version                     : running VTP1 (VTP2 capable)

Configuration Revision          : 0

Maximum VLANs supported locally : 255

Number of existing VLANs        : 5

VTP Operating Mode              : Client

VTP Domain Name                 :

VTP Pruning Mode                : Disabled

VTP V2 Mode                     : Disabled

VTP Traps Generation            : Disabled

MD5 digest                      : 0xC8 0xAB 0x3C 0x3B 0xAB 0xDD 0x34 0xCF

Configuration last modified by 0.0.0.0 at 3-1-93 15:47:34

How many VLANs can be supported locally on the 2960 switch?

Step 5: Configure VTP on DLS1.

- Create the VTP domain on DLS1 and create VLANs 100 and 200 for the domain.

DLS1(config)# vtp domain SWPOD

DLS1(config)# vtp version 2

DLS1(config)# vlan 100

DLS1(config-vlan)# name Finance

DLS1(config-vlan)# vlan 200

DLS1(config-vlan)# name Engineering

- Verify VTP information throughout the domain using the show vlan and show vtp status commands.

How many existing VLANs are in the VTP domain?

Step 6: Configure ports.

- Configure the host ports for the appropriate VLANs according to the diagram.

ALS1(config)# interface fastEthernet 0/6

ALS1(config-if)# switchport mode access

ALS1(config-if)# switchport access vlan 100

ALS2(config)# interface fastEthernet 0/6

ALS2(config-if)# switchport mode access

ALS2(config-if)# switchport access vlan 200

Ping from the host on VLAN 100 to the host on VLAN 200. Was the ping successful? Why do you think this is the case?

Ping from a host to the VLAN 1 management IP address of DLS1. Was the ping successful? Why do you think this is the case?

Step 7: Configure VLAN interfaces and enable routing.

- On DLS1, create the Layer 3 VLAN interfaces to route between VLANs using the interface vlan vlan-id command. These are known as SVIs. You do not need to set up VLAN 1, because this was done in Step 2.

DLS1(config)# interface vlan 100

DLS1(config-if)# ip add 172.16.100.1 255.255.255.0

DLS1(config-if)# no shut

DLS1(config-if)# interface vlan 200

DLS1(config-if)# ip address 172.16.200.1 255.255.255.0

DLS1(config-if)# no shutdown

- The ip routing command is also needed to allow the switch to act as a Layer 3 device to route between these VLANs. Because the VLANs are all considered directly connected, a routing protocol is not needed at this time.

DLS1(config)# ip routing

- Verify the configuration using the show ip route command on DLS1.

DLS1# show ip route

Codes: C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2, E - EGP

i - IS-IS, su - IS-IS summary, L1 - IS-IS level-1, L2 - IS-IS level-2

ia - IS-IS inter area, * - candidate default, U - per-user static route

o - ODR, P - periodic downloaded static route

Gateway of last resort is not set

172.16.0.0/24 is subnetted, 3 subnets

C       172.16.200.0 is directly connected, Vlan200

C       172.16.1.0 is directly connected, Vlan1

C       172.16.100.0 is directly connected, Vlan100

Step 8: Verify inter-VLAN routing by the internal route processor.

- Ping from the Engineering host to the Finance host. Was the ping successful this time?

- Telnet from one of the hosts to the VLAN 1 IP address of DLS1. Can this switch be remotely accessed from this host?

Step 9: Examine the CEF configuration.

CEF implements an advanced IP lookup and forwarding algorithm to deliver maximum Layer 3 switching performance. CEF is less CPU-intensive than route caching.

In dynamic networks, fast-switching cache entries are frequently invalidated because of routing changes. This can cause traffic to be process-switched using the routing table, instead of fast-switched using the route cache. CEF uses the Forwarding Information Base (FIB) lookup table to perform destination-based switching of IP packets.

CEF is enabled by default on the 3560 switch.

- Use the show ip cef command to display the CEF FIB.

DLS1# show ip cef

Prefix               Next Hop             Interface

0.0.0.0/32           receive

172.16.1.0/24        attached             Vlan1

172.16.1.0/32        receive

172.16.1.1/32        receive

172.16.1.101/32      attached             Vlan1

172.16.1.102/32      attached             Vlan1

172.16.1.255/32      receive

172.16.100.0/24      attached             Vlan100

172.16.100.0/32      receive

172.16.100.1/32      receive

172.16.100.255/32    receive

172.16.200.0/24      attached             Vlan200

172.16.200.0/32      receive

172.16.200.1/32      receive

172.16.200.255/32    receive

224.0.0.0/4          drop

224.0.0.0/24         receive

255.255.255.255/32   receive

- Use the show ip interface command to verify that CEF is enabled on an interface. The following output shows that CEF is enabled on VLAN 100.

DLS1# show ip interface vlan 100

Vlan100 is up, line protocol is up

Internet address is 172.16.100.1/24

Broadcast address is 255.255.255.255

Address determined by setup command

MTU is 1500 bytes

Helper address is not set

Directed broadcast forwarding is disabled

Outgoing access list is not set

Inbound  access list is not set

Proxy ARP is enabled

Local Proxy ARP is disabled

Security level is default

Split horizon is enabled

ICMP redirects are always sent

ICMP unreachables are always sent

ICMP mask replies are never sent

IP fast switching is enabled

IP CEF switching is enabled

IP CEF switching turbo vector

IP multicast fast switching is disabled

IP multicast distributed fast switching is disabled

IP route-cache flags are Fast, CEF

Router Discovery is disabled

IP output packet accounting is disabled

IP access violation accounting is disabled

TCP/IP header compression is disabled

RTP/IP header compression is disabled

Probe proxy name replies are disabled

Policy routing is disabled

Network address translation is disabled

WCCP Redirect outbound is disabled

WCCP Redirect inbound is disabled

WCCP Redirect exclude is disabled

BGP Policy Mapping is disabled

- Use the show ip cef summary command to display the CEF table summary.

DLS1# show ip cef summary

IPv4 CEF is enabled for distributed and running

VRF Default:

18 prefixes (18/0 fwd/non-fwd)

Table id 0, 0 resets

Database epoch: 1 (18 entries at this epoch)

- The show ip cef detail command shows CEF operation in detail for the switch.

DLS1# show ip cef detail

IPv4 CEF is enabled for distributed and running

VRF Default:

18 prefixes (18/0 fwd/non-fwd)

Table id 0, 0 resets

Database epoch: 1 (18 entries at this epoch)

0.0.0.0/32, epoch 1, flags receive

Special source: receive

receive

172.16.1.0/24, epoch 1, flags attached, connected

attached to Vlan1

172.16.1.0/32, epoch 1, flags receive

receive

172.16.1.1/32, epoch 1, flags receive

receive

172.16.1.101/32, epoch 1

Adj source: IP adj out of Vlan1, addr 172.16.1.101

attached to Vlan1

172.16.1.102/32, epoch 1

Adj source: IP adj out of Vlan1, addr 172.16.1.102

attached to Vlan1

172.16.1.255/32, epoch 1, flags receive

receive

172.16.100.0/24, epoch 1, flags attached, connected

attached to Vlan100

172.16.100.0/32, epoch 1, flags receive

receive

172.16.100.1/32, epoch 1, flags receive

receive

172.16.100.255/32, epoch 1, flags receive

receive

172.16.200.0/24, epoch 1, flags attached, connected

attached to Vlan200

172.16.200.0/32, epoch 1, flags receive

receive

172.16.200.1/32, epoch 1, flags receive

receive

172.16.200.255/32, epoch 1, flags receive

receive

224.0.0.0/4, epoch 1

Special source: drop

drop

224.0.0.0/24, epoch 1, flags receive

Special source: receive

receive

255.255.255.255/32, epoch 1, flags receive

Special source: receive

Receive