Lab – Configuring and Verifying IPv6 ACLs

Topology

Addressing Table

| Device | Interface | IP Address | Default Gateway |
| --- | --- | --- | --- |
| R1 | G0/0 | 2001:DB8:ACAD:B::1/64 | N/A |
|  | G0/1 | 2001:DB8:ACAD:A::1/64 | N/A |
|  | S0/0/0 (DCE) | 2001:DB8:AAAA:1::1/64 | N/A |
| R2 | S0/0/0 | 2001:DB8:AAAA:1::2/64 | N/A |
|  | S0/0/1 (DCE) | 2001:DB8:AAAA:2::2/64 | N/A |
| R3 | G0/1 | 2001:DB8:CAFE:C::1/64 | N/A |
|  | S0/0/1 | 2001:DB8:AAAA:2::1/64 | N/A |
| S1 | VLAN1 | 2001:DB8:ACAD:A::A/64 | N/A |
| S2 | VLAN1 | 2001:DB8:ACAD:B::A/64 | N/A |
| S3 | VLAN1 | 2001:DB8:CAFE:C::A/64 | N/A |
| PC-A | NIC | 2001:DB8:ACAD:A::3/64 | FE80::1 |
| PC-B | NIC | 2001:DB8:ACAD:B::3/64 | FE80::1 |
| PC-C | NIC | 2001:DB8:CAFE:C::3/64 | FE80::1 |

- Objectives

Part 1: Set Up the Topology and Initialize Devices

Part 2: Configure Devices and Verify Connectivity

Part 3: Configure and Verify IPv6 ACLs

Part 4: Edit IPv6 ACLs

Background / Scenario

You can filter IPv6 traffic by creating IPv6 access control lists (ACLs) and applying them to interfaces similarly to the way that you create IPv4 named ACLs. IPv6 ACL types are extended and named. Standard and numbered ACLs are no longer used with IPv6. To apply an IPv6 ACL to a vty interface, you use the new ipv6 access-class command. The ipv6 traffic-filter command is still used to apply an IPv6 ACL to interfaces.

In this lab, you will apply IPv6 filtering rules and then verify that they are restricting access as expected. You will also edit an IPv6 ACL and clear the match counters.

Note: The routers used with CCNA hands-on labs are Cisco 1941 Integrated Services Routers (ISRs) with Cisco IOS Release 15.2(4)M3 (universalk9 image). The switches used are Cisco Catalyst 2960s with Cisco IOS Release 15.0(2) (lanbasek9 image). Other routers, switches and Cisco IOS versions can be used. Depending on the model and Cisco IOS version, the commands available and output produced might vary from what is shown in the labs. Refer to the Router Interface Summary Table at the end of the lab for the correct interface identifiers.

Note: Make sure that the routers and switches have been erased and have no startup configurations. If you are unsure, contact your instructor.

Required Resources

3 Routers (Cisco 1941 with Cisco IOS Release 15.2(4)M3 universal image or comparable)

3 Switches (Cisco 2960 with Cisco IOS Release 15.0(2) lanbasek9 image or comparable)

3 PCs (Windows 7, Vista, or XP with terminal emulation program, such as Tera Term)

Console cables to configure the Cisco IOS devices via the console ports

Ethernet and serial cables as shown in the topology

Set Up the Topology and Initialize Devices

In Part 1, you set up the network topology and clear any configurations if necessary.

Cable the network as shown in the topology.

Initialize and reload the routers and switches.

Configure Devices and Verify Connectivity

In Part 2, you configure basic settings on the routers, switches and PCs. Refer to the Topology and Addressing Table at the beginning of this lab for device names and address information.

Configure IPv6 addresses on all PCs.

Configure IPv6 global unicast addresses according to the Addressing Table. Use the link-local address of FE80::1 for the default-gateway on all PCs.

Configure the switches.

Disable DNS lookup.

Assign the hostname.

Assign a domain-name of ccna-lab.com.

Encrypt plain text passwords.

Create a MOTD banner warning users that unauthorized access is prohibited.

Create a local user database with a username of admin and password as classadm.

Assign class as the privileged EXEC encrypted password.

Assign cisco as the console password and enable login.

Enable login on the VTY lines using the local database.

Generate a crypto rsa key for ssh using a modulus size of 1024 bits.

Change the transport input VTY lines to all for SSH and Telnet only.

Assign an IPv6 address to VLAN 1 according to the Addressing Table.

Administratively disable all inactive interfaces.

Configure basic settings on all routers.

Disable DNS lookup.

Assign the hostname.

Assign a domain-name of ccna-lab.com.

Encrypt plain text passwords.

Create a MOTD banner warning users that unauthorized access is prohibited.

Create a local user database with a username of admin and password as classadm.

Assign class as the privileged EXEC encrypted password.

Assign cisco as the console password and enable login.

Enable login on the VTY lines using the local database.

Generate a crypto rsa key for ssh using a modulus size of 1024 bits.

Change the transport input VTY lines to all for SSH and Telnet only.

Configure IPv6 settings on R1.

Configure the IPv6 unicast address on interface G0/0, G0/1, and S0/0/0.

Configure the IPv6 link-local address on interface G0/0, G0/1, and S0/0/0. Use FE80::1 for the link-local address on all three interfaces.

Set the clock rate on S0/0/0 to 128000.

Enable the interfaces.

Enable IPv6 unicast routing.

Configure an IPv6 default route to use interface S0/0/0.

R1(config)# ipv6 route ::/0 s0/0/0

Configure IPv6 settings on R2.

Configure the IPv6 unicast address on interface S0/0/0 and S0/0/1.

Configure the IPv6 link-local address on interface S0/0/0 and S0/0/1. Use FE80::2 for the link-local address on both interfaces.

Set the clock rate on S0/0/1 to 128000.

Enable the interfaces.

Enable IPv6 unicast routing.

Configure static IPv6 routes for traffic handling of R1 and R3 LAN subnets.

R2(config)# ipv6 route 2001:db8:acad::/48 s0/0/0

R2(config)# ipv6 route 2001:db8:cafe:c::/64 s0/0/1

Configure IPv6 settings on R3.

Configure the IPv6 unicast address on interface G0/1 and S0/0/1.

Configure the IPv6 link-local address on interface G0/1 and S0/0/1. Use FE80::1 for the link-local address on both interfaces.

Enable the interfaces.

Enable IPv6 unicast routing.

Configure an IPv6 default route to use interface S0/0/1.

R3(config)# ipv6 route ::/0 s0/0/1

Verify connectivity.

Each PC should be able to ping the other PCs in the topology.

Telnet to R1 from all PCs in the Topology.

SSH to R1 from all PCs in the Topology.

Telnet to S1 from all PCs in the Topology.

SSH to S1 from all PCs in the Topology.

Troubleshoot connectivity issues now because the ACLs that you create in Part 3 of this lab will restrict access to some areas of the network.

Note: Tera Term requires the target IPv6 address to be enclosed in brackets. Enter the IPv6 address as shown, click OK and then click Continue to accept the security warning and connect to the router.

Input the user credentials configured (username admin and password classadm) and select the Use plain password to log in in the SSH Authentication dialogue box. Click OK to continue.

Configure and Verify IPv6 ACLs

Configure and verify VTY restrictions on R1.

Create an ACL to only allow hosts from the 2001:db8:acad:a::/64 network to telnet to R1. All hosts should only be able to ssh to R1.

R1(config)# ipv6 access-list RESTRICT-VTY

R1(config-ipv6-acl)# permit tcp 2001:db8:acad:a::/64 any eq 23

R1(config-ipv6-acl)# permit tcp any any eq 22

Apply the RESTRICT-VTY ACL to R1’s VTY lines.

R1(config-ipv6-acl)# line vty 0 4

R1(config-line)# ipv6 access-class RESTRICT-VTY in

R1(config-line)# end

R1#

Show the new ACL.

R1# show access-lists

IPv6 access list RESTRICT-VTY

permit tcp 2001:DB8:ACAD:A::/64 any sequence 10

permit tcp any any eq 22 sequence 20

Verify that the RESTRICT-VTY ACL is only allowing Telnet traffic from the 2001:db8:acad:a::/64 network.

How does the RESTRICT-VTY ACL only allow hosts from the 2001:db8:acad:a::/64 network to telnet to R1?

____________________________________________________________________________________

____________________________________________________________________________________

____________________________________________________________________________________

____________________________________________________________________________________

What does the second permit statement in the RESTRICT-VTY ACL do?

____________________________________________________________________________________

Restrict Telnet access to the 2001:db8:acad:a::/64 network.

Create an ACL called RESTRICTED-LAN that will block Telnet access to the 2001:db8:acad:a::/64 network.

R1(config)# ipv6 access-list RESTRICTED-LAN

R1(config-ipv6-acl)# remark Block Telnet from outside

R1(config-ipv6-acl)# deny tcp any 2001:db8:acad:a::/64 eq telnet

R1(config-ipv6-acl)# permit ipv6 any any

Apply the RESTRICTED-LAN ACL to interface G0/1 for all outbound traffic.

R1(config-ipv6-acl)# int g0/1

R1(config-if)# ipv6 traffic-filter RESTRICTED-LAN out

R1(config-if)# end

Telnet to S1 from PC-B and PC-C to verify that Telnet has been restricted. SSH to S1 from PC-B to verify that it can still be reached using SSH. Troubleshoot if necessary.

Use the show ipv6 access-list command to view the RESTRICTED-LAN ACL.

R1# show ipv6 access-lists RESTRICTED-LAN

IPv6 access list RESTRICTED-LAN

deny tcp any 2001:DB8:ACAD:A::/64 eq telnet (6 matches) sequence 20

permit ipv6 any any (45 matches) sequence 30

Notice that each statement identifies the number of hits or matches that have occurred since the ACL was applied to the interface.

Use the clear ipv6 access-list to reset the match counters for the RESRICTED-LAN ACL.

R1# clear ipv6 access-list RESTRICTED-LAN

Redisplay the ACL with the show access-lists command to confirm that the counters were cleared.

R1# show access-lists RESTRICTED-LAN

IPv6 access list RESTRICTED-LAN

deny tcp any 2001:DB8:ACAD:A::/64 eq telnet sequence 20

permit ipv6 any any sequence 30

Edit IPv6 ACLs

In Part 4, you will edit the RESTRICTED-LAN ACL that you created in Part 3. It is always a good idea to remove the ACL from the interface to which it is applied before editing it. After you complete your edits, then reapply the ACL to the interface.

Note: Many network administrators will make a copy of the ACL and edit the copy. When editing is complete, the administrator will remove the old ACL and apply the newly edited ACL to the interface. This method keeps the ACL in place until you are ready to apply the edited copy of the ACL.

Remove the ACL from the interface.

R1(config)# int g0/1

R1(config-if)# no ipv6 traffic-filter RESTRICTED-LAN out

R1(config-if)# end

Use the show access-lists command to view the ACL.

R1# show access-lists

IPv6 access list RESTRICT-VTY

permit tcp 2001:DB8:ACAD:A::/64 any (4 matches) sequence 10

permit tcp any any eq 22 (6 matches) sequence 20

IPv6 access list RESTRICTED-LAN

deny tcp any 2001:DB8:ACAD:A::/64 eq telnet sequence 20

permit ipv6 any any (36 matches) sequence 30

Insert a new ACL statement using sequence numbering.

R1(config)# ipv6 access-list RESTRICTED-LAN

R1(config-ipv6-acl)# permit tcp 2001:db8:acad:b::/64 host 2001:db8:acad:a::a eq 23 sequence 15

What does this new permit statement do?

_______________________________________________________________________________________

Insert a new ACL statement at the end of the ACL.

R1(config-ipv6-acl)# permit tcp any host 2001:db8:acad:a::3 eq www

Note: This permit statement is only used to show how to add a statement to the end of an ACL. This ACL line would never be matched because the previous permit statement is matching on everything.

Use the do show access-lists command to view the ACL change.

R1(config-ipv6-acl)# do show access-list

IPv6 access list RESTRICT-VTY

permit tcp 2001:DB8:ACAD:A::/64 any (2 matches) sequence 10

permit tcp any any eq 22 (6 matches) sequence 20

IPv6 access list RESTRICTED-LAN

permit tcp 2001:DB8:ACAD:B::/64 host 2001:DB8:ACAD:A::A eq telnet sequence 15

deny tcp any 2001:DB8:ACAD:A::/64 eq telnet sequence 20

permit ipv6 any any (124 matches) sequence 30

permit tcp any host 2001:DB8:ACAD:A::3 eq www sequence 40

Note: The do command can be used to execute any privileged EXEC command while in global configuration mode or a submode.

Delete an ACL statement.

Use the no command to delete the permit statement that you just added.

R1(config-ipv6-acl)# no permit tcp any host 2001:DB8:ACAD:A::3 eq www

Use the do show access-list RESTRICTED-LAN command to view the ACL.

R1(config-ipv6-acl)# do show access-list RESTRICTED-LAN

IPv6 access list RESTRICTED-LAN

permit tcp 2001:DB8:ACAD:B::/64 host 2001:DB8:ACAD:A::A eq telnet sequence 15

deny tcp any 2001:DB8:ACAD:A::/64 eq telnet sequence 20

permit ipv6 any any (214 matches) sequence 30

Re-apply the RESTRICTED-LAN ACL to the interface G0/1.

R1(config-ipv6-acl)# int g0/1

R1(config-if)# ipv6 traffic-filter RESTRICTED-LAN out

R1(config-if)# end

Test ACL changes.

Telnet to S1 from PC-B. Troubleshoot if necessary.

Reflection

What is causing the match count on the RESTRICTED-LAN permit ipv6 any any statement to continue to increase?

_______________________________________________________________________________________

_______________________________________________________________________________________

_______________________________________________________________________________________

_______________________________________________________________________________________

What command would you use to reset the counters for the ACL on the VTY lines?

_______________________________________________________________________________________

Router Interface Summary Table

| Router Interface Summary |  |  |  |  |
| --- | --- | --- | --- | --- |
| Router Model | Ethernet Interface #1 | Ethernet Interface #2 | Serial Interface #1 | Serial Interface #2 |
| 1800 | Fast Ethernet 0/0 (F0/0) | Fast Ethernet 0/1 (F0/1) | Serial 0/0/0 (S0/0/0) | Serial 0/0/1 (S0/0/1) |
| 1900 | Gigabit Ethernet 0/0 (G0/0) | Gigabit Ethernet 0/1 (G0/1) | Serial 0/0/0 (S0/0/0) | Serial 0/0/1 (S0/0/1) |
| 2801 | Fast Ethernet 0/0 (F0/0) | Fast Ethernet 0/1 (F0/1) | Serial 0/1/0 (S0/1/0) | Serial 0/1/1 (S0/1/1) |
| 2811 | Fast Ethernet 0/0 (F0/0) | Fast Ethernet 0/1 (F0/1) | Serial 0/0/0 (S0/0/0) | Serial 0/0/1 (S0/0/1) |
| 2900 | Gigabit Ethernet 0/0 (G0/0) | Gigabit Ethernet 0/1 (G0/1) | Serial 0/0/0 (S0/0/0) | Serial 0/0/1 (S0/0/1) |
| Note: To find out how the router is configured, look at the interfaces to identify the type of router and how many interfaces the router has. There is no way to effectively list all the combinations of configurations for each router class. This table includes identifiers for the possible combinations of Ethernet and Serial interfaces in the device. The table does not include any other type of interface, even though a specific router may contain one. An example of this might be an ISDN BRI interface. The string in parenthesis is the legal abbreviation that can be used in Cisco IOS commands to represent the interface. |  |  |  |  |