Packet Tracer – Configuring Basic EIGRP with IPv4

Topology

- Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| R1 | G0/0 | 172.16.1.1 | 255.255.255.0 | N/A |
|  | S0/0/0 | 172.16.3.1 | 255.255.255.252 | N/A |
|  | S0/0/1 | 192.168.10.5 | 255.255.255.252 | N/A |
| R2 | G0/0 | 172.16.2.1 | 255.255.255.0 | N/A |
|  | S0/0/0 | 172.16.3.2 | 255.255.255.252 | N/A |
|  | S0/0/1 | 192.168.10.9 | 255.255.255.252 | N/A |
| R3 | G0/0 | 192.168.1.1 | 255.255.255.0 | N/A |
|  | S0/0/0 | 192.168.10.6 | 255.255.255.252 | N/A |
|  | S0/0/1 | 192.168.10.10 | 255.255.255.252 | N/A |
| PC1 | NIC | 172.16.1.10 | 255.255.255.0 | 172.16.1.1 |
| PC2 | NIC | 172.16.2.10 | 255.255.255.0 | 172.16.2.1 |
| PC3 | NIC | 192.168.1.10 | 255.255.255.0 | 192.168.1.1 |

Objectives

Part 1: Configure EIGRP

Part 2: Verify EIGRP Routing

Background

In this activity, you will implement basic EIGRP configurations including network commands, passive interfaces and disabling automatic summarization. You will then verify your EIGRP configuration using a variety of show commands and testing end-to-end connectivity.

Configure EIGRP

Enable the EIGRP routing process.

Enable the EIGRP routing process on each router using AS number 1. The configuration for R1 is shown.

R1(config)# router eigrp 1

What is the range of numbers that can be used for AS numbers? _______________________________

Advertise directly connected networks.

Use the show ip route command to display the directly connected networks on each router.

- How can you tell the difference between subnet addresses and interface addresses?

- ____________________________________________________________________________________

On each router, configure EIGRP to advertise the specific directly connected subnets. The configuration for R1 is shown.

R1(config-router)# network 172.16.1.0 0.0.0.255

R1(config-router)# network 172.16.3.0 0.0.0.3

R1(config-router)# network 192.168.10.4 0.0.0.3

Configure passive interfaces.

- Configure the LAN interfaces to not advertise EIGRP updates. The configuration for R1 is shown.

R1(config-router)# passive-interface g0/0

Disable automatic summarization.

The topology contains discontiguous networks. Therefore, disable automatic summarization on each router. The configuration for R1 is shown.

R1(config-router)# no auto-summary

Note: Prior to IOS 15 auto-summary had to be manually disabled.

Save the configurations.

Verify EIGRP Routing

Examine neighbor adjacencies.

Which command displays the neighbors discovered by EIGRP? _______________________________

All three routers should have two neighbors listed. The output for R1 should look similar to the following:

IP-EIGRP neighbors for process 1

H   Address         Interface      Hold Uptime    SRTT   RTO   Q   Seq

(sec)          (ms)        Cnt  Num

0   172.16.3.2      Se0/0/0        14   00:25:05  40     1000  0   28

1   192.168.10.6    Se0/0/1        12   00:13:29  40     1000  0   31

Display the EIGRP routing protocol parameters.

What command displays the parameters and other information about the current state of any active IPv4 routing protocol processes configured on the router? _______________________________

On R2, enter the command you listed for 2a and answer the following questions:

How many routers are sharing routing information with R2? __________

Where is this information located under? _______________________________

What is the maximum hop count? __________

Verify end-to-end connectivity

PC1, PC2 and PC3 should now be able to ping each other. If not, troubleshoot your EIGRP configurations.

Suggested Scoring Rubric

| Activity Section | Question Location | Possible Points | Earned Points |
| --- | --- | --- | --- |
| Part 1: Configure EIGRP | Step 1 | 2 |  |
|  | Step 2a | 2 |  |
| Part 1 Total |  | 4 |  |
| Part 2: Verify EIGRP Routing | Step 1a | 5 |  |
|  | Step 2a | 5 |  |
|  | Step 2b | 6 |  |
| Part 2 Total |  | 16 |  |
| Packet Tracer Score |  | 80 |  |
| Total Score |  | 100 |  |