Packet Tracer – Configuring Rapid PVST+ (Instructor Version)

Instructor Note: Red font color or Gray highlights indicate text that appears in the instructor copy only.

Topology

Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| S1 | VLAN 99 | 172.17.99.11 | 255.255.255.0 | N/A |
| S2 | VLAN 99 | 172.17.99.12 | 255.255.255.0 | N/A |
| S3 | VLAN 99 | 172.17.99.13 | 255.255.255.0 | N/A |
| PC1 | NIC | 172.17.10.21 | 255.255.255.0 | 172.17.10.254 |
| PC2 | NIC | 172.17.20.22 | 255.255.255.0 | 172.17.20.254 |
| PC3 | NIC | 172.17.30.23 | 255.255.255.0 | 172.17.30.254 |

Switch Port Assignment Specifications

| Ports | Assignments | Network |
| --- | --- | --- |
| S2 F0/6 | VLAN 30 | 172.17.30.0/24 |
| S2 F0/18 | VLAN 20 | 172.17.20.0/24 |
| S2 F0/11 | VLAN 10 | 172.17.10.0/24 |

Objectives

Part 1: Configure VLANs

Part 2: Configure Rapid Spanning Tree PVST+ Load balancing

Part 3: Configure PortFast and BPDU Guard

Background

In this activity, you will configure VLANs and trunks, Rapid Spanning Tree PVST+, primary and secondary root bridges, and examine the configuration results. You will also optimize the network by configuring PortFast, and BPDU Guard on edge ports.

Configure VLANs

Enable the user ports on S2 in access mode.

Refer to the topology diagram to determine which switch ports on S2 are activated for end-user device access. These three ports will be configured for access mode and enabled with the no shutdown command.

S2(config)# interface range f0/6,f0/11,f0/18

S2(config-if-range)# switchport mode access

S2(config-fi-range)# no shutdown

Create VLANs.

Using the appropriate command, create VLANs 10, 20, 30, 40, 50, 60, 70, 80, and 99 on all of the switches.

S1(config)# vlan 10

S1(config-vlan)# vlan 20

S1(config-vlan)# vlan 30

S1(config-vlan)# vlan 40

S1(config-vlan)# vlan 50

S1(config-vlan)# vlan 60

S1(config-vlan)# vlan 70

S1(config-vlan)# vlan 80

S1(config-vlan)# vlan 99

S2(config)# vlan 10

S2(config-vlan)# vlan 20

S2(config-vlan)# vlan 30

S2(config-vlan)# vlan 40

S2(config-vlan)# vlan 50

S2(config-vlan)# vlan 60

S2(config-vlan)# vlan 70

S2(config-vlan)# vlan 80

S2(config-vlan)# vlan 99

S3(config)# vlan 10

S3(config-vlan)# vlan 20

S3(config-vlan)# vlan 30

S3(config-vlan)# vlan 40

S3(config-vlan)# vlan 50

S3(config-vlan)# vlan 60

S3(config-vlan)# vlan 70

S3(config-vlan)# vlan 80

S3(config-vlan)# vlan 99

Assign VLANs to switch ports.

Port assignments are listed in the table at the beginning of the activity. Save your configurations after assigning switch ports to the VLANs.

S2(config)# interface f0/6

S2(config-if)# switchport access vlan 30

S2(config-if)# interface f0/11

S2(config-if)# switchport access vlan 10

S2(config-if)# interface f0/18

S2(config-if)# switchport access vlan 20

Verify the VLANs.

Use the show vlan brief command on all switches to verify that all VLANs are registered in the VLAN table.

Assign the trunks to native VLAN 99.

Use the appropriate command to configure ports F0/1 to F0/4 on each switch as trunk ports and assign these trunk ports to native VLAN 99.

S1(config)# interface range f0/1-4

S1(config-if-range)# switchport mode trunk

S1(config-if-range)# switchport trunk native vlan 99

S2(config)# interface range f0/1-4

S2(config-if-range)# switchport mode trunk

S2(config-if-range)# switchport trunk native vlan 99

S3(config)# interface range f0/1-4

S3(config-if-range)# switchport mode trunk

S3(config-if-range)# switchport trunk native vlan 99

Configure the management interface on all three switches with an address.

S1(config)# interface vlan99

S1(config-if)# ip address 172.17.99.11 255.255.255.0

S2(config)# interface vlan99

S2(config-if)# ip address 172.17.99.12 255.255.255.0

S3(config)# interface vlan99

S3(config-if)# ip address 172.17.99.13 255.255.255.0

Verify that the switches are correctly configured by pinging between them.

Configure Rapid Spanning Tree PVST+ Load Balancing

The Rapid Spanning Tree Protocol (RSTP; IEEE 802.1w) can be seen as an evolution of the 802.1D standard more so than a revolution. The 802.1D terminology remains primarily the same. Most parameters have been left unchanged so users familiar with 802.1D can rapidly configure the new protocol comfortably. In most cases, RSTP performs better than proprietary extensions of Cisco without any additional configuration. 802.1w can also revert back to 802.1D in order to interoperate with legacy bridges on a per-port basis.

Configure STP mode.

Use the spanning-tree mode command to configure the switches to use rapid PVST as the STP mode.

S1(config)# spanning-tree mode rapid-pvst

S2(config)# spanning-tree mode rapid-pvst

S3(config)# spanning-tree mode rapid-pvst

Configure Rapid Spanning Tree PVST+ load balancing.

Configure S1 to be the primary root for VLANs 1, 10, 30, 50, and 70. Configure S3 to be the primary root for VLANs 20, 40, 60, 80, and 99. Configure S2 to be the secondary root for all of the VLANs.

S1(config)# spanning-tree vlan 1,10,30,50,70 root primary

S2(config)# spanning-tree vlan 1,10,20,30,40,50,60,70,80,99 root secondary

S3(config)# spanning-tree vlan 20,40,60,80,99 root primary

Verify your configurations by using the show spanning-tree command.

Configure PortFast and BPDU Guard

Configuring PortFast on S2.

PortFast causes a port to enter the forwarding state almost immediately by dramatically decreasing the time of the listening and learning states. PortFast minimizes the time it takes for the server or workstation to come online. Configure PortFast on S2 interfaces that are connected to PCs.

S2(config)# interface range f0/6 , f0/11 , f0/18

S2(config-if-range)# spanning-tree portfast

Configuring BPDU Guard on S2.

The STP PortFast BPDU Guard enhancement allows network designers to enforce the STP domain borders and keep the active topology predictable. The devices behind the ports that have STP PortFast enabled are not able to influence the STP topology. At the reception of BPDUs, the BPDU Guard operation disables the port that has PortFast configured. The BPDU Guard transitions the port into err-disable state, and a message appears on the console. Configure BPDU Guard on S2 interfaces that are connected to PCs.

S2(config)# interface range f0/6 , f0/11 , f0/18

S2(config-if-range)# spanning-tree bpduguard enable

Verify your configuration.

Use the show run command to verify your configuration.