Packet Tracer – Skills Integration Challenge (Instructor Version)

Instructor Note: Red font color or gray highlights indicate text that appears in the instructor copy only.

Topology

Addressing Table

Instructor Note: The student version has blanks in place of all variables shown in red double brackets.

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| [[R1Name]] | G0/0.15 | [[R1G0sub15Add]] | [[R1G0sub15SM]] | N/A |
|  | G0/0.30 | [[R1G0sub30Add]] | [[R1G0sub30SM]] | N/A |
|  | G0/0.45 | [[R1G0sub45Add]] | [[R1G0sub45SM]] | N/A |
|  | G0/0.60 | [[R1G0sub60Add]] | [[R1G0sub60SM]] | N/A |
|  | S0/0/0 | [[R1S000Add]] | 255.255.255.252 | N/A |
|  | S0/0/1 | [[R1S001Add]] | 255.255.255.252 | N/A |
|  | S0/1/0 | [[R1S010Add]] | 255.255.255.252 | N/A |
| [[R2Name]] | G0/0 | [[R2G00Add]] | [[R2R3LanSM]] | N/A |
|  | S0/0/0 | [[R2S000Add]] | 255.255.255.252 | N/A |
|  | S0/0/1 | [[R2S001Add]] | 255.255.255.252 | N/A |
| [[R3Name]] | G0/0 | [[R3G00Add]] | [[R2R3LanSM]] | N/A |
|  | S0/0/0 | [[R3S000Add]] | 255.255.255.252 | N/A |
|  | S0/0/1 | [[R3S001Add]] | 255.255.255.252 | N/A |
| [[S1Name]] | VLAN 60 | [[S1VLAN60Add]] | [[R1G0sub60SM]] | [[R1G0sub60Add]] |
| [[PC1Name]] | NIC | DHCP Assigned | DHCP Assigned | DHCP Assigned |

VLANs and Port Assignments Table

| VLAN Number - Name | Port assignment | Network |
| --- | --- | --- |
| 15 - Servers | F0/11 - F0/20 | [[R1-VLANsrvNet]] |
| 30 - PCs | F0/1 - F0/10 | [[R1-VLANpcNet]] |
| 45 - Native | G0/1 | [[R1-VLANntvNet]] |
| 60 - Management | VLAN 60 | [[R1-VLANmanNet]] |

Scenario

This activity includes many of the skills that you have acquired during your CCNA studies. First, you will complete the documentation for the network. Make sure you have a printed version of the instructions. During implementation, you will configure VLANs, trunking, port security and SSH remote access on a switch. Then, you will implement inter-VLAN routing and NAT on a router. Finally, you will use your documentation to verify your implementation by testing end-to-end connectivity.

Documentation

You are required to fully document the network. You will need a print out of this instruction set, which will include an unlabeled topology diagram:

Label all the device names, network addresses and other important information that Packet Tracer generated.

Complete the Addressing Table and VLANs and Port Assignments Table.

Fill in any blanks in the Implementation and Verification steps. The information is supplied when you launch the Packet Tracer activity.

Implementation

Note: All devices in the topology except [[R1Name]], [[S1Name]], and [[PC1Name]] are fully configured. You do not have access to the other routers. You can access all the servers and PCs for testing purposes.

Implement to following requirements using your documentation:

[[S1Name]]

Configure remote management access including IP addressing and SSH:

Domain is cisco.com

Enable secret ciscoenpass

User [[UserText]] with password [[UserPass]]

Crypto key length of 1024

SSH version 2, limited to 2 authentication attempts and a 60 second timeout

Plaintext passwords should be encrypted.

Configure, name and assign VLANs. Ports should be manually configured as access ports.

Configure trunking.

Implement port security:

On F0/1, allow 2 MAC addresses that are automatically added to the configuration file when detected. The port should not be disabled, but a syslog message should be captured if a violation occurs.

Disable all other unused ports.

[[R1Name]]

Configure inter-VLAN routing.

Configure DHCP services for VLAN 30. Use LAN as the case-sensitive name for the pool.

Implement routing:

Use RIPv2.

Configure one network statement for the entire [[DisplayNet]] address space.

Disable interfaces that should not send RIPv2 messages.

Configure a default route to the Internet.

Implement NAT:

Configure a standard, one statement ACL number 1. All IP addresses belonging to the [[DisplayNet]] address space are allowed.

Refer to your documentation and configure static NAT for the File Server.

Configure dynamic NAT with PAT using a pool name of your choice, a /30 mask, and these two public addresses:

    - [[NATPoolText]]

[[PC1Name]]

Verify [[PC1Name]] has received full addressing information from [[R1Name]].

Verification

All devices should now be able to ping all other devices. If not, troubleshoot your configurations to isolate and solve problems. A few tests include:

Verify remote access to [[S1Name]] by using SSH from a PC.

Verify VLANs are assigned to appropriate ports and port security is in force.

Verify a complete routing table.

Verify NAT translations and statics.

Outside Host should be able to access File Server at the public address.

Inside PCs should be able to access Web Server.

Document any problems you encountered and the solutions in the Troubleshooting Documentation table below.

Troubleshooting Documentation

| Problem | Solution |
| --- | --- |

Suggested Scoring Rubric

- Packet Tracer scores 75 points. Documentation is worth 25 points.

ID:[[indexAdds]][[indexNames]]

*****************************************************

ISOMORPH ID KEY:

ID = XY where;

X = indexAdds for /24 private address space

Y = indexNAMES for device names

Note: Each seed contains variables that are independent

of the other seeds. You do not need to test all the

various combinations.

=======================================================

ISOMORPH ID = 00

=======================================================

!HQ!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

enable

conf t

ip dhcp excluded-address 172.16.15.33

ip dhcp pool LAN

network 172.16.15.32 255.255.255.224

default-router 172.16.15.33

interface GigabitEthernet0/0

no shutdown

interface GigabitEthernet0/0.15

encapsulation dot1Q 15

ip address 172.16.15.17 255.255.255.240

ip nat inside

interface GigabitEthernet0/0.30

encapsulation dot1Q 30

ip address 172.16.15.33 255.255.255.224

ip nat inside

interface GigabitEthernet0/0.45

encapsulation dot1Q 45 native

ip address 172.16.15.1 255.255.255.248

interface GigabitEthernet0/0.60

encapsulation dot1Q 60

ip address 172.16.15.9 255.255.255.248

router rip

version 2

no auto-summary

passive-interface GigabitEthernet0/0.15

passive-interface GigabitEthernet0/0.30

passive-interface GigabitEthernet0/0.45

passive-interface GigabitEthernet0/0.60

passive-interface Serial0/1/0

network 172.16.15.0

ip nat pool TEST 209.165.200.225 209.165.200.226 netmask 255.255.255.252

ip nat inside source list 1 pool TEST overload

ip nat inside source static 172.16.15.18 209.165.200.227

ip route 0.0.0.0 0.0.0.0 Serial0/1/0

access-list 1 permit 172.16.15.0 0.0.0.255

interface s0/0/0

ip nat inside

interface s0/0/1

ip nat inside

interface s0/1/0

ip nat outside

end

!!!!!!!!!!!!!!!!!!!!!!!!

!HQ-Sw!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

enable

conf t

int vlan 60

ip add 172.16.15.10 255.255.255.248

no shut

ip default-gateway 172.16.15.9

vlan 15

name Servers

vlan 30

name PCs

vlan 45

name Native

vlan 60

name Management

interface range f0/1 - 10

switchport mode access

switchport access vlan 30

interface f0/1

switchport port-security

switchport port-security maximum 2

switchport port-security mac-address sticky

switchport port-security violation restrict

interface range f0/11 - 20

switchport mode access

switchport access vlan 15

interface g0/1

switchport mode trunk

switchport trunk native vlan 45

interface range f0/21 - 24 , g0/2

shutdown

ip domain-name cisco.com

crypto key gen rsa

1024

enable secret ciscoenpass

user HQadmin pass ciscoclass

service password-encryption

ip ssh version 2

ip ssh auth 2

ip ssh time 60

line vty 0 15

login local

transport input ssh

end

=======================================================

ISOMORPH ID = 11

=======================================================

!Admin!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

enable

conf t

ip dhcp excluded-address 10.10.10.193

ip dhcp pool LAN

network 10.10.10.192 255.255.255.192

default-router 10.10.10.193

interface GigabitEthernet0/0

no shutdown

interface GigabitEthernet0/0.15

encapsulation dot1Q 15

ip address 10.10.10.161 255.255.255.224

ip nat inside

interface GigabitEthernet0/0.30

encapsulation dot1Q 30

ip address 10.10.10.193 255.255.255.192

ip nat inside

interface GigabitEthernet0/0.45

encapsulation dot1Q 45 native

ip address 10.10.10.129 255.255.255.240

interface GigabitEthernet0/0.60

encapsulation dot1Q 60

ip address 10.10.10.145 255.255.255.240

router rip

version 2

no auto-summary

passive-interface GigabitEthernet0/0.15

passive-interface GigabitEthernet0/0.30

passive-interface GigabitEthernet0/0.45

passive-interface GigabitEthernet0/0.60

passive-interface Serial0/1/0

network 10.0.0.0

interface s0/0/0

ip nat inside

interface s0/0/1

ip nat inside

interface s0/1/0

ip nat outside

ip nat pool TEST 198.133.219.128 198.133.219.129 netmask 255.255.255.252

ip nat inside source list 1 pool TEST overload

ip nat inside source static 10.10.10.162 198.133.219.130

ip route 0.0.0.0 0.0.0.0 Serial0/1/0

access-list 1 permit 10.10.10.0 0.0.0.255

end

!!!!!!!!!!!!!!!!!!!!!!!!

!Admin-Sw!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

enable

conf t

int vlan 60

ip add 10.10.10.146 255.255.255.240

no shut

ip default-gateway 10.10.10.145

vlan 15

name Servers

vlan 30

name PCs

vlan 45

name Native

vlan 60

name Management

interface range f0/1 - 10

switchport mode access

switchport access vlan 30

interface f0/1

switchport port-security

switchport port-security maximum 2

switchport port-security mac-address sticky

switchport port-security violation restrict

interface range f0/11 - 20

switchport mode access

switchport access vlan 15

interface g0/1

switchport mode trunk

switchport trunk native vlan 45

interface range f0/21 - 24 , g0/2

shutdown

ip domain-name cisco.com

crypto key gen rsa

1024

enable secret ciscoenpass

user Admin pass letmein

service password-encryption

ip ssh version 2

ip ssh auth 2

ip ssh time 60

line vty 0 15

login local

transport input ssh

end

===============================================================

ISOMORPH ID: 22

===============================================================

!Central!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

enable

conf t

ip dhcp excluded-address 192.168.45.129

ip dhcp pool LAN

network 192.168.45.128 255.255.255.192

default-router 192.168.45.129

interface GigabitEthernet0/0

no shutdown

interface GigabitEthernet0/0.15

encapsulation dot1Q 15

ip address 192.168.45.65 255.255.255.192

ip nat inside

interface GigabitEthernet0/0.30

encapsulation dot1Q 30

ip address 192.168.45.129 255.255.255.192

ip nat inside

interface GigabitEthernet0/0.45

encapsulation dot1Q 45 native

ip address 192.168.45.17 255.255.255.240

interface GigabitEthernet0/0.60

encapsulation dot1Q 60

ip address 192.168.45.33 255.255.255.240

router rip

passive-interface GigabitEthernet0/0.15

passive-interface GigabitEthernet0/0.30

passive-interface GigabitEthernet0/0.45

passive-interface GigabitEthernet0/0.60

passive-interface Serial0/1/0

network 192.168.45.0

no auto-summary

version 2

interface s0/0/0

ip nat inside

interface s0/0/1

ip nat inside

interface s0/1/0

ip nat outside

ip nat pool TEST 64.100.32.56 64.100.32.57 netmask 255.255.255.252

ip nat inside source list 1 pool TEST overload

ip nat inside source static 192.168.45.66 64.100.32.58

ip route 0.0.0.0 0.0.0.0 Serial0/1/0

access-list 1 permit 192.168.45.0 0.0.0.255

end

!!!!!!!!!!!!!!!!!!!!!!!!

!Cnt-Sw!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

enable

conf t

int vlan 60

ip add 192.168.45.34 255.255.255.240

no shut

ip default-gateway 192.168.45.33

vlan 15

name Servers

vlan 30

name PCs

vlan 45

name Native

vlan 60

name Management

interface range f0/1 - 10

switchport mode access

switchport access vlan 30

interface fa0/1

switchport port-security

switchport port-security maximum 2

switchport port-security mac-address sticky

switchport port-security violation restrict

interface range f0/11 - 20

switchport mode access

switchport access vlan 15

interface g0/1

switchport mode trunk

switchport trunk native vlan 45

interface range f0/21 - 24 , g0/2

shutdown

ip domain-name cisco.com

crypto key gen rsa

1024

enable secret ciscoenpass

user CAdmin pass itsasecret

service password-encryption

ip ssh version 2

ip ssh auth 2

ip ssh time 60

line vty 0 15

login local

transport input ssh

end