Packet Tracer – Skills Integration Challenge (Instructor Version)

Instructor Note: Red font color or gray highlights indicate text that appears in the instructor copy only.

Topology

- Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway | VLAN Association |
| --- | --- | --- | --- | --- | --- |
| R1 | G0/0.1 | 192.168.99.1 | 255.255.255.0 | N/A | VLAN 99 |
|  | G0/0.10 | 192.168.10.1 | 255.255.255.0 | N/A | VLAN 10 |
|  | G0/0.20 | 192.168.20.1 | 255.255.255.0 | N/A | VLAN 20 |
|  | S0/0/0 | 209.165.200.238 | 255.255.255.224 | N/A | N/A |
|  | S0/0/1 | 192.168.1.1 | 255.255.255.0 | N/A | N/A |
| R2 | G0/0.1 | 192.168.99.2 | 255.255.255.0 | N/A | VLAN 99 |
|  | G0/0.10 | 192.168.10.2 | 255.255.255.0 | N/A | VLAN 10 |
|  | G0/0.20 | 192.168.20.2 | 255.255.255.0 | N/A | VLAN 20 |
|  | S0/0/0 | 192.168.1.2 | 255.255.255.0 | N/A | N/A |
|  | S0/0/1 | 209.165.202.158 | 255.255.255.224 | N/A | N/A |
| ISP | S0/0/0 | 209.165.200.225 | 255.255.255.224 | N/A | N/A |
|  | S0/0/1 | 209.165.202.129 | 255.255.255.224 | N/A | N/A |
| Web | NIC | 64.104.13.130 | 255.255.255.252 | 64.104.13.129 | N/A |
| PC10A | NIC | 192.168.10.101 | 255.255.255.0 | 192.168.10.1 | VLAN 10 |
| PC10B | NIC | 192.168.10.102 | 255.255.255.0 | 192.168.10.1 | VLAN 10 |
| PC20A | NIC | 192.168.20.101 | 255.255.255.0 | 192.168.20.1 | VLAN 20 |
| PC20B | NIC | 192.168.20.102 | 255.255.255.0 | 192.168.20.1 | VLAN 20 |

Scenario

In this activity, two routers are configured to communicate with each other. You are responsible for configuring subinterfaces to communicate with the switches. You will configure inter-VLAN routing with RIPv2, VLANs with VTP, trunking, and EtherChannel with PVST. The PCs and Internet devices are all preconfigured.

Requirements

You are responsible for configuring routers R1 and R2 and switches S1, S2, S3, and S4.

Note: Packet Tracer does not allow assigning point values less than 1. Because this activity is checking over 150 items, not all configurations are assigned a point value. Click Check Results > Assessment Items to verify you have correctly configured all items.

Inter-VLAN Routing

On R1 and R2, enable and configure the subinterfaces with the following requirement:

Configure the appropriate dot1Q encapsulation.

Configure VLAN 99 as the native VLAN.

Configure the IP address for the subinterface according to the Addressing Table.

Routing

Configure RIPv2 using the following requirements:

Do not advertise the network connected to the Internet.

Disable autosummarization.

Disable RIP updates for each subinterface.

VTP and VLANs

Configure S1 as the VTP server. Configure all other switches as VTP clients. They are not allowed to create VLANs.

VTP domain is CCNA.

VTP password is cisco123.

Create VLAN 10, 20, and 99 on S1.

Configure the following static ports for S1 and S2:

F0/1 – 9 as access ports in VLAN 10.

F0/10 – 19 as access ports in VLAN 20.

F0/20 – F24 and G0/1 – 0/2 as the native trunk for VLAN 99.

Configure the following static ports for S3 and S4:

F0/1 – 9 as access ports in VLAN 10.

F0/10 – 20 as access ports in VLAN 20.

F0/21 – F24 and G0/1 – 0/2 as the native trunk for VLAN 99.

EtherChannels

All EtherChannels are configured as LACP.

All EtherChannels are statically configured to trunk all VLANs including VLAN 99 as the native VLAN.

Use the following table to configure the appropriate switch ports to form EtherChannels:

| Port Channel | Device: Ports | Device: Ports |
| --- | --- | --- |
| 1 | S1: G0/1 – 2 | S3: G0/1 – 2 |
| 2 | S2: G0/1 – 2 | S4: G0/1 – 2 |
| 3 | S1: F0/23 – 24 | S2: F0/23 – 24 |
| 4 | S3: F0/23 – 24 | S4: F0/23 – 24 |
| 5 | S1: F0/21 – 22 | S4: F0/21 – 22 |
| 6 | S2: F0/21 – 22 | S3: F0/21 - 22 |

Spanning Tree

Configure per-VLAN rapid spanning tree mode for all switches.

Configure spanning tree priorities according to the table below:

| Device | VLAN 10 Priority | VLAN 20 Priority |
| --- | --- | --- |
| S1 | 4096 | 8192 |
| S2 | 8192 | 4096 |
| S3 | 32768 | 32768 |
| S4 | 32768 | 32768 |

Connectivity

All PCs should be able to ping the Web and other PCs.

Scripts

Router R1

!R1

enable

configure t

interface GigabitEthernet0/0

no shut

!

interface GigabitEthernet0/0.1

encapsulation dot1Q 99 native

ip address 192.168.99.1 255.255.255.0

!

interface GigabitEthernet0/0.10

encapsulation dot1Q 10

ip address 192.168.10.1 255.255.255.0

!

interface GigabitEthernet0/0.20

encapsulation dot1Q 20

ip address 192.168.20.1 255.255.255.0

!

router rip

version 2

network 192.168.1.0

network 192.168.10.0

network 192.168.20.0

network 192.168.99.0

passive-interface GigabitEthernet0/0.1

passive-interface GigabitEthernet0/0.10

passive-interface GigabitEthernet0/0.20

no auto-summary

end

copy run start

Router R2

!R2

enable

configure t

!

interface GigabitEthernet0/0

no shut

!

interface GigabitEthernet0/0.1

encapsulation dot1Q 99 native

ip address 192.168.99.2 255.255.255.0

!

interface GigabitEthernet0/0.10

encapsulation dot1Q 10

ip address 192.168.10.2 255.255.255.0

!

interface GigabitEthernet0/0.20

encapsulation dot1Q 20

ip address 192.168.20.2 255.255.255.0

!

router rip

version 2

network 192.168.1.0

network 192.168.10.0

network 192.168.20.0

network 192.168.99.0

passive-interface GigabitEthernet0/0.1

passive-interface GigabitEthernet0/0.10

passive-interface GigabitEthernet0/0.20

no auto-summary

end

copy run start

Switch S1

!S1

enable

configure t

vtp mode server

vtp domain CCNA

vtp password cisco123

vlan 10

vlan 20

vlan 99

exit

interface range f0/1 - 9

switchport mode access

switchport access vlan 10

inte range f0/10 - 19

switchport mode access

switchport access vlan 20

interface range f0/20 - 24, g0/1-2

switchport mode trunk

switchport trunk native vlan 99

!

interface range g0/1 - 2

channel-group 1 mode active

interface range f0/21 - 22

channel-group 5 mode active

interface range f0/23 - 24

channel-group 3 mode active

!

interface po 1

switchport mode trunk

switchport trunk native vlan 99

interface po 3

switchport mode trunk

switchport trunk native vlan 99

interface po 5

switchport mode trunk

switchport trunk native vlan 99

!

spanning-tree mode rapid-pvst

spanning-tree vlan 10 priority 4096

spanning-tree vlan 20 priority 8192

end

copy run start

Switch S2

!S2

enable

configure t

vtp mode client

vtp domain CCNA

vtp password cisco123

interface range f0/1 - 9

switchport mode access

switchport access vlan 10

inte range f0/10 - 19

switchport mode access

switchport access vlan 20

inte range f0/20 - 24, g0/1-2

switchport mode trunk

switchport trunk native vlan 99

!

interface range g0/1 - 2

channel-group 2 mode active

interface range f0/21 - 22

channel-group 6 mode active

interface range f0/23 - 24

channel-group 3 mode active

!

interface po 2

switchport mode trunk

switchport trunk native vlan 99

interface po 3

switchport mode trunk

switchport trunk native vlan 99

interface po 6

switchport mode trunk

switchport trunk native vlan 99

!

spanning-tree mode rapid-pvst

spanning-tree vlan 10 priority 8192

spanning-tree vlan 20 priority 4096

end

copy run start

Switch S3

!S3

enable

configure t

vtp mode client

vtp domain CCNA

vtp password cisco123

interface range f0/1 - 9

switchport mode access

switchport access vlan 10

inte range f0/10 - 20

switchport mode access

switchport access vlan 20

inte range f0/21 - 24, g0/1-2

switchport mode trunk

switchport trunk native vlan 99

!

interface range g0/1 - 2

channel-group 1 mode active

interface range f0/21 - 22

channel-group 6 mode active

interface range f0/23 - 24

channel-group 4 mode active

!

interface po 1

switchport mode trunk

switchport trunk native vlan 99

interface po 4

switchport mode trunk

switchport trunk native vlan 99

interface po 6

switchport mode trunk

switchport trunk native vlan 99

!

spanning-tree mode rapid-pvst

spanning-tree vlan 10 priority 32768

spanning-tree vlan 20 priority 32768

end

copy run start

Switch S4

!S4

enable

configure t

vtp mode client

vtp domain CCNA

vtp password cisco123

interface range f0/1 - 9

switchport mode access

switchport access vlan 10

inte range f0/10 - 20

switchport mode access

switchport access vlan 20

inte range f0/21 - 24, g0/1-2

switchport mode trunk

switchport trunk native vlan 99

!

interface range g0/1 - 2

channel-group 2 mode active

interface range f0/21 - 22

channel-group 5 mode active

interface range f0/23 - 24

channel-group 4 mode active

interface po 2

switchport mode trunk

switchport trunk native vlan 99

interface po 4

switchport mode trunk

switchport trunk native vlan 99

interface po 5

switchport mode trunk

switchport trunk native vlan 99

!

spanning-tree mode rapid-pvst

spanning-tree vlan 10 priority 32768

spanning-tree vlan 20 priority 32768

end

copy run start