Packet Tracer – Investigating Convergence (Instructor Version)

Instructor Note: Red font color or Gray highlights indicate text that appears in the instructor copy only.

- Topology

- Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| R1 | G0/0 | 209.165.0.1 | 255.255.255.0 | N/A |
|  | G0/1 | 64.100.0.1 | 255.0.0.0 | N/A |
|  | S0/0/0 | 192.168.1.2 | 255.255.255.0 | N/A |
| R2 | G0/0 | 10.0.0.1 | 255.0.0.0 | N/A |
|  | S0/0/0 | 192.168.1.1 | 255.255.255.0 | N/A |
| PC1 | NIC | 64.100.0.2 | 255.0.0.0 | 64.100.0.1 |
| PC2 | NIC | 209.165.0.2 | 255.255.255.0 | 209.165.0.1 |
| PC3 | NIC | 10.0.0.2 | 255.0.0.0 | 10.0.0.1 |

Objectives

Part 1: View the Routing Table of a Converged Network

Part 2: Add a New LAN to the Topology

Part 3: Watch the Network Converge

Background

This activity will help you identify important information in routing tables and witness the process of network convergence.

View the Routing Table of a Converged Network

Use show commands and interpret the output.

Show the directly connected networks of R1. How many routes are connected to R1? 2

R1# show ip route connected

Show the running configuration of R1. What routing protocol is in use? RIP

Are the IP addresses in the configuration advertised by RIP the same as those that are connected? Yes

Are these IP addresses assignable, network, or broadcast? Network

Show the networks of R1 learned through RIP. How many routes are there? 1

R1# show ip route rip

Show all of the networks that R1 has in its routing table. What do the leading letters represent?

C=Connected, R=RIP L=local

R1# show ip route

Repeat step 1, a to f on R2. Compare the output of the two routers.

Verify the state of the topology.

Ping PC3 from PC2. The ping should be successful.

Show the interface status on R2. Two interfaces should have assigned addresses. Each address corresponds to a connected network.

R2# show ip interface brief

Show the interface status on R1. How many interfaces have assigned addresses? 3

R1# show ip interface brief

Add a New LAN to the Topology

Add an Ethernet cable.

Connect the correct Ethernet cable from S1 to the appropriate port on R1.

Ping from PC1 to PC2 after the affected S1 port turns green. Was the ping successful? Yes

Ping from PC1 to PC3. Was the ping successful? Why?

No, R1 is not advertising the 64.0.0.0 network to R2 which was unable to return packets.

Configure a route.

Switch from Realtime mode to Simulation mode.

Enter a new route on R1 for the 64.0.0.0 network.

R1(config)# router rip

R1(config-router)# network 64.0.0.0

Examine the PDUs leaving R1. What type are they? RIPv1

Watch the Network Converge

Use debug commands.

Enable debugging on R2.

R2# debug ip rip

R2# debug ip routing

For reference, show the routing table of R2 as in step 1f.

Click Capture / Forward from simulation mode. What notification appeared in the terminal of R2?

There was a RIPv1 update from R1.

According to the debugging output, how many hops away from R2 is 64.0.0.0? One hop

What interface does R2 send packets destined for the 64.0.0.0 network? S0/0/0

Show the routing table of R2. Record the new entry.

R    64.0.0.0/8 [120/1] via 192.168.1.2, 00:00:00, Serial0/0/0

Verify the state of the topology.

Ping from PC1 to PC3. Was the ping successful? Why?

Yes, R1 advertised the 64.0.0.0 network to R2 which was able to return packets.

Suggested Scoring Rubric

| Activity Section | Question Location | Possible Points | Earned Points |
| --- | --- | --- | --- |
| Part 1: View the Routing Table of a Converged Network. | Step 1-a | 6 |  |
|  | Step 1-b | 6 |  |
|  | Step 1-c | 6 |  |
|  | Step 1-d | 6 |  |
|  | Step 1-e | 6 |  |
|  | Step 1-f | 6 |  |
|  | Step 2-c | 6 |  |
| Part 1 Total |  | 42 |  |
| Part 2: Add a New LAN to the Topology | Step 1-b | 6 |  |
|  | Step 1-c | 6 |  |
|  | Step 2-c | 6 |  |
| Part 2 Total |  | 18 |  |
| Part 3: Watch the Network Converge | Step 1-c | 6 |  |
|  | Step 1-d | 6 |  |
|  | Step 1-e | 6 |  |
|  | Step 1-f | 6 |  |
|  | Step 2-a | 6 |  |
| Part 3 Total |  | 30 |  |
| Packet Tracer Score |  | 10 |  |
| Total Score |  | 100 |  |