Packet Tracer – Troubleshooting EtherChannel (Instructor Version)

Instructor Note: Red font color or Gray highlights indicate text that appears in the instructor copy only.

Topology

- Objectives

Part 1: Examine the Physical Layer and Correct Switch Port Mode Issues

Part 2: Identify and Correct Port Channel Assignment Issues

Part 3: Identify and Correct Port Channel Protocol Issues

- Background

Four switches were recently configured by a junior technician. Users are complaining that the network is running slow and would like you to investigate.

Examine the Physical Layer and Correct Switch Port Mode Issues

Look for access ports.

Examine the switches. When physical ports are assigned to an EtherChannel port, they behave as one. Each pair will either be operational or down. They will not be mixed with one port green and the other port orange.

Set ports to trunking mode.

    - Verify that all physical ports in the topology are set to trunking. Correct any that are in access mode.

S2(config)# interface range f0/21 - 24

S2(config-if-range)# switchport mode trunk

S2(config-if-range)# interface range g0/1-2

S2(config-if-range)# switchport mode trunk

    - Correct any EtherChannel ports that are not set to trunking mode.

Instructor Note: Packet Tracer 6.0.1 does not grade the switchport mode trunk command in port-channel interfaces.

S1(config)# interface port-channel 1

S1(config-if)# switchport mode trunk

S2(config)# interface port-channel 2

S2(config-if)# switchport mode trunk

S2(config-if)# interface port-channel 3

S2(config-if)# switchport mode trunk

S2(config-if)# interface Port-channel 6

S2(config-if)# switchport mode trunk

Identify and Correct Port Channel Assignment Issues

Examine port channel assignments.

The topology illustrates physical ports and their EtherChannel assignments. Verify that the switches are configured as indicated.

S1# show etherchannel summary

<output omitted>

1      Po1(SD)           LACP   Gig0/1(I) Gig0/2(I)

3      Po3(SU)           LACP   Fa0/23(P) Fa0/24(P)

5      Po5(SU)           LACP   Fa0/21(P) Fa0/22(P)

S2# show etherchannel summary

<output omitted>

2      Po2(SU)           LACP   Gig0/1(P) Gig0/2(P)

3      Po3(SU)           LACP   Fa0/23(P) Fa0/24(P)

6      Po6(SD)           LACP   Fa0/21(I) Fa0/22(I)

S3# show etherchannel summary

<output omitted>

1      Po1(SD)           PAgP   Gig0/1(I) Gig0/2(I)

4      Po4(SD)           PAgP   Fa0/23(I) Fa0/24(I)

6      Po6(SD)           PAgP   Fa0/21(I) Fa0/22(I)

S4# show etherchannel summary

<output omitted>

2      Po2(SU)           LACP   Gig0/1(P) Gig0/2(P)

4      Po4(SU)           LACP   Fa0/21(P) Fa0/22(P) Fa0/23(I) Fa0/24(I)

5      Po5(SD)           -

Correct port channel assignments.

Correct any switch ports that are not assigned to the correct EtherChannel port.

S4(config)# interface range f0/21 - 22

S4(config-if-range)# channel-group 5 mode active

Identify and Correct Port Channel Protocol Issues

Identify protocol issues.

In 2000, the IEEE released 802.3ad (LACP), which is an open standard version of EtherChannel. For compatibility reasons, the network design team chose to use LACP across the network. All ports that participate in EtherChannel need to actively negotiate the link as LACP, as opposed to PAgP. Verify that the physical ports are configured as indicated.

S3# show etherchannel summary

<output omitted>

1      Po1(SD)           PAgP   Gig0/1(I) Gig0/2(I)

4      Po4(SD)           PAgP   Fa0/23(I) Fa0/24(I)

6      Po6(SD)           PAgP   Fa0/21(I) Fa0/22(I)

Correct Protocol issues.

Correct any switch ports that are not negotiating using LACP.

S3(config)# interface range g0/1 - 2

S3(config-if-range)# no channel-group

S3(config-if-range)# channel-group 1 mode active

S3(config-if-range)# interface range f0/21 - 22

S3(config-if-range)# no channel-group

S3(config-if-range)# channel-group 6 mode active

S3(config-if-range)# interface range f0/23 - 24

S3(config-if-range)# no channel-group

S3(config-if-range)# channel-group 4 mode active