Packet Tracer – Troubleshoot VTP and DTP (Instructor Version)

Instructor Note: Red font color or gray highlights indicate text that appears in the instructor copy only.

Topology

Addressing Table

| Device | IP Address | Subnet Mask |
| --- | --- | --- |
| PC0 | 172.16.10.1 | 255.255.255.0 |
| PC1 | 172.16.20.1 | 255.255.255.0 |
| PC2 | 172.16.30.1 | 255.255.255.0 |
| PC3 | 172.16.30.2 | 255.255.255.0 |
| PC4 | 172.16.20.2 | 255.255.255.0 |
| PC5 | 172.16.10.2 | 255.255.255.0 |
| S1 | 172.16.99.1 | 255.255.255.0 |
| S2 | 172.16.99.2 | 255.255.255.0 |
| S3 | 172.16.99.3 | 255.255.255.0 |

Objectives

Part 1: Troubleshoot DTP

Part 2: Troubleshoot VTP

Background / Scenario

In this activity, the switches S2 and S3 are not implementing VTP information. You will verify that DTP and VTP configurations are correctly implemented. When all the issues are resolved, the PCs in the same VLAN will be able to communicate with each other.

Troubleshoot DTP

In Part 1, you will troubleshoot the trunk links among the switches. You will verify that permanent trunk links are used between the switches.

Enter show interfaces trunk at the privileged EXEC prompt on all the switches to determine the status of the trunk links. How many trunk links are configured currently?

____________________________________________________________________________________

There are no working trunk links between the switches.

Enter show interfaces g0/1 switchport at the privileged EXEC prompt on S1. Do the same for g0/2 interface on S1.

What is the operational mode on the GigabitEthernet interfaces on S1? 	 Static access

Repeat the commands for g0/1 on S2 and g0/2 on S3.

Correct the trunk links. Record the commands you used to correct the trunking issue.

____________________________________________________________________________________

____________________________________________________________________________________

____________________________________________________________________________________

____________________________________________________________________________________

S1(config)# interface range g0/1 - 2

S1(config-if-range)# switchport mode trunk

S2(config)# interface g0/1

S2(config-if)# switchport mode trunk

S3(config)# interface g0/2

S3(config-if)# switchport mode trunk

Verify the trunk links using the show commands.

Troubleshoot VTP

S1 will be configured as the VTP server. S2 and S3 will be configured as VTP clients, and will be receiving VTP updates from S1. The VTP domain should be CCNA and the VTP password should be cisco. Currently all the desired VLANs are already configured on S1.

Verify VLAN information

Use the show vlan brief command on all the switches. Do all the switches have the same number of VLANs? How many does each switch have?

_______________________________________________________________________________________

No. S1 has 10 VLANs. The other two switches have only 7 VLANs each.

Verify VTP configurations.

Use the show vtp status and show vtp password commands on all the switches to verify the VTP status.

Record the VTP status information in the table below.

| Device | Domain Name | Operating Mode | VTP Password |
| --- | --- | --- | --- |
| S1 | No domain name configured | Transparent | No password configured |
| S2 | ccna | Transparent | No password configured |
| S3 | CCNA | Transparent | Cisco |

Correct the VTP configurations.

Record the commands used to correct the VTP configurations.

_______________________________________________________________________________________

_______________________________________________________________________________________

_______________________________________________________________________________________

_______________________________________________________________________________________

S1(config)# vtp mode server

S1(config)# vtp domain CCNA

S1(config)# vtp password cisco

S2(config)# vtp mode client

S2(config)# vtp domain CCNA

S2(config)# vtp password cisco

S3(config)# vtp mode client

S3(config)# vtp password cisco

Verify port assignment.

The switchports connecting to the PCs need to be configured in the correct VLANs so the PCs can communicate with each other.

Use the show vlan brief command on S2 and S3 to determine if VLANs have been assigned to the switchports. Which VLAN is associated with these switchports? 	 1

| Ports | Assignments | Network |
| --- | --- | --- |
| S2 F0/1 S3 F0/8 | VLAN 10 (Staff) | 172.16.10.0/24 |
| S2 F0/9 S3 F0/16 | VLAN 20 (Student) | 172.16.20.0 /24 |
| S2 F0/17 S3 F0/24 | VLAN 30 (Faculty) | 172.16.30.0 /24 |

Using the table above, correct the VLAN assignments on S2 and S3. Record the VLAN assignment configurations below.

_______________________________________________________________________________________

_______________________________________________________________________________________

_______________________________________________________________________________________

_______________________________________________________________________________________

S2(config)# interface f0/1

S2(config-if)# switchport access vlan 10

S2(config-if)# interface f0/9

S2(config-if)# switchport access vlan 20

S2(config-if)# interface f0/17

S2(config-if)# switchport access vlan 30

S3(config)# interface f0/8

S3(config-if)# switchport access vlan 10

S3(config-if)# interface f0/16

S3(config-if)# switchport access vlan 20

S3(config-if)# interface f0/24

S2(config-if)# switchport access vlan 30

Verify end to end connectivity.

From PC0 ping PC5.

From PC1 ping PC4.

From PC2 ping PC3.

Script

Switch S1

enable

config t

vtp mode server

vtp domain CCNA

vtp password cisco

interface range g0/1 - 2

switchport mode trunk

end

Switch S2

enable

config t

vtp mode client

vtp domain CCNA

vtp password cisco

interface g0/1

switchport mode trunk

interface f0/1

switchport access vlan 10

interface f0/9

switchport access vlan 20

interface f0/17

switchport access vlan 30

end

Switch S3

enable

config t

vtp mode client

vtp password cisco

interface g0/2

switchport mode trunk

interface f0/8

switchport access vlan 10

interface f0/16

switchport access vlan 20

interface f0/24

switchport access vlan 30

end