Packet Tracer – Configuring Multiarea OSPFv3 (Instructor Version)

Instructor Note: Red font color or Gray highlights indicate text that appears in the instructor copy only.

Topology

Addressing Table

| Device | Interface | IPv6 Address | OSPF Area |
| --- | --- | --- | --- |
| RA | G0/0 | 2001:DB8:1:A1::1/64 | 1 |
|  | G0/1 | 2001:DB8:1:A2::1/64 | 1 |
|  | S0/0/0 | 2001:DB8:1:AB::2/64 | 0 |
|  | Link-Local | FE80::A | N/A |
| RB | G0/0 | 2001:DB8:1:B1::1/64 | 0 |
|  | S0/0/0 | 2001:DB8:1:AB::1/64 | 0 |
|  | S0/0/1 | 2001:DB8:1:BC::1/64 | 0 |
|  | Link-Local | FE80::B | N/A |
| RC | G0/0 | 2001:DB8:1:C1::1/64 | 2 |
|  | G0/1 | 2001:DB8:1:C2::1/64 | 2 |
|  | S0/0/1 | 2001:DB8:1:BC::2/64 | 0 |
|  | Link-Local | FE80::C | N/A |

Objectives

Part 1: Configure OSPFv3

Part 2: Verify Multiarea OSPFv3 Operations

Background

In this activity, you will configure multiarea OSPFv3. The network is already connected and interfaces are configured with IPv6 addressing. Your job is to enable multiarea OSPFv3, verify connectivity and examine the operation of multiarea OSPFv3.

Configure OSPFv3

Enable IPv6 routing and configure OSPFv3 on RA.

Enable IPv6 routing.

- RA(config)# ipv6 unicast-routing

Configure OSPFv3 on RA with a process ID of 1 and a router ID of 1.1.1.1.

RA(config)# ipv6 router ospf 1

RA(config-rtr)#  router-id 1.1.1.1

Advertise each directly connected network in OSPFv3 on RA.

Configure each active IPv6 interface with OSPFv3 assigning each to the area listed in the Addressing Table.

RA(config)# interface GigabitEthernet 0/0

RA(config-if)# ipv6 ospf 1 area 1

RA(config-if)# interface GigabitEthernet 0/1

RA(config-if)# ipv6 ospf 1 area 1

RA(config-if)# interface Serial 0/0/0

RA(config-if)# ipv6 ospf 1 area 0

Configure OSPFv3 on RB and RC

Repeat the Steps 1 and 2 for RB and RC, changing the router ID to 2.2.2.2 and 3.3.3.3 respectively.

RB(config)# ipv6 unicast-routing

RB(config)# ipv6 router ospf 1

RB(config-rtr)# router-id 2.2.2.2

RB(config-rtr)# interface GigabitEthernet0/0

RB(config-if)# ipv6 ospf 1 area 0

RB(config-if)# interface Serial0/0/0

RB(config-if)# ipv6 ospf 1 area 0

RB(config-if)# interface Serial0/0/1

RB(config-if)# ipv6 ospf 1 area 0

!

RC(config)# ipv6 unicast-routing

RC(config)# ipv6 router ospf 1

RC(config-rtr)# router-id 3.3.3.3

RC(config-rtr)# interface GigabitEthernet 0/0

RC(config-if)# ipv6 ospf 1 area 2

RC(config-if)# interface GigabitEthernet 0/1

RC(config-if)# ipv6 ospf 1 area 2

RC(config-if)# interface Serial 0/0/1

RC(config-if)# ipv6 ospf 1 area 0

Verify Multiarea OSPFv3 Operations

Verify connectivity to each of the OSPFv3 areas.

From RA, ping each of the following remote devices in area 0 and area 2: 2001:DB8:1:B1::2, 2001:DB8:1:A1::2, 2001:DB8:1:A2::2, 2001:DB8:1:C1::2, and 2001:DB8:1:C2::2.

Use show commands to examine the current OSPFv3 operations.

Use the following commands to gather information about your OSPFv3 multiarea implementation.

show ipv6 ospf

show ipv6 route

show ipv6 ospf database

show ipv6 ospf interface

show ipv6 ospf neighbor

Note: Packet Tracer output for show ipv6 protocols is currently not aligned with IOS 15 output. Refer to the real equipment labs for correct show command output.