# Lab 4: IPSec VPN

## Learning Objective

- Configure a IPSec tunnel

- Configure EIGRP on a routers

  - As a R2 will be used the L3 switch

  - Serial interfaces will be replaced by fastEthernet

- Configure and test routing the tunnel

## Topology Diagram

This lab is designed as an introduction to IPSec VPN tunnels.

Step 1. ISAKMP policy – Configure what parameters will be used for the IKE phase 1 tunnel

Step 2. Transform Set – Configure what parameters will eb used for the IKE phase 2 tunnel (aka the IPSEC tunnel)

Step 3. ACL – Create an ACL to define what “interesting” traffic will be sent over the VPN

Step 4. Crypto Map – Configured using the previous parameters.

Step 5. Apply – Apply the crypto map to an interface

## Step 1: Configure Addressing

| Device | Interface | IP Address | Subnet Mask | Comment |
| --- | --- | --- | --- | --- |
| R1 | FastEthernet0/0 | 192.168.12.1 | 255.255.255.0 |  |
| R1 | Loopback0 | 172.16.1.1 | 255.255.255.0 |  |
| R2 | FastEthernet0/1 | 192.168.12.2 | 255.255.255.0 |  |
| R2 | FastEthernet0/11 | 192.168.23.2 | 255.255.255.0 |  |
| R3 | FastEthernet0/1 | 192.168.23.3 | 255.255.255.0 |  |
| R3 | FastEthernet0/0 | 192.168.35.4 | 255.255.255.0 |  |
| R3 | Loopback0 | 172.16.3.1 | 255.255.255.0 |  |

Configure the loopback interfaces with the addresses shown in the diagram. Also configure the FastEthernet interfaces shown in the diagram. Do not forget issue the no shutdown command on all FastEthernet connections. Verify that you have connectivity across the local subnet using the ping command. Do not set up the tunnel interface until the next step.

## Step 2: Configure EIGRP AS 1

Configure EIGRP AS 1 for the major networks 192.168.12.0/24, 192.168.23.0/24 and 192.168.35.0/24. Do not include the networks in the diagram falling in the 172.16.0.0/16 range. The Class C networks will serve as the transit networks for the tunnel network. Make sure you disable EIGRP automatic summarization.

An example

R2(config)# router eigrp 1

R2(config-router)# no auto-summary

R2(config-router)# network 192.168.12.0

R2(config-router)# network 192.168.23.0

Verify that routers can see the remote transit network with the show ip route command. If they cannot see the remote transit network, troubleshoot. R3 will not learn any new routes because it is directly connected to both networks.

R1# show ip route

Codes: C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2

i - IS-IS, su - IS-IS summary, L1 - IS-IS level-1, L2 - IS-IS level-2

ia - IS-IS inter area, * - candidate default, U - per-user static route

o - ODR, P - periodic downloaded static route

Gateway of last resort is not set

C 192.168.12.0/24 is directly connected, FastEthernet0/0

172.16.0.0/24 is subnetted, 1 subnets

C 172.16.1.0 is directly connected, Loopback0

D 192.168.23.0/24 [90/2681856] via 192.168.12.2, 00:00:15, FastEthernet0/0

## Step 3: Configure an ISAKMP

Start witth enable isakmp, the policy number is quite important. When the router tries to negotiate an acceptable phase one policy it always starts with the policy closest to 1 then work up in order until a negotiation is successful.

Configure the authentication method. Acceptable options are pre-shared key, RSA-Sig and RSA-Encr. For simplicity we’ll use PSK at the moment. I’ll do another post soon to explain the other options.

Next is the hash method to be used. Options are MD5 and SHA-1 (SHA-1 is the default)

Last, configure the encryption algorithm we want to use. In order of strength AES 256, AES 192, AES 128, 3DES, DES (DES as the default if nothing is explicitly configured)

## Step 4: Transform Set (IPSec proposals)

We configure IPSEC tunnel mode using 256 bit AES ecryption and sha-1 hmac.
Various other options are:

Verify with “show crypto ipsec transform-set”

## Step 4: ACL

Set appropriate ACLs both on R1 and R3

## Step 4: Crypto Map

We configure the IP or hostname of the opposite end of the tunnel. Configure the “interesting” traffic with the match command then finally configure the transform set to be used.
Verify with “show crypto map”

## Step 4: Apply

Apply crypto maps on appropriate interfaces both on R1 and R3

## Step 4: Routing

Now that you have the tunnel set up, you can set up dynamic routing protocols over it. When the next hop address of a destination network is through the tunnel, the packet is encapsulated in an IP packet as described in the previous step.

Configure EIGRP AS 2 to route the entire 172.16.0.0 major network over the tunnel, but disable automatic summarization. Remember that R3 is not participating in this routing process so it will not need to be configured.

R1(config)# router eigrp 2

R1(config-router)# no auto-summary

R1(config-router)# network 172.16.0.0

You should see EIGRP neighbors come up with their messages logged to the console. Now issue the show ip eigrp neighbors 2 command on R1 and R3 (And R3 anad R4). The ‘2’ at the end of the command string specifies the AS number. If you omit this, you will get neighbor tables for both EIGRP processes.

## Step 5: Verify

The easest way to test is by using and extended ping. So here we use the 172.16.1.1 (fa 1/1) interface on R1 as the source to ping the 172.16.3.1 address on the R2 router.

R1#p 
Protocol [ip]:
Target IP address: 172.16.3.1
Repeat count [5]:
Datagram size [100]:
Timeout in seconds [2]:
Extended commands [n]: y
Source address or interface: 172.16.1.1
Type of service [0]:
Set DF bit in IP header? [no]:
Validate reply data? [no]:
Data pattern [0xABCD]:
Loose, Strict, Record, Timestamp, Verbose[none]:
Sweep range of sizes [n]:
Type escape sequence to abort.
Sending 5, 100-byte ICMP Echos to 172.16.3.1, timeout is 2 seconds:
Packet sent with a source address of 172.16.1.1
.!!!!
Success rate is 80 percent (4/5), round-trip min/avg/max = 4/5/8 ms

Of course, you can use (slightly different) one row ping command

R1#ping 172.16.3.1 source lo 0

Now the ping has setup the VPN because of its “interesting” traffic (the first ping is lost in the VPN creation). We can verify with “show crypto engine connections active”

R1#show crypto engine connections active
Crypto Engine Connections

ID	Interface	Type	Algorithm	Encrypt	Decrypt	IP-Address
1	Fa0/0	IPsec	AES256+SHA	0	4	192.168.1.1
2	Fa0/0	IPsec	AES256+SHA	4	0	192.168.1.1
1001	Fa0/0	IKE	SHA+AES256	0	0	192.168.1.1

You can see we have one IKE connection and an IPSes tunnel for each direction.