Acid base and blood gas analyser

Vocabulary warm-up

1 Match the words in English with their translations:

unknown			 			ponořit

inner side			 			tenká vrstva

coated				 			dosáhnout

immerse			 			neznámý

plastic jacket			 			nylonová síťka

basically			 			vnitřní strana

nylon net			 			plastikový obal

uncharged molecules	 				nenabité molekuly, molekuly bez náboje

pass through			 			v zásadě

thin layer			 			projít skrz

reach 				 			tím

thereby			 			potažený

Reading

1 Read the text and complete the gaps with the words from the box:

Electrode uncharged partial analysers diffuse electrolyte solution

Most acid base and blood gas ___________(1) measure pH, pCO2, pO2 by means of selective electrodes.

pH electrode

A pH electrode is a glass ________(2) with a pH-sensitive glass membrane to seal its tip. On the inner side of the membrane is a buffer solution with constant pH. A silver wire coated with AgCl is immersed in this solution and, via a plug, connected to the measuring instrument. On the other side of the glass membrane is a ___________(3) of unknown pH (the sample).

pCO2 electrode

pCO2 is defined as the _________ (4) pressure of CO2 in a gas phase in equilibrium with the blood, and is measured by a pCO2 electrode.

A pCO2 electrode is a combined glass and silver/silver chloride (Ag/AgCl) reference electrode mounted in a plastic jacket, which is filled with a bicarbonate_________ (5). The jacket is covered with a 20 μm silicon membrane moulded on a 50 μm nylon net.

The pCO2 electrode is basically a pH electrode with a silicon membrane added. This membrane allows only ___________(6) molecules (i.e. CO2, O2, N2) to pass through it. Charged ions, such as H+, will not pass. Consequently, dissolved CO2 from the sample will __________(7) into the thin layer of bicarbonate electrolyte until equilibrium is reached.

2 Now answer these questions:

1 How do the analysers find out about pH, pCO2 and pO2?

2 What is the connection between the buffer solution and the measuring instrument?

3 Where is the sample situated?

4 Where can you find the silicon membrane?

5 Which molecules pass through the membrane?

# Listening

A Pre-listening warm-up. Put the words in the correct order.

1 blood do a gas

2 sure gloves you have your make on

3 in password put your

4 machine put syringe into the the

5 hit “analyse” button the

6 the for patient ask ID

7 scanner use a

8 it manually put in

9 syringe the remove

10 the sharps syringe in container a discard

B Watch and answer these questions:

1 What is the first thing you should do before running the test?

2 Does the machine require a password?

3 Which type of blood is being tested?

4 How can you read the patients ID?

5 How long does the testing take?

6 How do you know the test is over?

7 How do you get your results?

8 What do you have to do with the syringe?

9 What is the last thing you do?