# Flow cytometry

A Vocabulary. Match the words on the left with suitable words on the right.

1 physical			a) light

2 a beam			b) light into signals

3 internal			c) signals

4 scatter			d) particles to detectors

5 fluorescence			e) of light

6 transport			f) characteristics

7 convert			g) intensity

8 produce			h) complexity

B Text

Flow cytometry is a technology that measures and then analyses multiple physical characteristics of single particles, usually cells, as they flow in a fluid stream through a beam of light. The properties measured include a particles’ relative size, relative granularity or internal complexity, and relative fluorescence intensity. An optical-to-electronic coupling system records how the cell or particle scatters laser light and emits fluorescence.

A flow cytometer is made up of three main systems: fluidics, optics and electronics.

The fluidics system transports particles in a stream to the laser beam for interrogation.

The optics system consists of lasers to illuminate the particles in the sample stream and optical filters to direct the resulting light signals to the appropriate detectors.

The electronics system converts the detected light signals into electronic signals. These signals can then be processed by the computer.

In the flow cytometer, particles are carried to the laser intercept in a fluid stream. Any suspended particle or cell from 0.2-150 micrometers in size is suitable for analysis. The portion of the fluid stream where particles are located is called the sample core. Particles pass through the laser intercept and they scatter laser light. Any fluorescent molecules present on the particle fluoresce. The scattered and fluorescent light is collected by lenses. The scattered and fluorescent light then enters the appropriate detectors. The detectors produce electronic signals. These signals correspond with the optical signals which strike the detectors.

C Questions. Find answers to the following questions:

1 What properties of a cell or particle can you measure by a flow cytometer?

2 What light source is used in most flow cytometer?

3 What are the three main systems in a flow cytometer?

4 What is the name of the portion of the fluid stream where the cells are located?

5 When cells pass through the focused laser beam, what two types of light signals do you get?

6 Which part of the flow cytometer collects light emitted by a particle?

# Vocabulary warm-up

1 Particles		příslušný

2 Fluid			vydávat, vyzařovat

3 Beam			čočky

4 Granularity		přeměnit

5 Complexity		zpracovat

6 Fluorescence		granularita

7 Emit			částice

8 Scatter		paprsek

9 Illuminate		komplexnost

10 Appropriate		kapalina

11 Convert		fluorescence

12 Process		osvítit, ozářit

13 Lenses 		rozptýlit

# Grammar point

Past perfect (předminulý čas)