SAR (specific absorption rate) is an indication of the amount of radiation that is absorbed into a head whilst using a cellular phone, the higher the SAR rating the more radiation that is absorbed into the head.

A SAR value is a measure of the maximum energy absorbed by a unit of mass of exposed tissue of a person using a mobile phone, over a given time or more simply the power absorbed per unit mass. SAR values are usually expressed in units of watts per kilogram (W/kg) in either 1g or 10g of tissue.

Exposure Limits

In Europe, the European Council Recommendation 519/1999/EC for exposure guidelines has adopted the recommendations made by the International Commission on Non-Ionising Radiation Protection (ICNIRP Guidelines 1998). In the US, the FCC, Federal Communications Commission, sets the radio frequency safety guidelines that all phones must meet before being sold in the US. Current reference standards and limits (status January 2001)

| Region / Country | -- Reference to -- SAR measuremant protocol | Reference to SAR limit | Limit |
| --- | --- | --- | --- |
| Europe | European Specification ES 59005 (1998) | ICNIRP Guidelines 1998 (ICNIRP 1998) | 2.0 W/Kg in 10g of tissue |
| Australia | Australian Communications Authority (ACA) Standard (ACA RS 1999) | Australian Standard AS/NZS 2772.1 | 1.6 W/Kg in 1g of tissue |
| US | Federal Communications Commission (FCC) Guidelines (FCC 1997) | American Standard ANSI C95.1 (ANSI 1992) | 1.6 W/Kg in 1g of tissue |

Specific absorption rate (SAR) is a measure of the maximum rate at which radio frequency (RF) energy is absorbed by the body when exposed to radio-frequency electromagnetic field. It is used for exposure to fields between 100kHz and 10 GHz[1]. The SAR value will depend heavily on the geometry of the part of the body that is exposed to the RF energy, and on the exact location and geometry of the source of the RF energy. Thus tests must be made with each specific source, such as a mobile phone model, and at the intended position of use. The value cited for a specific model is then the maximum level measured in the body part studied. The most common use is in relation to mobile phones, and here the mobile phone is placed at the head in a talk position. The SAR value measured is then value measured in the place that has the highest absorption rate in the entire head, which for a mobile phone often is as close to the phone as possible. Various governments have defined safety limits for exposure to RF energy produced by mobile devices that mainly exposes the head or a limb for the RF energy:

- United States: the FCC requires that phones sold have a SAR level at or below 1.6 watts per kilogram (W/kg) taken over a volume of 1 gram of tissue.

- European Union: the SAR limit is 2 W/kg, averaged over ten grams of tissue.

For whole body exposure there is a limit of 0,08 Watt/kg averaged over the whole body. [2]

SAR      VALUES
 The Facts and Figures on Cellular Phone Radiation!

NOKIA

Nokia 1100     0.67

Nokia 2100     0.55

Nokia 2300     0.86

Nokia 2600     0.80

Nokia 2650     0.54

Nokia 3100     0.76

Nokia 3110     1.24

Nokia 3200     0.56

Nokia 3210     0.81

Nokia 3300     0.77

Nokia 3310     0.96

Nokia 3330     0.96

Nokia 3350     0.52

Nokia 3410     0.81

Nokia 3510     0.66

Nokia 3510i    0.83

Nokia 3650     0.72

Nokia 3660     0.75

Nokia 5100     0.48

Nokia 5140     0.77

Nokia 5110     0.69

Nokia 5210     0.62

Nokia 5510     0.74

Nokia 6100     0.60

Nokia 6110     0.87

Nokia 6150     0.69

Nokia  6170    0.34

Nokia 6210     0.82

Nokia 6220     0.66

Nokia 6230     0.59

Nokia 6250     0.55

Nokia 6310     0.81

Nokia 6510     0.81

Nokia 6600     0.80

Nokia 6610     0.63

Nokia 6610i    0.63

Nokia 6650     0.64

Nokia 6800     0.61

Nokia 6810     0.82

Nokia 6820     0.72

Nokia 7110     0.90

Nokia 7200     0.36

Nokia 7210     0.63

Nokia 7250     0.60

Nokia 7250i    0.60

Nokia 7260     0.51

Nokia 7270     0.46

Nokia 7280     0.83

Nokia 7600     0.71

Nokia 7610     0.54

Nokia 7650    0.35

Nokia 8110    0.73

Nokia 8110i   0.73

Nokia 8210    1.00

Nokia 8310    0.82

Nokia 8810    1.14

Nokia 8810i   0.73

Nokia 8850    0.43

Nokia 8890    0.26

Nokia 8910    0.52

Nokia 8910i   0.26

Nokia 9210    0.34

Nokia 9210i   0.34

Nokia NGAUGE   0.57

Nokia THR850      0.35

Nokia THR880      0.52

SARVALUES The Facts and Figures on Cellular Phone Radiation!. The Ten Lowest Radiating Cell Phones (EUROPE). (Max exposure 10 Wkg in 10 Grams of Tissue). Manufacturer; Model; SAR rating W/Kg. Samsung; F210; 0.20. Nokia; 6267; 0.31. Emporia; Life; 0.37. HTC; TYTN11; 038. LG; KE970 Shine; 0.43. LG; KU970; 0.43. Nokia; 6290; 0.47. Samsung; U600; 0.48. Nokia; 8800i; 0.50. LG; KG130; 0.52..

SARVALUES The Facts and Figures on Cellular Phone Radiation!

The Ten Highest Radiating Cell Phones (EUROPE)

Maximum Absorption is 2 w/kg Averaged in 10gms of Tissue (ICNRP)

|  | Manufacturer | Model | SAR rating W/Kg |
| --- | --- | --- | --- |

|  | Sony Ericsson | T650 | 1.80 |
| --- | --- | --- | --- |

|  | Sony Ericsson | W880i | 1.45 |
| --- | --- | --- | --- |

|  | Nokia | E51 | 1.40 |
| --- | --- | --- | --- |

|  | Sony Ericsson | W950i | 1.35 |
| --- | --- | --- | --- |

|  | Sony Ericsson | Z610i | 1.32 |
| --- | --- | --- | --- |

|  | Sony Ericsson | K810i | 1.31 |
| --- | --- | --- | --- |

|  | Sony Ericsson | W610i | 1.31 |
| --- | --- | --- | --- |

|  | Sony Ericsson | W660i | 1.27 |
| --- | --- | --- | --- |

|  | Sony Ericsson | K550i | 1.25 |
| --- | --- | --- | --- |

|  | LG + Nokia | KU250 + N5700 | 1.24 |
| --- | --- | --- | --- |

Accurate RF dosimetry measurements in simulated humans are difficult to perform, yet they are essential in establishing the compliance of the phone with the relevant compliance standards. The new harmonised SAR test method adopted by CENELEC and IEEE will ensure that the measurement regimes used to test compliance with either the ICNIRP or IEEE exposure standards are largely the same. The following provides only an outline of the complicated process used in measuring the SAR values.

Essentially the SAR measurement system comprises a phantom human head, specially developed liquids, a robot fitted with a measurement probe, and a mobile phone powered up to its maximum certified power level.

The Phantom

In order to achieve a measure of SAR that will include all types of people, a phantom has been produced that is based upon the dimensions of a large adult male head (it has been shown that a larger head is exposed to a higher SAR) as measured on US Army personnel. This phantom has been constructed with compressed thin ears to simulate users with small ears. In addition – right and left model heads are used to ensure that the different exposure areas caused by the asymmetric location of the antenna in many phones are being measured.

The Liquid

The phantom is filled with a liquid that correlates with the dielectric properties of human head tissue.The dielectric properties of ‘head tissue’ have been calculated taking into account the properties of human brain tissue and the matching effects of the outer tissue layers of the head (e.g., skin and skull) to provide a conservative overestimate of the values. Different ‘recipes’ are used for tests on different frequencies.