MulCalc Application

_________________________________________________________________________

MULCALC program is an application for the calculation of the

ONE-DIMENSIONAL X-RAY DIFFRACTION PATTERNS

OF MIXED-LAYERED CLAYS.

This Window version was written in C++ based on the TRUE BASIC version 1.0 which was written by R.C. Reynolds (1985).

For more information about this application, please call or write to:

Dr. Ray E. Ferrell, Jr.

LSU Department of Geology & GeophysicsE235

Howe/Russell Geoscience Complex Baton Rouge,

LA 70803, USA

Phone:   (504)-388-5306Fax     :  (504)-388-2302

====================================================================

MulCalc 1.0 Installation

Welcome to MulCalc 1.0 for Window.  To install this application on your system, do the followings:

1.  Checking Program Directory

By default, C:\MULCALC is the directory of MULCALC for Window program.  All the data library Files and Executable files will

be in this directory.  A default sub directory will also be created called C:\MULCALC\DATA to store the samples data files.  You

can enter any directory name you want for this application.  Make sure this directory is the new directory in your system.

2.  Checking Window System Directory

The install program will automatically get the Window System directory on your system.  This action is very important because

the MULCALC application need to be run with a library file called BWCC.DLL.  If  this library file were not on your system or were

the older version then the install program will install or replace this file in your system.  Otherwise, it ignores extracting the file.

Most system, the default path is C:\WINDOWS\SYSTEM.  You can not change this path on the install window.

3.  Installing Application

Click on the Begin button to install the MULCALC 1.0 for Window application.  The following files will be extracted into yours

system:

Files			Descriptions					Directory (by default)

NEWMOD.EXE	MulCalc executable file				C:\MULCALC

MULCALC.HLP	MulCalc help file				C:\MULCALC

CLAYDATA.DAT	Clay data library file (don't ever change it)	C:\MULCALC

COEFDATA.DAT	Constant library file (don't ever change it)	C:\MULCALC

DEFVAR.DAT		Default values file				C:\MULCALC

ZDATA.DAT		Constant library file (don't ever change it)	C:\MULCALC

BWCC.DLL		Window Dynamic Linking Library file		C:\WINDOWS\SYSTEM

INSTALL.EXE		Install executable file				C:\MULCALC

All the samples data files will be extracted in to the C:\MULCALC\DATA sub-directory.  These files are the sample output files for plotting or editing examples.

Install program will also create the MULCALC 1.0 Application Group on your Window desktop.

4.  Exiting Install Program

Press the Exit button.

5.  Getting Install Help.

Press the Help button.  This Help Window will be displayed.

====================================================================

How To Use MULCALC 1.0

__________________________________________________________________________

This section will describe step by step how to run MULCALC application.

1. Change Default  Values

From the Run menu, clicking on Change Default Values command, you will review all the default values, and then make a change on them by replacing the new values.

Clicking on the OK button, a message box "Change the default file?" appears.  If you want to keep these values for future run, click on YES button (the default values file "DEFVAR.DAT" will be changed).  Otherwise, these values just be used for the current running of MULCALC.

_________________________________________________________________________________

2. Run MulCalc

From the Run menu, you can run either Single Run or  Multiple Run command.

Single Run	run a normal MULCALC data file.

Multiple Run	run several MULCALC data files with specified series of input parameters .

_________________________________________________________________________________

3. Plot

From the command menu, click on Plot to review the plotting of MULCALC data files.

Open Plot	Open a new plotting file to view on the plotting window.If the plot already has some data files, then a message box "Plot on the current plot?" will be displayed.  Clicking on YES button , this new data file will be added to the plot.  Clicking on NO button, the plot will erase all the previous data curves, and plot the new data only.

Zoom In	Zoom in a region on the plotting window.Press the Left Mouse Button at the upper left corner of the zoom in region, drag the mouse down to the lower right corner, then release the mouse.  To exit this zoom in, choose the zoom out command from Plot menu.

Zoom Out	Zoom out the zoom in region back to the original plot.

Print Plot	Print out the plotting or zoom in graph.  To cancel the print out, clicking on the CANCEL button on the Print Status Box.

Printer Setup		Set up the printer by calling system printer setup program.

Close All Plot	 	Close the Plotting Window.  All the curves will be erased.

On the plotting window, you will also have one or more rectangles to indicate each plotting curve.  Each

rectangle has its own color which is the same as the color of the current curves.  Clicking on these

rectangles, you can get more information about these curves.  You may also delete them by clicking YES

button on the information dialog.

____________________________________________________________________________________

4. Data

From the commands menu, click on Data command to review a single or multiple MULCALC data files.

Open Data File ...	Open a data file as a text file.

Save File As ...	Save the current data file to another input file name.

Print Data File ...	Print out the Data File.

Printer Setup ...	Set up the printer by calling system printer setup program.

Close Data File ...	Close the Data File Window.

Help			This command will open the help topics for MULCALC program.

_________________________________________________________________________________

5. Exit MulCalc

To Exit MULCALC program.

====================================================================

1. Change Default Values

____________________________________________________________________________________________

The Change Default Values command allows you to review and change the default values each time running

MULCALC.  Clicking on the OK button, a message box "Change the default file?" appears.  If you want to keep

these values for future run, click on YES button (the default values file "DEFVAR.DAT" will be changed).

Otherwise, these values just be used for the current running of MULCALC.

Following are the default values:

--------------------------------------------------------

Lambda(l)		1.5418		(wavelength of the incident radiation in Å)

Divergence Slit		1		(divergence for the primary beam in degrees)

Goniometer Radius	20		(cm)

Soller Slit 1		6.6		(primary = incident beam divergence in degrees)

Soller Slit 2		2		(diffracted beam slit in degrees)

Sample Length		1.3		(cm)

Quartz Ref. Intensity	2000		(cts/s at 3.34 Å for randomly oriented quartz powder, for abs. int.!)

Sigma-Star (s*)		12		(stadard deviation of the orientation function, this value + Soller slit

=> absolute intensity and angle dependent intesity through Lorentz factor)

Mu-Star (m*)	(CuKa)	45 (low Fe-clays)	(the mean sample mass absorption coefficient => absolute intensity)

Low N			3     (number of units cells stacked in z direction that make up the smallest crystallites)

High N			14     (number of units cells stacked in z direction that make up the largest crystallites)

D001A			10		(spacing of the most abundant layer type in the interstratification)

D001B			16.9		(spacing of the least abundant layer type in the interstratification)

Exchange Capacity	0.36		(only of smectite, number of univalent exch. cat. per (Si,Al)4 unit)

Exchange Cation		Ca		(Ca, Na, K, Mg, Sr,  in one-layer water/glycol smectite - Na)

Theta Comp. Slit		0		(0 - eliminates, 1 - incorporates slit)

Proportions of N's	1		(= Gaussian distribution, choose 1 when giving low and high N)

**  Mean Defect-Free Distance in N (d)	7	(in a crystallite not interrupted by stacking fault which breaks the crystallite into two or more X-ray coherent scattering domains. The realistic values of d for clays vary from 4 to 10)

**   If the d value were changed, then you are selecting the Defect Broadening Option.

__________________________________________________________________________________

2.1  Single Run

The Single Run option allows you to calculate a single data file which is the same as the TRUE BASIC version 2.

__________________________________________________________________________________

Select Clay Types

Click on the down arrow for more clay types and choose it by clicking on the name.

1.   Most abundant clay type: chosen as the first mixed clay type.

2.   Another clay type: chosen as the second mixed clay type.

__________________________________________________________________________________

Clay Compositions (Input for A Single Run)

__________________________________________________________________________________

The composition values for each clay layer type will be different depending on each layer type.  The following are the input required for each type:

Clay Type		Compositions values:

Fe	K	Fe1	Fe2	nHyd

DIMICA		0 - 1.7	0 - 1.0

TRIMICA		0 - 3.0	0 - 1.0

DISMECTITE-		0 - 1.7

TRISMECTITE-		0 - 3.0

DIVERMICULITE-	0 - 1.7

TRIVERMICULITE-	0 - 3.0

DI-DICHLORITE			0 - 2.0	0 - 2.0	0 - 1.0

DI-TRICHLORITE			0 - 2.0	0 - 3.0	0 - 1.0

TRI-DICHLORITE			0 - 3.0	0 - 2.0	0 - 1.0

TRI-TRICHLORITE			0 - 3.0	0 - 3.0	0 - 1.0

KAOLINITE		Fixed Compositions

SEPENTINE		0 - 3.0

__________________________________________________________________________________

Enter Decimal Fraction Value

The proportion of the most abundant clay type.  This values must be from .5 to .99.  The default is 0.9

Enter Reichweite Value

Enter one of these values 0, .5, 1, 1.5, 2, 2.5, or 3.  The default value is 0.

Input Clay Compositions

Enter 2-Theta (2q) Values

Low 2 must be not equal to 0.  The default for these includes: low 2 = 6, high 2 = 40, increment = 0.2.  Doporučuji použít 5-50 °2theta.

Enter Output File Name

The current working directory is the default directory for the output file.  This file name can be considered as a output file path which can be up to 50 characters long.  The file name itself has only 8 characters for the name

and 3 character for the extension on the DOS system.

====================================================================

2.2  Multiple Run

___________________________________________________________________________________

The Multiple Run option allows you to run several Single Run  output.  Each file has the same format as an output file from a single run.  These results will be stored in a sequence of files named automatically.  A listing file will also be created to hold the information for each data file.

Click on the following help topics:

Select Clay Types		as in single run

Enter Decimal Fraction Value (pA):

1.  Constant	Keep pA as a constant value.  Clicking on the CONSTANT CHECK BOX () to get the constant values.

2.  Vary	Let pA varies in a specified range.  This option will be automatically chosen by leaving the CONSTANT CHECK BOX () unchecked.  The range of a FROM value and a TO value must be from 0.5 to 0.99.

____________________________________________________________________________________

Enter Reichweite Value (R)

1. Constant	Keep R as a constant value.  Clicking on the CONSTANT CHECK BOX () to get the constant value.

2.  Vary	Let R varies in a specified range.  This option will be automatically chosen by leaving the CONSTANT CHECK BOX () unchecked.The values of FROM and TO include only the followings: 0, .5, 1, 1.5, 2, 2.5, and 3.

____________________________________________________________________________________

Input Clay Compositions  (Input for A Multiple Run)

The input composition values for each clay layer type in a MULTIPLE RUN is the same for a SINGLE RUN.  See the table of contents values of layer composition for a single run.

For a MULTIPLE RUN composition inputs, there are 2 options for each parameter value.

1.  CONSTANT:

Keep this parameter as a constant value.  Click to check on the CONSTANT CHECK BOX ().

2.  VARY:

Allows this parameter to varies in a certain range from a FROM value to a TO value with an increment of INCR value.  This option

will be automatically chosen by leaving the CONSTANT CHECK BOX  () un-checked.

If  this option were chosen, then each varied value of that parameter will generate a separated output file.  The number of output

file will be calculated based on the number of varied values.

____________________________________________________________________________________

Enter 2-Theta (2) Values

Low 2 must be not equal to 0.  The default for these includes: low 2 = 6, high 2 = 40, increm. = 0.2.

Enter OutPut File Name

The current working directory is the default directory for the output files.  This is really a path of the output files, and can be up to 50 characters long.  For the Multiple Run, these data files will be created automatically in a series along with  the information stored in the listing file.

For example:

If you enter the Output Files Names: A:\data*.dat, then the output files will be stored on the diskette in drive A with a series of data  001.dat, data002.dat, etc.

Enter Listing File Name

This is the main output file for each MULTIPLE RUN.  Whenever this option is chosen, it is better to change the name of  this file.  The default file name is "list1.dat".

_____________________________________________________________________________

Output Data File

Result of MULCALC calculation. The output file has the following format:

MULCALC DATA <Quartz Int Ref>	This first line is used to check the format and the Quartz Ref. Intensity value for this data. (*)

<Low 2 Theta>					The Low 2 Theta value

<High 2 Theta>					The High 2 Theta value

<Increment>					The Increment value

<Intensity>					These numbers are the intensities which were

<Intensity>					calculated by MULCALC program.  These values

...						are in the increased order of 2-Theta values from

...						Low 2-Theta to High 2-Theta with the step width

<Intensity>					of Increment value.

<Intensity>

...

| Clay 1: <Clay Name> <Chemical Compositions> | Clays Information: Clay type and its Chemical |
| --- | --- |
| Proportion: <Decimal Fraction Value (pA)> | compositions. |
| Clay 2: <Clay Name> <Chemical Composition> | Also the values of pA and R corresponding to |
| Reichweite: < Reichweite Value (R)> | these input parameters. |
| Molecular Weight: <Mole Value> | Molecular Weight and Density of this mixed clay |
| Density: <Density Value> | layer type. |
| Peaks: | These are the Peak Points of the data which were |
| 1 - (<2 Theta>, <Intensity>) | picked out from the plotting graph. The numbers |
| 2 - (<2 Theta>, <Intensity>) | of peaks is up to 5. Each peak is a point with the |
| ... | values of 2-Theta and the corresponding |
| 5 - (<2 Theta>, <Intensity>) | Intensity. |

____________________________________________________________________________________

(*) This format is very useful for both Window version and DOS version.  An output file of the TRUE BASIC version can be used to plot

on this plotting window by changing this line only.

____________________________________________________________________________________

Exit

This command allows you to close down the MULCALC application.

____________________________________________________________________________________

____________________________________________________________________________________

3. Plot Command

This command allows you to review the graph of any MULCALC  Data File file.

Open Plot	Open a data file to view on the plotting window.

Zoom In	Zoom into region on the plot.

Zoom Out	Zoom out back to the original plot.

Print Plot	Print out a plotting or a Zoom In region

Printer Setup	Set up the printer.

Close All Plot	Close the Plotting Window.  All the curves will be erased.

On the plotting window, you will also have one or more rectangles to indicate each plotting curve.  Each rectangle has its own color which is the same as the color of the current curves.  Clicking on these rectangles, you can get more information about these curves.  You may also delete them by clicking YES button on the information dialog.

____________________________________________________________________________________

File Open

In the File Open Dialog:

Click on the File List to choose a file to be opened.

Click the OK button on the File Open Dialog to open file.

Click the CANCEL button to cancel this action and close the Open File Dialog Box.

____________________________________________________________________________________

Zoom In

The Zoom In command allows you to view more closely at a region on the plotting window.

Press the LEFT MOUSE BUTTON at the upper left corner of the region that you  want to zoom in.

Hold the mouse button down and drag it to the lower right corner, and then release the mouse.

When you release the mouse, a zoom in window will be displayed.

To zoom back to the original plot, choose the Zoom Out command.

____________________________________________________________________________________

Zoom Out

The Zoom Out command allows you to return to the original plotting window.

You must be ZOOM OUT from a ZOOM IN window to make a another zoom in.

____________________________________________________________________________________

Print

Sent the current Plotting Graph or Data File to the printer.

Whenever this command is activated, you can cancel the printing job by clicking the CANCEL button on the Printing Status Dialog.

____________________________________________________________________________________

Printer Setup

This command allows you to set up the printer system.

To setup the Page Orientation layout, Fonts, etc., click the SETUP button on the SELECT PRINTER dialog.

Click on OK button to select the change.

Click on CANCEL button to cancel the change.

____________________________________________________________________________________

Close All Plots / Close Data File

This command allows you to Close the Plotting Window / Data Files Window.

____________________________________________________________________________________

____________________________________________________________________________________

4. Data Command

This command will allow you to view a Data File as a text file.  Click on the following topics:

Open Data File	Open a data file in text

Save File As		Save the current data file as another file.

Print Data		Print the current data file

Printer Setup		Set up Printer

Close Data File	Close Data File Window

Each time you open another data file, the previous data file will be erased from the view screen.

____________________________________________________________________________________

File Open

In the File Open Dialog:

Click on the File List to choose a file to be opened.

Click the OK button on the File Open Dialog to open file.

Click the CANCEL button to cancel this action and close the Open File Dialog Box.

____________________________________________________________________________________

Save File As...           In the File Save As Dialog:

Click on the Directory List to choose the directory.

Enter new file name in the File name to be saved as.

Click the OK button to save the file.

Click the CANCEL button to cancel this action, close the Dialog Box.

____________________________________________________________________________________

Print

Sent the current Plotting Graph or Data File to the printer.

Whenever this command is activated, you can cancel the printing job by clicking the CANCEL button on the Printing Status Dialog.

____________________________________________________________________________________

Printer Setup

This command allows you to set up the printer system.

To setup the Page Orientation layout, Fonts, etc., click the SETUP button on the SELECT PRINTER dialog.

Click on OK button to select the change.

Click on CANCEL button to cancel the change.

____________________________________________________________________________________

Close All Plots / Close Data File

This command allows you to Close the Plotting Window / Data Files Window.

____________________________________________________________________________________