G R U T I L

- - - - - -

A Grid Reference Utility program for DMAP

{ Digitising and Conversion Utilities }

Dr Alan Morton

Blackthorn Cottage

Chawridge Lane, Winkfield, Windsor, Berks SL4 4QR

GENERAL DESCRIPTION

This program enables the use of a graphics (digitising) tablet to

record grid reference data at points along boundary segments. A map is

placed on the graphics tablet, and the buttons on the cursor (or puck)

are clicked at points where grid reference data are to be recorded. The

grid references are automatically written to a file which can then be

read directly by DMAP as a boundary definition file. The program can

also be used to digitise species grid references from existing

distribution maps.

Various utilities are also provided for manipulating data files

containing boundary or species grid references. These include:

conversion of DMAP 'short form' grid references to 'long forms';

conversion of numeric grid references to alphanumeric form; a facility

for comparing two distribution data files for change; export and import

facilities for data exchange between DMAP and databases via comma-

delimited text files.

GRUTIL is a DOS program and should be run directly from DOS. It should

not be run from Windows (or the MS-DOS Prompt from Windows) because

there may well be setup conflicts for serial port communication during

digitizing.

GRUTIL is essentially a utility program for Grid References and is not

suitable for the digitizing or manipulation of lat/long co-ordinates.

OUTLINE INSTRUCTIONS FOR DIGITISING

1.   Switch on the graphics tablet and secure the map to the graphics

tablet using masking tape. The grid system of the map should be

parallel to the edges of the graphics tablet.

2.   Check which serial port the graphics tablet is connected to, i.e.

COM1 or COM2. Run the MS-DOS MODE command to set up your computer in

the appropriate way for your graphics tablet. Information about this

should be found in your graphics tablet instruction manual. For

example, the default setting for the SummaSketch is 9600 baud, odd

parity, 8 data bits, 1 stop bit. So, assuming COM1, enter:

MODE COM1:9600,o,8,1

3.   Enter:

GRUTIL

and follow the on-screen instructions.

4.   The first stage of digitising is calibration. When asked to click

on the SW reference point, select a grid intersection near the SW

corner of your area to be digitised. You will then be asked to give the

actual grid reference of that point. The supplied grid reference must

be given in 8- 10- 12- or 14-digit numeric format, so, for example an

alphanumeric grid reference like SN700800 must be given in the numeric

form 27002800 (or 22/700800). IF, however, your area contains grid

references beyond the 1000km grid line (Orkney & Shetland), 10 digits

must be supplied (see the description of grid references in DMAP.DOC).

The 12- and 14-digit options are for digitising small areas (e.g.

sites, nature reserves etc.) when 10m or 1m precision is needed. After

defining a SW reference point, you will be asked to repeat the

procedure for a NE reference point. Finally, you will be asked to click

on the NW and SE corners of the calibration rectangle, to allow for

skew.

5.   Boundaries: Follow the on-screen instructions for digitising the

boundary segments. Don't forget to terminate the final segment before

finishing. If you make any mistakes, you will need to make a note of

them and, after leaving GRUTIL, use an editor to correct them in the

boundary file, so be careful!!

6.   Species: Follow the on-screen instructions. You will be asked to

select the precision which you require for the digitised grid

references. This will determine how they are written to the data file

created by GRUTIL. You will then be given an opportunity to assign two

of the cursor buttons to particular DMAP symbol types before digitising

commences. As with boundary digitising, any mistakes you make whilst

digitising will need to be noted down and corrected later using a text

editor.

7.   It is strongly recommended that before getting down to 'serious'

digitising, you should do a 'practice run' to familiarise yourself with

the graphics tablet and GRUTIL.

8.   The files created by digitising with GRUTIL will contain grid

references in numeric forms such as 28122964 or 22/812964 (boundaries),

or 28002950 or 22/800950 (species, 5km precision). Which of the two

alternative types of numeric grid references is generated will be

determined by how you specify the calibration grid references (e.g.

either 27002800 or 22/700800). If you prefer the grid references to be

in alphanumeric form, you can use a conversion utility provided in

GRUTIL to convert your boundary or species files. Grid references such

as 28122964 or 22/812964 will then be converted to SN812964 etc.

ADDITIONAL TECHNICAL INFORMATION

(needed for graphics tablets other than SummaSketch)

1. Button Assignments

The button colours referred to in the screen instructions are those of

SummaSketch cursors. Other makes of graphics tablet may have completely

different colour codes. You have two alternatives for untangling this:

trial and error ! or checking the button colour assignments in the

instruction manual. You may find it convenient to temporarily change

the colour of your buttons, e.g. with coloured sticky tape, whilst

using GRUTIL.

If your cursor (puck or stylus) has only 2 buttons, or has 3 buttons

numbered 1, 2 and 4, you can simulate button 3 by simultaneously

pressing buttons 1 and 2.

2. Graphics Tablet Configuration

The following information may help you to configure your graphics

tablet if it is not already set up as a SummaSketch II. This usually

involves setting DIP switches appropriately:

'Point' mode or 'switch stream' mode.

Serial output.

9600 baud.

8 data bits.

Odd, even or no parity, but it must be matched by your MODE command.

Binary mode (not ASCII).

(If several binary modes are available, you may have to experiment with

each to find out which matches GRUTIL, or, if you have technical

information about your tablet, compare it with the binary formats

listed below).

3. Binary formats recognized by GRUTIL.

Type 1  (e.g. SummaSketch) :

Bits      7    6    5    4    3    2    1    0

-----------------------------------------------

Byte 1    PH   PR   T    SX   SY   B    B    B

Byte 2    0    X6   X5   X4   X3   X2   X1   X0

Byte 3    0    X13  X12  X11  X10  X9   X8   X7

Byte 4    0    Y6   Y5   Y4   Y3   Y2   Y1   Y0

Byte 5    0    Y13  Y12  Y11  Y10  Y9   Y8   Y7

Type 2 :

Bits      7    6    5    4    3    2    1    0

-----------------------------------------------

Byte 1    PA   PH   B    B    B    B    0    0

Byte 2    PA   0    X5   X4   X3   X2   X1   X0

Byte 3    PA   0    X11  X10  X9   X8   X7   X6

Byte 4    PA   0    Y5   Y4   Y3   Y2   Y1   Y0

Byte 5    PA   0    Y11  Y10  Y9   Y8   Y7   Y6

Key :

PA = Parity Bit

PH = Phasing Bit (always 1)

PR = Proximity Bit

T = Tablet identifier Bit

SX,SY = Sign Bits for X,Y

B = Bits flagging the cursor buttons

X0,X1, & Y0,Y1, etc. = X & Y co-ordinate Bits

GRID REFERENCE CONVERSION UTILITIES

The following utilities are provided for changing the formats of grid

references within boundary or species files. In each case you will be

asked for the name of an input file (the original data file) and an

output file (a new file name which will contain the converted grid

references). The original file is left unchanged.

1. Conversion of DMAP 'short forms' to 'long forms'.

Refer to the DMAP documentation (file DMAP.DOC) for a description of

these forms. At present, the only 'short forms' which are accepted for

conversion are the all-digit forms without tetrad letters.

The conversion involves the insertion of the 100km co-ordinates into

the grid references. This is supplied as the value which you use as the

Plotting Parameter XYMIN (the SW corner of the map area) in DMAP, so

make sure you know this value before starting conversion. XYMIN has to

be given in 8-digit form, so, for example, if your current value is

670730 in 100km grid square SN (22), you would enter it as 26702730

when requested for it.

2. Conversion of numeric grid references to alphanumeric form.

This conversion is for DMAP 'long form' grid references only, and is

particularly useful for converting files of grid references generated

by digitising with a graphics tablet. Grid references such as 28412967

or 22/841967 will be converted to SN841967 etc.

3. Conversion of alphanumeric grid references to tetrad letter form.

This conversion is only for grid references which are already in

alphanumeric form. For example, grid references such as SN8292 or

SN837925 will be converted to SN89G etc. This conversion is only

available for species files, not boundaries.

4. Truncation of alphanumeric grid references to fewer digits.

This conversion is only for grid references which are already in

alphanumeric form. You will be asked to specify the number of digits

which you require. For example, if you specify 2 digits, grid

references such as SN8292, SN823925 or SN800900 will be truncated to

the 10km grid reference SN89. This conversion is only available for

species files, not boundaries.

5. Compare two Distribution Data files for change.

You will be asked for the names of two Distribution Data files. The

second file will be compared with the first. You will also be asked to

assign DMAP symbol types for gains, losses, and no change.

6. Create comma-delimited text file from Distribution Data file.

This creates a comma-delimited text file suitable for importing into

other databases. Each line in the file has the following fields:

SpeciesCode,GridReference,SymbolCode

7. Create Distribution Data from comma-delimited text file file.

This is the reverse process of the previous facility and allows the

import of data into DMAP from a comma-delimited text file produced by a

database. It is assumed that the database file has previously been

sorted by Species Code and that each line in the comma-delimited text

file has the following fields (although SymbolCode is optional) :

SpeciesCode,GridReference,SymbolCode

8. Extract Sub-area from Boundary data file.

This routine copies boundary segments from an existing boundary data

file and writes these to a new file. It is normally used to create a

smaller boundary data file from a larger one to describe a particular

study area which is part of the larger area represented in the original

file. The user is asked to define a rectangular area for extraction by

supplying grid references for the south-west and north-east corners.

All segments which contain grid references occurring within the defined

rectangle are copied to the new file.

Alan Morton

October 1995

* End of file GRUTIL.DOC *

##################################################################################